Airbag ECU Portfolio HS Harz — Functional Safety AIRBAG ELECTRONIC CONTROL UNIT Measurement and Processing of Sensor Signals to Control Airbag Operation

LIFE CYCLE PHASE 1 — CONCEPT

| Document Number | LCP01-AIRBAG-ECU-001 |
| --- | --- |
| Version | 1.00 |
| Date | 2026-05-17 |
| Status | RELEASED |
| Author / Safety Manager | JF, HS Harz FuSa — Systems Safety Engineering Team |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| Overall ASIL / SIL | ASIL D / SIL 3 |
| File | LCP01-AIRBAG-ECU-001_Concept_v100.docx |

# Contents

# 1  Unresolved Topics

None. This document has been reviewed and accepted.

# 2  Meetings within the Project

The following project meetings were held during the Concept Phase:

| Topic | Date / Time | Participants |
| --- | --- | --- |
| Kick-off: Airbag ECU Project — Concept Phase Review | 2026-05-03, 10:00 | PM, Safety Eng. Team, SW Dev, HW Dev, Test Eng. |
| HARA Review and Safety Goal Approval | 2026-05-06, 14:00 | JF, Safety Eng. Team, Project Lead |
| Concept Phase Closure and Document Acceptance | 2026-05-17, 09:00 | All team members |

# 3  Glossary

Table 1 lists all abbreviations and terms used in this document and the wider Airbag ECU project portfolio.

Table 1: Glossary

| # | Term / Abbreviation | Definition |
| --- | --- | --- |
| A | ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
|  | ACU | Airbag Control Unit — synonymous with Airbag ECU |
| C | CAN | Controller Area Network — serial communication bus for vehicle ECUs |
|  | CRC | Cyclic Redundancy Check — error-detection code for data integrity verification |
| D | DTC | Diagnostic Trouble Code — fault code stored in non-volatile memory |
|  | DWI | Diagnostic Warning Indicator — airbag fault lamp on instrument cluster |
| E | ECU | Electronic Control Unit |
|  | EMC | Electromagnetic Compatibility |
|  | EUC | Equipment Under Control — the item/system being safety-assessed |
| F | FSRS | Functional Safety Requirements Specification |
| H | HARA | Hazard and Risk Analysis |
|  | HW | Hardware |
| I | ISO 26262 | Road Vehicles — Functional Safety standard (2018 edition) |
|  | IEC 61508 | Functional Safety of E/E/PE Safety-related Systems (2010 edition) |
| L | LCP01 | Life Cycle Phase 01 — Concept Phase (first of 16 IEC 61508 / ISO 26262 phases) |
| M | MCU | Microcontroller Unit |
| N | NVRAM | Non-Volatile Random Access Memory |
| O | OCS | Occupant Classification System |
| P | PMHF | Probabilistic Metric for Hardware random Failures (< 10 FIT for ASIL D) |
|  | POST | Power-On Self-Test |
| Q | QM | Quality Management — no specific ASIL requirement |
| S | SIL | Safety Integrity Level per IEC 61508 (1–4) |
|  | SPFM | Single-Point Fault Metric (≥ 99% required for ASIL D) |
|  | SW | Software |
| 2 | 2oo2 | Two-out-of-Two — voting architecture requiring agreement of both redundant channels |

# 4  Basics — Concept of Operation and Safety

This document covers Life Cycle Phase 1 (LCP01) — Concept — of the Airbag Electronic Control Unit (Airbag ECU) project, conducted at HS Harz as a Functional Safety exercise in summer semester 2026.

## 4.1  Purpose of the Concept Phase

The Concept Phase (Phase 1 of 16 per IEC 61508 and ISO 26262) establishes the functional and safety baseline for all subsequent lifecycle phases. The key outputs of this phase are:

- A clear item definition and system boundary for the Airbag ECU

- A description of the operational environment, including use cases and environmental conditions

- Identification of all safety-relevant functions performed by the item

- A Hazard and Risk Analysis (HARA) establishing ASIL / SIL levels for all identified hazardous events

- Safety goals with their corresponding ASIL / SIL classifications

- A preliminary functional safety concept describing the architectural approach to achieving the safety goals

## 4.2  Applicable Standards

This project is developed in accordance with:

- ISO 26262:2018 — Road Vehicles — Functional Safety (Parts 1–12)

- IEC 61508:2010 — Functional Safety of Electrical / Electronic / Programmable Electronic Safety-related Systems (Parts 1–7)

- LV124:2013 — Electric and Electronic Components in Motor Vehicles (Environmental Requirements)

## 4.3  Team and Responsibilities

The following virtual departments and roles are established for the project:

| Name / Team | Role / Department | Responsibilities |
| --- | --- | --- |
| JF | Project Leader / Safety Manager | Overall project responsibility; HARA approval; concept release authorisation; liaison to HS Harz FuSa. |
| Safety Engineering Team | Functional Safety Engineer(s) | HARA, Safety Concept, FSRS authoring; ASIL / SIL determination; safety review and sign-off. |
| SW Dev | Software Developer | Embedded software implementation on Arduino Due; safety requirements realisation in code; unit and integration testing. |
| HW Dev | Electrical / Hardware Developer | Schematic and breadboard design; sensor interface; dual-channel PCB architecture; hardware review. |
| Test Engineer | Verification & Validation Engineer | Test plan creation; test execution (sensor input ranges, CAN, outputs); V&V reporting. |
| Mechanical Dev | Mechanical / Systems Engineer | Mounting concept; environmental requirements (vibration, temperature, sealing); LV124 compliance. |

# 5  System Description

The Airbag Electronic Control Unit (ACU) is a safety-critical automotive component responsible for real-time detection of crash events and controlled deployment of occupant restraint systems (frontal, side, and curtain airbags together with seatbelt pretensioners).

In the laboratory implementation, the system is realized on two Arduino Due microcontroller boards operating in a dual-channel (2oo2) redundant architecture. Three potentiometers on a breadboard simulate the analog crash sensor inputs; one digital switch simulates the baby-seat / Occupant Classification System (OCS) input. A CAN bus interface connects the ECU to the OMS laboratory PC for output verification.

## 5.1  System Operation Environment

### 5.1.1  Environmental Conditions

The Airbag ECU is designed to operate throughout the full vehicle lifetime (15 years / 300,000 km) under the following conditions:

- Ambient temperature: −40 °C to +85 °C (storage: −40 °C to +105 °C)

- Mechanical shocks and vibration per LV124 and ISO 16750-3

- EMC requirements per CISPR 25 and ISO 11452 (ECE R10 for type approval)

- Supply voltage: 9–16 V (nominal 12 V); post-crash powering via internal reservoir capacitor for ≥ 150 ms

- Vehicle category: M1 passenger car with a maximum design speed of 250 km/h

### 5.1.2  Item Boundary

The item boundary encompasses the Airbag ECU hardware and embedded software only. The following are treated as adjacent systems outside the item boundary:

- Mechanical airbag modules, inflators, and gas generators

- Pyrotechnic seatbelt pretensioners

- Vehicle body structure and passenger compartment

- Occupant Classification System (OCS) — represented in lab by digital switch

### 5.1.3  Sensor Inputs — Laboratory Implementation

Table 2 defines the sensor inputs of the Airbag ECU and their laboratory equivalents used in the HS Harz exercise hardware setup.

Table 2: Sensor Inputs and Laboratory Substitutes

| Real Sensor | Lab Substitute | Signal Type | Voltage / Range | Function |
| --- | --- | --- | --- | --- |
| #1 — Accelerometer | POTI-1 (Potentiometer) | Analog (ADC) | 0–3.3 V → 0–100 g | Crash pulse detection |
| #2 — Gyroscope / Angular Rate | POTI-2 (Potentiometer) | Analog (ADC) | 0–3.3 V → 0–360°/s | Rollover / yaw detection |
| #3 — Pressure Sensor | POTI-3 (Potentiometer) | Analog (ADC) | 0–3.3 V → 0–500 kPa | Side / frontal impact pressure |
| #4 — Baby-Seat / OCS | Digital Switch (active-LOW) | Digital (GPIO) | LOW / HIGH (3.3 V) | LOW = child seat → suppress pax airbag |

### 5.1.4  Processing Architecture

Two Arduino Due boards implement a Two-out-of-Two (2oo2) dual-channel redundant architecture. Each board independently reads all sensor inputs, executes the crash detection algorithm, and computes a deployment decision. A deployment output is only issued when both Channel A and Channel B independently agree. Each board calculates an independent CRC over the CAN frame payload to verify data integrity. Failure of either channel suppresses deployment for the affected output, activates the DWI lamp, and logs a DTC.

### 5.1.5  Output and Communication

The deployment decision and sensor status are transmitted as a CAN bus frame from the Arduino Due to the OMS lab PC CAN interface. A parallel bitstream readback detects transmission errors. Outputs include:

- Driver frontal airbag squib channel (simulated by output pin)

- Passenger frontal airbag squib channel (suppressed when baby-seat input LOW)

- Diagnostic Warning Indicator (DWI) / airbag fault signal lamp output

- CAN bus crash notification frame to main ECU (fuel cutoff, door unlock, eCall trigger)

## 5.2  Use Cases

Table 3 identifies all operating and use-case scenarios considered for hazard identification and exposure rating.

Table 3: Use Cases

| UC-ID | Use Case | Description / Safety Relevance |
| --- | --- | --- |
| UC-01 | Normal Highway Driving | Vehicle at 80–250 km/h. Highest crash severity potential. Full sensor fusion active. |
| UC-02 | Urban / Low-Speed Driving | Vehicle at 0–50 km/h in urban environment. Frequent speed changes; minor impacts common. |
| UC-03 | Frontal Crash (Barrier Impact) | Full-width or offset frontal impact. Primary scenario for frontal airbag and seatbelt pretensioner activation. |
| UC-04 | Side Crash (Pole / MDB) | Lateral impact. Requires rapid (< 8 ms) side curtain and thorax airbag deployment. |
| UC-05 | Rear Impact | Impact from rear. Seatbelt pretensioners activated; whiplash protection. |
| UC-06 | Rollover | Vehicle roll detected by yaw/roll rate sensors. Curtain airbags must remain deployed for ≥ 5 s. |
| UC-07 | Minor Non-Deployable Impacts | Pothole, speed bump, minor collision (< 15 g). No deployment must occur. |
| UC-08 | Vehicle Stationary / Ignition Off | Engine off; ECU remains powered. Risk of inadvertent activation due to maintenance or voltage transients. |
| UC-09 | Maintenance / Workshop | Technician accessing vehicle; airbag system not fully isolated. Risk of inadvertent deployment. |
| UC-10 | Vehicle Wash / Flood Wading | Vibration from wash equipment; immersion risk. Sensor noise must not trigger deployment. |
| UC-11 | Child / Small Occupant Presence | OCS / baby-seat switch classifies occupant as child or empty seat. Passenger airbag must be suppressed. |
| UC-12 | End-of-Life / Scrapping | Airbag must be safely disarmed before vehicle is scrapped. |

## 5.3  Safety-Relevant Functionalities

The following functions performed by the Airbag ECU are identified as safety-relevant and form the basis for hazard analysis:

- F1 — Crash Detection: continuous monitoring and fusion of accelerometer, gyroscope, and pressure sensor signals; detection of crash events above deployment threshold within ≤ 15 ms.

- F2 — Airbag Deployment: controlled actuation of driver and passenger frontal airbag squib channels upon valid deployment decision; suppression of passenger channel when OCS / baby-seat input is active.

- F3 — Sensor Monitoring and Plausibility Check: continuous range checking of all analog inputs (POTI-1, POTI-2, POTI-3) and digital input (baby-seat switch); out-of-range or stuck faults trigger DTC and DWI activation.

- F4 — System Self-Test and Diagnosis: Power-On Self-Test (POST) within 800 ms; background monitoring at ≤ 100 ms intervals; squib loop resistance monitoring; Flash CRC check; watchdog supervision.

- F5 — Fail-Safe / Fallback at Failure States: safe-state management including deployment inhibit (SS-01), partial-channel isolation (SS-04), and passenger airbag suppression on OCS fault (SS-03).

- F6 — Communication of Safety-Relevant States: CAN crash notification frame transmitted within ≤ 15 ms of deployment decision; DWI lamp activation within ≤ 800 ms of any detected fault.

# 6  Hazard and Risk Analysis Summary

The HARA (Document HARA-AIRBAG-ECU-001, Rev. 2.1, 2026-05-06) was conducted in accordance with ISO 26262 Part 3 and IEC 61508, evaluating all identified hazardous conditions using Severity (S), Exposure (E), and Controllability (C) parameters. Table 4 summarises the results.

## 6.1  ASIL/SIL Rating Criteria

Severity (S0–S3), Exposure (E0–E4), and Controllability (C0–C3) are rated per ISO 26262:2018 Annex B. The ASIL is read from the standard risk graph (ISO 26262-3 Table 4). Equivalence to SIL per IEC 61508 is applied per the mapping: ASIL A ≈ SIL 1, ASIL B ≈ SIL 1–2, ASIL C ≈ SIL 2–3, ASIL D ≈ SIL 3.

## 6.2  HARA Results

Table 4: Hazard and Risk Analysis Summary

| ID | Hazardous Event | S | E | C | ASIL | SIL |
| --- | --- | --- | --- | --- | --- | --- |
| H1-01 | Unintended frontal airbag deployment — highway driving | S3 | E4 | C3 | ASIL D | SIL 3 |
| H1-02 | Unintended side/curtain airbag deployment — urban driving | S3 | E3 | C3 | ASIL D | SIL 3 |
| H1-03 | Inadvertent deployment — vehicle stationary / maintenance | S2 | E2 | C2 | ASIL B | SIL 2 |
| H1-04 | Passenger airbag deployment with child seat / empty seat | S3 | E3 | C3 | ASIL D | SIL 3 |
| H1-05 | EMC-induced spurious deployment | S3 | E2 | C3 | ASIL C | SIL 2 |
| H2-01 | Non-deployment — frontal airbag in severe crash | S3 | E3 | C3 | ASIL D | SIL 3 |
| H2-02 | Non-deployment — side curtain / thorax in lateral crash | S3 | E3 | C3 | ASIL D | SIL 3 |
| H2-03 | Non-deployment — seatbelt pretensioners in crash | S2 | E3 | C3 | ASIL C | SIL 2 |
| H2-04 | Delayed deployment (> 15 ms from threshold) | S2 | E2 | C3 | ASIL C | SIL 2 |
| H3-01 | Incorrect airbag zone deployment | S3 | E1 | C3 | ASIL B | SIL 2 |
| H3-02 | Single squib failure in dual-stage inflator | S2 | E2 | C2 | ASIL A | SIL 1 |
| H4-01 | Loss of DWI / airbag warning lamp (silent fault) | S1 | E3 | C2 | ASIL A | SIL 1 |
| H4-02 | Loss of post-crash CAN broadcast | S2 | E3 | C2 | ASIL B | SIL 2 |

Overall Item ASIL: ASIL D (ISO 26262)  |  Overall Item SIL: SIL 3 (IEC 61508)

# 7  Safety Goals

Safety goals were derived directly from the HARA and represent the top-level safety requirements for the Airbag ECU item. All functional safety requirements, architectural decisions, and verification activities must be traceable to these safety goals.

Table 5: Safety Goals Summary

| SG-ID | Safety Goal | ASIL | SIL | Governing Hazard(s) |
| --- | --- | --- | --- | --- |
| SG-01 | The Airbag ECU shall NOT command deployment of any airbag or pretensioner in the absence of a valid crash event. | ASIL D | SIL 3 | H1-01, H1-02, H1-04 |
| SG-02 | The Airbag ECU shall command deployment of the appropriate airbag(s) within the required time window when a crash event exceeding the deployment threshold is detected. | ASIL D | SIL 3 | H2-01, H2-02 |
| SG-03 | The Airbag ECU shall suppress the passenger airbag when the OCS / baby-seat input indicates a child occupant or empty seat. | ASIL D | SIL 3 | H1-04 |
| SG-04 | The Airbag ECU shall command seatbelt pretensioner activation within 5 ms of a valid deployment decision. | ASIL C | SIL 2 | H2-03, H2-04 |
| SG-05 | The Airbag ECU shall not command deployment in response to an EMC or electrical disturbance that does not represent a crash event. | ASIL C | SIL 2 | H1-05 |
| SG-06 | The Airbag ECU shall activate the Diagnostic Warning Indicator (DWI / airbag lamp) within 800 ms of detecting a safety-critical system fault. | ASIL A | SIL 1 | H4-01 |
| SG-07 | The Airbag ECU shall transmit a crash notification CAN frame within 15 ms of a deployment decision. | ASIL B | SIL 2 | H4-02 |

The Airbag ECU item is classified ASIL D. All hardware and software components realising Safety Goals SG-01, SG-02, and SG-03 shall be developed in accordance with ISO 26262 Parts 4 and 6 at ASIL D, including:

- Dual-core lockstep MCU architecture with hardware safety monitor (Channel A / Channel B on separate Arduino Due boards)

- Hardware-enforced firing inhibit path independent of software (hardware AND-gate and hardware safety switch)

- 100% MC/DC code coverage for all safety-relevant software modules

- Single-Point Fault Metric (SPFM) ≥ 99% for ASIL D functions

- Probabilistic Metric for Hardware random Failures (PMHF) < 10 FIT for ASIL D safety goals

# 8  Preliminary Functional Safety Concept

The Functional Safety Concept describes the high-level strategy by which the Airbag ECU achieves its safety goals. This section provides the Concept Phase view; a detailed Safety Concept Document (SC-AIRBAG-ECU-001) is issued as a separate deliverable in the subsequent lifecycle phases.

## 8.1  Dual-Channel Redundant Architecture (2oo2)

The most critical architectural measure is the deployment of two independent processing channels (Channel A: Arduino Due #1; Channel B: Arduino Due #2) operating in a Two-out-of-Two voting logic configuration. This architecture is required to achieve the ASIL D Single-Point Fault Metric (SPFM ≥ 99%). A deployment output is issued if and only if both channels independently and simultaneously arrive at the same deployment decision (AND-gate authorisation logic). No single hardware or software fault in either channel can independently cause an unintended deployment.

## 8.2  Deployment Authorization Logic

A deployment command shall only be issued when all six of the following conditions are simultaneously satisfied:

- Crash sensor threshold exceeded on at least one axis (accelerometer > 30 g, OR gyroscope > 120°/s, OR pressure > 200 kPa)

- Channel A independently confirms crash detection

- Channel B independently confirms crash detection

- Both channels reach identical deployment decisions (AND-gate agreement)

- No critical fault condition exists in either channel

- Passenger suppression condition is not active (baby-seat digital input HIGH / no child seat detected)

## 8.3  Fault Detection and Safe-State Management

The system implements continuous background diagnostics at ≤ 100 ms intervals covering: MCU core and memory integrity, watchdog timer operation, sensor input plausibility, squib loop resistance, supply voltage monitoring, and CAN bus communication health. Upon detection of a fault, the system transitions to the appropriate safe state:

- SS-01 — No deployment (inhibit state): all firing transistors off; DWI lamp ON (H1-01, H1-02, H1-03, H1-05)

- SS-02 — Deployment armed, awaiting valid crash signal: normal standby; all squib channels armed (H2-01, H2-02, H2-03)

- SS-03 — Passenger airbag suppressed (OCS fault / CAN loss > 50 ms): suppress-on-fault strategy; driver and side airbags remain armed (H1-04)

- SS-04 — Affected channel isolated: faulted squib channel disabled; remaining channels continue; DWI ON; DTC logged (H3-01, H3-02, H4-01)

- SS-05 — System shutdown: all outputs forced off by hardware safety switch; watchdog reset; permanent shutdown after 3 failed resets (H1-01 related)

## 8.4  Power-On Self-Test (POST)

Upon every ignition cycle the Airbag ECU performs a complete self-test sequence within 800 ms covering: MCU CPU core and Flash CRC diagnostic, internal clock and watchdog timer self-test, squib loop resistance measurement, sensor plausibility check (all ADC channels within valid range 0–4095 on 12-bit ADC), supply voltage monitoring, and CAN bus communication verification. Any POST failure stores a DTC and illuminates the DWI lamp; the system remains in a safe degraded state.

# 9  Figure and Table Listing

Table 1: Glossary ............................................... Section 3

Table 2: Sensor Inputs and Laboratory Substitutes ............. Section 5.1.3

Table 3: Use Cases ............................................. Section 5.2

Table 4: Hazard and Risk Analysis Summary ...................... Section 6.2

Table 5: Safety Goals Summary .................................. Section 7

Table 6: Project Meetings ...................................... Section 2

Table 7: Team and Responsibilities ............................. Section 4.3

# 10  List of Dependent Documents

Table 8 lists all documents referenced by or related to this Concept Phase document.

Table 8: Dependent Documents

| No. | Document Name | Version | Date | File Name |
| --- | --- | --- | --- | --- |
| 1 | Hazard and Risk Analysis — Airbag ECU | 2.1 | 2026-05-06 | HARA-AIRBAG-ECU-001_Rev2.1.pdf |
| 2 | Safety Concept Document — Airbag ECU | 2.0 | 2026-05-12 | SC-AIRBAG-ECU-001_Rev2.0.pdf |
| 3 | Functional Safety Requirements Specification | 2.1 | 2026-05-12 | FSRS-AIRBAG-ECU-001_Rev2.1.pdf |
| 4 | ISO 26262:2018 — Road Vehicles — Functional Safety | 2018 | 2018 |  |
| 5 | IEC 61508:2010 — Functional Safety of E/E Systems | 2010 | 2010 |  |
| 6 | LV124 — Electric and Electronic Components Validation | 2013 | 2013 |  |
| 7 | ISO 16750-3 — Road Vehicle Environmental Conditions | 2007 | 2007 |  |
| 8 | CISPR 25 — EMC Requirements (Road Vehicles) | 2016 | 2016 |  |

# 11  Literature

ISO 26262:2018 — Road Vehicles — Functional Safety, Parts 1–12. International Organization for Standardization.

IEC 61508:2010 — Functional Safety of Electrical/Electronic/Programmable Electronic Safety-related Systems, Parts 1–7. International Electrotechnical Commission.

ISO 21448:2022 — Safety of the Intended Functionality (SOTIF). ISO.

ISO 16750-3:2007 — Road Vehicles — Environmental Conditions and Testing — Mechanical Loads. ISO.

CISPR 25:2016 — Vehicles, Boats and Internal Combustion Engines — Radio Disturbance Characteristics. IEC.

ISO 11452 — Road Vehicles — Component Test Methods for Electrical Disturbances from Narrowband Radiated Electromagnetic Energy. ISO.

ISO 14229-1:2020 — Unified Diagnostic Services (UDS). ISO.

SAE J211 — Instrumentation for Impact Tests. SAE International.

FMVSS No. 208 — Occupant Crash Protection. NHTSA, USA.

UN Regulation No. 94/95 — Frontal / Side Collision Protection. UNECE.

LV124:2013 — Electric and Electronic Components in Motor Vehicles — General Requirements, Test Conditions and Tests. VDA.

# 12  Version History

| Document Name / File | Date | Change Log |
| --- | --- | --- |
| LCP01-AIRBAG-ECU-001_Concept_v100.docx | 2026-05-17 | Version 1.00 — Initial release of the Concept Phase (LCP01) document for the Airbag ECU project, covering all IEC 61508 / ISO 26262 Concept Phase deliverables: system description, operational environment, safety-relevant functionalities, use cases, HARA summary, safety goals, and safety concept overview. |