SOFTWARE REQUIREMENTS

SPECIFICATION

Airbag ECU — Dual Arduino Due Redundant System

| Document No. | SRS-AIRBAG-SW-001 |
| --- | --- |
| Revision | 1.0 |
| Date | 2026-05-31 |
| Status | RELEASED |
| ASIL / SIL | ASIL D / SIL 3 |
| Target HW | Arduino Due (SAM3X8E ARM Cortex-M3) × 2 |
| Language | C++ (Arduino framework, due_can library) |
| Standards | ISO 26262:2018 / IEC 61508:2010 |
| Based On | FSRS-AIRBAG-ECU-001 Rev 2.1 ¦ SC-AIRBAG-ECU-001 Rev 2.0 ¦ HARA-AIRBAG-ECU-001 Rev 2.1 |
| Author | Hardware Safety Engineering Team |

# 1. Purpose and Scope

This Software Requirements Specification (SRS) defines all software-level requirements for the Airbag ECU embedded software running on two Arduino Due microcontroller boards Channel A (Due 1, Master) and Channel B (Due 2, Slave). It is derived from and fully traceable to the Functional Safety Requirements Specification (FSRS-AIRBAG-ECU-001 Rev 2.1), the Safety Concept Document (SC-AIRBAG-ECU-001 Rev 2.0), and the Hazard and Risk Analysis (HARA-AIRBAG-ECU-001 Rev 2.1).

The software is classified ASIL D per ISO 26262:2018 and SIL 3 per IEC 61508:2010. All requirements defined in this document are mandatory unless explicitly marked optional. No requirement may be waived without a formal change impact assessment and re-evaluation of the HARA and ASIL classification.

This document covers the complete software behaviour for:

- Power-on self-test and hardware verification at startup

- Continuous sensor acquisition from three analog crash sensor channels

- Crash detection algorithm using sensor threshold fusion

- Deployment output driving logic with 2oo2 AND gate integration

- Baby seat occupant detection and passenger airbag suppression

- Hardware watchdog supervision and safe-state enforcement

- Inter-board UART communication and mutual liveness monitoring

- CAN bus communication to the OMS laboratory PC (Due 1 only)

- Diagnostic Warning Indicator (DWI) control

- Serial diagnostic output for development and test

# 2. Document Information

## 2.1 Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-31 | Hardware Safety Engineering Team | Initial release - full SRS for dual Arduino Due airbag ECU with UART bridge and CAN output. |

## 2.2 Reference Documents

| Document Number | Title | Revision |
| --- | --- | --- |
| HARA-AIRBAG-ECU-001 | Hazard and Risk Analysis | Rev 2.1 |
| FSRS-AIRBAG-ECU-001 | Functional Safety Requirements Specification | Rev 2.1 |
| SC-AIRBAG-ECU-001 | Safety Concept Document | Rev 2.0 |
| HADS-AIRBAG-ECU-001 | Hardware Architecture and Design Specification | Rev 1.0 |
| ISO 26262:2018 | Road Vehicles — Functional Safety (Parts 3, 5, 6) | 2018 Edition |
| IEC 61508:2010 | Functional Safety of E/E/PE Systems (Parts 2, 3) | 2010 Edition |

## 2.3 Abbreviations

| Term | Definition |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| ASIL | Automotive Safety Integrity Level (ISO 26262) |
| CAN | Controller Area Network |
| DWI | Diagnostic Warning Indicator |
| ECU | Electronic Control Unit |
| FSR | Functional Safety Requirement (from FSRS) |
| GPIO | General Purpose Input/Output |
| OMS | Original Manufacturer System (laboratory monitoring PC) |
| POST | Power-On Self-Test |
| SIL | Safety Integrity Level (IEC 61508) |
| SRS | Software Requirements Specification (this document) |
| SWR | Software Requirement (individual requirement in this document) |
| UART | Universal Asynchronous Receiver/Transmitter |
| WDT | Hardware Watchdog Timer |
| 2oo2 | Two-out-of-Two voting architecture |

# 3. System Overview

## 3.1 Hardware Platform

The software runs on two Arduino Due boards, each featuring the Atmel SAM3X8E ARM Cortex-M3 microcontroller running at 84 MHz. Each board independently reads all three sensor inputs, executes the crash detection algorithm, and drives its own AND gate input pins. The hardware DM74LS08N AND gate enforces the physical 2oo2 deployment constraint, both boards must simultaneously assert their output HIGH for the deployment LED to activate. UART Serial1 provides inter-board heartbeat and status communication. CAN0 on Due 1 connects to a Waveshare SN65HVD230 CAN transceiver for OMS lab PC communication at 500 kbit/s.

## 3.2 Board Roles and Responsibilities

| Board | Role | CAN | UART |
| --- | --- | --- | --- |
| Arduino Due 1 (Channel A) | Master | CAN0 TX → Waveshare → USB-CAN-A → OMS PC | TX1/RX1 ↔ Due 2. Receives crash/baby/fault status from Due 2 and uses it in DWI and deployment logic. |
| Arduino Due 2 (Channel B) | Slave | None — no CAN module | TX1/RX1 ↔ Due 1. Sends crash/baby/fault status every 100 ms to Due 1. |

## 3.3 Safety Classification

| Software Function | ISO 26262 | IEC 61508 | Governing FSR |
| --- | --- | --- | --- |
| POST self-tests, crash detection, deployment logic, watchdog, UART supervision | ASIL D | SIL 3 | FSR-03, FSR-07, FSR-17, FSR-22 |
| Sensor acquisition and physical conversion | ASIL D | SIL 3 | FSR-01, FSR-02 |
| Baby seat detection and passenger suppression | ASIL D | SIL 3 | FSR-09, FSR-10, FSR-11 |
| Diagnostic monitoring and safe-state transitions | ASIL D | SIL 3 | FSR-18, FSR-20 |
| CAN communication to OMS PC | ASIL B | SIL 2 | FSR-13, FSR-16 |
| Diagnostic Warning Indicator (DWI) lamp | ASIL A | SIL 1 | FSR-19 |
| Serial monitor diagnostic output | ASIL A | SIL 1 | FSR-21 |

# 4. Software Module Definitions

The following table defines every software module constituting the complete embedded software. Each module maps to one or more FSR requirements and is assigned an ASIL classification.

| Module | Function | Runs On | Cycle | FSR | ASIL |
| --- | --- | --- | --- | --- | --- |
| cpu_test() | Arithmetic self-test of CPU core (12345×6789=83 810 205) | Both | Once at boot | FSR-17 | ASIL C |
| ram_test() | Checkerboard write/read pattern test of RAM (0xAAAA / 0x5555) | Both | Once at boot | FSR-17, FSR-22 | ASIL D |
| adc_test() | Verify analogRead(A0) returns a value in valid range 0–4095 | Both | Once at boot | FSR-17 | ASIL C |
| watchdog_setup() | Configure SAM3X8E hardware WDT with 10 ms reset-on-timeout | Both | Once at boot | FSR-22 | ASIL D |
| watchdog_feed() | Kick hardware WDT — unconditionally first call every loop | Both | < 10 ms | FSR-22 | ASIL D |
| updateBabySeat() | Read D11, apply 10 ms debounce, set stable babySeat state | Both | Every loop | FSR-09, 10, 11 | ASIL D |
| toPhys() | Scale 12-bit ADC reading to physical unit (g / °/s / kPa) | Both | Every loop | FSR-01, 02 | ASIL D |
| Sensor read block | Read A0/A1/A2, pass to toPhys(), produce physical values | Both | Every loop | FSR-01, 02 | ASIL D |
| Threshold check block | Compare physical values against THRESH_x constants | Both | Every loop | FSR-03, 04 | ASIL D |
| crashLocal decision | 2oo3 majority vote of accelEx, gyroEx, pressEx flags | Both | Every loop | FSR-03 | ASIL D |
| driveAirbag1/2 logic | Drive D8/D9 HIGH only when crash confirmed and no fault | Both | Every loop | FSR-03, 04, 08 | ASIL D |
| DWI logic | Drive D10 HIGH on any local or remote fault flag | Both | Every loop | FSR-19, 20 | ASIL A/D |
| enterSafeState logic | Force D8/D9 LOW immediately when system_fault TRUE | Both | Every loop | FSR-20 | ASIL D |
| UART heartbeat TX | Transmit ECUx:HB every 500 ms via Serial1 | Both | 500 ms | FSR-07, 08 | ASIL D |
| UART heartbeat RX | Receive and parse peer heartbeat messages from Serial1 | Both | Every loop | FSR-07, 08 | ASIL D |
| UART timeout check | Set fault_dueX if no heartbeat received within timeout period | Both | Every loop | FSR-15 | ASIL D |
| parseUART() / status TX | Due 2 sends crash/baby/fault; Due 1 parses remote state | Due 1 + Due 2 | 100 ms | FSR-07 | ASIL D |
| crc16() | Calculate CRC-16 (poly 0x1021, seed 0xFFFF) over 6 CAN bytes | Due 1 only | On CAN TX | FSR-13 | ASIL B |
| canSend() | Pack 8-byte CAN frame with CRC and transmit via CAN0 | Due 1 only | 100 ms | FSR-13, 16, 16A | ASIL B |
| Serial monitor output | Print diagnostic summary every 1000 ms to Serial | Both | 1000 ms | FSR-21 | ASIL A |

# 5. Software Functional Requirements

## 5.1 Startup and Self-Test (POST)

Before the system arms itself and enters the main monitoring loop, every safety-critical hardware resource must be verified. The POST sequence provides this assurance. Any failure detected during POST results in the system remaining permanently disarmed within that ignition cycle, the deployment outputs D8 and D9 are never set HIGH until all POST checks pass.

| SWR-01 POST Execution | The software shall execute cpu_test(), ram_test(), and adc_test() sequentially before entering the main loop. The arithmetic result of cpu_test() (12345 × 6789) shall equal exactly 83 810 205. ram_test() shall write the pattern 0xAAAA and then 0x5555 across a 128-word buffer and verify each value. adc_test() shall call analogRead(A0) and verify the result is within [0, 4095]. Results shall be stored in fault_cpu, fault_ram, and fault_adc boolean flags respectively. |
| --- | --- |
| ASIL: C | FSR trace: FSR-17 |

| SWR-02 POST Completion Before Arming | The software shall not set any airbag output pin (D8 or D9) HIGH until all POST tests have completed and their result flags have been evaluated. If any POST flag is TRUE (fault detected), both D8 and D9 shall remain LOW for the entire current ignition cycle. POST faults are non-recoverable without a full power cycle — no software reset path exists for fault_cpu, fault_ram, or fault_adc. |
| --- | --- |
| ASIL: D | FSR trace: FSR-17 |

| SWR-03 ADC 12-Bit Resolution Configuration | The software shall call analogReadResolution(12) during setup() to configure the Arduino Due ADC to 12-bit mode. This produces ADC readings in the range 0–4095 corresponding to 0–3.3V. Without this call the ADC defaults to 10-bit mode (0–1023), causing all physical conversion and threshold comparison calculations to produce incorrect results. |
| --- | --- |
| ASIL: D | FSR trace: FSR-01, FSR-02 |

## 5.2 Sensor Reading

The three potentiometer-based sensor simulations produce analog voltages from 0V to 3.3V on ADC pins A0, A1, and A2. The software must read these values on every loop iteration, convert them to physical units, and validate their range before using them in the crash detection algorithm.

| SWR-04 Sensor Acquisition Cycle | The software shall read analog pins A0 (accelerometer simulation), A1 (gyroscope simulation), and A2 (pressure sensor simulation) on every main loop iteration without exception. No blocking call to delay() or any other time-consuming function shall appear anywhere in the main loop. Readings shall be taken in the same fixed order every cycle. |
| --- | --- |
| ASIL: D | FSR trace: FSR-01, FSR-02 |

| SWR-05 Physical Unit Conversion | Raw ADC readings shall be converted to physical units using the formula: physical = (raw / 4095.0) × MAX_VALUE, where MAX_VALUE is: 100.0 g for the accelerometer channel, 360.0 °/s for the gyroscope channel, and 500.0 kPa for the pressure channel. The division operand 4095.0 shall be a floating-point literal to prevent integer division truncation. Physical values shall be stored as float variables. |
| --- | --- |
| ASIL: D | FSR trace: FSR-01 |

| SWR-06 Threshold Comparison | Physical sensor values shall be compared against the following compile-time constants: THRESH_ACCEL = 30.0 g (frontal crash), THRESH_GYRO = 120.0 °/s (rollover), THRESH_PRESS = 200.0 kPa (side impact). A boolean threshold-exceeded flag (accelEx, gyroEx, pressEx) shall be set TRUE independently for each sensor when its physical value meets or exceeds the threshold. These flags shall be cleared to FALSE at the start of each loop iteration before recomputation. |
| --- | --- |
| ASIL: D | FSR trace: FSR-03, FSR-04 |

## 5.3 Baby Seat Detection and Passenger Suppression

The passenger airbag suppression logic is an ASIL D safety function protecting against airbag deployment into a child seat. The hardware provides a 10 kΩ pull-up resistor to 3.3V on pin D11. The software implements debouncing and fail-safe default behaviour on top of this hardware measure.

| SWR-07 Baby Seat Input Pin Configuration | Pin D11 (baby seat switch) shall be configured as INPUT (not INPUT_PULLUP) because an external 10 kΩ pull-up resistor to 3.3V is already provided on the hardware. Configuring D11 as INPUT_PULLUP would result in two pull-up resistors in parallel, which does not affect safety but should be avoided. Active-LOW logic applies: LOW (0V) = baby seat present, HIGH (3.3V) = adult occupant or empty seat. |
| --- | --- |
| ASIL: D | FSR trace: FSR-09 |

| SWR-08 10 ms Debounce Filter | updateBabySeat() shall be called on every main loop iteration. A state transition of the babySeat flag shall only be accepted after the D11 pin has remained at the new logic level for a continuous minimum of 10 milliseconds. A stable-state timer shall restart from zero on every detected pin change. Debounce time is defined by compile-time constant DEBOUNCE_MS = 10. |
| --- | --- |
| ASIL: D | FSR trace: FSR-11 |

| SWR-09 Suppress-on-Fault Default | If the baby seat pin signal becomes absent, intermittent, or produces level transitions faster than the 10 ms debounce window (indicating wire fault or electrical noise), the software shall default babySeat = TRUE. This suppresses the passenger airbag in the absence of a verified safe signal. Suppression is the fail-safe direction: deploying an airbag into an unknown occupancy state is more dangerous than not deploying it. |
| --- | --- |
| ASIL: D | FSR trace: FSR-10 |

## 5.4 Crash Detection Algorithm

The crash detection algorithm fuses the three independent sensor threshold flags into a single crashLocal boolean decision. This decision is used by both boards independently to drive their respective AND gate input pins.

| SWR-10 Sensor Fusion — 2oo3 Majority Vote | crashLocal shall be set TRUE if at least two of the three threshold flags (accelEx, gyroEx, pressEx) are simultaneously TRUE. This implements a Two-out-of-Three (2oo3) majority vote. A single sensor exceeding threshold alone shall not set crashLocal TRUE. This protects against a single stuck or faulty potentiometer channel triggering a spurious crash detection. The vote count shall be calculated as: vote = (accelEx ? 1 : 0) + (gyroEx ? 1 : 0) + (pressEx ? 1 : 0); crashLocal = (vote >= 2). |
| --- | --- |
| ASIL: D | FSR trace: FSR-03 Note: The original SRS-AIRBAG-SW-001 specified OR fusion (any single sensor). This has been updated to 2oo3 majority vote per safety review finding to prevent single-sensor faults causing spurious deployment. |

| SWR-11 No Blocking Code in Main Loop | No call to delay(), while(condition) without watchdog kick, or any other blocking construct shall appear in the main loop. All timing-based tasks (UART heartbeat TX, CAN TX, status print) shall use non-blocking millis() comparison. The target main loop cycle time is less than 1 ms. Loop execution time is supervised by the hardware watchdog (10 ms timeout) which will reset the MCU if the loop stalls. |
| --- | --- |
| ASIL: D | FSR trace: FSR-02 |

## 5.5 Deployment Output Logic

The deployment output logic evaluates the crash decision, fault state, and occupant classification simultaneously on every loop iteration. The result drives D8 and D9, which are inputs to the physical DM74LS08N AND gate. The AND gate output, and therefore the actual deployment signal is only HIGH when both boards simultaneously drive their respective pins HIGH.

| SWR-12 AND Gate Input Drive Logic — Airbag 1 (Driver) | Pin D8 (Airbag 1 AND gate input) shall be set HIGH if and only if all three of the following conditions are simultaneously TRUE: (1) crashLocal is TRUE, (2) system_fault is FALSE — no POST or runtime fault on the local board, (3) the other board is alive — fault_due1 or fault_due2 is FALSE (no UART timeout). If any condition is FALSE, D8 shall be driven LOW immediately within the same loop iteration. |
| --- | --- |
| ASIL: D | FSR trace: FSR-03, FSR-04, FSR-08 |

| SWR-13 Passenger Airbag Suppression — Airbag 2 | Pin D9 (Airbag 2 AND gate input) shall satisfy all conditions of SWR-12 and additionally require babySeat to be FALSE. If babySeat is TRUE — regardless of crash state, fault state, or other board state — D9 shall be driven and held LOW. Baby seat suppression takes unconditional priority over all other deployment conditions. |
| --- | --- |
| ASIL: D | FSR trace: FSR-09, FSR-10 |

| SWR-14 Fault Inhibit — Safe State Enforcement | If system_fault is TRUE (composed of: fault_cpu OR fault_ram OR fault_adc OR fault_due1 OR fault_due2), both D8 and D9 shall immediately be driven LOW in the same loop iteration that the fault is detected. This is evaluated before the deployment drive logic and takes precedence over all other conditions. A board that detects any fault forces its AND gate inputs LOW, which collapses the AND gate output LOW regardless of the other board's state. |
| --- | --- |
| ASIL: D | FSR trace: FSR-20 |

## 5.6 Watchdog Supervision

The SAM3X8E hardware watchdog timer operates completely independently of software. It cannot be disabled by software once configured. Its sole purpose is to detect software lockup and force a hardware MCU reset, driving all GPIO to their default LOW state before software re-initialises.

| SWR-15 Hardware Watchdog Configuration | watchdog_setup() shall configure the SAM3X8E WDT with the following settings: WDT_MR_WDRSTEN (reset-on-timeout mode, not interrupt-only), WDT_MR_WDV(256 × 10) producing a timeout of approximately 10 ms. The watchdog shall be configured during setup() before POST tests execute. Once configured, the watchdog cannot be disabled by any software instruction. |
| --- | --- |
| ASIL: D | FSR trace: FSR-22 |

| SWR-16 Watchdog Feed | watchdog_feed() shall be the unconditionally first function call executed in every iteration of loop(). It shall precede all sensor reads, fault checks, and output drives. Writing the service key 0xA5 to WDT_CR constitutes a valid watchdog kick. If any loop iteration fails to call watchdog_feed() within the 10 ms timeout window, the hardware will reset the MCU and all GPIO default to LOW, forcing AND gate inputs LOW and preventing deployment. |
| --- | --- |
| ASIL: D | FSR trace: FSR-22 |

## 5.7 Inter-Board UART Communication

Both boards communicate via UART Serial1 at 115200 baud. Two independent UART data paths operate simultaneously: a heartbeat path detecting total board failure, and a status path providing fine-grained remote state (crash, baby seat, fault). Neither path can be bypassed without physical access to the UART wiring.

| SWR-17 Heartbeat Transmission | Each board shall transmit its heartbeat string via Serial1 every 500 ms using a non-blocking millis() timer. Due 1 transmits 'ECU1:HB\n' and Due 2 transmits 'ECU2:HB\n'. The heartbeat string shall be followed by a newline character to allow line-by-line parsing on the receiving board. Heartbeat interval is defined by compile-time constant HEARTBEAT_MS = 500. |
| --- | --- |
| ASIL: D | FSR trace: FSR-07 |

| SWR-18 Heartbeat Reception and Timeout Fault | Each board shall monitor Serial1 continuously for incoming heartbeat messages from the other board. On every loop iteration, if Serial1.available() indicates incoming data, the board shall read and parse the message. Upon receipt of a valid peer heartbeat, the UART receive timer shall be reset to zero and the corresponding fault flag (fault_due1 or fault_due2) shall be cleared to FALSE. If no valid heartbeat is received for a period exceeding COMM_TIMEOUT milliseconds, the fault flag shall be set TRUE and both D8 and D9 shall be driven LOW via SWR-14. |
| --- | --- |
| ASIL: D | FSR trace: FSR-07, FSR-15 |

| SWR-19 Due 2 Status Transmission and Parsing | Due 2 shall transmit its complete status to Due 1 every 100 ms in the format: 'ECU2,<crash>,<baby>,<fault>\n' where each field is the ASCII character '0' or '1'. Due 1 shall parse each received status string and extract the remote state into variables: due2_crash, due2_baby, and due2_fault. These remote state variables shall be used by Due 1 in the DWI logic (SWR-25) and as a confirmation cross-check of the remote deployment decision. |
| --- | --- |
| ASIL: D | FSR trace: FSR-07 |

## 5.8 CAN Bus Communication (Due 1 Only)

Due 1 transmits system status to the OMS laboratory PC via CAN bus at 500 kbit/s. CAN communication is classified ASIL B and carries monitoring data only. Loss of CAN has no effect on the ASIL D deployment decision path.

| SWR-20 CAN Initialisation | Due 1 shall initialise CAN0 at 500 kbit/s by calling Can0.begin(CAN_BPS_500K, 255) during setup(). CAN initialisation shall complete before the main loop is entered. Due 2 does not have a CAN module and shall not call any CAN initialisation function. |
| --- | --- |
| ASIL: B | FSR trace: FSR-13 |

| SWR-21 CAN Frame Structure | Every CAN frame transmitted by Due 1 shall use identifier 0x100 and DLC = 8 bytes with the following fixed layout: Byte 0 = flags byte (Bit0=crashA, Bit1=crashB, Bit2=deploy1, Bit3=deploy2, Bit4=babySeat, Bit5=faultA, Bit6=faultB, Bit7=DWI), Byte 1 = Due 1 accelerometer value scaled to 0–255, Byte 2 = Due 1 gyroscope value scaled to 0–255, Byte 3 = Due 2 compact status (Bit0=due2_crash, Bit1=due2_baby, Bit2=due2_fault, Bit3=UART_link_lost), Byte 4 = reserved (0x00), Byte 5 = rolling sequence counter (0–255 wrapping), Bytes 6–7 = CRC-16 big-endian. |
| --- | --- |
| ASIL: B | FSR trace: FSR-13, FSR-16 |

| SWR-22 CRC-16 Frame Protection | The crc16() function shall calculate a CRC-16 checksum using polynomial 0x1021 with initial seed value 0xFFFF over exactly the first 6 bytes (Bytes 0–5) of each CAN frame. The 16-bit result shall be stored big-endian: the most significant byte in Byte 6 and the least significant byte in Byte 7. The OMS PC shall validate the CRC of every received frame and discard frames with CRC mismatch. |
| --- | --- |
| ASIL: B | FSR trace: FSR-13 |

| SWR-23 CAN Transmission Rate | Due 1 shall transmit one CAN frame every 100 ms using a non-blocking millis() timer. No delay() shall be used. The rolling sequence counter in Byte 5 shall increment by 1 on every transmitted frame and wrap from 255 back to 0. The OMS PC can use gaps in the sequence counter to detect any missed or dropped frames. |
| --- | --- |
| ASIL: B | FSR trace: FSR-16 |

## 5.9 Diagnostic Warning Indicator

The DWI warning lamp (amber LED on pin D10) is the primary mechanism for alerting the driver that the airbag system is degraded. Each board independently drives its D10 output. The DWI activates on any local or remote fault, ensuring that a fault on either board illuminates the lamp.

| SWR-24 DWI Activation on Local Fault | Pin D10 (DWI warning lamp) shall be driven HIGH within one main loop cycle of any local fault flag being set TRUE. Local fault flags are: fault_cpu (CPU self-test failure), fault_ram (RAM test failure), fault_adc (ADC test failure), fault_due1 (UART timeout to peer board). D10 shall remain HIGH for as long as any fault flag remains TRUE. D10 shall only return to LOW when all fault flags are FALSE simultaneously. |
| --- | --- |
| ASIL: A | FSR trace: FSR-19 |

| SWR-25 DWI Activation on Remote Fault | Pin D10 shall additionally be driven HIGH if the received due2_fault flag parsed from the UART status string is TRUE, even if all local fault flags are FALSE. This ensures that a fault on the other board illuminates the DWI lamp on both boards, providing redundant warning indication regardless of which board is faulty. |
| --- | --- |
| ASIL: A | FSR trace: FSR-19 |

# 6. Non-Functional Requirements

Non-functional requirements define the performance, timing, and configuration constraints that all software modules must satisfy. These requirements are derived directly from FSR timing constraints and are verified during integration and system testing.

| ID | Requirement | Value / Limit | FSR | ASIL |
| --- | --- | --- | --- | --- |
| SWR-NF-01 | Main loop cycle time -- the complete loop() execution from watchdog_feed() to return shall not exceed this limit under any operating condition | < 1 ms | FSR-02 | ASIL D |
| SWR-NF-02 | Hardware watchdog timeout -- the WDT shall be configured and shall trigger an MCU reset if loop execution exceeds this interval | 10 ms | FSR-22 | ASIL D |
| SWR-NF-03 | Baby seat debounce period -- D11 must remain stable for this minimum duration before a state change is accepted | 10 ms minimum stable | FSR-11 | ASIL D |
| SWR-NF-04 | UART heartbeat transmit interval -- each board transmits its heartbeat string to the peer at this interval | 500 ms | FSR-07 | ASIL D |
| SWR-NF-05 | UART communication timeout --absence of a valid peer heartbeat for this duration sets the UART fault flag | 100 ms without heartbeat | FSR-15 | ASIL D |
| SWR-NF-06 | CAN frame transmit interval -- Due 1 transmits one 8-byte CAN frame to OMS PC at this rate | 100 ms | FSR-16 | ASIL B |
| SWR-NF-07 | Serial diagnostic print interval --diagnostic status summary printed to Serial at this rate | 1000 ms | FSR-21 | ASIL A |
| SWR-NF-08 | ADC resolution — the Arduino Due ADC shall be configured to 12-bit mode producing counts in the range 0–4095 | 12-bit (0–4095) | FSR-01 | ASIL D |
| SWR-NF-09 | CAN bus baud rate — CAN0 shall be initialised at this speed | 500 kbit/s | FSR-13 | ASIL B |
| SWR-NF-10 | UART baud rate — Serial1 inter-board link shall operate at this baud rate on both boards | 115200 baud | FSR-07 | ASIL D |

# 7. Software Constants and Thresholds

All safety-critical constants shall be defined as compile-time literals using const or #define. They shall not be stored as runtime variables that could be overwritten by software faults. Any change to a value in this table requires a formal change impact assessment and re-verification of the affected test cases.

| Constant | Value | Unit | Meaning | Rationale | FSR |
| --- | --- | --- | --- | --- | --- |
| ADC_MAX | 4095.0 | — | Arduino Due 12-bit ADC maximum count | Maximum count at 3.3V full-scale input | FSR-01 |
| ACCEL_MAX | 100.0 | g | Accelerometer full-scale physical range | Represents 100g maximum crash deceleration | FSR-01 |
| GYRO_MAX | 360.0 | °/s | Gyroscope full-scale physical range | Represents 360°/s maximum rollover rate | FSR-01 |
| PRESS_MAX | 500.0 | kPa | Pressure sensor full-scale physical range | Represents 500 kPa maximum impact pressure | FSR-01 |
| THRESH_ACCEL | 30.0 | g | Frontal crash detection threshold | 30g represents severe frontal impact requiring deployment | FSR-03 |
| THRESH_GYRO | 120.0 | °/s | Rollover detection threshold | 120°/s represents rollover event requiring deployment | FSR-03 |
| THRESH_PRESS | 200.0 | kPa | Side impact detection threshold | 200 kPa represents severe side impact requiring deployment | FSR-06 |
| DEBOUNCE_MS | 10 | ms | Baby seat pin debounce time | Minimum stable signal duration per FSR-11 | FSR-11 |
| HEARTBEAT_MS | 500 | ms | UART heartbeat transmit interval | Peer liveness check interval | FSR-07 |
| COMM_TIMEOUT | 100 | ms | UART link fault threshold | Updated from 2000 ms per safety review — fault must be detected within 100 ms | FSR-15 |
| CAN_TX_MS | 100 | ms | CAN frame transmit interval | OMS PC monitoring update rate | FSR-16 |
| CAN_TX_ID | 0x100 | hex | CAN message identifier for OMS PC | Fixed ID for OMS PC filtering | FSR-16 |
| CAN_BAUD | CAN_BPS_500K | — | CAN bus speed 500 kbit/s | Standard automotive CAN speed | FSR-13 |

Note: COMM_TIMEOUT has been updated to 100 ms from the original value of 2000 ms. A 2000 ms fault detection window is 50–100× longer than the crash event itself (20–40 ms) and is therefore not acceptable for an ASIL D system. The updated value of 100 ms ensures that a silent board is detected and flagged within one diagnostic cycle.

# 8. Full Requirements Traceability Matrix

Every software requirement traces back to a functional safety requirement (FSR) and forward to an implementing code module and test case. This matrix ensures complete bidirectional traceability as required by ISO 26262-6 and IEC 61508-3.

| SWR ID | Description | FSR | ASIL | Code Module | Test Case |
| --- | --- | --- | --- | --- | --- |
| SWR-01 | POST execution at boot | FSR-17 | ASIL C | cpu_test, ram_test, adc_test | TC-17 |
| SWR-02 | No output before POST complete | FSR-17 | ASIL D | setup() sequence — POST before loop entry | TC-17 |
| SWR-03 | ADC 12-bit resolution | FSR-01, FSR-02 | ASIL D | analogReadResolution(12) in setup() | TC-01 |
| SWR-04 | Sensor read every loop | FSR-01, FSR-02 | ASIL D | analogRead A0/A1/A2 every loop iteration | TC-01, TC-02 |
| SWR-05 | Physical conversion formula | FSR-01 | ASIL D | toPhys() function | TC-01 |
| SWR-06 | Threshold comparison | FSR-03, FSR-04 | ASIL D | accelEx, gyroEx, pressEx flags | TC-03, TC-04 |
| SWR-07 | Baby seat INPUT mode | FSR-09 | ASIL D | pinMode(D11, INPUT) in setup() | TC-09 |
| SWR-08 | 10 ms debounce filter | FSR-11 | ASIL D | updateBabySeat() debounce timer | TC-11 |
| SWR-09 | Suppress-on-fault default | FSR-10 | ASIL D | updateBabySeat() fault branch — babySeat=true | TC-10 |
| SWR-10 | 2oo3 majority vote fusion | FSR-03 | ASIL D | crashLocal vote >= 2 | TC-03 |
| SWR-11 | No blocking delay in loop | FSR-02 | ASIL D | millis() timers throughout | TC-02 |
| SWR-12 | D8 AND gate drive logic | FSR-03, FSR-08 | ASIL D | driveAirbag1 logic — D8 output | TC-03, TC-08 |
| SWR-13 | D9 passenger suppression | FSR-09, FSR-10 | ASIL D | driveAirbag2 logic + babySeat gate | TC-09, TC-10 |
| SWR-14 | Fault inhibit both outputs | FSR-20 | ASIL D | system_fault → D8/D9 LOW — enterSafeState | TC-20 |
| SWR-15 | WDT configuration | FSR-22 | ASIL D | watchdog_setup() — WDV(256×10) ≈10 ms | TC-22 |
| SWR-16 | WDT kick every loop | FSR-22 | ASIL D | watchdog_feed() — first call in loop() | TC-22 |
| SWR-17 | UART heartbeat TX 500 ms | FSR-07 | ASIL D | Serial1 HB TX non-blocking millis() | TC-07 |
| SWR-18 | UART timeout fault | FSR-07, FSR-15 | ASIL D | fault_due1/2 timeout detection | TC-15 |
| SWR-19 | Due 2 status TX / parseUART | FSR-07 | ASIL D | sendToMaster() / parseUART() | TC-07 |
| SWR-20 | CAN0 init 500 kbps | FSR-13 | ASIL B | Can0.begin(CAN_BPS_500K, 255) | TC-13 |
| SWR-21 | CAN 8-byte frame structure | FSR-13, FSR-16 | ASIL B | canSend() frame layout | TC-13, TC-16 |
| SWR-22 | CRC-16 on CAN frame | FSR-13 | ASIL B | crc16(poly=0x1021, seed=0xFFFF) | TC-13 |
| SWR-23 | CAN TX every 100 ms | FSR-16 | ASIL B | millis() non-blocking CAN timer | TC-16 |
| SWR-24 | DWI on local fault | FSR-19 | ASIL A | D10 HIGH on local fault flag | TC-19 |
| SWR-25 | DWI on remote fault | FSR-19 | ASIL A | due2_fault → D10 HIGH | TC-19 |

# 9. Mandatory Coding Constraints

The following coding constraints are mandatory for all ASIL D software in this system. They apply to all modules listed in Section 4. Violations must be identified and resolved before V&V testing begins.

| ID | Constraint | Rationale | How to Verify |
| --- | --- | --- | --- |
| CC-01 | Dynamic memory allocation (malloc, new, delete, realloc) is prohibited throughout the entire codebase | Dynamic allocation can fail at runtime in unpredictable ways and can cause memory fragmentation, leading to undefined behaviour in safety-critical code | Code review — grep for malloc, new, delete in all source files |
| CC-02 | Recursive function calls are prohibited | Recursion depth is not statically bounded and can cause stack overflow, corrupting safety-critical variables | Code review — verify call graph has no cycles |
| CC-03 | All global and static variables shall be explicitly initialised before first use | Uninitialised variables contain undefined values on startup, potentially causing incorrect deployment decisions | Code review + static analysis tool |
| CC-04 | No call to delay() anywhere in the main loop | delay() blocks the entire CPU, preventing watchdog feed, sensor reads, and fault detection during the blocked period | Code review — grep for delay() in loop() scope |
| CC-05 | All safety-critical thresholds and timing constants shall be compile-time const literals, not runtime variables | Runtime variables can be corrupted by software faults or stack overflow, changing deployment thresholds without detection | Code review — verify all THRESH_x and timing values are const |
| CC-06 | Integer arithmetic shall not be used where floating-point precision is required in conversion formulas | Integer division of (raw / 4095) truncates to zero for all valid raw values below 4095, producing incorrect physical unit conversion | Code review — verify toPhys() uses 4095.0 not 4095 |
| CC-07 | Requirement numbers shall be referenced as comments above the implementing code block | Provides direct traceability from code to requirements, enabling efficient verification and future change impact assessment | Code review — every SWR must appear as a comment in the code |
| CC-08 | The hardware watchdog_feed() call shall always be the first statement in loop(), with no conditional guard | Any conditional guard could allow the watchdog to expire during a fault condition, which is the correct behaviour — but if the guard prevents the feed during normal operation, it creates a false reset | Code review — verify watchdog_feed() is unconditional and first |

# 10. Approval and Sign-Off

This document is at Revision 1.0 RELEASED status. All approvals below must be confirmed before this SRS is used as the baseline for software implementation and V&V execution.

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Software Safety Engineer |  | _________________ | ___________ |
| Hardware Safety Engineer |  | _________________ | ___________ |
| Functional Safety Engineer |  | _________________ | ___________ |
| V&V Lead |  | _________________ | ___________ |
| Project Supervisor |  | _________________ | ___________ |