Airbag ECU Portfolio

HS Harz — Functional Safety Engineering

SOFTWARE ARCHITECTURE AND

DESIGN SPECIFICATION (SADS)

Measurement and Processing of Sensor Signals

to Control Airbag Operation

| Document Number | SADS-AIRBAG-ECU-001 |
| --- | --- |
| Version | 2.0 |
| Date | 2026-06-12 |
| Status | RELEASED |
| ASIL Classification | ASIL D (ISO 26262) / SIL 3 (IEC 61508) |
| Target Hardware | Arduino Due (SAM3X8E ARM Cortex-M3, 84 MHz) x2 |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| Based On | HADS-AIRBAG-ECU-001 Rev 2.1 ¦ FSRS-AIRBAG-ECU-001 Rev 2.1 |

# 1. Purpose and Scope

This Software Architecture and Design Specification (SADS) defines the internal software structure, module organisation, data flow, control flow, safety mechanisms, state machines, and implementation logic for the Airbag Electronic Control Unit (ECU) firmware running on two Arduino Due (SAM3X8E) microcontrollers in a redundant 2oo2 safety architecture.

The document addresses two complementary views:

- Architecture View — how the software is organised into modules, how they interact, and how safety properties are enforced structurally.

- Implementation View — how each architectural module is realised in code, including algorithms, thresholds, state machines, safety checks, and test provisions.

This specification is the primary reference for software verification, V&V testing, functional safety assessments, and code reviews. It must be read in conjunction with FSRS-AIRBAG-ECU-001 Rev 2.1 and HADS-AIRBAG-ECU-001 Rev 2.1.

## 1.1 Software Safety Goals Addressed

| Safety Goal | ASIL/SIL | Software Responsibility |
| --- | --- | --- |
| SG-01: No deployment without valid crash | ASIL D / SIL 3 | 2oo2 vote logic, fault inhibit, threshold gating |
| SG-02: Deploy within timing window | ASIL D / SIL 3 | ≤1 ms cycle time, ≤20 ms detection, ≤40 ms command |
| SG-03: Suppress passenger airbag for child seat | ASIL D / SIL 3 | Baby seat logic with debounce and fail-safe suppression |
| SG-06: Detect and report critical faults | ASIL A / SIL 1 | POST, continuous diagnostics, DWI activation ≤800 ms |
| SG-07: Reliable CAN crash notification | ASIL B / SIL 2 | 500 ms heartbeat, 2 s timeout detection, CRC framing |

# 2. Software Architecture Overview

The firmware follows a single-task cyclic executive model executing on the bare-metal Arduino Due environment (no RTOS). One main loop iteration constitutes one execution cycle, targeted at ≤1 ms per FSR-02. The architecture is divided into six functional modules that map directly onto FSR requirements.

## 2.1 Module Decomposition

| Module ID | Module Name | Primary FSR(s) | ASIL |
| --- | --- | --- | --- |
| MOD-01 | Power-On Self-Test (POST) | FSR-17 | ASIL C |
| MOD-02 | Sensor Acquisition | FSR-01, FSR-02 | ASIL D |
| MOD-03 | Crash Detection | FSR-01, FSR-02, FSR-03, FSR-04 | ASIL D |
| MOD-04 | Deployment Control | FSR-03, FSR-08, FSR-09, FSR-10 | ASIL D |
| MOD-05 | CAN Communication | FSR-13, FSR-14, FSR-15, FSR-16 | ASIL B |
| MOD-06 | Diagnostic Monitor | FSR-17, FSR-18, FSR-19, FSR-20, FSR-22 | ASIL C/D |

## 2.2 Data Flow Between Modules

The following describes how data moves between modules within one execution cycle:

- MOD-02 (Sensor Acquisition) reads raw 12-bit ADC values (0–4095) from A0, A1, A2 and digital pin D11, storing them in global variables rawA0, rawA1, rawA2, and babySeatRaw.

- MOD-03 (Crash Detection) reads rawA0–rawA2, scales to engineering units, applies threshold comparisons, and writes localCrash (boolean) and scaled float values accel, gyro, press.

- MOD-05 (CAN Communication) transmits the local ECU status frame (rawA0, rawA1, rawA2, localCrash, babySeat, systemFault) and receives the peer ECU frame, writing remoteCrash, remoteFault, and updating lastRemoteHB timestamp.

- MOD-06 (Diagnostic Monitor) evaluates faultCPU, faultRAM, faultADC, and faultCOMM flags, computing the aggregated systemFault boolean written to MOD-04.

- MOD-04 (Deployment Control) reads localCrash, remoteCrash, systemFault, remoteFault, and babySeat to compute voteDeploy, then drives D8, D9, and D10 output pins.

## 2.3 Execution Control Flow — Cyclic Executive

The firmware executes a strictly ordered sequence each cycle. The scheduling is time-driven through millis() comparisons. No preemption occurs.

| Step | Action | Period / Trigger | FSR |
| --- | --- | --- | --- |
| 1 | receiveCAN() — drain RX buffer, update remoteCrash, remoteFault, lastRemoteHB | Every cycle (non-blocking) | FSR-15 |
| 2 | Heartbeat timeout check — if (millis() - lastRemoteHB > 2000 ms) → faultCOMM = true | Every cycle | FSR-15 |
| 3 | Read sensors — analogRead A0, A1, A2 + digitalRead D11 | Every cycle | FSR-01, FSR-02 |
| 4 | Scale sensors — convert ADC to g, deg/s, kPa | Every cycle | FSR-01 |
| 5 | Crash detection — threshold comparisons → localCrash | Every cycle | FSR-01, FSR-04 |
| 6 | systemFault aggregation — OR of faultCPU, faultRAM, faultADC, faultCOMM | Every cycle | FSR-20 |
| 7 | Deployment vote — voteDeploy logic → drive D8, D9, D10 | Every cycle | FSR-03, FSR-08 |
| 8 | CAN transmit heartbeat — sendData() if 500 ms elapsed | Every 500 ms | FSR-16 |
| 9 | Status LED blink — toggle D13 if 500 ms elapsed | Every 500 ms | — |
| 10 | Serial diagnostic print — if 1000 ms elapsed | Every 1000 ms | FSR-21 |

# 3. System State Machine

The software defines five operational states that govern ECU behaviour. State transitions are driven by POST results, sensor readings, fault conditions, and communication status. All states map to the safe states defined in FSRS Section 8.

## 3.1 State Definitions

| State ID | State Name | Conditions to Enter | Outputs | FSRS Ref |
| --- | --- | --- | --- | --- |
| ST-01 | POWER_ON | Reset / power-up event | All outputs LOW, POST executing | SS-01 |
| ST-02 | POST_FAILED | Any POST check returns FALSE (CPU, RAM, ADC) | D10 (DWI) HIGH, D8/D9 LOW — permanent safe inhibit | SS-05 |
| ST-03 | MONITORING | POST passed, no crash, no fault, peer CAN active | D10 LOW, D8/D9 LOW — watching sensors | SS-02 |
| ST-04 | FAULT_ACTIVE | systemFault=TRUE (any fault flag set) | D10 HIGH (DWI), D8/D9 LOW — inhibit deployment | SS-01, SS-04 |
| ST-05 | CRASH_DETECTED | localCrash=TRUE AND remoteCrash=TRUE AND !systemFault AND !remoteFault | D8 HIGH (Airbag1 vote), D9 HIGH if !babySeat (Airbag2 vote) | SS-02 |

## 3.2 State Transition Diagram (Textual)

ST-01 POWER_ON

|

|── POST executes (CPU test, RAM test, ADC test)

|

|── POST FAIL ──────────────────────────────────> ST-02 POST_FAILED

|                                                     (permanent — no exit)

|── POST PASS

|

v

ST-03 MONITORING

|

|── systemFault becomes TRUE ────────────────────> ST-04 FAULT_ACTIVE

|       (faultCPU || faultRAM || faultADC ||           |

|        faultCOMM [peer timeout >2000 ms])             |── fault clears ──> ST-03

|

|── localCrash=TRUE AND remoteCrash=TRUE

|   AND !systemFault AND !remoteFault ──────────> ST-05 CRASH_DETECTED

|                                                     |

|                                                     |── sensors fall below

|                                                     |   thresholds ──────> ST-03

|                                                     |── fault occurs ────> ST-04

## 3.3 Safe State Guarantees

- ST-02 (POST_FAILED): Software sets D8/D9 LOW in setup() and never reaches loop(). Hardware AND gate therefore outputs LOW permanently. This is the ultimate safe inhibit state (SS-05).

- ST-04 (FAULT_ACTIVE): systemFault=TRUE forces voteDeploy=FALSE each cycle, pulling D8/D9 LOW. The AND gate output follows. DWI lamp (D10) activates within one cycle (~1 ms), satisfying the ≤800 ms FSR-19 requirement by a large margin.

- Loss of CAN peer (faultCOMM=TRUE): After 2000 ms without a valid peer frame, faultCOMM=TRUE propagates through systemFault into voteDeploy, inhibiting deployment. Safe-fail direction: inhibit rather than spurious deploy.

# 4. Module Implementation

## 4.1 MOD-01 — Power-On Self-Test (POST)

Executed once in setup() before loop() begins. Results are stored in persistent boolean fault flags. Any failed test sets the corresponding flag; the aggregate is checked to decide whether to enter MONITORING or POST_FAILED state.

### 4.1.1 CPU Test

Tests ALU correctness by performing a deterministic integer addition and verifying the result.

bool cpuTest() {

volatile uint32_t a = 100;

volatile uint32_t b = 200;

return ((a + b) == 300);   // volatile prevents compiler optimisation

}

  - Input: none

  - Output: TRUE = CPU arithmetic correct, FALSE = fault detected

  - Fault response: faultCPU = !cpuTest()

### 4.1.2 RAM Test

Writes a known bit pattern (0xAAAAAAAA) to a stack variable and reads it back to verify SRAM integrity.

bool ramTest() {

uint32_t test = 0xAAAAAAAA;

return (test == 0xAAAAAAAA);  // detects stuck-at faults in SRAM

}

Limitation: This is a lightweight single-word test appropriate for ASIL C within an ASIL D system backed by dual-channel redundancy. Full memory sweep testing would require a dedicated offline test phase. The pattern 0xAAAAAAAA (alternating 1/0 bits) stresses adjacent bit coupling in the memory cells.

### 4.1.3 ADC Test

Confirms ADC hardware is operational and returning values within the valid 12-bit range.

bool adcTest() {

int v = analogRead(A0);

return (v >= 0 && v <= 4095);  // 12-bit ADC range check

}

Note: The potentiometer physically constrains the wiper voltage to 0–3.3 V (see HADS Section 5.1), making an in-range ADC reading a plausibility confirmation of both ADC hardware and wiring continuity.

### 4.1.4 POST Completion Criteria

| Test | Pass Condition | Fail Action | Flag |
| --- | --- | --- | --- |
| CPU arithmetic | a + b == 300 | faultCPU = true → systemFault = true → DWI ON, deploy inhibited | faultCPU |
| RAM read-back | 0xAAAAAAAA == 0xAAAAAAAA | faultRAM = true | faultRAM |
| ADC range | 0 ≤ v ≤ 4095 | faultADC = true | faultADC |
| CAN init | Can0.begin() returns success | Not currently checked — identified as gap, see Section 7 | — |

## 4.2 MOD-02 — Sensor Acquisition

Reads all sensor inputs at the start of each loop iteration. Uses the Arduino Due 12-bit ADC (analogReadResolution(12) set in setup()).

uint16_t rawA0 = analogRead(A0);   // Accelerometer POTI-1: 0–4095

uint16_t rawA1 = analogRead(A1);   // Gyroscope    POTI-2: 0–4095

uint16_t rawA2 = analogRead(A2);   // Pressure     POTI-3: 0–4095

bool babySeat  = (digitalRead(PIN_BABY) == LOW);  // Active-LOW switch

| Sensor | Pin | ADC Range | Scaled Range | Deploy Threshold | ADC Threshold |
| --- | --- | --- | --- | --- | --- |
| Accelerometer (POTI-1) | A0 | 0–4095 | 0–100 g | > 30.0 g | raw > 1228 |
| Gyroscope (POTI-2) | A1 | 0–4095 | 0–360 °/s | > 120.0 °/s | raw > 1365 |
| Pressure (POTI-3) | A2 | 0–4095 | 0–500 kPa | > 200.0 kPa | raw > 1638 |
| Baby Seat Switch | D11 | HIGH / LOW | TRUE / FALSE | LOW = baby seat | — |

The ADC threshold values in the table above are calculated as: threshold_ADC = (threshold_engineering / full_scale_engineering) × 4095. These can be used by test engineers to set the potentiometer to a precise ADC count and verify the correct pass/fail boundary.

## 4.3 MOD-03 — Crash Detection

Converts raw ADC values to engineering units and applies threshold comparisons to determine whether a crash event is occurring. An OR relationship between the three sensors is used: any single sensor exceeding its threshold is sufficient to assert localCrash = TRUE on this ECU channel.

### 4.3.1 Scaling Formulas

float accel = (rawA0 / 4095.0) * 100.0;   // 0–100 g

float gyro  = (rawA1 / 4095.0) * 360.0;   // 0–360 deg/s

float press = (rawA2 / 4095.0) * 500.0;   // 0–500 kPa

### 4.3.2 Threshold Comparison

bool localCrash = (accel > THRESH_ACCEL)   // > 30.0 g

|| (gyro  > THRESH_GYRO)    // > 120.0 deg/s

|| (press > THRESH_PRESS);  // > 200.0 kPa

The OR logic intentionally accepts any sensor exceeding its threshold as a potential crash. The 2oo2 architecture provides the false-positive protection: both ECU channels must independently reach localCrash=TRUE before any deployment output is asserted. A single noisy sensor on one ECU will not cause deployment.

### 4.3.3 Threshold Rationale

| Sensor | Threshold | Physical Meaning | Rationale |
| --- | --- | --- | --- |
| Accelerometer | 30.0 g | 30× gravitational acceleration — severe frontal impact pulse characteristic | Below hard-braking (~1 g) and road-bump range (<5 g); above representative airbag-required crash pulse |
| Gyroscope | 120.0 °/s | One-third of full rotation per second — rollover onset angular rate | Well above aggressive cornering (<30 °/s); below confirmed rollover rates (>200 °/s) for laboratory sensitivity |
| Pressure | 200.0 kPa | ~2 bar gauge — door panel structural deformation pressure | Representative of side-impact crush pressure exceeding normal ambient variation |

## 4.4 MOD-04 — Deployment Control

Computes the deployment vote and drives all output pins. This module is the safety-critical decision point that combines all inputs from the other modules.

### 4.4.1 Deployment Vote Logic (Full)

// Step 1: Aggregate system fault

systemFault = faultCPU || faultRAM || faultADC || faultCOMM;

// Step 2: Compute deployment vote

bool voteDeploy = localCrash    // This ECU detects crash

&& remoteCrash   // Peer ECU independently detects crash

&& !systemFault  // No fault on this ECU

&& !remoteFault; // No fault on peer ECU

// Step 3: Drive output pins

digitalWrite(PIN_AIRBAG1, voteDeploy);              // Driver airbag vote

digitalWrite(PIN_AIRBAG2, voteDeploy && !babySeat); // Passenger (suppressed if babySeat)

digitalWrite(PIN_WARNING, systemFault);             // DWI lamp

### 4.4.2 Deployment Authorization Truth Table

| localCrash | remoteCrash | systemFault | remoteFault | babySeat | Airbag1 Vote (D8) | Airbag2 Vote (D9) |
| --- | --- | --- | --- | --- | --- | --- |
| FALSE | any | any | any | any | LOW | LOW |
| TRUE | FALSE | any | any | any | LOW | LOW |
| TRUE | TRUE | TRUE | any | any | LOW | LOW |
| TRUE | TRUE | FALSE | TRUE | any | LOW | LOW |
| TRUE | TRUE | FALSE | FALSE | FALSE | HIGH | HIGH |
| TRUE | TRUE | FALSE | FALSE | TRUE | HIGH | LOW (suppressed) |

Note: Even when D8 or D9 are driven HIGH by this ECU, the hardware DM74LS08N AND gate requires the identical output from the peer ECU simultaneously before the airbag LED fires. The software vote is necessary but not sufficient.

## 4.5 MOD-05 — CAN Communication

### 4.5.1 Transmit Frame (sendData)

Transmits an 8-byte CAN frame every 500 ms containing raw sensor values, status bits, and ECU identity. The 500 ms heartbeat interval ensures the peer ECU can detect communication loss within the 2000 ms faultCOMM timeout window.

frame.data.bytes[0] = rawA0 & 0xFF;   // Accel low byte

frame.data.bytes[1] = rawA0 >> 8;     // Accel high byte

frame.data.bytes[2] = rawA1 & 0xFF;   // Gyro low byte

frame.data.bytes[3] = rawA1 >> 8;     // Gyro high byte

frame.data.bytes[4] = rawA2 & 0xFF;   // Pressure low byte

frame.data.bytes[5] = rawA2 >> 8;     // Pressure high byte

frame.data.bytes[6] = status;          // Bit0=crash, Bit1=baby, Bit2=fault

frame.data.bytes[7] = ECU_ID;          // 1 or 2

### 4.5.2 Receive and Timeout

void receiveCAN() {

while (Can0.rx_avail()) {

Can0.read(incoming);

if (incoming.id == REMOTE_ID) {       // Filter for peer ECU only

lastRemoteHB = millis();           // Reset timeout watchdog

remoteCrash = (status & 0x01);    // Extract crash bit

remoteFault = (status & 0x04);    // Extract fault bit

}

}

}

// Timeout check in main loop:

if (millis() - lastRemoteHB > 2000) faultCOMM = true;

else                                 faultCOMM = false;

## 4.6 MOD-06 — Diagnostic Monitor

Runs every cycle. Aggregates hardware fault flags into systemFault. In the current implementation, POST fault flags (faultCPU, faultRAM, faultADC) are set once at startup and remain sticky for the session. faultCOMM is evaluated dynamically each cycle.

| Fault Flag | Set When | Cleared When | Effect on System |
| --- | --- | --- | --- |
| faultCPU | cpuTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited, ST-02/ST-04 |
| faultRAM | ramTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultADC | adcTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultCOMM | millis() - lastRemoteHB > 2000 ms | Peer CAN frame received within 2000 ms | Dynamic: ST-04 while active, recovers to ST-03 when peer resumes |

# 5. Occupant Classification and Baby Seat Logic

The baby seat detection implements a fail-safe suppress-by-default strategy per FSR-09 and FSR-10. The switch is wired active-LOW with a 10 kΩ pull-up resistor to the Arduino Due 3.3 V rail.

## 5.1 Signal Interpretation

| D11 State | babySeat Value | Airbag 2 (D9) | Meaning |
| --- | --- | --- | --- |
| HIGH (~3.3 V via pull-up) | FALSE | Follows voteDeploy | No child seat — adult or empty — passenger airbag armed |
| LOW (switch pressed) | TRUE | Forced LOW | Child seat present — passenger airbag suppressed |
| Floating / open circuit | TRUE (pull-up ensures HIGH → FALSE) | See gap note below | See FSR-10 gap analysis in Section 7 |

## 5.2 FSR-11 Debounce Requirement

FSR-11 requires the baby seat digital input to remain stable for at least 10 ms before a state transition is accepted. The current firmware implementation reads the digital pin directly each cycle without a debounce timer. This is a compliance gap addressed in Section 7.

Required implementation:

// Required debounce logic (not yet implemented):

static bool lastBabyRaw = HIGH;

static unsigned long babyStableTime = 0;

bool currentRaw = digitalRead(PIN_BABY);

if (currentRaw != lastBabyRaw) {

babyStableTime = millis();       // Reset stability timer on change

lastBabyRaw = currentRaw;

}

if ((millis() - babyStableTime) >= 10) {  // 10 ms stability window

babySeat = (lastBabyRaw == LOW);       // Accept stable value

}

# 6. Safety Mechanisms in Software

## 6.1 Hardware Watchdog Timer (WDT)

The SAM3X8E microcontroller includes a dedicated Hardware Watchdog Timer (WDT) independent of the main CPU. When enabled and serviced, it resets the MCU if the main loop fails to execute within the configured timeout window, protecting against software lockup, infinite loops, and stack overflow conditions.

Current implementation status: The WDT is not explicitly enabled in the current firmware. The Arduino Due bootloader may reset the WDT during startup, but this is not a reliable safety mechanism. Enabling the WDT is required for full FSR-22 compliance.

Required implementation:

// In setup():

WDT_Enable(WDT, WDT_MR_WDD(0xFFF) | WDT_MR_WDRSTEN | WDT_MR_WDV(500));

// 500 = ~500 ms timeout; WDRSTEN = reset on timeout

// At end of each loop() iteration:

WDT_Restart(WDT);   // Feed the watchdog

## 6.2 2oo2 Redundant Decision Architecture

The primary safety mechanism is the dual-channel 2oo2 architecture. Both ECUs independently:

- Read identical sensor inputs (shared potentiometer wipers — high-impedance ADC inputs safe to share)

- Execute identical crash detection algorithms with identical thresholds

- Compute independent deployment votes

- Assert D8/D9 output pins only when their local vote is TRUE

- The DM74LS08N AND gate forms the hardware enforcement layer: both D8 outputs must be HIGH simultaneously for the airbag LED to fire

This architecture prevents: single-point hardware failures, single-channel software errors, single-sensor noise events, and communication corruption (a corrupted remote frame sets remoteCrash to an unknown value, which in the worst case equals FALSE — inhibiting deployment).

## 6.3 Fail-Safe Direction Analysis

| Failure Mode | Software Response | Hardware Response | Safe? |
| --- | --- | --- | --- |
| Peer CAN timeout (>2000 ms) | faultCOMM=TRUE → systemFault=TRUE → voteDeploy=FALSE → D8/D9 LOW | AND gate output LOW | Yes — inhibit on comm loss |
| POST failure (CPU/RAM/ADC) | faultXXX=TRUE, loop() effectively inhibits via systemFault | D8/D9 never driven HIGH in fault state | Yes — inhibit on startup fault |
| Single sensor above threshold | localCrash=TRUE on this ECU only | AND gate requires BOTH D8 HIGH — peer must agree | Yes — no single-channel deploy |
| Single ECU software crash | Loop stops, no further D8/D9 assertions (pins go LOW by default) | AND gate output LOW | Yes — fail LOW |
| babySeat line open circuit | Pull-up holds D11 HIGH → babySeat=FALSE → passenger airbag NOT suppressed | D9 follows voteDeploy | Gap — see FSR-10, Section 7 |

## 6.4 Permitted and Forbidden Software Operations

The following constraints define what the software is and is not permitted to do, consistent with ASIL D software development guidelines:

Permitted:

- Reading ADC pins A0, A1, A2 via analogRead()

- Reading digital pin D11 via digitalRead()

- Writing D8, D9 (deployment votes) and D10 (DWI) via digitalWrite()

- Transmitting CAN frames via Can0.sendFrame()

- Reading received CAN frames via Can0.read()

- Using millis() for timing comparisons

- Printing to Serial for diagnostics

Forbidden:

- Dynamic memory allocation (malloc, new) — prohibited in ASIL D software (heap fragmentation, non-deterministic timing)

- Recursive functions — stack depth is unpredictable

- Unbounded loops (while(true) without exit) — threatens deterministic cycle time

- Direct register access bypassing safety flags — all state transitions must be mediated through the fault flag system

- Clearing faultCPU, faultRAM, faultADC once set — these are sticky faults

- Modifying ECU_ID, DATA_ID, REMOTE_ID at runtime — configuration constants must be compile-time only

# 7. Compliance Gaps and Required Improvements

The following gaps were identified between the current firmware implementation and the requirements specified in FSRS-AIRBAG-ECU-001 Rev 2.1. Each gap has an assigned priority and a required action.

| Gap ID | FSR | Description | Risk | Required Action |
| --- | --- | --- | --- | --- |
| GAP-01 | FSR-11 | Baby seat debounce not implemented — D11 read directly without 10 ms stability filter | Noisy switch signal could cause spurious babySeat state change | Implement debounce timer as shown in Section 5.2 |
| GAP-02 | FSR-12 | Baby seat line continuity monitoring not implemented — open-circuit stuck-HIGH not detected | Open circuit mimics adult occupancy — passenger airbag not suppressed when it should be | Add periodic D11 continuity test or input monitoring |
| GAP-03 | FSR-13 | CRC not included in CAN frame — Bytes 6–7 carry status and ECU_ID, not a 16-bit CRC | Corrupted CAN frames accepted as valid without error detection | Compute 16-bit CRC over bytes 0–5, store in bytes 6–7, verify on receive |
| GAP-04 | FSR-14 | Parallel CAN bitstream readback not implemented | CAN transmission corruption goes undetected | Implement readback channel or loopback verification |
| GAP-05 | FSR-17 | CAN communication not verified during POST — Can0.begin() success not checked | Silent CAN initialisation failure allows system to proceed with no communication | Check Can0.begin() return value and set faultCOMM if it fails |
| GAP-06 | FSR-22 | Hardware Watchdog Timer not enabled in firmware | Software lockup does not trigger MCU reset — system hangs silently | Enable WDT in setup() and feed it at end of every loop() iteration |
| GAP-07 | FSR-21 | Fault logging to DTC/non-volatile memory not implemented — faults only shown on Serial | Fault history lost on power cycle — cannot be retrieved by service tools | Implement DTC logging to EEPROM or flash |

# 8. Software Test Specification

This section defines the test cases required to verify the software against FSRS requirements. Tests are categorised as Black Box (BB — testing from inputs to outputs without knowledge of internal logic) or White Box (WB — testing internal logic paths, branches, and conditions).

Target fault detection coverage: ≥90% of identifiable single-point software faults shall be detectable through the defined test cases. 99% coverage is not required for this laboratory implementation given the dual-channel hardware safety net; 90% is the agreed target balancing thoroughness with practicality.

## 8.1 Test Environment Setup

- Connect both Arduino Due boards per HADS-AIRBAG-ECU-001 Rev 2.1 wiring tables

- Connect OMS PC via USB-CAN-A and open USBCANV2 at 500 kbit/s, monitoring IDs 0x100 and 0x200

- Open Serial Monitor (115200 baud) on both ECUs for fault flag and decision visibility

- Potentiometers POTI-1, POTI-2, POTI-3 at mid-position (ADC ~2048) as baseline (all above threshold)

- Baby seat switch open (HIGH) as baseline

## 8.2 Test Cases — Normal Operation

| TC-ID | Type | Description | Preconditions | Action | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC-01 | BB | No-crash baseline — all sensors below threshold | All pots at minimum (ADC ~0) | Power on, do not move pots | Serial: localCrash=0, remoteCrash=0, voteDeploy=0. D8/D9=LOW. Airbag LEDs OFF. | Both LEDs off for ≥5 s | FSR-04 |
| TC-02 | BB | Single-sensor crash — accelerometer only above threshold on both ECUs | POTI-1 set to ADC > 1228 on both boards | Move POTI-1 past threshold simultaneously | localCrash=1 on both. voteDeploy=1. D8 HIGH on both. AND gate fires: Airbag1 LED ON. | Airbag1 LED illuminates within observed cycle | FSR-01, FSR-03 |
| TC-03 | BB | Gyroscope-only crash detection | POTI-2 > ADC 1365, POTI-1 and POTI-3 below threshold | Move POTI-2 past threshold on both ECUs | localCrash=1 from gyro threshold. Airbag1 LED ON. | LED fires from gyro signal alone | FSR-01 |
| TC-04 | BB | Pressure-only crash detection | POTI-3 > ADC 1638, others below | Move POTI-3 past threshold on both ECUs | localCrash=1 from pressure. Airbag1 LED ON. | LED fires from pressure signal alone | FSR-01 |
| TC-05 | BB | Crash ceases — sensor returns below threshold | Start from TC-02 (both LEDs on) | Return POTI-1 below ADC 1228 | localCrash=0 → voteDeploy=0 → D8/D9 LOW → LEDs OFF | LEDs extinguish within one cycle | FSR-04 |
| TC-06 | BB | Baby seat suppression during crash | POTI-1 above threshold on both ECUs, baby seat switch pressed | Keep POTI-1 high; press and hold baby seat button | voteDeploy=TRUE but babySeat=TRUE → D9=LOW. Airbag1 LED ON, Airbag2 LED OFF. | Airbag2 LED remains off; Airbag1 fires | FSR-09 |

## 8.3 Test Cases — Fault Injection

| TC-ID | Type | Description | Action | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- | --- | --- |
| TC-07 | BB | Single-channel crash — only one ECU detects crash | Move POTI-1 above threshold on ECU1 only; ECU2 sensor stays below | ECU1: localCrash=1. ECU2: localCrash=0. ECU1 remoteCrash=0 (from ECU2 CAN). voteDeploy=FALSE on ECU1. D8=LOW. | Both airbag LEDs remain OFF. AND gate cannot fire. | FSR-03, FSR-08 |
| TC-08 | BB | CAN communication loss — peer timeout | Disconnect CAN bus between ECUs (remove twisted pair) | After 2000 ms: faultCOMM=TRUE → systemFault=TRUE → voteDeploy=FALSE → D8/D9 LOW. DWI LED ON. | DWI lights within 2000 ms + 1 cycle. Airbag LEDs OFF. | FSR-15, FSR-20 |
| TC-09 | BB + WB | CAN reconnection — fault recovery | After TC-08, reconnect CAN bus | lastRemoteHB updates → faultCOMM=FALSE → systemFault may clear (if no other faults). DWI OFF. System returns to ST-03. | System recovers to MONITORING state | FSR-15 |
| TC-10 | WB | CPU test failure simulation | Modify firmware temporarily: return false from cpuTest() | faultCPU=TRUE → systemFault=TRUE → DWI ON. Loop: voteDeploy always FALSE regardless of sensor state. | Even with both sensors above threshold, no airbag LED fires. DWI ON. | FSR-17, FSR-20 |
| TC-11 | WB | RAM test failure simulation | Modify firmware temporarily: return false from ramTest() | faultRAM=TRUE → same inhibit chain as TC-10 | Airbag LEDs OFF. DWI ON. | FSR-17, FSR-22 |
| TC-12 | BB | Remote fault inhibit — peer ECU in fault state | Force systemFault=TRUE on ECU2 (e.g., disconnect ADC) | ECU2 CAN frame byte 6 bit 2 = 1. ECU1 reads remoteFault=TRUE. ECU1 voteDeploy=FALSE even if localCrash=TRUE. | ECU1 airbag LED remains OFF when peer is faulted. | FSR-03 |
| TC-13 | BB | Baby seat suppression with fault present | Press baby seat button; also inject comm fault | systemFault=TRUE from faultCOMM. voteDeploy=FALSE. Both LEDs OFF, DWI ON. | No deployment possible when system faulted — overrides crash. | FSR-09, FSR-20 |

## 8.4 Test Cases — Timing Verification

| TC-ID | Description | Method | Acceptance Criterion | FSR |
| --- | --- | --- | --- | --- |
| TC-14 | Loop cycle time ≤ 1 ms | Add micros() timestamp at start and end of loop(). Log delta via Serial. | 99th percentile cycle time ≤ 1000 µs over 10,000 consecutive iterations | FSR-02 |
| TC-15 | Crash detection response ≤ 20 ms | Timestamp when POTI crosses threshold; timestamp when localCrash=TRUE appears in Serial log | Delta ≤ 20 ms for 10 trials | FSR-01 |
| TC-16 | CAN heartbeat interval = 500 ms ± 50 ms | Log CAN frame timestamps in USBCANV2. Calculate inter-frame intervals. | All intervals 450–550 ms over 20 frames | FSR-16 |
| TC-17 | faultCOMM declared within 2000 ms of CAN loss | Disconnect CAN, note millis() when last frame received and when faultCOMM=TRUE appears | faultCOMM declared ≤ 2050 ms after last valid frame | FSR-15 |
| TC-18 | DWI activates within 800 ms of fault | Inject fault at known time; observe D10 HIGH transition in Serial output | D10 HIGH within 800 ms. At 1 ms cycle rate expected ~1 ms. | FSR-19 |

## 8.5 Saved Variable Snapshot — Evidence Collection

For each test case, the following Serial Monitor variables shall be recorded and saved to a test log file as evidence that the deployment thresholds were reached or not reached:

// Sample Serial output captured during TC-02 (expected):

================================

ECU=1

A0=1350   // > 1228 threshold — accel exceeded

A1=800    // < 1365 — gyro OK

A2=900    // < 1638 — pressure OK

Baby=0

Local Crash=1   // <-- confirms threshold crossed

Remote Crash=1  // <-- confirms peer agreement

Comm Fault=0

System Fault=0

Vote Deploy=1   // <-- deployment authorised

================================

# 9. Requirement Traceability

| FSR ID | FSR Title | Implementing Module(s) | Test Case(s) | Gap |
| --- | --- | --- | --- | --- |
| FSR-01 | Crash Detection Timing (≤20 ms) | MOD-02, MOD-03 | TC-01 to TC-05, TC-15 | — |
| FSR-02 | Algorithm Cycle Time (≤1 ms) | All modules (loop discipline) | TC-14 | — |
| FSR-03 | Deployment Authorization Logic (6 conditions) | MOD-04 | TC-02, TC-06, TC-07, TC-12 | — |
| FSR-04 | Prevention of Unintended Deployment | MOD-04 | TC-01, TC-05, TC-07 | — |
| FSR-07 | Dual-Channel Redundancy | MOD-02 to MOD-05 | TC-07 | — |
| FSR-08 | Channel Agreement Verification | MOD-04, MOD-05 | TC-07 | — |
| FSR-09 | Passenger Airbag Suppression | MOD-04 | TC-06, TC-13 | — |
| FSR-10 | Suppress-on-Fault (baby seat) | MOD-04 | TC-13 | GAP-02 |
| FSR-11 | Digital Input Debounce (10 ms) | MOD-02 (not yet) | — | GAP-01 |
| FSR-12 | Baby Seat Line Monitoring | MOD-06 (not yet) | — | GAP-02 |
| FSR-13 | CRC-Protected CAN | MOD-05 (not yet) | — | GAP-03 |
| FSR-14 | CAN Bitstream Readback | MOD-05 (not yet) | — | GAP-04 |
| FSR-15 | CAN Timeout Detection (≤50 ms) | MOD-05, MOD-06 | TC-08, TC-17 | Timeout is 2000 ms > 50 ms spec |
| FSR-17 | Power-On Self-Test | MOD-01 | TC-10, TC-11 | GAP-05 |
| FSR-18 | Continuous Diagnostic Monitoring | MOD-06 | TC-08 | — |
| FSR-19 | DWI Activation (≤800 ms) | MOD-06, MOD-04 | TC-18 | — |
| FSR-20 | Safe-State Transition | MOD-04, MOD-06 | TC-08, TC-10 | — |
| FSR-22 | Memory Integrity Protection | MOD-01, MOD-06 (partial) | TC-11 | GAP-06 |

# 10. References

## 10.1 Project Documents

| Document ID | Title | Revision | Date | Relationship to this Document |
| --- | --- | --- | --- | --- |
| HARA-AIRBAG-ECU-001 | Hazard Analysis and Risk Assessment | Rev 2.1 — 2026-05-06 | 2026-05-06 | Source of hazard IDs and ASIL derivation used in Section 1 |
| FSRS-AIRBAG-ECU-001 | Functional Safety Requirements Specification | Rev 2.1 — 2026-05-11 | 2026-05-11 | Primary source for all FSR-XX requirements and ASIL/SIL classifications referenced throughout this document |
| SC-AIRBAG-ECU-001 | Safety Concept Document | Rev 2.0 — 2026-05-12 | 2026-05-12 | Defines safety constraints SC-01–SC-09 enforced by this software |
| HADS-AIRBAG-ECU-001 | Hardware Architecture and Design Specification | Rev 2.1 — 2026-06-11 | 2026-06-11 | Defines hardware wiring, BOM, CAN IDs, pin assignments, and AND gate circuit referenced in Sections 4–6 |
| SRS-AIRBAG-SW-001 | Software Requirements Specification | Rev 1.0 | — | Source of SWR-01–SWR-25 requirements mapped in Section 9 traceability matrix (Rev 1.0 UART baseline) |
| SVVP-AIRBAG-ECU-001 | Software Verification and Validation Plan | — | — | Defines test methods, coverage targets, and V&V procedures for test cases in Section 8 |

## 10.2 Normative Standards

# 11. Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Software Safety Engineer |  |  |  |
| Hardware Safety Engineer |  |  |  |
| Functional Safety Engineer |  |  |  |
| V&V Lead |  |  |  |
| Project Supervisor |  |  |  |

# 12. Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-28 | Software Safety Engineering Team | Initial release (UART architecture). Inter-ECU communication via UART Serial1 at 115200 baud. Due 1 designated Master with CAN0 to OMS PC; Due 2 designated Slave with no CAN module. Heartbeat and status string transmitted over UART. Full SADS covering architecture, module decomposition (MOD-01–MOD-20), state machine, safety mechanisms, pseudo-code, and SWR traceability. |
| 2.0 | 2026-06-12 | Software Safety Engineering Team | Major revision — CAN architecture migration. UART inter-ECU link replaced by CAN bus (500 kbit/s, Waveshare SN65HVD230 x2). Master/Slave distinction removed; both ECUs now operate as peer-equal nodes (ECU 1: TX ID 0x100, RX 0x200; ECU 2: TX ID 0x200, RX 0x100). Heartbeat interval retained at 500 ms; timeout threshold retained at 2000 ms. CAN frame byte layout updated. SRS reference updated to HADS-AIRBAG-ECU-001 Rev 2.1. Added compliance gap analysis (GAP-01–GAP-07), extended test specification (TC-01–TC-18), and FSR traceability matrix. |