FUNCTIONAL SAFETY

HARDWARE ARCHITECTURE AND DESIGN SPECIFICATION

Airbag Electronic Control Unit (ECU)

ISO 26262:2018 / IEC 61508:2010 | ASIL D / SIL 3

# 1. Document Information

| Document Title | Hardware Architecture and Design Specification |
| --- | --- |
| Document Number | HADS-AIRBAG-ECU-001 |
| Revision | 1.0 |
| Date | 2026-05-31 |
| System | Airbag Electronic Control Unit (ECU) |
| Standards | ISO 26262:2018 / IEC 61508:2010 |
| ASIL / SIL | ASIL D / SIL 3 |
| Classification | Confidential Functional Safety Document |
| Based On | FSRS-AIRBAG-ECU-001 Rev 2.1 ¦ SC-AIRBAG-ECU-001 Rev 2.0 |
| Author | Hardware Safety Engineering Team |

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-31 | Hardware Safety Engineering Team | Initial release |

## 1.1 Revision History

# 2. Purpose and Scope

This Hardware Architecture and Design Specification (HADS) describes the complete hardware design of the Airbag ECU laboratory system. It defines the physical component inventory, electrical connections, signal specifications, power architecture, safety-relevant hardware mechanisms, and the rationale for every hardware decision made to achieve ASIL D / SIL 3 compliance.

The document covers the confirmed laboratory implementation consisting of two Arduino Due boards, three potentiometer-based analog sensor simulations, one digital baby-seat switch, a DM74LS08N hardware AND gate enforcing 2oo2 deployment logic, LED deployment indicators, a CAN transceiver, and all associated passive components.

The physical airbag inflator, pyrotechnic squib circuits, and vehicle body structure are outside the laboratory scope and are treated as adjacent systems beyond the item boundary.

# 3. System Overview and Design Principles

## 3.1 Defence in Depth - Three Independent Protection Layers

The hardware architecture implements three independent layers of protection against unintended airbag deployment. No single hardware or software failure can bypass all three layers simultaneously.

| Layer | Mechanism | Description |
| --- | --- | --- |
| 1 | Software threshold check | Each board independently evaluates sensor readings against defined crash thresholds. Neither board drives its output HIGH unless its own sensor data confirms a crash event. |
| 2 | UART mutual supervision | Each board continuously monitors the the other board via UART heartbeat. A silent board forces its own output LOW, preventing a dead board from leaving a stuck-HIGH on an AND gate input. |
| 3 | Hardware AND gate (DM74LS08N) | A physical logic gate independent of any software, requires both boards to simultaneously assert their output pins HIGH before the deployment signal propagates. |

## 3.2 2oo2 Safety Architecture

A single Arduino Due processing channel does not satisfy the ASIL D single-point fault metric requirements of ISO 26262-5. A single microcontroller board has no protection against its own internal faults causing unintended deployment. The 2oo2 (Two-out-of-Two) architecture resolves this by requiring independent agreement from two physically separate processing channels before any deployment output is activated.

The physical implementation of the 2oo2 constraint is the DM74LS08N AND gate. Both Arduino Due boards must simultaneously drive their respective AND gate input pins HIGH for the AND gate output to go HIGH and activate the LED deployment indicators. A software crash, MCU reset, or watchdog expiry on either board will immediately drive that board's output LOW, collapsing the AND gate output to LOW regardless of the other board's state.

# 4. Hardware Architecture

## 4.1 Bill of Materials

| Qty | Component | Role | Interface | ASIL |
| --- | --- | --- | --- | --- |
| 2 | Arduino Due (SAM3X8E) | Processing channels A and B | ADC, GPIO, UART, CAN0 | ASIL D |
| 2 | Potentiometer 10 kΩ — POTI-1 (Accelerometer) | Simulate accelerometer on A0 of both boards | Analog 0–3.3 V | ASIL D |
| 2 | Potentiometer 10 kΩ — POTI-2 (Gyroscope) | Simulate gyroscope on A1 of both boards | Analog 0–3.3 V | ASIL D |
| 2 | Potentiometer 10 kΩ — POTI-3 (Pressure sensor) | Simulate pressure sensor on A2 of both boards | Analog 0–3.3 V | ASIL D |
| 1 | Tactile push button | Baby seat detection switch on D11 | Digital GPIO active-LOW | ASIL D |
| 2 | 10 kΩ resistor | Ensure D11 defaults HIGH (no seat) on open circuit | Passive | ASIL D |
| 1 | Quad 2-Input AND gate (DM74LS08N) | Physical 2oo2 deployment enforcement | Logic 3.3 V | ASIL D |
| 1 | Airbag 1 LED (driver airbag indicator) | Visual indicator for driver airbag deployment | Via 220 Ω directly from AND gate output | ASIL B |
| 1 | Airbag 2 LED (passenger airbag indicator) | Visual indicator for passenger airbag deployment | Via 220 Ω directly from AND gate output | ASIL B |
| 1 | DWI warning lamp LED | Diagnostic Warning Indicator - illuminates on any fault | D10 via 220 Ω on each board - OR logic | ASIL A |
| 5 | 220 Ω resistor | Limit LED current to safe level | Passive | QM |
| 1 | Waveshare CAN transceiver board (SN65HVD230) | CAN bus interface for Due 1 to OMS lab PC | CANTX/CANRX 3.3 V, CANH/CANL diff pair | ASIL B |
| 1 | USB-CAN-A adapter | Converts CAN bus differential pair to USB for OMS PC | CANH/CANL in, USB out | ASIL B |
| 1 | 120 Ω resistor | Terminate CAN bus at USB-CAN-A end impedance matching | Across CANH/CANL | ASIL B |

3.2 Sensor connections

| Sensor | Lab sub | Due 1 pin | Due 2 pin | Signal | Physical range |
| --- | --- | --- | --- | --- | --- |
| Accelerometer | POTI-1 | A0 | A0 | Analog 0–3.3 V | 0–100 g crash pulse |
| Gyroscope | POTI-2 | A1 | A1 | Analog 0–3.3 V | 0–360 deg/s rollover |
| Pressure sensor | POTI-3 | A2 | A2 | Analog 0–3.3 V | 0–500 kPa side impact |
| Baby seat switch | Push button | D11 | D11 | Digital active-LOW | LOW=seat present |

## 3.3 Output connections

| Output | Source | Hardware path | Active condition |
| --- | --- | --- | --- |
| Airbag 1 LED (red) | AND gate 1 output | Due1 D8 + Due2 D8 → DM74LS08N pin3 → MOSFET → LED | Both D8 pins HIGH simultaneously |
| Airbag 2 LED (blue) | AND gate 2 output | Due1 D9 + Due2 D9 → DM74LS08N pin6 → MOSFET → LED | Both D9 pins HIGH, pax not suppressed |
| DWI lamp (amber) | OR logic | Due1 D10 or Due2 D10 → LED | Either board detects any fault |
| CAN to OMS PC | Due 1 CAN0 | pin69 → Waveshare → USB-CAN-A → laptop | Every 100 ms, always transmitting |

# 4. Software Architecture

## 4.1 Execution model

Both boards run a single-threaded bare-metal Arduino loop. There is no RTOS. The main loop executes continuously with no blocking calls. All timing is managed via millis() non-blocking timers. The hardware watchdog resets the MCU if the loop fails to complete within 10 ms.

## 4.2 Loop task schedule

| Task | Trigger | Max time | Safety impact |
| --- | --- | --- | --- |
| watchdog_feed() | Every loop — first call | < 0.1 ms | CRITICAL — must be first |
| updateBabySeat() | Every loop | < 0.1 ms | ASIL D — debounce logic |
| LED blink | millis() > 500 ms | < 0.1 ms | QM — diagnostic only |
| UART heartbeat TX | millis() > 500 ms | < 0.5 ms | ASIL D — peer supervision |
| UART receive | Serial1.available() | < 1 ms | ASIL D — peer data |
| UART timeout check | Every loop | < 0.1 ms | ASIL D — fault detection |
| Sensor read A0/A1/A2 | Every loop | < 0.3 ms | ASIL D — crash detection |
| Threshold evaluation | Every loop | < 0.1 ms | ASIL D — deployment decision |
| Baby seat + fault eval | Every loop | < 0.1 ms | ASIL D — suppression |
| D8/D9/D10 output drive | Every loop | < 0.1 ms | ASIL D — AND gate inputs |
| CAN TX (Due 1 only) | millis() > 100 ms | < 1 ms | ASIL B — OMS reporting |
| Serial print | millis() > 1000 ms | < 2 ms | ASIL A — diagnostic logging |

# 5. Communication Architecture

## 5.1 UART inter-board link

| Parameter | Value |
| --- | --- |
| Baud rate | 115200 baud |
| Interface | Serial1 — Due 1 TX1(pin18)/RX1(pin19) ↔ Due 2 TX1(pin18)/RX1(pin19) |
| Heartbeat interval | 500 ms |
| Timeout fault threshold | 2000 ms |
| Data format | "ECU2,<crash>,<baby>,<fault>\n" — 3 binary fields |
| Heartbeat format | "ECU1:HB\n" / "ECU2:HB\n" |
| Purpose | Mutual liveness monitoring + Due 2 status forwarding to Due 1 |

## 5.2 CAN bus (Due 1 to OMS lab PC)

| Parameter | Value |
| --- | --- |
| Baud rate | 500 kbit/s |
| CAN ID | 0x100 |
| DLC | 8 bytes |
| Transmit interval | 100 ms |
| Transceiver | Waveshare SN65HVD230 / VP230 |
| Termination | 120 Ω at USB-CAN-A end |
| OMS software | PCAN-View (Windows) or can-utils (Linux) |
| CRC protection | CRC-16 poly 0x1021, over bytes 0–5, stored in bytes 6–7 |

## 5.3 CAN frame byte map

| Byte | Field | Bits / encoding |
| --- | --- | --- |
| 0 | Flags | Bit0=crashA, Bit1=crashB, Bit2=deploy1, Bit3=deploy2, Bit4=babySeat, Bit5=faultA, Bit6=faultB, Bit7=DWI |
| 1 | Due1 accel | (accel_g / 100.0) * 255 — 0=0g, 255=100g |
| 2 | Due1 gyro | (gyro_dps / 360.0) * 255 — 0=0deg/s, 255=360deg/s |
| 3 | Due2 status | Bit0=due2_crash, Bit1=due2_baby, Bit2=due2_fault, Bit3=UART_link_lost |
| 4 | Reserved | 0x00 |
| 5 | Sequence | Rolling counter 0–255, wraps around |
| 6–7 | CRC-16 | CRC-16/CCITT over bytes 0–5, big-endian |

# 6. Safety Mechanisms

| Mechanism | Implementation | Protects against | FSR |
| --- | --- | --- | --- |
| Hardware AND gate 2oo2 | DM74LS08N — both D8 pins must be HIGH | Single-board failure causing deployment | FSR-03, FSR-08 |
| Hardware watchdog | SAM3X8E WDT — 10 ms timeout, reset-on-expire | Software lockup or infinite loop | FSR-22 |
| UART mutual supervision | Heartbeat every 500 ms, fault on 2000 ms silence | Silent board leaving AND gate input stuck HIGH | FSR-07, FSR-15 |
| Suppress-on-fault (baby seat) | Default to babySeat=TRUE on signal instability | Passenger airbag deploying into child seat | FSR-10 |
| 10 ms debounce filter | updateBabySeat() stable timer | Noisy baby seat signal causing false suppression | FSR-11 |
| POST self-tests | cpu_test, ram_test, adc_test at boot | Faulty hardware being armed | FSR-17 |
| Fault inhibit | system_fault → D8/D9 forced LOW | Faulty system deploying airbag | FSR-20 |
| DWI on any fault | D10 HIGH on local or remote fault flag | Silent fault going undetected by driver | FSR-19 |
| CRC-16 on CAN frames | crc16() over first 6 bytes | Corrupted CAN frame causing incorrect OMS data | FSR-13 |
| Rolling sequence counter | canSeq++ in every frame | Detecting dropped or duplicate CAN frames | FSR-16 |