Airbag ECU Portfolio HS Harz — Functional Safety AIRBAG ELECTRONIC CONTROL UNIT Measurement and Processing of Sensor Signals to Control Airbag Operation

LIFE CYCLE PHASE 2 TASK OF THE PROCESS OR PRODUCT

| Document Number | LCP02-AIRBAG-ECU-001 |
| --- | --- |
| Version | 1.10 |
| Date | 2026-05-03 |
| Editor | Alita, Fateme, HS Harz FuSa — Project Management Team |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| File | LCP02-AIRBAG-ECU-001_Concept_v_110.docx |

Table of Contents

1  Meetings within the Project	3

2  Unresolved Topics	4

3  Glossary	5

4  Basics — Concept of Operation and Safety	6

4.1  Purpose of the Concept Phase	6

4.2  Applicable Standards	6

4.3  Team and Responsibilities	6

5  System Description	8

5.1  System Operation Environment	8

5.2  Safety-Relevant Functionalities	8

6  Figure and Table Listing	9

8  List of Dependent Documents	10

9  Literature	11

12  Version History	12

# 1  Meetings within the Project

The following project meetings were held during the Concept Phase:

Table 1  Meetings

| Topic | Date / Time | Participants | Minutes / Decisions |
| --- | --- | --- | --- |
| Concept Phase, Task & Scope Definition alignment | 2026-05-02, 9:00 | Safety and Analysis Team, Quality Manager, Project Manager, HW and SW team | Confirmed item boundary from LCP01. Assigned hazard brainstorming tasks. Divided members into groups (Software Dev, Hardware Dev, Test, Project Management). |

# 2  Unresolved Topics

Table 2: Unresolved Topics

| No. | Topic / Question | Responsible | Target Date | Status |
| --- | --- | --- | --- | --- |
| No unresolved topics at time of issue. Any topics arising during review shall be recorded here before the document status is changed to RELEASED. |  |  |  |  |

# 3  Glossary

Table 2 lists all abbreviations and terms used in this document and the wider Airbag ECU project portfolio.

Table 3: Glossary

| Term / Abbreviation | Definition |
| --- | --- |
| ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
| ACU | Airbag Control Unit — synonymous with Airbag ECU |
| CAN | Controller Area Network — serial communication bus for vehicle ECUs |
| ECU | Electronic Control Unit |
| EMC | Electromagnetic Compatibility |
| EUC | Equipment Under Control — the item/system being safety-assessed |
| FSRS | Functional Safety Requirements Specification |
| HARA | Hazard and Risk Analysis |
| HW | Hardware |
| ISO 26262 | Road Vehicles — Functional Safety standard (2018 edition) |
| IEC 61508 | Functional Safety of E/E/PE Safety-related Systems (2010 edition) |
| LCP01 | Life Cycle Phase 01 — Concept Phase (first of 16 IEC 61508 / ISO 26262 phases) |
| POST | Power-On Self-Test |
| QM | Quality Management — no specific ASIL requirement |
| SIL | Safety Integrity Level per IEC 61508 (1–4) |

# 4  Introduction & Objectives of LCP02

## The primary purpose of Lifecycle Phase 2 (LCP02 — Task of the Process or Product) is to deliver an exhaustive, deterministic description of the exact behavioral requirements, operational envelopes, interactions, and design tasks allocated to the Airbag Electronic Control Unit (ACU).

## This document directly references the high-level baseline specifications established in LCP01-AIRBAG-ECU-001_Concept_v110.docx as its foundational input. By expanding those initial functional requirements into an explicit, detailed operational model, LCP02 creates the solid technical basis required to execute the upcoming LCP03 (Hazard and Risk Analysis — HARA). This guarantees full end-to-end requirement traceability across the development lifecycle.

# 5  Detailed Functional Description

The Airbag ECU acts as the central safety-critical supervisor for crash tracking and rapid energy deployment inside modern vehicle passenger compartments. In our specialized laboratory implementation, the functional core relies on two separate Arduino Due microcontroller units operating in an active dual-channel cross-voted architecture (2oo2 voting format). Both microcontrollers must independently process sensory fields, cross-check telemetry indices, and execute simultaneous matching firing decisions before an actual squib deployment can proceed.

The complete functional loop consists of continuous high-frequency digital sensing, signal filtering, parameter fusion, and physical state matching. It runs parallel real-time diagnostic routines to verify structural board execution health and prevent accidental triggering.

# 6. Inputs and Outputs (I/O) Architecture

The specific configuration of physical connections, sensor mappings, and output drivers allocated to the system test board includes the following interfaces:

## 6.1 Physical System Inputs

- POTI-1 (Analog Acceleration Channel): Linear potentiometer tracking continuous longitudinal deceleration ranges. Simulates crash-impact force vectors fed into the internal algorithmic thresholds.

- POTI-2 (Analog Gyroscope Angular Velocity Channel): Linear potentiometer mapping rotational angular velocities to track vehicle yawing, pitching, and rolling changes. Necessary for detecting rollover events.

- POTI-3 (Analog Door-Panel Pressure Channel): Linear potentiometer tracking barometric pressure changes within localized side-door volumes to detect rapid lateral compression during side impacts.

- Digital Switch (Occupant Classification System / OCS Input): Hardware switch simulating front passenger seat presence classification. Toggling the input flags whether a rear-facing infant safety carrier seat occupies the position, which strictly disables the passenger-side explosive airbag loop.

## 6.2 Physical System Outputs

- Driver Airbag Squib Line: Low-impedance dual-pin ignition channel firing driver-side steering column occupant protection upon confirmed collision detection.

- Passenger Airbag Squib Line: Low-impedance dual-pin ignition channel firing passenger dashboard occupant protection, provided suppression is not active via the OCS interface.

- DWI (Diagnostic Warning Indicator) Lamp: High-visibility control-panel illumination indicator displaying persistent fault status to users when any interior monitoring routine reports errors.

- CAN Bus Interface: Serial controller area network transceiver linkage broadcasting periodic health logs and real-time high-priority accident notification frames directly to the central lab PC.

# 7. System Operating Modes

The internal software code state machine transitions between four distinct operational execution zones to maintain functional safety targets:

Table 4: System Operating Modes

| Operating Mode | Functional Description and Behavioral Rules |
| --- | --- |
| 1. Initialization Mode (POST) | Triggered at power application. Executes full internal Power-On Self-Test (POST) routines within a strict maximum window of 800 ms. Verifies Flash CRC integrity, tests internal RAM registers, verifies watchdog communication, and executes electrical squib line continuity assessments. Deployment outputs are safely clamped low. |
| 2. Normal Operation Mode | The main system operation zone. Performs high-speed cyclic sampling of analog sensory interfaces at background intervals ≤ 100 ms. Runs continuous sensor fusion algorithms to cross-reference acceleration trends with interior internal door pressure waves. Monitors the state of the OCS baby-seat switch. |
| 3. Deployment Mode | Entered instantly if acceleration or pressure inputs cross verified collision trigger thresholds. In this mode, both Arduino channels must validate the trigger within ≤ 15 ms, prompting coordinated squib deployment fire pulses. The high-priority CAN crash frame is sent out within ≤ 15 ms of this decision. |
| 4. Safe / Fallback Mode | Activated within ≤ 800 ms if any routine catches out-of-range anomalies or corrupted memory flags. Based on the fault classification, the system moves to specific states: SS-01 (full deployment inhibit), SS-04 (partial-channel isolation), or SS-03 (passenger airbag suppression due to an OCS switch fault), while lighting the dashboard DWI lamp. |

# 8. Existing Safety Measures

To manage random hardware faults and prevent accidental deployment commands caused by systematic bugs, the architecture features the following built-in safety controls:

- Dual-Channel Microcontroller Integration (2oo2): Hardware design requiring both microcontrollers to confirm impact parameters before routing power to the low-resistance firing squibs. A fault on one channel immediately stops the deployment loop.

- Onboard Energy Storage Autarky: Internal capacitive backup reservoirs that maintain independent system power for ≥ 150 ms if a vehicle crash breaks the battery connection line.

- Hardware Loop Pull-Down Restraints: Physical pull-down resistors positioned directly on the squib lines to hold the firing lines in a stable unexcited state, preventing accidental triggering from electrical transients or floating pins.

- Continuous Memory and Algorithmic Surveillance: Background cyclic software surveillance checking internal Flash memory bounds via a persistent CRC matrix, verifying inputs with range filters, and resetting via hardware watchdogs.

# 9. List of Dependent Documents

The functional parameters and system operational definitions documented in this LCP02 phase establish dependencies with the following upstream and downstream project files:

Table 5: List of Dependent Documents

| No. | Document Name | Version | Date | File Name |
| --- | --- | --- | --- | --- |
| 1 | Life Cycle Phase 1 — Concept Document | 1.10 | 2026-05-03 | LCP01-AIRBAG-ECU-001_Concept_v110 |
| 2 | Hazard and Risk Analysis | 1.1 | 2026-05-05 | LCP03-AIRBAG-ECU-001_HARA_v110 |
| 3 | Safety Concept Document | 1.1 | 2026-05-06 | LCP04-AIRBAG-ECU-001_HARA_v110 |

# 10. Figure and Table Listing

Table 1  Meetings	3

Table 2: Unresolved Topics	4

Table 3: Glossary	5

Table 4: System Operating Modes	8

Table 5: List of Dependent Documents	9

Table 6:Version History	10

# 12. Literature

[1] ISO 26262:2018 — Road Vehicles — Functional Safety, Parts 1–12. International Organization for Standardization.

[2] IEC 61508:2010 — Functional Safety of Electrical/Electronic/Programmable Electronic Safety-related Systems, Parts 1–7. International Electrotechnical Commission.

[3] LV124:2013 — Electric and Electronic Components in Motor Vehicles (Environmental Requirements).

# 13. Version History

Table 6:Version History

| Document Name / File | Date | Change Log |
| --- | --- | --- |
| LCP02-AIRBAG-ECU-001_Task_of_the_Process_or_Product_v100.docx | 2026-06-10 | Version 1.00 — Fresh creation of Lifecycle Phase 2 document tracking full functional descriptions, I/O parameters, and operating state logic mapping. |