Airbag Electronic Control Unit (ECU) Portfolio HS Harz — Functional Safety Engineering INTEGRATION TEST PLAN Measurement and Processing of Sensor Signals to Control Airbag Operation

ASIL D / SIL 3

| Field | Value |
| --- | --- |
| Document Number | ITP-AIRBAG-ECU-001 |
| Revision | 3.0 |
| Date | 2026-06-13 |
| Status | RELEASED |
| ASIL / SIL | ASIL D / SIL 3 |
| Target Hardware | Arduino Due (SAM3X8E) x2 |
| Author | Atharva Banarase, Mrunal Ghotekar |
| Based On | SADS-AIRBAG-ECU-001 Rev 3.0 ¦ SC-AIRBAG-ECU-001 Rev 2.0 ¦ FSRS-AIRBAG-ECU-001 Rev 2.1 ¦ HADS-AIRBAG-ECU-001 Rev 2.0 Final |

Table of Contents

1 Glossary	3

2 Purpose and Scope	3

3 Test Strategy	5

3.1 Position in the V&V Lifecycle	5

3.2 Integration Test Level Overview	5

3.3 Test Approach	5

4 Test Environment	6

4.1 Hardware Configuration	6

4.2 Software / Tooling	6

4.3 Interfaces Under Test	6

4.4 Baseline Configuration	7

5 Entry and Exit Criteria	7

5.1 Entry Criteria	7

5.2 HW/SW Integration Bring-Up as Gating Step	8

5.3 Exit Criteria	8

6 Integration Test Case Format	8

7 Integration Test Cases	8

7.1 Normal Operation Tests	8

IT-01 — No-crash baseline — all sensors below threshold	9

IT-02 — Full dual-channel crash — both ECUs above threshold simultaneously	9

IT-03 — Crash ceases — sensors returned below threshold	9

IT-04 — Baby seat suppression during crash	10

IT-05 — CAN heartbeat visible on USBCANV2	10

7.2 Fault Injection Tests	11

IT-06 — Single-channel crash — only ECU1 above threshold, ECU2 below	11

IT-07 — CAN bus disconnected — peer timeout after 2000 ms	11

IT-08 — CAN reconnected after timeout — system recovers to MONITORING	12

IT-09 — Remote fault bit set — peer broadcasts fault in CAN byte 6	12

IT-10 — POST failure simulation — faultCPU forced TRUE on ECU1	13

7.3 Timing / Interface Latency Tests	13

IT-11 — Loop cycle time ≤ 1 ms	14

IT-12 — Crash detection response ≤ 20 ms	14

IT-13 — CAN heartbeat interval 500 ms ± 50 ms	14

IT-14 — faultCOMM declared ≤ 2000 ms after CAN loss	15

IT-15 — DWI activates within 800 ms of fault	15

8 HW/SW Integration Bring-Up Procedure	16

9 Coverage Targets	17

9.1 Interface Agreement / Disagreement Pairs	17

10 Test Evidence and Reporting	18

10.1 Required Evidence per Test Case	18

10.2 Test Report Structure	18

11 Compliance Gaps Affecting Integration Test Scope	18

12 Integration Requirement Traceability Matrix	20

13 Roles, Responsibilities, and Sign-Off	21

14 Revision History	22

# 1 Glossary

| Abbreviation | Definition |
| --- | --- |
| 2oo2 | Two-out-of-Two voting. |
| ADC | Analog-to-Digital Converter. |
| ASIL | Automotive Safety Integrity Level (QM, A, B, C, D) per ISO 26262. |
| BB | Black-Box test. |
| BOM | Bill of Materials. |
| CAN | Controller Area Network. |
| DLC | Data Length Code. |
| DWI | Diagnostic Warning Indicator. |
| ECU | Electronic Control Unit. |
| FSRS | Functional Safety Requirements Specification. |
| HADS | Hardware Architecture and Design Specification. |
| IT | Integration Test. |
| ITP | Integration Test Plan |
| MC/DC | Modified Condition/Decision Coverage. |
| NCR | Non-Conformance Report. |
| OMS PC | Laboratory monitoring PC |
| POST | Power-On Self-Test. |
| POTI | Potentiometer |
| SADS | Software Architecture and Design Specification. |
| SIL | Safety Integrity Level (1–4) per IEC 61508. |
| SWR | Software Requirement. |
| WB | White-Box test. |
| WDT | Watchdog Timer. |

# 2 Purpose and Scope

This Integration Test Plan (ITP) defines the test strategy, test environment, entry and exit criteria, test case catalogue, and requirement traceability for verifying the interfaces and interactions between the Airbag ECU software modules, between the two redundant Arduino Due (SAM3X8E) channels, and between the firmware and the surrounding hardware (sensors, AND-gate enforcement layer, CAN bus, and the OMS laboratory PC).

Integration testing sits between unit testing and validation testing in the V&V lifecycle. Unit-level correctness of individual modules (MOD-01 through MOD-04) is established separately in the Unit Test Plan, and system-level demonstration of the safety goals (SG-01 through SG-07) is established separately in the Validation Test Plan. This plan assumes unit testing has been completed successfully and focuses exclusively on:

- Module-to-module data flow on the assembled firmware — e.g., sensor acquisition feeding crash detection, crash detection feeding deployment control, diagnostic monitor feeding deployment control and the DWI output.

- Inter-channel (ECU1 ↔ ECU2) interaction over the shared CAN bus, including heartbeat exchange, status frame parsing, and communication-loss handling.

- Hardware/software interaction — ADC sampling, digital I/O, the DM74LS08N AND-gate 2oo2 enforcement layer, indicator LEDs, and the CAN transceiver interface to the OMS laboratory PC.

- Timing behaviour of the integrated system — loop cycle time, crash detection latency, CAN heartbeat interval, communication-loss detection, and DWI activation latency.

- The 12-step HW/SW Integration Bring-Up procedure that must be completed before any of the above interface tests are executed.

The plan covers the integration test cases IT-01 through IT-15 as specified in SADS-AIRBAG-ECU-001 Rev 3.0 Section 9, and the HW/SW integration bring-up procedure specified in SADS-AIRBAG-ECU-001 Rev 3.0 Section 9.1. Unit test cases and validation test cases are out of scope for this document. SADS-AIRBAG-ECU-001 Rev 3.0 records unit, integration, and validation testing, together with the code review checklist, as the responsibility of the Implementation Test Plan (ITP-AIRBAG-ECU-001) and the Test Report (TR-AIRBAG-ECU-001). This document, ITP-AIRBAG-ECU-001 (Integration Test Plan), is a focused companion document that extracts and elaborates only the integration-level test cases and bring-up procedure relevant to module-to-module, inter-channel, and HW/SW interface verification; it does not replace ITP-AIRBAG-ECU-001.

This plan must be read in conjunction with:

- SADS-AIRBAG-ECU-001 Rev 3.0 — Software Architecture Design Specification and Implementation

- SC- AIRBAG-ECU-001 Rev 3.1 — Safety Concept

- FSRS-AIRBAG-ECU-001 Rev 2.1 — Functional Safety Requirements Specification

- HADS-AIRBAG-ECU-001 Rev 2.0 Final — Hardware Architecture and Design Specification

- ITP-AIRBAG-ECU-001 Rev 3.0 — Implementation Test Plan (unit, integration, and validation test cases; code review checklist)

NOTE: Eight open compliance gaps (GAP-01 through GAP-08) exist in the current firmware, as documented in SADS-AIRBAG-ECU-001 Rev 3.0 Section 10. Section 10 of this plan documents which of those gaps affect the integration test cases in scope here, with a test-execution disposition for each. Gaps relevant only to unit-level or validation-level testing are noted but not dispositioned in this document.

# 3 Test Strategy

## 3.1 Position in the V&V Lifecycle

| Test Level | Scope | Document |
| --- | --- | --- |
| Unit Testing | Each firmware module (MOD-01 to MOD-04) verified in isolation on target hardware. | ITP-AIRBAG-ECU-001 (Implementation Test Plan) |
| Integration Testing | Module-to-module, inter-channel (ECU1/ECU2), and HW/SW interface verification on the assembled system, plus the bring-up procedure. | This document (ITP-AIRBAG-ECU-001) |
| Validation Testing | Safety goal (SG-01 to SG-07) demonstration at complete-system level. | ITP-AIRBAG-ECU-001 (Implementation Test Plan) |

## 3.2 Integration Test Level Overview

SADS-AIRBAG-ECU-001 Rev 3.0 Section 9 (HW/SW Integration Procedure) defines the 12-step bring-up procedure referenced in Section 7 of this plan. The integration test cases IT-01 through IT-15 below elaborate the module-to-module and inter-channel interfaces implied by the architecture in SADS Sections 4 through 8 (module decomposition, data flow, state machine, and safety mechanisms) and are grouped as follows:

| Sub-Level | Focus | SADS Reference | Test Case Range |
| --- | --- | --- | --- |
| HW/SW Integration Bring-Up | Flash, pin, CAN bus, and AND-gate verification — prerequisite for all interface tests below. | Section 9 / 9.1 | Bring-up Steps 1–12 |
| Normal Operation Interface Tests | Sensor → crash detection → deployment → AND gate data flow; ECU-to-ECU CAN heartbeat. | Sections 4.2, 6.2, 6.3, 6.5 | IT-01 to IT-05 |
| Fault Injection Interface Tests | Diagnostic monitor → deployment control interaction; CAN link-loss handling between channels; AND-gate single-channel rejection. | Sections 6.6, 8.2, 8.3 | IT-06 to IT-10 |
| Timing / Interface Latency Tests | Cross-module and cross-channel timing of the integrated system. | Sections 4.3, 5.2 | IT-11 to IT-15 |

## 3.3 Test Approach

All tests are executed on target hardware (two Arduino Due SAM3X8E boards) to avoid simulator discrepancies, per SADS Section 8.1. Black-box (BB) tests drive inputs (potentiometers, baby seat switch, CAN bus connection) and observe outputs (LEDs, CAN frames, Serial diagnostics) without reference to internal code structure. White-box (WB) tests additionally inspect internal state via Serial Monitor logs (e.g., fault flags, voteDeploy, remoteCrash) to confirm the correct internal interface values are being exchanged between modules and between channels.

The 2oo2 dual-channel architecture is the central interface under test: both ECU boards are operated simultaneously, exchanging status over the shared CAN bus, and the hardware AND gate enforces physical agreement between their D8/D9 outputs. The integration tests explicitly verify that the CAN interface correctly carries crash, fault, and baby-seat status between channels, and that a single-channel fault or single-channel crash detection cannot, by itself, fire any airbag LED.

# 4 Test Environment

## 4.1 Hardware Configuration

| Component | Specification | Role |
| --- | --- | --- |
| Arduino Due x 2 | SAM3X8E ARM Cortex-M3, 84 MHz | ECU 1 (ID=1, TX=0x100) and ECU 2 (ID=2, TX=0x200) |
| POTI-1 (Accelerometer) | 10 kΩ potentiometer on A0, 0–3.3 V → 0–100 g | Crash sensor simulation |
| POTI-2 (Gyroscope) | 10 kΩ potentiometer on A1, 0–3.3 V → 0–360 °/s | Rollover sensor simulation |
| POTI-3 (Pressure) | 10 kΩ potentiometer on A2, 0–3.3 V → 0–500 kPa | Side-impact sensor simulation |
| Baby Seat Switch | Momentary push switch on D11, 10 kΩ pull-up to 3.3 V, active-LOW | Occupant classification |
| DM74LS08N AND Gate | Quad 2-input AND, pins 1/2 → Airbag1 out, pins 4/5 → Airbag2 out | Hardware 2oo2 enforcement |
| Airbag1 LED (Red) | 3 mm RED LED + 220 Ω from AND gate pin 3 | Driver airbag deployment indicator |
| Airbag2 LED (Red) | 3 mm RED LED + 220 Ω from AND gate pin 6 | Passenger airbag deployment indicator |
| DWI LED (Red) | 3 mm LED + 220 Ω shared between D10 of ECU1 and ECU2 | Diagnostic Warning Indicator |
| Status LED (D13) | Built-in Arduino LED | Heartbeat / status blink |
| Waveshare SN65HVD230 x 2 | CAN transceiver 3.3 V compatible | CAN bus interface for each ECU |
| USB-CAN-A adapter | 500 kbit/s CAN to USB bridge | OMS PC CAN monitoring interface |

## 4.2 Software / Tooling

| Tool | Version | Role |
| --- | --- | --- |
| Arduino IDE | 2.x | Firmware compilation and upload |
| GCC ARM (avr-g++) | 4.8.3 (bundled) | Cross-compiler for SAM3X8E |
| due_can library | v1.0 (open source) | CAN0 hardware abstraction |
| USBCANV2 (OMS PC) | Latest stable | Passive CAN frame monitoring at 500 kbit/s |
| Serial Monitor | Bundled in Arduino IDE | 115200 baud diagnostic/Serial log capture |
| Git | Any | Firmware version control and release tagging |

## 4.3 Interfaces Under Test

Both ECU boards share the same POTI-1/2/3 analog lines and the same baby seat switch (D11) as defined in HADS-AIRBAG-ECU-001 Rev 2.0 Final. The following table summarises the discrete interfaces exercised by the integration test cases in Section 6, each of which corresponds to a data-flow link described in SADS Section 3.2.

| Interface | Endpoints | Carried Data | Exercised By |
| --- | --- | --- | --- |
| Sensor → Software (ADC) | POTI-1/2/3, D11 → MOD-02 (Sensor Acquisition) | rawA0, rawA1, rawA2, babySeatRaw | IT-01 to IT-04, IT-12 |
| MOD-02 → MOD-03 | Sensor Acquisition → Crash Detection | Scaled accel, gyro, press; localCrash | IT-01 to IT-04, IT-06, IT-12 |
| MOD-03/MOD-06 → MOD-04 | Crash Detection + Diagnostic Monitor → Deployment Control | localCrash, systemFault, babySeat | IT-01 to IT-04, IT-06, IT-09, IT-10 |
| MOD-05 (ECU1) ↔ MOD-05 (ECU2) | CAN0 TX/RX, shared CANH/CANL bus | remoteCrash, remoteFault, lastRemoteHB (status byte bits 0/1/2) | IT-05 to IT-09, IT-13, IT-14 |
| MOD-04 → AND Gate / LEDs | D8, D9, D10 outputs → DM74LS08N → Airbag1/Airbag2/DWI LEDs | voteDeploy, voteDeploy && !babySeat, systemFault | IT-01 to IT-04, IT-06, IT-09, IT-10, IT-15 |
| MOD-05 → OMS PC | CAN0 TX → USB-CAN-A → USBCANV2 | 8-byte status frame (rawA0/A1/A2, status byte, ECU_ID) | IT-05, IT-13 |

## 4.4 Baseline Configuration

Unless an individual test case specifies otherwise, the following baseline configuration applies for all integration tests in Section 6 (per SADS §9.1):

- Both ECUs powered, flashed with the build under test, and communicating (faultCOMM = 0 on both).

- All three potentiometers (POTI-1/2/3) set below their deployment thresholds (ADC < 1000) unless the test requires a crash condition.

- Baby seat switch open (D11 = HIGH, babySeat = FALSE).

- Serial Monitor open at 115200 baud on both ECUs; USBCANV2 running and logging CAN IDs 0x100 and 0x200.

- The shared CAN bus is intact and the USB-CAN-A adapter is in passive monitoring mode — it must not inject frames unless a test case specifies otherwise.

# 5 Entry and Exit Criteria

## 5.1 Entry Criteria

The following conditions must be satisfied before integration test execution begins:

| # | Entry Criterion | Evidence Required |
| --- | --- | --- |
| EC-01 | Firmware build corresponding to SADS-AIRBAG-ECU-001 Rev 3.0 complete and version-tagged in Git for both ECUs. | Git tags present for ECU1 and ECU2; confirmed in startup Serial banner (ECU 1 / ECU 2 STARTED). |
| EC-02 | Unit testing (per ITP-AIRBAG-ECU-001) completed with all results recorded. | ITP-AIRBAG-ECU-001 execution record / sign-off reference. |
| EC-03 | Code review checklist (per ITP-AIRBAG-ECU-001, referenced from SADS §11.1) completed with all items PASS. | Signed code review form, two-person review. |
| EC-04 | Test hardware assembled per HADS-AIRBAG-ECU-001 Rev 2.0 Final wiring tables; continuity check passed. | Continuity check sign-off. |
| EC-05 | Known gaps GAP-01 through GAP-08 (SADS §10) reviewed for relevance to integration test cases. | Gap disposition table in Section 10 of this ITP updated. |

## 5.2 HW/SW Integration Bring-Up as Gating Step

The 12-step HW/SW Integration Bring-Up procedure (Section 7 of this plan) is itself part of integration testing, but it additionally serves as an entry gate for the interface test cases IT-01 through IT-15: those tests shall not commence until all 12 bring-up steps have a recorded PASS and the Integration Sign-Off in Section 7 has been completed.

## 5.3 Exit Criteria

| # | Exit Criterion |
| --- | --- |
| EX-01 | All 12 HW/SW Integration Bring-Up steps executed; results recorded as PASS with sign-off completed. |
| EX-02 | All integration tests IT-01 through IT-15 executed; results recorded as PASS, FAIL, or WAIVED with justification. |
| EX-03 | Integration-level coverage targets in Section 8 met, or a documented shortfall and rationale recorded. |
| EX-04 | All FAIL results have an approved deviation report or corrective action plan. |
| EX-05 | Test evidence archived per Section 9: Serial logs, USBCANV2 logs, LED photos/video, operator/date/firmware version for every executed test case. |
| EX-06 | Integration-relevant gap dispositions in Section 11 reviewed and accepted. |
| EX-07 | V&V lead and functional safety engineer sign-off obtained (Section 12). |

Completion of this plan's exit criteria is a precondition for the Validation Test Plan (VT-01 to VT-07) to commence, since validation testing assumes that the integrated system's module-to-module, channel-to-channel, and HW/SW interfaces have already been verified.

# 6 Integration Test Case Format

Each integration test case below is documented using a common field structure: Test ID, Test Type (BB = black-box, WB = white-box, BB+WB = combined), Test Name, the interface(s) under test, FSR trace, ASIL, preconditions, test procedure, expected result, pass criterion, evidence required, and a Result/Notes field to be completed during execution. The ‘Interface Under Test’ field, specific to this Integration Test Plan, identifies which data-flow link from Section 3.3 is being exercised.

# 7 Integration Test Cases

## 7.1 Normal Operation Tests

These tests verify the primary data-flow path — sensors through crash detection and deployment control to the AND-gate outputs — and the basic ECU1↔ECU2 CAN heartbeat interface, under fault-free conditions.

### IT-01 — No-crash baseline — all sensors below threshold

| Test ID | IT-01 |
| --- | --- |
| Test Type | BB (Black-Box) |
| Test Name | No-crash baseline — all sensors below threshold |
| Interface Under Test | Sensor → MOD-02 → MOD-03 → MOD-04 → AND Gate / LEDs (Section 3.3) |
| FSR Trace | FSR-04 |
| ASIL | ASIL D |
| Preconditions | Full assembled system: both ECUs, CAN bus, AND gate, LEDs. USBCANV2 running. Serial Monitor open on both ECUs. Baseline configuration per Section 3.4. |
| Test Procedure | Both ECUs powered and communicating. Set all POTIs well below threshold (ADC < 1000 on all channels). Baby seat switch OPEN. Run for minimum 5 minutes. |
| Expected Result | Both Airbag LEDs remain OFF for entire 5-minute run. DWI OFF. Serial: localCrash=0, voteDeploy=0 on both ECUs continuously. |
| Pass Criterion | Zero false deployments; Airbag LEDs OFF for ≥ 5 minutes without interruption. |
| Evidence Required | Serial logs from both ECUs (timestamped). USBCANV2 CAN frame log. LED state photograph or video. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-02 — Full dual-channel crash — both ECUs above threshold simultaneously

| Test ID | IT-02 |
| --- | --- |
| Test Type | BB (Black-Box) |
| Test Name | Full dual-channel crash — both ECUs above threshold simultaneously |
| Interface Under Test | MOD-03 → MOD-04 → AND Gate (2oo2 agreement); MOD-05 ECU1 ↔ ECU2 status exchange |
| FSR Trace | FSR-01, FSR-03 |
| ASIL | ASIL D |
| Preconditions | Full assembled system: both ECUs, CAN bus, AND gate, LEDs. USBCANV2 running. Serial Monitor open on both ECUs. Baseline configuration per Section 3.4. |
| Test Procedure | Both ECUs communicating. Simultaneously advance POTI-1 above threshold on both ECUs (ADC > 1228). Observe LEDs and Serial on both ECUs. Monitor USBCANV2 for crash bit in frames 0x100 and 0x200. |
| Expected Result | Airbag1 LED fires ON. Serial: voteDeploy=1 on both ECUs. USBCANV2 shows bit 0 of byte 6 = 1 in both 0x100 and 0x200 frames. |
| Pass Criterion | Airbag1 LED ON within observed cycle; USBCANV2 confirms crash bit in both CAN frames. |
| Evidence Required | Serial logs from both ECUs (timestamped). USBCANV2 CAN frame log. LED state photograph or video. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-03 — Crash ceases — sensors returned below threshold

| Test ID | IT-03 |
| --- | --- |
| Test Type | BB (Black-Box) |
| Test Name | Crash ceases — sensors returned below threshold |
| Interface Under Test | MOD-03 → MOD-04 → AND Gate / LEDs — verifies the interface returns to its rest state without latching |
| FSR Trace | FSR-04 |
| ASIL | ASIL D |
| Preconditions | Full assembled system: both ECUs, CAN bus, AND gate, LEDs. USBCANV2 running. Serial Monitor open on both ECUs. Baseline configuration per Section 3.4. |
| Test Procedure | Following IT-02, return POTI-1 below threshold on both ECUs. |
| Expected Result | Airbag1 and Airbag2 LEDs extinguish within one loop cycle of POTI-1 dropping below threshold. Serial: localCrash=0, voteDeploy=0. |
| Pass Criterion | LEDs extinguish within one cycle of sensor drop below threshold. |
| Evidence Required | Serial logs from both ECUs (timestamped). USBCANV2 CAN frame log. LED state photograph or video. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-04 — Baby seat suppression during crash

| Test ID | IT-04 |
| --- | --- |
| Test Type | BB (Black-Box) |
| Test Name | Baby seat suppression during crash |
| Interface Under Test | Baby seat switch (D11) → MOD-02 → MOD-04 → D9 (Airbag2) — verifies the babySeat interface independently suppresses only the passenger channel |
| FSR Trace | FSR-09 |
| ASIL | ASIL D |
| Preconditions | Full assembled system: both ECUs, CAN bus, AND gate, LEDs. USBCANV2 running. Serial Monitor open on both ECUs. Baseline configuration per Section 3.4. |
| Test Procedure | Both ECUs above threshold (crash condition). Press and hold baby seat switch (D11 LOW). Observe D8 and D9. |
| Expected Result | Airbag1 LED ON (D8 HIGH on both ECUs). Airbag2 LED OFF (D9 LOW on both ECUs). Serial: babySeat=1. |
| Pass Criterion | Airbag2 LED stays OFF; Airbag1 LED remains ON during baby seat + crash condition. |
| Evidence Required | Serial logs from both ECUs (timestamped). USBCANV2 CAN frame log. LED state photograph or video. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-05 — CAN heartbeat visible on USBCANV2

| Test ID | IT-05 |
| --- | --- |
| Test Type | BB (Black-Box) |
| Test Name | CAN heartbeat visible on USBCANV2 |
| Interface Under Test | MOD-05 → OMS PC (CAN0 TX → USB-CAN-A → USBCANV2) — verifies the ECU-to-OMS monitoring interface |
| FSR Trace | FSR-16 |
| ASIL | ASIL D |
| Preconditions | Full assembled system: both ECUs, CAN bus, AND gate, LEDs. USBCANV2 running. Serial Monitor open on both ECUs. Baseline configuration per Section 3.4. |
| Test Procedure | Both ECUs powered and communicating normally. Open USBCANV2 at 500 kbit/s. Record frame timestamps for CAN IDs 0x100 and 0x200 over 20 consecutive frames each. |
| Expected Result | Frames for both 0x100 and 0x200 arrive at ~500 ms intervals. All inter-frame gaps within 450–550 ms. |
| Pass Criterion | All 20 intervals for both CAN IDs within 450–550 ms. |
| Evidence Required | Serial logs from both ECUs (timestamped). USBCANV2 CAN frame log. LED state photograph or video. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

## 7.2 Fault Injection Tests

These tests verify the diagnostic-monitor-to-deployment-control interface, the inter-channel fault-status exchange over CAN, and the AND-gate's hardware enforcement of channel agreement when one side of an interface reports a fault or an out-of-agreement crash state.

### IT-06 — Single-channel crash — only ECU1 above threshold, ECU2 below

| Test ID | IT-06 |
| --- | --- |
| Test Type | BB |
| Test Name | Single-channel crash — only ECU1 above threshold, ECU2 below |
| Interface Under Test | MOD-04 (ECU1) → D8 → AND Gate ← D8 MOD-04 (ECU2) — verifies the AND gate's 2oo2 interface rejects a single-channel assertion; MOD-05 ECU1 ↔ ECU2 remoteCrash exchange |
| FSR Trace | FSR-03, FSR-08 |
| ASIL | ASIL D |
| Preconditions | Full assembled system. USBCANV2 running. Serial Monitor on both ECUs. System in ST-03 MONITORING state unless stated. |
| Test Procedure | Set POTI-1 above threshold on ECU1 only (ADC > 1228). Keep all POTIs on ECU2 below threshold. Observe all LEDs and Serial remoteCrash on ECU1. |
| Expected Result | Both Airbag LEDs OFF. ECU1 Serial: localCrash=1, remoteCrash=0 (ECU2 not detecting crash), voteDeploy=0. AND gate cannot assert because ECU2 D8 is LOW. |
| Pass Criterion | No airbag LED fires when only one ECU detects a crash; AND gate hardware prevents single-channel deployment. |
| Evidence Required | Serial logs from both ECUs. USBCANV2 log. LED photograph. Timing measurement where applicable. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-07 — CAN bus disconnected — peer timeout after 2000 ms

| Test ID | IT-07 |
| --- | --- |
| Test Type | BB |
| Test Name | CAN bus disconnected — peer timeout after 2000 ms |
| Interface Under Test | MOD-05 ECU1 ↔ ECU2 CAN link (heartbeat/timeout) → MOD-06 (faultCOMM) → MOD-04 / DWI — verifies loss of the inter-channel interface is correctly propagated to the deployment and warning outputs |
| FSR Trace | FSR-15, FSR-20 |
| ASIL | ASIL D |
| Preconditions | Full assembled system. USBCANV2 running. Serial Monitor on both ECUs. System in ST-03 MONITORING state unless stated. |
| Test Procedure | Both ECUs in normal monitoring state (POTIs below threshold). Physically disconnect the CAN bus (remove CANH or CANL wire). Start timing from disconnection. Observe DWI LED and Serial faultCOMM flag. |
| Expected Result | After 2000 ms without a valid peer CAN frame: faultCOMM=TRUE on both ECUs, DWI LED ON, D8=LOW, D9=LOW. Serial shows systemFault=1. |
| Pass Criterion | DWI ON within 2000 ms of CAN disconnection; airbag LEDs remain OFF. |
| Evidence Required | Serial logs from both ECUs. USBCANV2 log. LED photograph. Timing measurement where applicable. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-08 — CAN reconnected after timeout — system recovers to MONITORING

| Test ID | IT-08 |
| --- | --- |
| Test Type | BB+WB |
| Test Name | CAN reconnected after timeout — system recovers to MONITORING |
| Interface Under Test | MOD-05 ECU1 ↔ ECU2 CAN link — verifies the inter-channel interface re-establishes cleanly and faultCOMM clears without manual intervention |
| FSR Trace | FSR-15 |
| ASIL | ASIL D |
| Preconditions | Full assembled system. USBCANV2 running. Serial Monitor on both ECUs. System in ST-03 MONITORING state unless stated. |
| Test Procedure | Following IT-07, reconnect CAN bus. Observe faultCOMM and DWI LED after next valid peer CAN frame is received. |
| Expected Result | faultCOMM clears to FALSE within one CAN frame period (~500 ms). DWI LED OFF. System returns to ST-03 MONITORING. Serial: faultCOMM=0. |
| Pass Criterion | System recovery within one CAN heartbeat period after CAN reconnection; DWI extinguishes. |
| Evidence Required | Serial logs from both ECUs. USBCANV2 log. LED photograph. Timing measurement where applicable. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-09 — Remote fault bit set — peer broadcasts fault in CAN byte 6

| Test ID | IT-09 |
| --- | --- |
| Test Type | WB |
| Test Name | Remote fault bit set — peer broadcasts fault in CAN byte 6 |
| Interface Under Test | MOD-05 (ECU2) → CAN status byte bit 2 → MOD-05 (ECU1) remoteFault → MOD-04 — verifies the remote-fault field of the CAN status interface correctly inhibits deployment on the receiving channel |
| FSR Trace | FSR-03 |
| ASIL | ASIL D |
| Preconditions | Full assembled system. USBCANV2 running. Serial Monitor on both ECUs. System in ST-03 MONITORING state unless stated. |
| Test Procedure | Force ECU2 to set bit 2 of its CAN status byte (systemFault=TRUE broadcast). Simultaneously set all sensors above threshold on both ECUs. Observe voteDeploy on ECU1. |
| Expected Result | ECU1: remoteFault=TRUE received; voteDeploy=0; D8=LOW, D9=LOW even with both sensors above threshold. Airbag LEDs OFF. |
| Pass Criterion | Deployment inhibited when peer ECU reports fault, regardless of sensor state. |
| Evidence Required | Serial logs from both ECUs. USBCANV2 log. LED photograph. Timing measurement where applicable. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-10 — POST failure simulation — faultCPU forced TRUE on ECU1

| Test ID | IT-10 |
| --- | --- |
| Test Type | BB |
| Test Name | POST failure simulation — faultCPU forced TRUE on ECU1 |
| Interface Under Test | MOD-01 (POST, ECU1) → MOD-06 (systemFault) → MOD-04 → D8 → AND Gate — verifies a startup-fault interface on one channel propagates through to the shared 2oo2 hardware interface and prevents the peer's otherwise-valid crash signal from firing the LED |
| FSR Trace | FSR-17, FSR-20 |
| ASIL | ASIL D |
| Preconditions | Full assembled system. USBCANV2 running. Serial Monitor on both ECUs. System in ST-03 MONITORING state unless stated. |
| Test Procedure | Build ECU1 firmware with cpuTest() returning FALSE. Flash. Power-cycle. Set all POTIs above threshold on both ECUs. Observe for entire session. |
| Expected Result | DWI ON from startup. voteDeploy=0 on ECU1. D8 on ECU1 remains LOW. Since AND gate requires both D8 HIGH simultaneously, Airbag1 LED stays OFF even with ECU2 in full crash state. |
| Pass Criterion | No airbag LED fires throughout entire session when ECU1 has POST fault; DWI ON from startup. |
| Evidence Required | Serial logs from both ECUs. USBCANV2 log. LED photograph. Timing measurement where applicable. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

## 7.3 Timing / Interface Latency Tests

These tests measure the timing behaviour of the integrated cross-module and cross-channel interfaces — the loop cycle that ties MOD-02 through MOD-06 together, the sensor-to-output latency through the crash-detection-to-deployment interface, the CAN heartbeat interface timing, the CAN-loss detection latency, and the fault-to-DWI interface latency.

### IT-11 — Loop cycle time ≤ 1 ms

| Test ID | IT-11 |
| --- | --- |
| Test Name | Loop cycle time ≤ 1 ms |
| Interface Under Test | Cyclic executive — aggregate timing of the MOD-02 → MOD-03 → MOD-06 → MOD-04 → MOD-05 interface chain within one loop() iteration |
| FSR Trace | FSR-02 |
| ASIL | ASIL D |
| Preconditions | Full assembled system in MONITORING state. Timing-instrumented firmware build where stated. |
| Test Procedure | Add micros() timestamps at start and end of loop() in a timing-instrumented build. Log via Serial over 10,000 consecutive iterations. |
| Measurement Method | Record maximum, mean, and 99th percentile cycle time from the 10,000-sample log. |
| Pass Criterion | 99th percentile cycle time ≤ 1000 µs (1 ms). |
| Evidence Required | Serial timing log with sample count. USBCANV2 timestamp log. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-12 — Crash detection response ≤ 20 ms

| Test ID | IT-12 |
| --- | --- |
| Test Name | Crash detection response ≤ 20 ms |
| Interface Under Test | Sensor (POTI-1) → MOD-02 → MOD-03 (localCrash) interface latency |
| FSR Trace | FSR-01 |
| ASIL | ASIL D |
| Preconditions | Full assembled system in MONITORING state. Timing-instrumented firmware build where stated. |
| Test Procedure | Log millis() immediately before POTI-1 is moved above threshold and immediately when localCrash=TRUE appears in Serial output. Repeat 10 trials. |
| Measurement Method | Delta between threshold crossing and localCrash=TRUE logged for each trial. |
| Pass Criterion | All 10 deltas ≤ 20 ms. |
| Evidence Required | Serial timing log with sample count. USBCANV2 timestamp log. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-13 — CAN heartbeat interval 500 ms ± 50 ms

| Test ID | IT-13 |
| --- | --- |
| Test Name | CAN heartbeat interval 500 ms ± 50 ms |
| Interface Under Test | MOD-05 → OMS PC CAN interface — transmit timing of the periodic status frame |
| FSR Trace | FSR-16 |
| ASIL | ASIL D |
| Preconditions | Full assembled system in MONITORING state. Timing-instrumented firmware build where stated. |
| Test Procedure | Log USBCANV2 frame timestamps for 20 consecutive frames from each ECU (0x100 and 0x200). |
| Measurement Method | Calculate inter-frame intervals from USBCANV2 timestamps. |
| Pass Criterion | All intervals within 450–550 ms for both CAN IDs. |
| Evidence Required | Serial timing log with sample count. USBCANV2 timestamp log. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-14 — faultCOMM declared ≤ 2000 ms after CAN loss

| Test ID | IT-14 |
| --- | --- |
| Test Name | faultCOMM declared ≤ 2000 ms after CAN loss |
| Interface Under Test | MOD-05 ECU1 ↔ ECU2 CAN link — timeout-detection latency of the inter-channel heartbeat interface |
| FSR Trace | FSR-15 |
| ASIL | ASIL D |
| Preconditions | Full assembled system in MONITORING state. Timing-instrumented firmware build where stated. |
| Test Procedure | Log millis() at last received valid CAN frame before disconnecting CAN. Log millis() when faultCOMM=TRUE appears in Serial. Calculate delta. |
| Measurement Method | Delta = time from last CAN frame to faultCOMM=TRUE in Serial log. |
| Pass Criterion | faultCOMM declared ≤ 2050 ms after last valid peer frame (NOTE: GAP-08 — FSR-15 requires ≤50 ms; current implementation uses 2000 ms; documented deviation, see Section 10). |
| Evidence Required | Serial timing log with sample count. USBCANV2 timestamp log. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

### IT-15 — DWI activates within 800 ms of fault

| Test ID | IT-15 |
| --- | --- |
| Test Name | DWI activates within 800 ms of fault |
| Interface Under Test | MOD-06 (fault flags) → MOD-04 → D10 (DWI) — latency of the fault-to-warning-indicator interface |
| FSR Trace | FSR-19 |
| ASIL | ASIL D |
| Preconditions | Full assembled system in MONITORING state. Timing-instrumented firmware build where stated. |
| Test Procedure | Inject fault at known millis() timestamp (disconnect CAN or force faultCPU). Log millis() at D10=HIGH transition in Serial. Calculate delta. |
| Measurement Method | Delta from fault injection to D10 HIGH in Serial log. |
| Pass Criterion | D10 HIGH within 800 ms of fault; per design D10 fires within ~1 ms (one cycle), well within FSR-19 limit. |
| Evidence Required | Serial timing log with sample count. USBCANV2 timestamp log. Operator name, date, firmware version. |
| Result (Pass/Fail) | Pass |

# 8 HW/SW Integration Bring-Up Procedure

The 12-step bring-up procedure from SADS-AIRBAG-ECU-001 Rev 3.0 Section 9.1 verifies the physical and logical interfaces — power rails, firmware flashing, CAN bus bring-up, and the AND-gate hardware interface — before the interface test cases in Section 6 are executed. All steps must show PASS, per the entry criteria in Section 4.2.

| Step | Action | Acceptance Criterion | Owner | Result |
| --- | --- | --- | --- | --- |
| 1 | Assemble hardware per HADS wiring tables. Verify breadboard wiring, POTIs, baby seat switch, CAN transceivers, AND gate, LEDs. | All wiring verified against HADS Tables 5–14. No short circuits. Continuity on all signal lines. | HW Engineer | Pass |
| 2 | Verify power rails: 5 V rail to AND gate VCC; 3.3 V rail to CAN transceivers; GND common to all. | 5 V ± 0.25 V at AND gate pin 14. 3.3 V ± 0.15 V at Waveshare VCC. GND continuity confirmed. | HW Engineer | Pass |
| 3 | Flash ECU 1 firmware (ECU_ID=1, DATA_ID=0x100, REMOTE_ID=0x200) via Arduino IDE. | Upload completes 'Done uploading'. Serial: 'ECU 1 STARTED' at 115200 baud. | SW Engineer | Pass |
| 4 | Flash ECU 2 firmware (ECU_ID=2, DATA_ID=0x200, REMOTE_ID=0x100) via Arduino IDE. | Upload completes. Serial: 'ECU 2 STARTED' at 115200 baud. | SW Engineer | Pass |
| 5 | Verify POST results on both ECUs via Serial Monitor. | Serial: faultCPU=0, faultRAM=0, faultADC=0. D13 blinking at ~500 ms on both ECUs. | SW Engineer | Pass |
| 6 | Verify CAN bus bring-up — connect OMS PC via USB-CAN-A, open USBCANV2 at 500 kbit/s. | CAN frames with ID 0x100 and 0x200 visible at ~500 ms intervals in USBCANV2 log. | HW/SW Engineer | Pass |
| 7 | Verify CAN communication between ECUs — check faultCOMM=0 on both ECUs. | Serial: faultCOMM=0 on both ECUs. Remote crash and fault bits correctly parsed. | SW Engineer | Pass |
| 8 | Verify pin output levels at rest — all sensors below threshold. | D8=LOW, D9=LOW, D10=LOW, D13 blinking. Airbag LEDs OFF. DWI OFF. | HW Engineer | Pass |
| 9 | Verify AND gate logic — set POTI-1 on ECU1 only above threshold. | Airbag1 LED stays OFF. AND gate pin 3 LOW. Only ECU1 asserting D8. | HW Engineer | Pass |
| 10 | Verify full crash scenario — set both ECU POTI-1 above threshold simultaneously. | Airbag1 LED fires (AND gate pin 3 HIGH). Serial: voteDeploy=1 on both ECUs. | HW/SW Engineer | Pass |
| 11 | Verify baby seat suppression — press baby seat switch during step 10 crash. | Airbag1 LED ON; Airbag2 LED OFF. D9 LOW on both ECUs. | HW/SW Engineer | Pass |
| 12 | Sign off HW/SW integration — record results and release to interface testing (Section 6). | All steps PASS. Integration sign-off form completed. | Project Lead | Pass |

# 9 Coverage Targets

Coverage at the integration level is defined in terms of interfaces and cross-module/crosschannel interaction paths exercised, rather than the statement- and branch-level MC/DC coverage applicable to unit testing (which is addressed in the Unit Test Plan). The following targets apply to this Integration Test Plan.

| Coverage Type | Target | Method | Test Cases |
| --- | --- | --- | --- |
| Data-Flow Interface Coverage | Every interface listed in Section 3.3 exercised by at least one passing integration test case. | Cross-check each row of the Section 3.3 interface table against the ‘Exercised By’ column. | IT-01 to IT-15 |
| Inter-Channel (2oo2) Coverage | Both the agreement path (both channels assert) and the disagreement path (single channel asserts, or one channel faulted) exercised. | Dedicated agreement/disagreement test pairs. | IT-02/IT-06, IT-09/IT-02, IT-10 |
| CAN Link Coverage | Normal heartbeat, link loss, and link recovery all exercised on the inter-channel CAN interface. | Sequential normal → loss → recovery test sequence. | IT-05, IT-07, IT-08 |
| Timing Interface Coverage | Every timing requirement associated with a cross-module or cross-channel interface (FSR-02, FSR-01, FSR-16, FSR-15, FSR-19) measured at least once. | Instrumented timing builds with Serial/USBCANV2 timestamp logging. | IT-11 to IT-15 |
| HW/SW Interface Coverage | Power, flashing, CAN bus bring-up, pin-level rest state, and AND-gate logic all verified before interface testing begins. | 12-step bring-up procedure, all steps PASS. | Bring-up Steps 1–12 |

## 9.1 Interface Agreement / Disagreement Pairs

The following table cross-references the dual-channel (2oo2) interface conditions that must each be exercised by at least one integration test case, ensuring both the agreement path and every disagreement path through the AND-gate interface are covered.

| 2oo2 Interface Condition | Agreement / Disagreement | Covered By |
| --- | --- | --- |
| Both channels assert D8 (crash confirmed on both) | Agreement — LED fires | IT-02 |
| Only ECU1 asserts D8 (ECU2 below threshold) | Disagreement — AND gate blocks | IT-06 |
| ECU1 D8 forced LOW by local POST fault, ECU2 in full crash | Disagreement — AND gate blocks | IT-10 |
| ECU1 sensors above threshold but remoteFault=TRUE received from ECU2 | Disagreement — AND gate blocks via inhibited vote | IT-09 |
| Both channels report crash, baby seat asserted — D8 agree, D9 individually suppressed | Agreement on D8, independent suppression on D9 | IT-04 |
| CAN link lost → faultCOMM on both channels → both D8/D9 forced LOW | Disagreement avoided by mutual inhibit (fail-safe) | IT-07 |

# 10 Test Evidence and Reporting

## 10.1 Required Evidence per Test Case

The following evidence must be collected and archived for every integration test case executed:

| Evidence Item | Format | Storage Location |
| --- | --- | --- |
| Serial Monitor log (both ECUs) | Timestamped .txt file (115200 baud capture) | Test archive — date-stamped folder per test session |
| USBCANV2 CAN frame log | Exported .csv or .log from USBCANV2 | Test archive — same folder as Serial logs |
| LED state photograph or video | .jpg / .mp4 capturing Airbag1, Airbag2, DWI LED states at key moments | Test archive — labelled with test case ID |
| Firmware version record | From startup Serial banner: firmware revision, ECU ID, build date/time (__DATE__, __TIME__) | Included in Serial log and test report header |
| Test operator record | Operator name, date, hardware revision, pass/fail determination | Test report sign-off sheet |

## 10.2 Test Report Structure

A test report shall be produced at the completion of integration test execution. The report shall include:

- Document title, document number, revision, date, and firmware version tested.

- Summary table of all integration test cases: Test ID, name, result (Pass/Fail), and any deviations.

- HW/SW Integration Bring-Up sign-off record (Section 7) attached or referenced.

- For each FAIL: defect description, root cause (if known), corrective action plan or deviation approval reference.

- Coverage summary against the targets in Section 8.

- Test evidence log listing all archived files with their paths and checksums.

- Sign-off table completed by test executor, reviewer (independent), V&V lead, and functional safety engineer.

# 11 Compliance Gaps Affecting Integration Test Scope

The following gaps between the current firmware and FSRS requirements have been identified in SADS-AIRBAG-ECU-001 Rev 3.0 Section 10 (Compliance Gaps and Required Improvements). Only gaps that affect an interface exercised by an integration test case (IT-01 to IT-15) or the HW/SW bring-up procedure are dispositioned below. Gaps relevant only to unit-level or system-level (validation) testing — GAP-02, GAP-07 — are noted for completeness but carry no integration-test disposition; their required actions are tracked in ITP-AIRBAG-ECU-001.

| Gap ID | FSR | Description | Affected Integration Interface | Test Plan Disposition |
| --- | --- | --- | --- | --- |
| GAP-01 | FSR-11 | Baby seat debounce not implemented — D11 read directly without 10 ms stability filter. | Baby seat switch (D11) → MOD-02 → MOD-04 interface (IT-04) | IT-04 executes as-is. Switch bounce may cause intermittent false suppression during crash vibration. Deviation accepted for lab integration testing; closure required before production. Test marked with deviation flag. |
| GAP-02 | FSR-12 | Baby seat line continuity monitoring not implemented — open-circuit stuck-HIGH not detected. | Not directly exercised by an IT-XX case | No integration test case defined for this unimplemented function. Out of scope for this ITP — required action (periodic D11 continuity test) tracked in ITP-AIRBAG-ECU-001 / SADS §10 (GAP-02). |
| GAP-03 | FSR-13 | CRC not included in CAN frame — bytes 6–7 carry status and ECU_ID, not a 16-bit CRC. | MOD-05 → OMS PC CAN interface (IT-05, IT-13) | IT-05 and IT-13 proceed without CRC verification; USBCANV2 log used to verify correct byte content directly. GAP-03 closure required before full SG-07 validation. |
| GAP-04 | FSR-14 | Parallel CAN bitstream readback not implemented. | MOD-05 → OMS PC CAN interface | No integration test case defined for this unimplemented feature. Deferred — the AND-gate hardware interface provides the second-layer protection exercised instead (IT-02, IT-06). |
| GAP-05 | FSR-17 | CAN communication not verified during POST — Can0.begin() return value not checked. | MOD-01 (POST) → MOD-05 CAN interface (Bring-Up Step 5/6) | Bring-up steps 5 and 6 proceed without an explicit POST-time CAN init check. Functional CAN interface verification is provided by IT-05 / IT-06 after bring-up. Deviation noted. |
| GAP-06 | FSR-22 | Hardware WDT not enabled in firmware. | Cyclic executive interface chain (IT-11, IT-14, IT-15) | WDT absence means a software lockup during the timing tests would not be caught by hardware. IT-11, IT-14, and IT-15 proceed as specified. WDT implementation required before validation sign-off; flagged as a residual risk for this integration cycle. |
| GAP-07 | FSR-21 | Fault logging to DTC/non-volatile memory not implemented. | Not directly exercised by an IT-XX case | No integration test case for EEPROM/flash DTC storage. Out of scope for this ITP — Serial Monitor logs serve as the fault-evidence interface for all integration tests in Section 6. |
| GAP-08 | FSR-15 | CAN timeout threshold is 2000 ms vs FSR-15 requirement of ≤50 ms. | MOD-05 ECU1 ↔ ECU2 CAN link timeout interface (IT-07, IT-14) | IT-07 and IT-14 test at the 2000 ms threshold actually implemented. IT-14 pass criterion states ≤ 2050 ms with an explicit note that FSR-15 requires ≤50 ms. Safety rationale: a 2000 ms timeout does not create a deployment risk since CAN loss causes inhibit (fail-safe direction). Formal deviation justification required at validation level. |

# 12 Integration Requirement Traceability Matrix

This matrix lists the FSRs that are verified, in whole or in part, by an integration test case (IT-01 to IT-15) or the HW/SW Integration Bring-Up procedure, together with the implementing Software Requirement (SWR) from SADS-AIRBAG-ECU-001 Rev 3.0 Section 12. FSRs whose verification relies solely on unit testing or validation testing (per ITP-AIRBAG-ECU-001) are not repeated here. Where an FSR is only partially covered at integration level (the remainder being covered at unit or validation level), this is noted in the Coverage Note column.

| FSR ID | SWR(s) | FSR Title | ASIL | Integration Interface (Section 3.3) | Integration Test Case(s) | Coverage Note |
| --- | --- | --- | --- | --- | --- | --- |
| FSR-01 | SWR-04, SWR-05 | Crash Detection Timing (≤20 ms) | ASIL D | Sensor → MOD-02 → MOD-03 | IT-02, IT-12 | Threshold logic itself unit-tested; latency verified here. |
| FSR-02 | SWR-11 | Algorithm Cycle Time (≤1 ms) | ASIL D | Full cyclic executive interface chain | IT-11 | Integration-only requirement — measured on assembled system. |
| FSR-03 | SWR-06, SWR-10, SWR-12 | Deployment Authorization Logic | ASIL D | MOD-03/MOD-06 → MOD-04 → AND Gate | IT-02, IT-04, IT-06, IT-09 | Vote logic unit-tested; cross-channel agreement verified here. |
| FSR-04 | SWR-06 | Prevention of Unintended Deployment | ASIL D | MOD-03 → MOD-04 → AND Gate / LEDs | IT-01, IT-03, IT-06 | — |
| FSR-07 | SWR-17, SWR-18, SWR-19 | Dual-Channel Redundancy | ASIL D | MOD-05 ECU1 ↔ ECU2 | IT-06 | Full redundancy demonstration at validation level (ITP-AIRBAG-ECU-001). |
| FSR-08 | SWR-12 | Channel Agreement Verification | ASIL D | MOD-04 → D8/D9 → AND Gate | IT-06 | — |
| FSR-09 | SWR-07, SWR-13 | Passenger Airbag Suppression | ASIL D | Baby seat switch → MOD-02 → MOD-04 → D9 | IT-04 | Switch-level logic unit-tested; cross-module suppression verified here. |
| FSR-15 | SWR-18 | CAN Timeout Detection (≤50 ms) | ASIL B | MOD-05 ECU1 ↔ ECU2 link | IT-07, IT-14 | GAP-08 — tested at 2000 ms implemented threshold (see Section 10). |
| FSR-16 | SWR-21, SWR-22 | Crash Notification Transmission | ASIL B | MOD-05 → OMS PC | IT-05, IT-13 | GAP-03 — frame transmitted without 16-bit CRC; content/timing verified here, full CRC verification per ITP-AIRBAG-ECU-001. |
| FSR-17 | SWR-01, SWR-02 | Power-On Self-Test | ASIL C | MOD-01 → MOD-05/MOD-06 interface (Bring-up, IT-10) | Bring-up Step 5, IT-10 | POST internal logic unit-tested per ITP-AIRBAG-ECU-001; interface to deployment/CAN verified here. GAP-05 — CAN init not checked during POST. |
| FSR-19 | SWR-22, SWR-23 | DWI Activation (≤800 ms) | ASIL A | MOD-06 → MOD-04 → D10 | IT-15 | — |
| FSR-20 | SWR-14 | Safe-State Transition | ASIL D | MOD-06 → MOD-04 → D8/D9/D10 | IT-07, IT-10 | — |

Note: SADS-AIRBAG-ECU-001 Rev 3.0 Section 12 lists SWR-22 twice (once for CAN TX, once for DWI on local fault) — both instances are referenced above against FSR-16 and FSR-19 respectively, as they appear in the source traceability matrix.

FSR-10, FSR-11, FSR-12, FSR-13, FSR-14, FSR-18, FSR-21, and FSR-22 are addressed in ITP-AIRBAG-ECU-001 (unit and validation test cases) and are referenced in this document only through the gap dispositions in Section 10 where they affect an integration interface (GAP-01, GAP-03, GAP-04, GAP-05, GAP-06, GAP-08).

# 13 Roles, Responsibilities, and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Software Safety Engineer | Mariana Chavez Avila |  |  |
| Hardware Safety Engineer | Om Naik |  |  |
| Functional Safety Engineer | Kruthika S. Gowda |  |  |
| V&V Lead | Shivbaraje Bhosale |  |  |
| Project Supervisor |  |  |  |

# 14 Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-06-12 | Atharva Banarase | Initial release. IT-01 to IT-15. 12-step bring-up. GAP-01 to GAP-08 documented. |
| 2.0 | 2026-06-13 | Atharva Banarase | Firmware baseline updated to Rev 3.3. GAP-01 CLOSED (debounce). GAP-03 CLOSED (HW CRC). GAP-06 CLOSED (WDT). IT-16 WDT test added. LED colours corrected to all RED 3mm. Every test case expanded with numbered steps, evidence file naming, and screenshot/photo requirements. |
| 3.0 | 2026-06-13 | Atharva Banarase | Merged improvements from ITP-AIRBAG-ECU-003 (team member document). Added: Section 3 Test Strategy (V&V lifecycle, integration level overview, test approach). Added: Section 4.3 Interfaces Under Test table (all data-flow links with endpoints and exercised-by). Added: Interface Under Test field to every test case. Added: Section 8.2 2oo2 Agreement/Disagreement pairs table. Added: Measurement Method field to timing test cases IT-11 to IT-15. All Rev 2.0 content (gap statuses, IT-16, numbered steps, evidence naming) preserved unchanged. |