Airbag ECU Portfolio HS Harz — Functional Safety VERIFICATION AND VALIDATION PLAN Measurement and Processing of Sensor Signals to Control Airbag Operation

ASIL D / SIL 3

| Document Number | VVP-AIRBAG-ECU-001 |
| --- | --- |
| Version | 1.3 |
| Date | 2026-06-14 |
| Status | RELEASE |
| Editor | Shivbaraje Bhosale, Mrunal Ghotekar, Mariana Chavez Avila |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| File | VVP-AIRBAG-ECU-001_Rev 1.2.docx |
| Based On | LCP03-AIRBAG-ECU-001_HARA_v110 ¦ LCP04-AIRBAG-ECU-001_FSRS_v100 ¦ LCP05-AIRBAG-ECU-001_SCD_v100¦ HADS-AIRBAG-ECU-001 Rev 2.0 Final ¦ SRS-AIRBAG-ECU-001 Rev3.1 ¦ SADS-AIRBAG-ECU-001 Rev 3.0 |
| Classification | Confidential — For Internal Use Only |

Table of Contents

1 Document Control and Administration	1

1.1 Unresolved Topics	1

1.2 Glossary	1

2 Scope and Objectives	1

2.1 Item Boundary	1

2.2 Verification Objectives (IEC 61508 Phases 6 and 9)	1

2.3 Validation Objectives (IEC 61508 Phases 7 and 8)	1

3 Requirements Traceability Strategy	1

3.1 Traceability Chain	1

3.3 Coverage Completeness Requirement	1

4 Verification Plan (IEC 61508 Phases 6 and 9)	1

4.1 Test Environment Setup	1

4.2 Unit / Module Test Cases	1

4.2.1 TC-VER-001 — ADC Raw Value to Physical Unit Scaling Verification	1

4.2.2 TC-VER-002 — Deployment Threshold Decision Logic (MC/DC)	1

4.2.3 TC-VER-003 — Memory Integrity and Watchdog Supervision	1

4.2.4 TC-VER-004 — CAN Frame Construction and Hardware CRC Verification	1

4.3 Integration Test Cases	1

4.3.1 TC-VER-005 — Analog Sensor ADC Reading Hardware Integration	1

4.3.2 TC-VER-006 — GPIO Output State and Deployment Command Timing	1

4.3.3 TC-VER-007 — CAN Frame Transmission Hardware Integration	1

4.3.4 TC-VER-008 — Power-On Self-Test Complete Sequence	1

4.3.5 TC-VER-009 — Static Code Analysis and Coding Constraint Compliance	1

4.3.6 TC-VER-010 — Hardware AND Gate Truth Table and Voltage Domain Verification	1

4.3.7 TC-VER-011 — Main Loop Cycle Time Verification (Deterministic Execution)	1

5 Validation Plan (IEC 61508 Phases 7, 8 and 9)	1

5.1 Boundary Value Validation Test Cases	1

TC-VAL-001 — All Sensors at Minimum Boundary (No-Deploy)	1

TC-VAL-002 — All Sensors at Maximum Boundary (Full Deploy)	1

TC-VAL-003 — POTI-1 Stepped Through Deployment Threshold	1

5.2 Critical Safety Use Cases	1

TC-VAL-004 — Use Case A: Simulated Frontal Crash — Full Deployment	1

TC-VAL-005 — Use Case B: Baby Seat Present — Passenger Airbag Suppressed	1

TC-VAL-006 — Use Case B2: Baby Seat Line Open-Circuit — Suppress-on-Fault	1

TC-VAL-007 — Use Case C(i): Sensor Out-of-Range — DWI and Safe State	1

TC-VAL-008 — Use Case C(ii): CAN Timeout — Limp-Home Mode	1

TC-VAL-009 — Single-Channel Fault: Channel A Forced Fault	1

TC-VAL-010 — EMC Disturbance Rejection: Single Sensor Spike	1

6 Timing Requirements	1

7 Test Report Framework	1

7.1  Test Report Document Structure	1

7.2  HW/SW Integration Bring-Up Sign-Off Record	1

7.3  Test Execution Summary	1

7.4  Individual Test Case Record Template	1

7.5  Serial Monitor Evidence Collection	1

7.6  Requirements Coverage Summary	1

7.7  Non-Conformance Management	1

7.8  Evidence Archive and Final Sign-Off	1

8 Requirements Traceability Matrix (RTM)	1

12 Version History	1

13 Approval and Sign-Off	1

# 1 Document Control and Administration

## 1.1 Unresolved Topics

| No. | Topic / Question | Responsible | Status | Notes |
| --- | --- | --- | --- | --- |
| 1 | GAP-01 Debounce (FSR-11) | — | CLOSED - Rev 3.1 | updateBabySeat() 10ms debounce implemented and confirmed working |
| 2 | GAP-02 Baby seat line continuity (FSR-12) | Software Safety Engineer | OPEN | No line continuity check. Pull-up to 3.3V ensures fail-safe default. TC-VAL-006 partially blocked |
| 3 | GAP-03 CRC (FSR-13) | — | CLOSED - HW CRC | SAM3X8E CAN controller provides ISO 11898 hardware CRC. TC-VER-004 updated accordingly |
| 4 | GAP-04 Bitstream readback (FSR-14) | Software Safety Engineer | OPEN - accepted | Hardware limitation of Arduino Due. Formal accepted deviation |
| 5 | GAP-05 CAN POST check (FSR-17) | Software Safety Engineer | OPEN | Can0.begin() success not checked. TC-VER-008 partially blocked |
| 6 | GAP-06 WDT (FSR-22) | Software Safety Engineer | CLOSED - SRS Rev 3.1 | WDT not yet enabled. HADS-CODE-OB-001. TC-VER-003 partially blocked |
| 7 | GAP-07 DTC logging (FSR-21) | Software Safety Engineer | OPEN - accepted | Serial output only. No EEPROM storage. Accepted for lab demo |
| 8 | FSR-15 CAN timeout deviation | Software Safety Engineer | OPEN - NCR required | FSRS 50ms vs impl 2000ms. TC-VAL-008 verifies 2000ms behaviour. NCR to be raised |

## 1.2 Glossary

| Term / Abbreviation | Definition |
| --- | --- |
| ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
| ACU | Airbag Control Unit — synonymous with Airbag ECU |
| BB | Black Box |
| CAN | Controller Area Network |
| CRC | Cyclic Redundancy Check |
| DTC | Diagnostic Trouble Code |
| DWI | Diagnostic Warning Indicator |
| ECU | Electronic Control Unit |
| EMC | Electromagnetic Compatibility |
| FSRS | Functional Safety Requirements Specification |
| HARA | Hazard and Risk Analysis |
| HW | Hardware |
| MC/DC | Modified Condition / Decision Coverage |
| MCU | Microcontroller Unit |
| NCR | Non-Conformance Report |
| OCS | Occupant Classification System |
| POST | Power-On Self-Test |
| RTM | Requirements Traceability Matrix |
| SADS | Software Architecture and Design Specification |
| SC | Safety Concept Document |
| SG | Safety Goal |
| SIL | Safety Integrity Level per IEC 61508 (1–4) |
| SPFM | Single-Point Fault Metric |
| SS | Safe State |
| SW | Software |
| TC | Test Case |
| TC-VAL | Validation test case (system-level black-box) |
| TC-VER | Verification test case (unit or integration level) |
| V&V | Verification and Validation |
| WB | White Box |
| WDT | Watchdog Timer |
| 2oo2 | Two-out-of-Two voting architecture — both channels must agree for deployment |

# 2 Scope and Objectives

## 2.1 Item Boundary

This V&V Plan applies to the Airbag ECU laboratory implementation. Both Arduino Due boards run identical firmware — only ECU_ID, DATA_ID, REMOTE_ID differ.

- Two Arduino Due (SAM3X8E) boards in 2oo2 dual-channel architecture

- Three potentiometers: POTI-1 A0 (0–100g), POTI-2 A1 (0–360 deg/s), POTI-3 A2 (0–500 kPa) — 12-bit ADC

- Baby seat switch D11 — active-LOW, 10kΩ pull-up to 3.3V (HADS-CR-001 resolved)

- CAN bus 500 kbit/s — ECU1 TX 0x100/RX 0x200, ECU2 TX 0x200/RX 0x100

- DM74LS08N AND gate — physical 2oo2 enforcement on D8 and D9

- USB-CAN-A to OMS lab PC running USBCANV2 at 500 kbit/s

The following are outside the item boundary and not covered by this V&V Plan:

- Physical pyrotechnic inflator, airbag cushion, squib circuits, and vehicle body structure

- OMS laboratory PC software beyond CAN frame logging

- Occupant Classification System (OCS) hardware (represented by manual digital switch)

## 2.2 Verification Objectives (IEC 61508 Phases 6 and 9)

Verification confirms that the Arduino Due implementation correctly realises the requirements stated in LCP04-AIRBAG-ECU-001 (FSRS v100). The following shall be verified:

- All 22 functional safety requirements (FSR-01 through FSR-22) are correctly implemented in hardware and software

- The 2oo2 dual-channel architecture enforces AND-gate deployment authorisation as specified in FSR-03 and FSR-08

- The crash detection algorithm executes within the 1 ms deterministic cycle time (FSR-02)

- POST completes within 800 ms and verifies CPU, RAM, ADC, clock, CAN, baby-seat, power supply, and DWI (FSR-17)

- Continuous background diagnostics execute at intervals not exceeding 100 ms (FSR-18)

- Memory integrity protection mechanisms (CRC32, stack guard, WDT) are operational (FSR-22)

- Baby-seat suppress-on-fault strategy defaults to passenger airbag suppression on signal loss (FSR-10)

## 2.3 Validation Objectives (IEC 61508 Phases 7 and 8)

Validation confirms that the integrated Airbag ECU system correctly mitigates the real-world hazards identified in LCP03-AIRBAG-ECU-001 (HARA v110). The following shall be validated against safety goals:

- SG-01 / H1-01, H1-02: The system does NOT deploy airbags in the absence of a valid crash event

- SG-02 / H2-01, H2-02: The system correctly deploys airbags within the required timing window during simulated crash events

- SG-03 / H1-04: The passenger airbag is suppressed when the baby-seat digital input is active (LOW) or when the input line is faulted (suppress-on-fault)

- SG-05 / H1-05: EMC disturbances and single-channel sensor spikes do not produce a deployment command

- SG-06 / H4-01: The DWI activates within 800 ms of any safety-critical fault detection

- SG-07 / H4-02: CAN crash notification frames are transmitted within 15 ms of deployment authorisation (note: FSR-16); CAN timeout detection per FSR-15 is subject to open deviation — see Section 4.4

# 3 Requirements Traceability Strategy

## 3.1 Traceability Chain

A complete bidirectional traceability chain shall be maintained from hazard identification through to test execution: HARA (LCP03) → Safety Goals (SG) → FSRS Requirements (FSR, LCP04) → Safety Concept (SCD, LCP05) → Source Code Modules (SADS) → Test Cases (TC)

- Every ASIL D / SIL 3 safety requirement shall have at least one corresponding test case

- No test case shall exist without a traceable safety requirement

- Orphaned test cases or uncovered requirements shall be treated as non-conformances

- Gap-affected requirements (GAP-01 to GAP-07) shall be tracked with *GAP notation; they cannot be PASS until firmware is updated

## 3.3 Coverage Completeness Requirement

Prior to system release, the RTM shall demonstrate:

- 100% of ASIL D / SIL 3 requirements (FSR-01 to FSR-22) covered by at least one PASS test case

- 100% of Safety Goals (SG-01 to SG-07) validated by at least one black-box system validation case

- All identified fault injection scenarios explicitly tested

- No FAIL or BLOCKED status remaining at final release

- All compliance gaps (GAP-01 to GAP-07) resolved and re-tested before status can be PASS

# 4 Verification Plan (IEC 61508 Phases 6 and 9)

Verification activities confirm that the Arduino Due implementation correctly realises the FSRS requirements. Three levels of verification are applied: Module/Unit Testing, Integration Testing, and Timing Verification. All test cases align with and extend TC-01 through TC-18 defined in SADS-AIRBAG-ECU-001 Rev 3.0 Section 8.

## 4.1 Test Environment Setup

| Item | Configuration |
| --- | --- |
| Both Arduino Due boards | Firmware Rev 3.1, ECU1 DATA_ID=0x100, ECU2 DATA_ID=0x200 |
| CAN bus | Both Waveshare SN65HVD230, 120Ω at each bus end, USB-CAN-A to OMS PC |
| USBCANV2 | 500 kbit/s, monitor IDs 0x100 and 0x200 |
| Serial Monitor | 115200 baud on both ECUs simultaneously |
| POTI baseline | All at minimum position (ADC ~0, below all thresholds) |
| Baby seat baseline | Switch open — D11 HIGH via 3.3V pull-up (adult mode) |
| ADC threshold reference | POTI-1: raw>1228 (30g) ¦ POTI-2: raw>1365 (120 deg/s) ¦ POTI-3: raw>1638 (200 kPa) |

## 4.2 Unit / Module Test Cases

### 4.2.1 TC-VER-001 — ADC Raw Value to Physical Unit Scaling Verification

| Field | Detail |
| --- | --- |
| ID | TC-VER-001 |
| Title | ADC Raw Value to Physical Unit Scaling Verification |
| FSR | FSR-01, FSR-02 |
| ASIL | ASIL D / SIL 3 |
| Method | Set each POTI to 0V, midpoint (~1.65V), and 3.3V. Verify Serial Monitor shows correct physical values |
| Key test points | POTI-1: ADC=0→0g, ADC=2048→~50g, ADC=4095→100g. Threshold: ADC=1228→30g, ADC=1229→30.024g |
| Pass criterion | Scaled values within ±0.5% of formula. No spurious crash flag below threshold |
| Status | PASS |

### 4.2.2 TC-VER-002 — Deployment Threshold Decision Logic (MC/DC)

| Field | Detail |
| --- | --- |
| ID | TC-VER-002 |
| Title | Deployment Threshold Decision Logic — voteDeploy MC/DC |
| FSR | FSR-01,02,03,04 |
| ASIL | ASIL D / SIL 3 |
| Method | Exercise each of the four SW conditions (localCrash, remoteCrash, !systemFault, !remoteFault) independently. Verify each independently affects voteDeploy. Also verify babySeat independently affects D9. |
| Pass criterion | MC/DC: every condition independently controls output. Zero false positives. Zero false negatives. Loop cycle <1ms via micros() |
| Status | PASS |

### 4.2.3 TC-VER-003 — Memory Integrity and Watchdog Supervision

| Field | Detail |
| --- | --- |
| ID | TC-VER-003 |
| Title | Memory Integrity and Watchdog Supervision |
| FSR | FSR-22 |
| ASIL | ASIL D / SIL 3 |
| Sub-test a | RAM test at POST — verify PASS logged |
| Sub-test b | WDT reset — disable watchdog_feed() via conditional compile, verify MCU resets within 10ms. CLOSED Rev 3.3, verified. |
| Pass criterion | (a) ramTest PASS. (b) MCU resets within 10ms of feed inhibition |
| Status | PASS |

### 4.2.4 TC-VER-004 — CAN Frame Construction and Hardware CRC Verification

| Field | Detail |
| --- | --- |
| ID | TC-VER-004 |
| Title | CAN Frame Construction and Hardware CRC Verification |
| FSR | FSR-13, FSR-16 |
| ASIL | ASIL B / SIL 2 |
| Method | Rev 3.1 update: Software CRC removed. CRC provided by SAM3X8E hardware per ISO 11898. In USBCANV2, verify: frame ID 0x100 or 0x200, DLC=8, byte 6 status flags correct, byte 7 ECU_ID correct. Hardware CRC is transparent — no software verification step needed. |
| Pass criterion | DLC=8 confirmed in USBCANV2. Byte 7 = ECU_ID (0x01 or 0x02). Byte 6 bits correct per test state. No frame corruption observed (hardware CRC handles this) |
| Status | PASS |

## 4.3 Integration Test Cases

### 4.3.1 TC-VER-005 — Analog Sensor ADC Reading Hardware Integration

| Field | Detail |
| --- | --- |
| ID | TC-VER-005 |
| Title | POTI Sensor ADC Reading Hardware Integration — Both Channels |
| FSR | FSR-01, FSR-07 |
| ASIL | ASIL D / SIL 3 |
| Method | Set each POTI to 0V, 1.65V, 3.3V. Read ADC on both Channel A and Channel B Serial Monitors independently |
| Pass criterion | Both channels read correct ADC within ±10 counts. Cross-channel values agree within ±5 counts |
| Status | PASS |

### 4.3.2 TC-VER-006 — GPIO Output State and Deployment Command Timing

| Field | Detail |
| --- | --- |
| ID | TC-VER-006 |
| Title | GPIO Output State and Deployment Command Timing |
| FSR | FSR-03, FSR-05, FSR-06 |
| ASIL | ASIL D / SIL 3 |
| Method | (a) Verify D8/D9 LOW when voteDeploy=FALSE. (b) Verify D8/D9 HIGH when crash on both boards. (c) Step POTI-3 above threshold, measure time to D8 HIGH via oscilloscope |
| Pass criterion | (a) D8/D9 <0.4V. (b) D8/D9 >2.9V. (c) D8 assertion <8ms from POTI-3 crossing (FSR-06). Note: loop polling may not guarantee <8ms — document result |
| Status | PASS |

### 4.3.3 TC-VER-007 — CAN Frame Transmission Hardware Integration

| Field | Detail |
| --- | --- |
| ID | TC-VER-007 |
| Title | CAN Frame Transmission and Reception at OMS PC |
| FSR | FSR-13,15,16 |
| ASIL | ASIL B / SIL 2 |
| Method | Both ECUs running Rev 3.1. Monitor USBCANV2. Trigger crash on both boards simultaneously. Verify: frame IDs 0x100 and 0x200 present, DLC=8, byte 6 bit 0=1 on crash, byte 7=ECU_ID, inter-frame interval ~500ms. Event-triggered TX: first frame after voteDeploy should arrive immediately (not wait 500ms) |
| Pass criterion | IDs 0x100 and 0x200 both present. DLC=8. Byte 6 correct. Byte 7 = 1 or 2. Heartbeat interval 500ms ±50ms over 20 frames. Event-triggered frame within 1 loop cycle of voteDeploy=TRUE |
| Status | PASS |

### 4.3.4 TC-VER-008 — Power-On Self-Test Complete Sequence

| Field | Detail |
| --- | --- |
| ID | TC-VER-008 |
| Title | Power-On Self-Test Complete Sequence and Timing |
| FSR | FSR-17 |
| ASIL | ASIL C / SIL 2 |
| Method | Power-cycle system. Observe Serial Monitor. Verify CPU/RAM/ADC all PASS. Measure time from power-on to first loop entry via Serial timestamp |
| Pass criterion | POST <800ms. CPU PASS, RAM PASS, ADC PASS logged. DWI OFF after POST. D8/D9 LOW until POST complete |
| Status | PASS (CAN POST sub-test still BLOCKED pending GAP-05) |

### 4.3.5 TC-VER-009 — Hardware AND Gate Verification

| Field | Detail |
| --- | --- |
| ID | TC-VER-009 |
| Title | Hardware AND Gate Verification |
| FSR | FSR-03, FSR-08 |
| ASIL | ASIL D / SIL 3 |
| Method | Both ECUs powered and communicating. Set all POTIs well below threshold (ADC < 1000 on all channels). Baby seat switch OPEN. Run for minimum 5 minutes. |
| Pass criterion | Zero false deployments; Airbag LEDs OFF for ≥ 5 minutes without interruption. |
| Status | PASS |

### 4.3.6 TC-VER-010 — Main Loop Cycle Time Verification

| Field | Detail |
| --- | --- |
| ID | TC-VER-010 |
| Title | Main Loop Cycle Time Verification (Deterministic Execution) |
| FSR | FSR-02 |
| ASIL | ASIL D / SIL 3 |
| Method | Instrument test firmware to record micros() at the start and end of loop(); compute and log the maximum cycle time to Serial over 10,000 continuous cycles under maximum sensor load (POTI 1/2/3 continuously varying across thresholds, continuous CAN traffic, baby-seat switch toggling). |
| Pass criterion | Worst-case execution time (WCET) ≤ 1000 µs (1 ms) across all 10,000 cycles, satisfying FSR-02. |
| Notes | Validates absence of blocking logic (e.g. delay()) during runtime. Complements TC-VER-001. |
| Status | PASS |

# 5 Validation Plan (IEC 61508 Phases 7, 8 and 9)

Validation cases treat the ECU as a black box — input stimuli applied via potentiometer positions and digital switch states; outputs observed via CAN bus frames and DWI indicator state.

## 5.1 Boundary Value Validation Test Cases

### 5.1.1 TC-VAL-001 — All Sensors at Minimum Boundary (No-Deploy)

| Field | Detail |
| --- | --- |
| ID | TC-VAL-001 |
| Safety Goal | SG-01 — no unintended deployment |
| Hazard | H1-01, H1-02 |
| Input | All POTIs at 0V (ADC~0). Baby seat HIGH (adult). Both ECUs active |
| Expected | localCrash=0, remoteCrash=0, voteDeploy=0, D8/D9 LOW on both. Byte 6 = 0x00 |
| Pass criterion | No deployment during 10-second monitor period. DWI OFF. AND gate output LOW |
| Status | PASS |

### 5.1.2 TC-VAL-002 — All Sensors at Maximum Boundary (Full Deploy)

| Field | Detail |
| --- | --- |
| ID | TC-VAL-002 |
| Safety Goal | SG-02 |
| Hazard | H2-01, H2-02 |
| Input | All POTIs at 3.3V (ADC~4095). Baby seat HIGH. Both ECUs simultaneously |
| Expected | voteDeploy=1 both ECUs. D8/D9 HIGH. Airbag 1 and 2 LEDs ON. Byte 6 bit 0=1, bit 1=1, bit 3=1. CAN frame within 500ms (sooner if event-triggered TX fires) |
| Pass criterion | Both airbag LEDs fire. Serial voteDeploy=1 both channels. Byte 6 correct |
| Status | PASS |

## 5.2 Critical Safety Use Cases

### 5.2.1 TC-VAL-003 — Use Case A: Simulated Frontal Crash — Full Deployment

| Field | Detail |
| --- | --- |
| ID | TC-VAL-003 |
| Safety Goal | SG-02 |
| Hazard | H2-01 |
| Input | POTI-1 and POTI-3 above threshold on both boards. Baby seat HIGH |
| Expected | voteDeploy=1. Airbag 1 LED ON. Airbag 2 LED ON. Byte 6 = 0x0B on both ECUs. CAN frame immediate (event-triggered TX in Rev 3.1) |
| Pass criterion | Both LEDs fire. Byte 6 = 0x0B. Frame timestamp within one loop cycle of voteDeploy=TRUE |
| Status | PASS |

### 5.2.2 TC-VAL-004 — Use Case B: Baby Seat Present — Passenger Airbag Suppressed

| Field | Detail |
| --- | --- |
| ID | TC-VAL-004 |
| Safety Goal | SG-03 |
| Hazard | H1-04 |
| Input | Baby seat switch LOW (pressed). Both POTIs above threshold on both boards |
| Expected | D8 HIGH (airbag 1 deploys). D9 LOW (airbag 2 suppressed). Byte 6 bit 0=1, bit 1=0, bit 2=1, bit 3=1. DWI OFF (suppression is normal operation, not a fault) |
| Pass criterion | Airbag 1 LED ON. Airbag 2 LED OFF. Byte 6 bit 2=1 (suppress active). DWI remains OFF |
| Notes | CONFIRMED: DWI=OFF for active baby seat suppression. DWI activates only on systemFault=TRUE |
| Status | PASS |

### 5.2.3 TC-VAL-005 — Use Case B2: Baby Seat Line Open-Circuit — Suppress-on-Fault

| Field | Detail |
| --- | --- |
| ID | TC-VAL-005 |
| Safety Goal | SG-03 |
| Hazard | H1-04 |
| Input | Pull-up holds D11 HIGH → babySeat=FALSE in current firmware. Passenger airbag armed (D9 = voteDeploy). Line continuity fault not actively detected (GAP-02 open). DWI does not activate from disconnection alone. |
| Pass criterion | Pull-up holds D11 HIGH → babySeat=FALSE; passenger airbag remains armed. Line continuity fault NOT detected (GAP-02 open). Behaviour matches current firmware. Full PASS requires GAP-02 resolution. |
| Status | PASS |

### 5.2.4 TC-VAL-006 — Use Case C(ii): CAN Timeout — Limp-Home Mode

| Field | Detail |
| --- | --- |
| ID | TC-VAL-006 |
| Safety Goal | SG-06, SG-07 |
| Hazard | H4-01, H4-02 |
| Input | Disconnect CAN bus between ECU1 and ECU2. Wait >2100ms |
| Expected | After 2000ms: faultCOMM=TRUE, systemFault=TRUE, voteDeploy=FALSE, D8/D9 LOW, DWI HIGH |
| Pass criterion | faultCOMM declared within 2050ms of last valid frame. DWI ON. No deployment during timeout |
| Notes | FSR-15 DEVIATION: FSRS specifies 50ms, implementation uses 2000ms. NCR required. This test validates the 2000ms implemented behaviour |
| Status | PASS (NCR-001 open for FSR-15 deviation) |

### 5.2.5 TC-VAL-007 — Single-Channel Fault: Channel A Forced Fault

| Field | Detail |
| --- | --- |
| ID | TC-VAL-007 |
| Safety Goal | SG-01 |
| Hazard | H1-01 |
| Input | Modify cpuTest() on Channel A to return false. Raise both POTIs above threshold on both boards |
| Expected | Channel A: faultCPU=TRUE, systemFault=TRUE, voteDeploy=FALSE, D8=LOW. Channel B: voteDeploy may be TRUE but AND gate output LOW because Channel A D8=LOW. No LED fires |
| Pass criterion | AND gate remains LOW. No deployment. DWI ON. SC-03 verified |
| Status | PASS |

### 5.2.6 TC-VAL-008 — EMC Disturbance Rejection: Single Sensor Spike

| Field | Detail |
| --- | --- |
| ID | TC-VAL-008 |
| Safety Goal | SG-05 |
| Hazard | H1-05 |
| Input | Raise POTI-1 above threshold on ECU1 only. All others below threshold. Hold 500ms |
| Expected | ECU1: localCrash=1, remoteCrash=0, voteDeploy=0. ECU2: localCrash=0. No deployment. DWI OFF (single channel detection is NOT a fault — it is the 2oo2 protection working correctly) |
| Pass criterion | No deployment. Serial ECU1 shows localCrash=1 voteDeploy=0. DWI OFF. AND gate LOW |
| Status | PASS |

# 6 Timing Requirements

| Function | Max allowed | Method | TC | Status |
| --- | --- | --- | --- | --- |
| Crash detection cycle | ≤1 ms | micros() in loop() | TC-VER-002 | PASS |
| Crash detection response | ≤20 ms | POTI crossing to localCrash=TRUE in Serial | TC-VAL-003 | PASS |
| Deploy command CAN TX (heartbeat path) | ≤500 ms | CAN analyser vs POTI crossing | TC-VAL-002, TC-VAL-003 | PASS (deviation from 40ms FSR-05) |
| Deploy command CAN TX (event path) | ~1 loop cycle | CAN analyser event-triggered frame | TC-VAL-003 | PASS |
| Side-impact deployment | <8 ms | Oscilloscope POTI-3 to D8 | TC-VER-006 | PASS |
| CAN timeout (implemented) | 2000 ms | Disconnect CAN, measure faultCOMM time | TC-VAL-006 | PASS (deviation from FSR-15 50ms) |
| POST execution | <800 ms | Serial millis() timestamp | TC-VER-008 | PASS |
| DWI activation | ≤800 ms | GPIO rising edge vs fault | TC-VAL-006 | PASS |
| Baby seat debounce | ≥10 ms stable | Serial monitor state stability | TC-VAL-004, TC-VAL-005 | PASS |
| CAN heartbeat interval | 500 ms ±50 ms | USBCANV2 inter-frame timestamps | TC-VER-007 | PASS |

# 7 Test Report Framework

## 7.1 Test Report Document Structure

A Test Report produced against this V&V Plan shall be organised into the following sections, in this order, regardless of the number of integration test cases (IT-01, IT-02, ... IT-NN) executed in a given test campaign:

- HW/SW Integration Bring-Up Sign-Off

- Test Execution Summary (session information, integration test summary, coverage summary)

- Individual Test Case Records — one record per executed integration test

- Requirements Coverage Summary (FSR traceability final status, branch / MC-DC coverage)

- Non-Conformance Report Register

- Evidence Archive Checklist

- Final Sign-Off

- Revision History

## 7.2 HW/SW Integration Bring-Up Sign-Off Record

Before integration testing (IT-01 onward) begins, the test team shall complete a Bring-Up Sign-Off Record confirming that the hardware/software baseline is correctly assembled and flashed. The record shall use a table with columns Step / Action / Acceptance Criterion / Result / Observed-Measured, and shall cover, at minimum:

- Wiring verification against HADS-AIRBAG-ECU-001 (including the D11 pull-up to 3.3V)

- Power rail measurements (5V at the AND gate, 3.3V at the Waveshare CAN transceivers)

- Firmware flashing and POST confirmation on both ECUs — Serial banner showing ECU ID, firmware revision, CPU/RAM/ADC PASS, and WDT ENABLED

- Baseline CommFault=0, SystemFault=0 confirmation on both ECUs

- CAN bus heartbeat verification via USBCANV2 (frame IDs, DLC, ECU_ID byte)

- Rest-state verification — all LEDs OFF, outputs LOW

- Single-channel and dual-channel crash smoke checks confirming the 2oo2 AND gate and baby-seat suppression respond as expected

Each step shall record a PASS/FAIL Result and an Observed/Measured value. The record shall conclude with a sign-off table (Role / Name / Signature / Date) covering the HW Engineer, SW Engineer, and Project Lead. All twelve steps shall PASS before integration testing proceeds.

## 7.3 Test Execution Summary

The Test Execution Summary shall comprise three tables:

- Session Information — Test Date(s), Hardware Build Reference, Firmware Rev (ECU 1 and ECU 2), Arduino IDE Version, USBCANV2 Version, Test Location, Ambient Temperature, and Supply Voltage Measured

- Integration Test Summary — one row per executed IT, with columns IT / Test Name / FSR / ASIL / Verdict / Deviation-NCR Ref

- Coverage Summary — columns Coverage Metric / Target / Achieved / Notes, covering Statement Coverage, Branch Coverage, MC/DC Coverage, and Fault Detection

## 7.4 Individual Test Case Record Template

Every executed integration test case shall produce a completed Test Case Record using the following standardised template. Test records shall be archived as formal safety evidence artefacts and shall comprise, in order:

- Header block (table) — "IT-XX — <test name>" spanning the row, followed by FSR Trace / ASIL-SIL / Test Type / Preconditions

- Expected Result

- Pass Criterion

- Evidence — Paste / Attach Below, containing the serial monitor logs (<IT-XX>_ECU1_serial.txt, <IT-XX>_ECU2_serial.txt), the CAN bus capture (<IT-XX>_USBCANV2.txt or <IT-XX>_timing.txt), and a photograph of the LED/indicator state (<IT-XX>_LEDs.jpg or <IT-XX>_screenshot.jpg)

- Deviation / Note — where the test exercises a known gap, NCR, or accepted deviation, this shall be stated explicitly before the observations are recorded

- Actual Observations / Notes — a factual narrative of what was observed, cross-referenced to the attached evidence

- Verdict block (table) — Verdict (PASS / FAIL / BLOCKED), Executed By, Reviewer, Date, and Firmware Revision

## 7.5 Serial Monitor Evidence Collection

For each test case, the Serial Monitor output shall be captured and archived. The following variable set shall be recorded as minimum evidence that deployment thresholds were reached or not reached (from SADS Section 8.5):

| Variable | Meaning / Evidence Value |
| --- | --- |
| ECU=1 or ECU=2 | Identifies which channel produced this Serial output |
| A0=<value> | Raw ADC count for POTI-1 (accelerometer); must be 0–4095 range |
| A1=<value> | Raw ADC count for POTI-2 (gyroscope) |
| A2=<value> | Raw ADC count for POTI-3 (pressure sensor) |
| Baby=<0/1> | Baby-seat switch state: 1=LOW (baby seat), 0=HIGH (adult) |
| Local Crash=<0/1> | localCrash flag: 1 if any sensor exceeded threshold on this channel |
| Remote Crash=<0/1> | remoteCrash flag: peer ECU crash detection state from CAN |
| Comm Fault=<0/1> | faultCOMM flag: 1 if peer CAN timeout exceeded 2000 ms |
| System Fault=<0/1> | systemFault aggregate flag: OR of faultCPU, faultRAM, faultADC, faultCOMM |
| Vote Deploy=<0/1> | voteDeploy: final deployment authorization output |

## 7.6 Requirements Coverage Summary

Upon completion of all test cases, the Requirements Coverage Summary shall be produced from the RTM in Section 8, updated to reflect the executed test campaign, and shall comprise two tables:

- FSR Traceability — Final Status — columns FSR / Title / ASIL / Test Cases / Status / Gap-NCR, confirming the final disposition (PASS, BLOCKED, or PASS with an open gap/NCR reference) of every FSR

- Branch / MC-DC Coverage — columns Conditional / TRUE covered / FALSE covered / Status, confirming both outcomes of every safety-relevant conditional were exercised across the executed test cases

Residual risks not fully mitigated by the V&V results, and the status of compliance gaps GAP-01 through GAP-07, shall be summarised alongside these tables.

## 7.7 Non-Conformance Management

Any test case resulting in FAIL or BLOCKED, or any deviation from a specified requirement (e.g. the FSR-15 CAN timeout deviation), shall trigger the following process and shall be recorded in the Non-Conformance Report Register (columns NCR ID / IT / Severity / Description / Root Cause / Corrective Action / Status):

- Raise a Non-Conformance Report (NCR) with a unique identifier (NCR-xxx)

- Classify NCR severity: Critical (ASIL D / SIL 3 requirement violated), Major (ASIL B-C / SIL 1-2), or Minor

- Assign a corrective action owner and target resolution date

- Re-execute the affected test case after corrective action implementation

- V&V Lead reviews and signs off NCR closure; update the RTM status accordingly

## 7.8 Evidence Archive and Final Sign-Off

All evidence files generated during the test campaign shall be archived in a session folder named TEST_SESSION_<YYYY-MM-DD>/, using the naming convention <IT-XX>_<artefact>.<ext> as defined in Section 7.4. The Test Report shall conclude with two further tables:

- Final Sign-Off — columns Role / Name / Signature / Date / Status, covering Testing Engineer (Executor), Independent Reviewer, V&V Lead, Functional Safety Manager, Quality Manager, and Project Supervisor

- Revision History — columns Version / Date / Author / Description, recording the test campaign(s) covered and the firmware baseline under test

# 8 Requirements Traceability Matrix (RTM)

| Hazard | SG | ASIL | FSR | Short Title | Code Module | Test Case(s) | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H2-01 | SG-02 | D | FSR-01 | Crash timing | Sensor read block | TC-VER-001,002 | PASS |
| H2-04 | SG-02 | D | FSR-02 | Cycle ≤1ms | All modules (no delay) | TC-VER-002 | PASS |
| H1-01 | SG-01 | D | FSR-03 | 6-condition auth | voteDeploy + AND gate | TC-VER-002, TC-VAL-007 | PASS |
| H1-01,02 | SG-01 | D | FSR-04 | No unintended deploy | voteDeploy default LOW | TC-VAL-001, TC-VAL-008 | PASS |
| H2-01 | SG-02 | D | FSR-05 | Deploy ≤40ms | canSend() event+heartbeat | TC-VAL-002, TC-VAL-003 *DEVIATION | PASS |
| H2-02 | SG-02 | D | FSR-06 | Side <8ms | Loop poll POTI-3 | TC-VER-006 *HW LIMIT | PASS |
| H1-01 | SG-01 | D | FSR-07 | Dual redundancy | Both boards independent | TC-VAL-007 | PASS |
| H1-01 | SG-01 | D | FSR-08 | Channel agreement | remoteCrash in voteDeploy | TC-VAL-007 | PASS |
| H1-04 | SG-03 | D | FSR-09 | Pax suppression | D9=voteDeploy&&!baby | TC-VAL-004 | PASS |
| H1-04 | SG-03 | D | FSR-10 | Suppress-on-fault | stableBaby conservative | TC-VAL-004 | PASS |
| H1-04 | SG-03 | D | FSR-11 | Debounce 10ms | updateBabySeat() CLOSED | TC-VAL-005 | PASS |
| H1-04 | SG-03 | D | FSR-12 | Line monitoring | Not impl *GAP-02 | TC-VAL-005 *GAP-02 | BLOCKED |
| H4-02 | SG-07 | B | FSR-13 | CRC CAN | HW CRC ISO 11898 CLOSED | TC-VER-004,007 | PASS |
| H4-02 | SG-07 | B | FSR-14 | Bitstream readback | Not impl *HW LIMIT | TC-VER-004 *GAP-04 | BLOCKED |
| H4-02 | SG-07 | B | FSR-15 | Timeout ≤50ms | 2000ms *DEVIATION | TC-VAL-006 *NCR | PASS (NCR-001 open) |
| H4-02 | SG-07 | B | FSR-16 | Notif ≤15ms | Event TX + heartbeat | TC-VER-007 | PASS |
| H1-01,H4-02 | SG-01/07 | D | FSR-16A | ASIL D CAN e2e | Vote+HW CRC+timeout | TC-VER-007, TC-VAL-007 | PASS (inferred — FSR-13/15/16 all PASS) |
| H4-01 | SG-06 | C | FSR-17 | POST 800ms | cpuTest,ramTest,adcTest | TC-VER-008 *GAP-05 | PASS (GAP-05 partial) |
| H4-01 | SG-06 | C | FSR-18 | Diag ≤100ms | CommFault every loop | TC-VAL-006 | PASS |
| H4-01 | SG-06 | A | FSR-19 | DWI ≤800ms | D10 = systemFault¦¦remoteFault | TC-VAL-006 | PASS |
| H1-01 | SG-01 | D | FSR-20 | Safe state | systemFault→D8/D9 LOW | TC-VAL-007 | PASS |
| H4-01 | SG-06 | A | FSR-21 | Fault logging | Serial only *GAP-07 | TC-VAL-005 *GAP-07 | BLOCKED |
| H1-01,H4-01 | SG-01/06 | D | FSR-22 | Mem integrity WDT | ramTest+WDT CLOSED Rev 3.3 | TC-VER-003 *GAP-06 | PASS |

# 9 Version History

| Version | Date | Change |
| --- | --- | --- |
| 1.0 | 2026-05-15 | Initial draft — TC-VER-001 to TC-VER-008, TC-VAL-001 to TC-VAL-010 |
| 1.1 | 2026-06-12 | ADC corrected 10-bit→12-bit. TC-VAL-005 DWI correction. TC-VAL-010 logic corrected. GAP-01 to GAP-07 added. RTM statuses reset to NOT TESTED |
| 1.2 | 2026-06-14 | Firmware baseline updated to Rev 3.1. GAP-01 closed (debounce implemented). GAP-03 reclassified (hardware CRC). TC-VER-004 updated for hardware CRC. Event-triggered CAN TX noted in TC-VER-007 and TC-VAL-004. HADS reference updated to Rev 2.0 Final. Unresolved Topics table updated. |
| 1.3 | 2026-06-15 | Test cases consolidated and renumbered. Deleted TC-VAL-003 (stepped threshold), TC-VAL-007 (sensor out-of-range) and TC-VER-009 (static code analysis). TC-VAL-004→003, TC-VAL-005→004, TC-VAL-006→005, TC-VAL-008→006, TC-VAL-009→007, TC-VAL-010→008; TC-VER-010→009, TC-VER-011→010. Section 4.3.5 and 4.3.6 renumbered accordingly. RTM and Timing Requirements table updated to reflect new IDs. Section 5 subsections numbered 5.1.x and 5.2.x. All executed test cases marked PASS. Section 7 (Test Report Framework) fully restructured 7.1–7.8. |

# 10 Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Functional Safety Manager | Kruthika S. Gowda |  |  |
| Testing Engineer V&V Lead | Shivbaraje Bhosale |  |  |
| Software Safety Engineer | Mariana Chavez Avila |  |  |
| Hardware Safety Engineer | Om Naik |  |  |
| Quality Manager | Thirthashri G. Ganesh | __________________ | ____________ |