Airbag ECU Portfolio HS Harz — Functional Safety AIRBAG ELECTRONIC CONTROL UNIT Measurement and Processing of Sensor Signals to Control Airbag Operation

VERIFICATION AND VALIDATION PLAN (V&V PLAN)

| Document Number | VVP-AIRBAG-ECU-001 |
| --- | --- |
| Version | 1.10 |
| Date | 2026-06-12 |
| Editor | Shivbaraje Bhosale |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| File | VVP-AIRBAG-ECU-001_v110.docx |
| Based On | LCP03-AIRBAG-ECU-001 (HARA v2.00) / LCP04-AIRBAG-ECU-001 (FSRS v1.00) / LCP05-AIRBAG-ECU-001 (SCD v1.00) / SADS-AIRBAG-ECU-001 (Rev 2.0) |
| Classification | Confidential — For Internal Use Only |

Table of Contents

1  Document Control and Administration	4

1.1  Meetings within the Project	4

1.2  Unresolved Topics	4

1.3  Glossary	4

1.4  Organisational Structure and Role Separation	6

2  Scope and Objectives	7

2.1  Item Boundary	7

2.2  Verification Objectives (IEC 61508 Phases 6 and 9)	7

2.3  Validation Objectives (IEC 61508 Phases 7 and 8)	7

2.4  Known Compliance Gaps Affecting V&V Scope	8

3  Requirements Traceability Strategy	9

3.1  Traceability Chain	9

3.2  Traceability Matrix Format	9

3.3  Coverage Completeness Requirement	9

3.4  Code Coverage Goals	9

4  Verification Plan (IEC 61508 Phases 6 and 9)	11

4.1  Test Environment Setup	11

4.2  Unit / Module Test Cases	11

4.2.1  TC-VER-001 — ADC Raw Value to Physical Unit Scaling Verification	11

4.2.2  TC-VER-002 — Deployment Threshold Decision Logic (MC/DC)	12

4.2.3  TC-VER-003 — Memory Integrity and Watchdog Supervision	12

4.2.4  TC-VER-004 — CAN Frame Construction and CRC Calculation	13

4.3  Integration Test Cases	15

4.3.1  TC-VER-005 — Analog Sensor ADC Reading Hardware Integration	15

4.3.2  TC-VER-006 — GPIO Output State and Deployment Command Timing	15

4.3.3  TC-VER-007 — CAN Frame Transmission Hardware Integration	16

4.3.4  TC-VER-008 — Power-On Self-Test Complete Sequence	16

4.4  FSR-15 CAN Timeout — Known Deviation	17

5  Validation Plan (IEC 61508 Phases 7, 8 and 9)	18

5.1  Boundary Value Validation Test Cases	18

TC-VAL-001 — All Sensors at Minimum Boundary (No-Deploy)	18

TC-VAL-002 — All Sensors at Maximum Boundary (Full Deploy)	18

TC-VAL-003 — POTI-1 Stepped Through Deployment Threshold	19

5.2  Critical Safety Use Cases	19

TC-VAL-004 — Use Case A: Simulated Frontal Crash — Full Deployment	19

TC-VAL-005 — Use Case B: Baby Seat Present — Passenger Airbag Suppressed	20

TC-VAL-006 — Use Case B2: Baby Seat Line Open-Circuit — Suppress-on-Fault	20

TC-VAL-007 — Use Case C(i): Sensor Out-of-Range — DWI and Safe State	21

TC-VAL-008 — Use Case C(ii): CAN Timeout — Limp-Home Mode	21

TC-VAL-009 — Single-Channel Fault: Channel A Forced Fault	22

TC-VAL-010 — EMC Disturbance Rejection: Single Sensor Spike	23

6  Timing Requirements Verification Summary	24

7  Test Report Framework	25

7.1  Test Case Record Template	25

7.2  Serial Monitor Evidence Collection	26

7.3  Test Summary Report Structure	26

7.4  Non-Conformance Management	26

8  Requirements Traceability Matrix (RTM)	28

9  References and Normative Standards	30

10  List of Dependent Documents	31

11  Figure and Table Listing	31

12  Version History	32

13  Approval and Sign-Off	32

# 1  Document Control and Administration

## 1.1  Meetings within the Project

The following project meetings were held during the V&V planning phase:

Table 1: Meetings within the Project

| Topic | Date / Time | Participants | Minutes / Decisions |
| --- | --- | --- | --- |
| V&V Plan Kick-off and Scope Review | 2026-05-15, 10:00 | Testing Engineer V&V, Functional Safety Manager, Software Architect, Quality Manager | Confirmed FSR scope (FSR-01 to FSR-22). Reviewed SADS-AIRBAG-ECU-001 Rev 2.0 test cases TC-01 to TC-18. Assigned verification and validation task ownership. |
| V&V Plan Review and Gap Assessment | 2026-06-10, 14:00 | Testing Engineer V&V, Safety Requirement Engineers, Quality Manager, Project Manager | Reviewed compliance gaps GAP-01 to GAP-07 from SADS Rev 2.0. Confirmed impact on testability. Test cases updated to reflect gap status. Plan approved for DRAFT release. |

## 1.2  Unresolved Topics

Table 2: Unresolved Topics

| No. | Topic / Question | Responsible | Target Date | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | FSR-15 CAN timeout: FSRS specifies 50 ms; SADS implements 2000 ms. Discrepancy must be resolved before final release. | Software Safety Engineer | 2026-06-30 | OPEN | See Section 4.4 (GAP-15 notation in RTM). Acceptance rationale required or firmware update needed. |
| 2 | GAP-01 to GAP-07 (SADS Rev 2.0): Seven firmware compliance gaps. Verification of FSR-11, FSR-12, FSR-13, FSR-14, FSR-21, FSR-22, POST CAN check remain blocked pending gap resolution. | Software Safety Engineer / V&V Lead | 2026-07-15 | OPEN | Test cases referencing gaps are marked *GAP-xx and cannot achieve PASS status until firmware is updated. |

## 1.3  Glossary

Table 3 lists all abbreviations and terms used in this document and the wider Airbag ECU project portfolio.

Table 3: Glossary

| Term / Abbreviation | Definition |
| --- | --- |
| ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
| ACU | Airbag Control Unit — synonymous with Airbag ECU |
| BB | Black Box — test method that exercises inputs/outputs without knowledge of internal logic |
| CAN | Controller Area Network — serial communication bus for vehicle ECUs |
| CRC | Cyclic Redundancy Check — error-detection mechanism for data integrity verification |
| DTC | Diagnostic Trouble Code |
| DWI | Diagnostic Warning Indicator — airbag fault lamp in the instrument cluster |
| ECU | Electronic Control Unit |
| EMC | Electromagnetic Compatibility |
| FSR | Functional Safety Requirement — from LCP04-AIRBAG-ECU-001 FSRS |
| FSRS | Functional Safety Requirements Specification |
| GAP | Compliance gap identified in SADS-AIRBAG-ECU-001 Rev 2.0 between specification and implementation |
| HARA | Hazard and Risk Analysis |
| HW | Hardware |
| IEC 61508 | Functional Safety of E/E/PE Safety-related Systems (2010 edition) |
| ISO 26262 | Road Vehicles — Functional Safety standard (2018 edition) |
| MC/DC | Modified Condition / Decision Coverage — mandatory for ASIL D software per ISO 26262 Part 6 |
| MCU | Microcontroller Unit |
| MOD | Software module identifier from SADS-AIRBAG-ECU-001 (MOD-01 through MOD-06) |
| NCR | Non-Conformance Report |
| OCS | Occupant Classification System |
| POST | Power-On Self-Test |
| RTM | Requirements Traceability Matrix |
| SADS | Software Architecture and Design Specification — SADS-AIRBAG-ECU-001 |
| SCD | Safety Concept Document — LCP05-AIRBAG-ECU-001 |
| SG | Safety Goal — high-level safety requirement derived from HARA |
| SIL | Safety Integrity Level per IEC 61508 (1–4) |
| SPFM | Single-Point Fault Metric |
| SS | Safe State — defined system condition upon fault detection (SS-01 through SS-05) |
| SW | Software |
| TC | Test Case |
| TC-VAL | Validation test case (system-level black-box) |
| TC-VER | Verification test case (unit or integration level) |
| V&V | Verification and Validation |
| WB | White Box — test method that exercises internal code paths and conditions |
| WDT | Watchdog Timer |
| 2oo2 | Two-out-of-Two voting architecture — both channels must agree for deployment |

## 1.4  Organisational Structure and Role Separation

In accordance with ISO 26262 Part 8 and IEC 61508-1 Clause 6, verification and validation activities shall be conducted by personnel independent of the development team. The following virtual organisational structure enforces this separation:

Table 4: Organisational Roles and Independence

| Role | Responsibility | Independence Level |
| --- | --- | --- |
| Functional Safety Manager (Kruthika S. Gowda) | Overall V&V programme governance; sign-off authority on all test reports; ASIL D compliance oversight. | Independent of SW/HW development |
| Testing Engineer V&V (Shivbaraje Bhosale) | V&V plan management; test execution lead; traceability matrix maintenance; test environment configuration control. | Independent of SW/HW development |
| Quality Manager (Thirthashri G. Ganesh) | Execution oversight; recording actual results; issuing pass/fail verdicts; NCR management. | Independent of SW development |
| Software Architect (Mariana Chavez Avila) | Unit code authoring; module implementation; embedded algorithm development. | Development role — not permitted to execute own test cases |
| System Architect / HW Engineer (Om Naik / Atharva Avsare) | Breadboard wiring verification; ADC signal calibration; hardware fault injection setup. | Independent of SW development |
| Safety Requirement Engineers (Fateme Hosseinzadeh, Mobina Ketabi) | Safety analysis support; FSRS clarification; traceability review. | Independent of SW development |
| Project Supervisor | Final approval authority; regulatory compliance review. | Senior independent oversight |

# 2  Scope and Objectives

## 2.1  Item Boundary

This V&V Plan applies to the Airbag ECU laboratory implementation as defined in LCP01-AIRBAG-ECU-001 and SADS-AIRBAG-ECU-001 Rev 2.0. The item under verification comprises:

- Two Arduino Due (SAM3X8E, 84 MHz ARM Cortex-M3) microcontroller boards operating in a 2oo2 dual-channel redundant architecture

- Three potentiometer-simulated analog crash sensors on a breadboard: POTI-1 (accelerometer, pin A0), POTI-2 (gyroscope, pin A1), POTI-3 (pressure sensor, pin A2) — all with 12-bit ADC resolution (0–4095 counts, 0–3.3 V)

- One digital baby-seat switch input (active-LOW, 10 kΩ pull-up to 3.3 V, digital pin D11)

- CAN bus inter-ECU communication (500 kbit/s, Waveshare SN65HVD230 transceivers; ECU1 TX ID 0x100 / RX 0x200; ECU2 TX ID 0x200 / RX 0x100)

- CAN bus output to OMS laboratory PC via USB-CAN-A interface (USBCANV2 at 500 kbit/s)

- Hardware AND-gate (DM74LS08N) forming the physical 2oo2 deployment enforcement layer

- Embedded firmware implementing MOD-01 (POST), MOD-02 (Sensor Acquisition), MOD-03 (Crash Detection), MOD-04 (Deployment Control), MOD-05 (CAN Communication), MOD-06 (Diagnostic Monitor)

The following are outside the item boundary and not covered by this V&V Plan:

- Physical pyrotechnic inflator, airbag cushion, squib circuits, and vehicle body structure

- OMS laboratory PC software beyond CAN frame logging

- Occupant Classification System (OCS) hardware (represented by manual digital switch)

## 2.2  Verification Objectives (IEC 61508 Phases 6 and 9)

Verification confirms that the Arduino Due implementation correctly realises the requirements stated in LCP04-AIRBAG-ECU-001 (FSRS v1.00). The following shall be verified:

- All 22 functional safety requirements (FSR-01 through FSR-22) are correctly implemented in hardware and software

- The 2oo2 dual-channel architecture enforces AND-gate deployment authorisation as specified in FSR-03 and FSR-08

- The crash detection algorithm executes within the 1 ms deterministic cycle time (FSR-02)

- POST completes within 800 ms and verifies CPU, RAM, ADC, clock, CAN, baby-seat, power supply, and DWI (FSR-17)

- Continuous background diagnostics execute at intervals not exceeding 100 ms (FSR-18)

- Memory integrity protection mechanisms (CRC32, stack guard, WDT) are operational (FSR-22)

- Baby-seat suppress-on-fault strategy defaults to passenger airbag suppression on signal loss (FSR-10)

## 2.3  Validation Objectives (IEC 61508 Phases 7 and 8)

Validation confirms that the integrated Airbag ECU system correctly mitigates the real-world hazards identified in LCP03-AIRBAG-ECU-001 (HARA v2.00). The following shall be validated against safety goals:

- SG-01 / H1-01, H1-02: The system does NOT deploy airbags in the absence of a valid crash event

- SG-02 / H2-01, H2-02: The system correctly deploys airbags within the required timing window during simulated crash events

- SG-03 / H1-04: The passenger airbag is suppressed when the baby-seat digital input is active (LOW) or when the input line is faulted (suppress-on-fault)

- SG-05 / H1-05: EMC disturbances and single-channel sensor spikes do not produce a deployment command

- SG-06 / H4-01: The DWI activates within 800 ms of any safety-critical fault detection

- SG-07 / H4-02: CAN crash notification frames are transmitted within 15 ms of deployment authorisation (note: FSR-16); CAN timeout detection per FSR-15 is subject to open deviation — see Section 4.4

## 2.4  Known Compliance Gaps Affecting V&V Scope

IMPORTANT: SADS-AIRBAG-ECU-001 Rev 2.0 Section 7 identifies seven firmware compliance gaps (GAP-01 to GAP-07) between the current Arduino Due implementation and the FSRS requirements. These gaps directly affect the achievability of PASS verdicts for the associated test cases. All gap-affected test cases are marked *GAP-xx in the RTM (Section 8) and in test case descriptions below. Gap closure is required before system-level release.

Table 5: Firmware Compliance Gaps Affecting V&V

| Gap ID | FSR | Description | Risk | Required Action |
| --- | --- | --- | --- | --- |
| GAP-01 | FSR-11 | Baby seat debounce not implemented — D11 read directly without 10 ms stability filter | Noisy switch could cause spurious state change | Implement debounce timer in MOD-02 (code shown in SADS Section 5.2) |
| GAP-02 | FSR-12 | Baby seat line continuity monitoring not implemented — open-circuit stuck-HIGH not detected | Open circuit mimics adult occupancy; passenger airbag not suppressed | Add periodic D11 continuity test in MOD-06 |
| GAP-03 | FSR-13 | CRC not included in CAN frame — bytes 6–7 carry status and ECU_ID, not a 16-bit CRC | Corrupted frames accepted without error detection | Compute 16-bit CRC over bytes 0–5, store in bytes 6–7 |
| GAP-04 | FSR-14 | Parallel CAN bitstream readback not implemented | CAN transmission corruption undetected | Implement readback channel or CAN loopback verification |
| GAP-05 | FSR-17 | CAN communication not verified during POST — Can0.begin() success not checked | Silent CAN initialisation failure allows system to proceed | Check Can0.begin() return value; set faultCOMM if it fails |
| GAP-06 | FSR-22 | Hardware Watchdog Timer not enabled in firmware | Software lockup does not trigger MCU reset — system hangs silently | Enable WDT in setup(); feed at end of every loop() iteration |
| GAP-07 | FSR-21 | Fault logging to DTC/non-volatile memory not implemented — faults shown on Serial only | Fault history lost on power cycle; cannot be retrieved by service tools | Implement DTC logging to EEPROM or flash |

# 3  Requirements Traceability Strategy

## 3.1  Traceability Chain

A complete bidirectional traceability chain shall be maintained from hazard identification through to test execution:

HARA (LCP03)  →  Safety Goals (SG)  →  FSRS Requirements (FSR, LCP04)  →  Safety Concept (SCD, LCP05)  →  Source Code Modules (SADS)  →  Test Cases (TC)

- Every ASIL D / SIL 3 safety requirement shall have at least one corresponding test case

- No test case shall exist without a traceable safety requirement

- Orphaned test cases or uncovered requirements shall be treated as non-conformances

- Gap-affected requirements (GAP-01 to GAP-07) shall be tracked with *GAP notation; they cannot be PASS until firmware is updated

## 3.2  Traceability Matrix Format

The master Requirements Traceability Matrix (RTM) in Section 8 uses the following column structure:

Table 6: RTM Column Definitions

| RTM Column | Content / Source |
| --- | --- |
| Hazard ID | Source hazardous event from LCP03 HARA (e.g., H1-01, H2-02) |
| Safety Goal | Governing safety goal from HARA Section 10 (e.g., SG-01) |
| ASIL / SIL | Required integrity level for the requirement |
| FSR ID | Functional safety requirement from LCP04 FSRS (e.g., FSR-03) |
| FSR Short Title | Abbreviated requirement title for readability |
| SC Mechanism | Corresponding safety mechanism from LCP05 SCD |
| Code Module(s) | Arduino Due firmware module(s) from SADS-AIRBAG-ECU-001 Rev 2.0 |
| Test Case ID(s) | One or more V&V test case identifiers (TC-VER-xxx or TC-VAL-xxx); *GAP-xx denotes gap dependency |
| Coverage Type | Unit / Integration / Black-box / Fault Injection / Timing / WB MC/DC |
| Status | NOT TESTED / PASS / FAIL / BLOCKED — must be NOT TESTED at plan issue |

## 3.3  Coverage Completeness Requirement

Prior to system release, the RTM shall demonstrate:

- 100% of ASIL D / SIL 3 requirements (FSR-01 to FSR-22) covered by at least one PASS test case

- 100% of Safety Goals (SG-01 to SG-07) validated by at least one black-box system validation case

- All identified fault injection scenarios explicitly tested

- No FAIL or BLOCKED status remaining at final release

- All compliance gaps (GAP-01 to GAP-07) resolved and re-tested before status can be PASS

## 3.4  Code Coverage Goals

In accordance with ISO 26262 Part 6 Table 12 (ASIL D), the following code coverage targets are mandatory for all safety-relevant software modules:

Table 7: Code Coverage Targets

| Coverage Metric | ASIL D Target | Applicable Modules |
| --- | --- | --- |
| Statement Coverage | 100% | All safety-relevant modules: MOD-01 (POST), MOD-02 (Sensor), MOD-03 (Crash Detect), MOD-04 (Deployment), MOD-05 (CAN TX), MOD-06 (Diagnostics) |
| Branch Coverage | 100% | Crash threshold evaluation, baby-seat suppression logic, safe-state transitions, POST pass/fail paths |
| MC/DC Coverage | 100% | Deployment authorisation 6-condition AND-gate (FSR-03); suppress-on-fault logic (FSR-10); cross-channel agreement (FSR-08) |
| Path Coverage | Best effort | Complex conditional paths in dual-channel cross-comparison routine |

NOTE: Code coverage shall be measured using a code instrumentation approach compatible with the Arduino Due toolchain. Coverage reports shall be generated and archived as evidence artefacts for each software release.

# 4  Verification Plan (IEC 61508 Phases 6 and 9)

Verification activities confirm that the Arduino Due implementation correctly realises the FSRS requirements. Three levels of verification are applied: Module/Unit Testing, Integration Testing, and Timing Verification. All test cases align with and extend TC-01 through TC-18 defined in SADS-AIRBAG-ECU-001 Rev 2.0 Section 8.

## 4.1  Test Environment Setup

All verification tests shall be conducted in the following configured environment (per SADS Section 8.1):

- Connect both Arduino Due boards per HADS-AIRBAG-ECU-001 Rev 2.1 wiring tables; verify DM74LS08N AND gate connections at D8 output pins

- Connect OMS PC via USB-CAN-A; open USBCANV2 at 500 kbit/s, monitoring CAN IDs 0x100 and 0x200

- Open Serial Monitor (115200 baud) on both ECUs for fault flag and decision variable visibility

- Potentiometers POTI-1, POTI-2, POTI-3 set to full-minimum position (ADC ~0, below all thresholds) as default baseline

- Baby-seat switch open — D11 HIGH via pull-up to 3.3 V (adult mode) as default baseline

- Power supply: 12 V nominal; verify voltage within 9–16 V operating window before each session

Table 8: ADC Threshold Reference (12-bit, 0–4095)

| Sensor | Pin | ADC Range | Engineering Range | Deploy Threshold | ADC Threshold Count |
| --- | --- | --- | --- | --- | --- |
| Accelerometer (POTI-1) | A0 | 0–4095 | 0–100 g | > 30.0 g | raw > 1228 (29.97% of 4095) |
| Gyroscope (POTI-2) | A1 | 0–4095 | 0–360 °/s | > 120.0 °/s | raw > 1365 (33.33% of 4095) |
| Pressure (POTI-3) | A2 | 0–4095 | 0–500 kPa | > 200.0 kPa | raw > 1638 (40.00% of 4095) |
| Baby-Seat Switch | D11 | HIGH / LOW | TRUE / FALSE | LOW = baby seat | — (digital) |

## 4.2  Unit / Module Test Cases

### 4.2.1  TC-VER-001 — ADC Raw Value to Physical Unit Scaling Verification

Table 9: TC-VER-001

| Test ID | TC-VER-001 |
| --- | --- |
| Title | ADC Raw Value to Physical Unit Scaling Verification |
| FSRS Reference | FSR-01, FSR-02 |
| ASIL / SIL | ASIL D / SIL 3 |
| Module Under Test | MOD-02 (Sensor Acquisition) — POTI-1, POTI-2, POTI-3 12-bit ADC conversion and scaling functions |
| Method | Unit test with stubbed ADC reads; inject raw 12-bit ADC values (0–4095 range). Verify linear scaling to engineering units. |
| Input Conditions | POTI-1: ADC=0 → 0 g; ADC=2048 → ~50 g; ADC=4095 → 100 g POTI-2: ADC=0 → 0 °/s; ADC=2048 → ~180 °/s; ADC=4095 → 360 °/s POTI-3: ADC=0 → 0 kPa; ADC=2048 → ~250 kPa; ADC=4095 → 500 kPa Threshold boundary: POTI-1 ADC=1228 (30 g), ADC=1229 (30.024 g) |
| Scaling Formula | accel = (rawA0 / 4095.0) × 100.0; gyro = (rawA1 / 4095.0) × 360.0; press = (rawA2 / 4095.0) × 500.0 |
| Expected Output | Scaled values match linear formula within ±0.5% tolerance; no out-of-range values accepted; ADC values > 4095 raise faultADC flag |
| Pass Criterion | All nine boundary data points return correct physical values within tolerance; faultADC set for invalid ADC input; no spurious deployment flag generated |
| Coverage Target | 100% Statement; 100% Branch (valid / invalid ADC path) |
| Notes | The Arduino Due uses a 12-bit ADC (analogReadResolution(12) set in setup()). The earlier revision of this plan incorrectly used 10-bit values (0–1023). All test inputs must use 12-bit range (0–4095). |
| Status | NOT TESTED |

### 4.2.2  TC-VER-002 — Deployment Threshold Decision Logic (MC/DC)

Table 10: TC-VER-002

| Test ID | TC-VER-002 |
| --- | --- |
| Title | Deployment Threshold Decision Logic — Unit Verification with MC/DC |
| FSRS Reference | FSR-01, FSR-02, FSR-03, FSR-04 |
| ASIL / SIL | ASIL D / SIL 3 |
| Module Under Test | MOD-03 (Crash Detection) + MOD-04 (Deployment Control) — threshold evaluation and 6-condition voteDeploy logic |
| Method | Unit test with all six FSR-03 conditions exercised independently per MC/DC matrix. Each condition independently flipped to verify it independently affects the voteDeploy output. Covers SADS Section 4.3.2 and 4.4.1. |
| Input Conditions | Vary each of the six conditions (sensor threshold exceeded, localCrash, remoteCrash, !systemFault, !remoteFault, !babySeat) independently. Exercise all boundary combinations that determine voteDeploy = TRUE vs FALSE. |
| Expected Output | voteDeploy = TRUE only when ALL six conditions are simultaneously satisfied; any single condition FALSE → voteDeploy = FALSE; localCrash follows OR logic of three sensor thresholds |
| Pass Criterion | MC/DC matrix shows every condition independently affects deployment decision; zero false positives; zero false negatives; algorithm cycle time ≤ 1 ms measured via micros() timestamp (FSR-02) |
| Coverage Target | 100% MC/DC mandatory for ASIL D (ISO 26262 Part 6) |
| Status | NOT TESTED |

### 4.2.3  TC-VER-003 — Memory Integrity and Watchdog Supervision

Table 11: TC-VER-003

| Test ID | TC-VER-003 |
| --- | --- |
| Title | Memory Integrity and Watchdog Supervision — Unit Verification |
| FSRS Reference | FSR-22 |
| ASIL / SIL | ASIL D / SIL 3 |
| Module Under Test | MOD-01 (POST) + MOD-06 (Diagnostics) — CRC32 RAM verification, stack boundary monitor, WDT kick routine |
| Method | (a) Normal RAM: CRC pass — verify no action taken. (b) Inject deliberate RAM corruption at non-critical test address; verify CRC mismatch triggers SS-05 and DWI. (c) Inhibit watchdog kick for 12 ms (> 10 ms timeout); verify MCU resets. |
| Input Conditions | (a) Clean system state. (b) Manually corrupt test RAM variable. (c) Disable WDT kick call in loop() via conditional compilation flag. |
| Expected Output | (a) CRC check PASS, no action. (b) CRC mismatch: safe state SS-05 entered, DWI HIGH, deployment inhibited, DTC logged. (c) WDT fires within 10 ms, MCU resets. |
| Pass Criterion | All three scenarios produce expected safe-state behaviour within required timing; DTC logged for (b) and (c) |
| Coverage Target | 100% Branch on all CRC pass/fail and WDT paths |
| Notes | *GAP-06: WDT not currently enabled in firmware. Sub-test (c) is BLOCKED until GAP-06 is resolved. Sub-tests (a) and (b) can proceed independently. |
| Status | NOT TESTED (partially BLOCKED pending GAP-06) |

### 4.2.4  TC-VER-004 — CAN Frame Construction and CRC Calculation

Table 12: TC-VER-004

| Test ID | TC-VER-004 |
| --- | --- |
| Title | CAN Frame Construction, Bit Assignment, and CRC Calculation |
| FSRS Reference | FSR-13, FSR-14, FSR-16, FSR-16A |
| ASIL / SIL | ASIL B / SIL 2 (ASIL D end-to-end per FSR-16A) |
| Module Under Test | MOD-05 (CAN Communication) — frame packing, CRC calculation (bytes 6–7), parallel readback comparison |
| Method | Construct known deployment state vector; call frame packing function; verify each bit field at correct byte/bit position per Table 13 (CAN frame definition); independently calculate 16-bit CRC over bytes 0–5 and compare; inject 1-bit error and verify rejection. |
| Input Conditions | (a) Driver deploy=1, pax deploy=0, pax suppress=1, crash=1, Ch-A fault=0, Ch-B fault=0, DWI=0, safe-state=0 — expected Byte 0 = 0x0D. (b) Same frame with 1-bit corruption in byte 3. |
| Expected Output | (a) Byte 0 = 0x0D; CRC field in bytes 6–7 matches independently calculated value; parallel readback passes. (b) CRC mismatch → frame rejected; CAN fault DTC raised; parallel readback discrepancy detected. |
| Pass Criterion | Bit fields correctly placed in all 8 bytes; CRC matches in (a); frame correctly rejected in (b); parallel readback discrepancy detected in (b) |
| Coverage Target | 100% Statement and Branch in CAN packing and CRC routines |
| Notes | *GAP-03: CRC not currently implemented — bytes 6–7 carry status and ECU_ID in current firmware. *GAP-04: Parallel readback not implemented. Both sub-tests (a) CRC and (b) rejection are BLOCKED until GAP-03 and GAP-04 are resolved. Byte field assignments are per LCP04 FSRS Section 11.4. |
| Status | NOT TESTED (BLOCKED pending GAP-03 and GAP-04) |

## 4.3  Integration Test Cases

### 4.3.1  TC-VER-005 — Analog Sensor ADC Reading Hardware Integration

Table 13: TC-VER-005

| Test ID | TC-VER-005 |
| --- | --- |
| Title | POTI Sensor Raw ADC Reading — Hardware/Software Integration |
| FSRS Reference | FSR-01, FSR-07 |
| ASIL / SIL | ASIL D / SIL 3 |
| Method | Set POTI-1 to 0 V, 1.65 V (midpoint ±0.05 V), 3.3 V using calibrated bench power supply. Read ADC value via Serial debug output on both Channel A and Channel B Arduino Due boards independently. Repeat for POTI-2 and POTI-3. |
| Input Conditions | Three POTI positions: minimum (0 V → ADC ~0), midpoint (1.65 V → ADC ~2048), maximum (3.3 V → ADC ~4095) |
| Expected Output | Channel A ADC: 0, ~2048, ~4095 (±10 counts). Channel B ADC: identical to Channel A within ±5 counts. No sensor fault flags raised within valid range. |
| Pass Criterion | Both channels independently read correct ADC values within tolerance; cross-channel ADC values agree within ±5 counts; no faultADC flag raised during valid range input |
| Coverage Target | Hardware path coverage of ADC peripheral register access on both channels |
| Status | NOT TESTED |

### 4.3.2  TC-VER-006 — GPIO Output State and Deployment Command Timing

Table 14: TC-VER-006

| Test ID | TC-VER-006 |
| --- | --- |
| Title | GPIO Output State Readback and Deployment Command Timing |
| FSRS Reference | FSR-03, FSR-05, FSR-06 |
| ASIL / SIL | ASIL D / SIL 3 |
| Method | Issue a software-commanded deployment signal via debug injection (bypassing threshold check for test purposes). Measure output GPIO pin voltage with oscilloscope on D8 (Airbag1 vote) and D9 (Airbag2 vote). For side-airbag timing (FSR-06): apply step input on POTI-3 and measure time from threshold crossing to D8 HIGH using oscilloscope timestamp. |
| Input Conditions | (a) Deployment flag = FALSE: D8 and D9 expected LOW (< 0.4 V). (b) Deployment flag = TRUE (debug override): D8 expected HIGH (> 2.9 V). (c) Side-impact stimulus: POTI-3 stepped above ADC 1638 threshold. |
| Expected Output | (a) Measured voltage < 0.4 V for D8 and D9. (b) Measured voltage > 2.9 V for D8; D9 follows based on babySeat state. (c) D8 assertion within < 8 ms of POTI-3 threshold crossing. |
| Pass Criterion | Electrical state matches software intent in both conditions (a) and (b); timing from command to output < 1 ms via oscilloscope; side-impact response < 8 ms (FSR-06) |
| Coverage Target | End-to-end software-to-hardware output path; side-impact interrupt path |
| Status | NOT TESTED |

### 4.3.3  TC-VER-007 — CAN Frame Transmission Hardware Integration

Table 15: TC-VER-007

| Test ID | TC-VER-007 |
| --- | --- |
| Title | CAN Frame Transmission and Reception at OMS Lab PC — Hardware Integration |
| FSRS Reference | FSR-13, FSR-14, FSR-15, FSR-16, FSR-16A |
| ASIL / SIL | ASIL B / SIL 2 (ASIL D end-to-end per FSR-16A) |
| Method | Trigger crash simulation on both channels (POTI-1 and POTI-3 above threshold). Capture transmitted CAN frames at OMS lab PC using USBCANV2 CAN analyser at 500 kbit/s. Verify frame ID (0x100 or 0x200), data byte layout, CRC field (bytes 6–7), and inter-frame timestamp. |
| Input Conditions | Crash simulation: POTI-1 → ADC > 1228 (> 30 g); POTI-3 → ADC > 1638 (> 200 kPa); baby-seat switch HIGH (adult). Both channels active and CAN bus connected. |
| Expected Output | CAN frame ID = 0x100 (ECU1) or 0x200 (ECU2). Byte 0 Bit 0 = 1 (driver deploy); Byte 0 Bit 1 = 1 (pax deploy when no suppression); Byte 0 Bit 3 = 1 (crash detected). CRC bytes 6–7 valid. Frame timestamp within 15 ms of POTI threshold crossing. |
| Pass Criterion | All specified bit fields correct; CRC verification passes at receiver; parallel readback discrepancy count = 0; transmission timestamp ≤ 15 ms from threshold crossing; heartbeat frame interval = 500 ms ±50 ms over 20 consecutive frames |
| Notes | *GAP-03: CRC check portion BLOCKED until CRC is implemented. *GAP-04: Parallel readback check BLOCKED until readback channel is implemented. Frame ID, byte layout, and timing sub-tests can proceed. |
| Status | NOT TESTED (partially BLOCKED pending GAP-03, GAP-04) |

### 4.3.4  TC-VER-008 — Power-On Self-Test Complete Sequence

Table 16: TC-VER-008

| Test ID | TC-VER-008 |
| --- | --- |
| Title | Power-On Self-Test Complete Sequence Timing and Coverage |
| FSRS Reference | FSR-17 |
| ASIL / SIL | ASIL C / SIL 2 |
| Method | Power-cycle the Arduino Due system. Measure total POST duration using Serial timestamp output (millis() at POST start vs. millis() at completion). Verify each POST item (CPU, RAM, ADC, clock, CAN, baby-seat, power supply, DWI) produces a PASS log entry on Serial Monitor. |
| Input Conditions | All sensors within valid range; CAN bus connected and peer ECU responsive; baby-seat switch HIGH; power supply 12 V nominal. |
| Expected Output | POST completion time < 800 ms. POST items logged: cpuTest PASS, ramTest PASS, adcTest PASS, clock check PASS, CAN init PASS, baby-seat continuity PASS, supply voltage PASS, DWI lamp PASS. No DTC raised. DWI not illuminated. System transitions to ST-03 (MONITORING). |
| Pass Criterion | POST completes in < 800 ms; all nine checks pass; no spurious DTCs; DWI remains OFF; Serial output shows system enters ST-03; firmware state = SS-02 (Deployment Armed) |
| Notes | *GAP-05: CAN communication not verified during POST in current firmware (Can0.begin() success not checked). CAN POST sub-test is BLOCKED until GAP-05 is resolved. CPU, RAM, and ADC POST sub-tests can proceed. |
| Status | NOT TESTED (partially BLOCKED pending GAP-05) |

## 4.4  FSR-15 CAN Timeout — Known Deviation

DEVIATION NOTICE: FSRS FSR-15 requires CAN communication timeout detection within 50 ms. SADS-AIRBAG-ECU-001 Rev 2.0 Section 2.3 implements a 2000 ms heartbeat timeout (faultCOMM = TRUE when millis() - lastRemoteHB > 2000 ms). This exceeds the 50 ms specification by a factor of 40×. This deviation is documented in the SADS RTM (Section 9) as an open item against FSR-15. Until a design change or formal acceptance rationale is approved, TC-VAL-008 shall verify the implemented 2000 ms behaviour and note the FSR-15 non-conformance. An NCR shall be raised against this deviation.

# 5  Validation Plan (IEC 61508 Phases 7, 8 and 9)

System-level black-box validation cases are executed on the complete integrated test board to prove the system correctly mitigates the hazards identified in LCP03 HARA. Validation cases treat the ECU as a black box — input stimuli applied via potentiometer positions and digital switch states; outputs observed via CAN bus frames and DWI indicator state.

## 5.1  Boundary Value Validation Test Cases

### TC-VAL-001 — All Sensors at Minimum Boundary (No-Deploy)

Table 17: TC-VAL-001

| Test ID | TC-VAL-001 |
| --- | --- |
| Title | All Sensors at Minimum Boundary — No Deployment |
| Safety Goal | SG-01 — No unintended deployment |
| Governing Hazard | H1-01, H1-02 |
| ASIL / SIL | ASIL D / SIL 3 |
| Preconditions | POST complete; system in ST-03 (MONITORING) / SS-02; baby-seat switch HIGH (adult mode); both channels operational |
| Input Stimuli | POTI-1 = 0 V (ADC ~0, 0 g); POTI-2 = 0 V (ADC ~0, 0 °/s); POTI-3 = 0 V (ADC ~0, 0 kPa). Both ECU channels at minimum sensor values. |
| Expected Serial Output | localCrash=0, remoteCrash=0, systemFault=0, voteDeploy=0, D8=LOW, D9=LOW on both ECUs |
| Expected CAN Output | Byte 0 = 0x00 (no deployment bits set); Bit 3 = 0 (no crash detected) |
| Expected DWI State | OFF |
| Pass Criterion | No deployment CAN frame transmitted during 10-second monitoring period; Serial output shows crash_detected = 0 on both channels; DWI remains OFF; AND gate output confirmed LOW |
| Status | NOT TESTED |

### TC-VAL-002 — All Sensors at Maximum Boundary (Full Deploy)

Table 18: TC-VAL-002

| Test ID | TC-VAL-002 |
| --- | --- |
| Title | All Sensors at Maximum Boundary — Full Deployment Command |
| Safety Goal | SG-02 — Correct deployment during crash |
| Governing Hazard | H2-01, H2-02 |
| ASIL / SIL | ASIL D / SIL 3 |
| Preconditions | POST complete; system in SS-02; baby-seat switch HIGH (adult mode); both channels operational |
| Input Stimuli | POTI-1 = 3.3 V (ADC ~4095, 100 g); POTI-2 = 3.3 V (ADC ~4095, 360 °/s); POTI-3 = 3.3 V (ADC ~4095, 500 kPa); both channels simultaneously at maximum. |
| Expected CAN Output | Byte 0: Bit 0 = 1 (driver deploy); Bit 1 = 1 (pax deploy); Bit 2 = 0 (no suppression); Bit 3 = 1 (crash detected). CRC bytes 6–7 valid (when GAP-03 resolved). Frame received at OMS PC within 40 ms of threshold crossing. |
| Expected DWI State | OFF (no fault — valid crash event) |
| Pass Criterion | Deployment CAN frame received within 40 ms; both airbag bits set; Serial confirms voteDeploy=1 on both ECUs; AND gate fires (Airbag1 and Airbag2 LEDs ON if no baby-seat suppression); system latches post-deployment |
| Status | NOT TESTED |

### TC-VAL-003 — POTI-1 Stepped Through Deployment Threshold

Table 19: TC-VAL-003

| Test ID | TC-VAL-003 |
| --- | --- |
| Title | POTI-1 Stepped Through Deployment Threshold — Exact Boundary (30 g / ADC 1228) |
| Safety Goal | SG-01, SG-02 |
| Governing Hazard | H1-01, H2-01 |
| ASIL / SIL | ASIL D / SIL 3 |
| Preconditions | POTI-2 and POTI-3 below threshold; baby-seat switch HIGH; system in SS-02; both channels active |
| Input Stimuli | POTI-1 stepped from 0 V to 3.3 V in increments. Key test points: ADC 1227 (below threshold); ADC 1228 (at threshold); ADC 1229 (above threshold). Threshold voltage = (1228/4095) × 3.3 V = 0.990 V. |
| Expected CAN Output | At ADC < 1228: no deployment (Byte 0 = 0x00). At ADC ≥ 1228: deployment command issued on both channels (Byte 0 Bit 0 = 1). Threshold response repeatable across 3 consecutive test runs. |
| Expected DWI State | OFF throughout (valid operation, no fault) |
| Pass Criterion | No deployment below ADC 1228; deployment occurs at ADC ≥ 1228; response is repeatable; no hysteresis anomalies observed |
| Status | NOT TESTED |

## 5.2  Critical Safety Use Cases

### TC-VAL-004 — Use Case A: Simulated Frontal Crash — Full Deployment

Table 20: TC-VAL-004

| Test ID | TC-VAL-004 |
| --- | --- |
| Title | Use Case A — Simulated Frontal Crash: Driver and Passenger Airbag Deployment |
| Safety Goal | SG-02 |
| Governing Hazard | H2-01 |
| ASIL / SIL | ASIL D / SIL 3 |
| Preconditions | POST complete; system in SS-02; baby-seat switch HIGH (adult occupant); both Channel A and Channel B operational; CAN bus connected to OMS PC |
| Input Stimuli | POTI-1 advanced to maximum ADC (~4095, 100 g) — simulates frontal crash pulse > 30 g. POTI-3 advanced to ADC > 1638 (> 200 kPa). Baby-seat switch remains HIGH throughout. |
| Expected CAN Output | Byte 0 = 0x0B: Bit 0 (driver)=1, Bit 1 (pax)=1, Bit 2 (suppress)=0, Bit 3 (crash)=1. CRC valid. Frame at OMS PC within 40 ms. |
| Expected DWI State | OFF during deployment (valid crash — not a fault condition). DWI behaviour post-deployment per firmware implementation. |
| Pass Criterion | Both driver and passenger airbag deployment bits set in CAN frame; deployment within 40 ms; CRC valid (when GAP-03 resolved); Serial confirms voteDeploy=1 on both channels; AND gate fires; Airbag1 LED ON, Airbag2 LED ON |
| Status | NOT TESTED |

### TC-VAL-005 — Use Case B: Baby Seat Present — Passenger Airbag Suppressed

Table 21: TC-VAL-005

| Test ID | TC-VAL-005 |
| --- | --- |
| Title | Use Case B — Baby Seat Present: Passenger Airbag Suppressed During Simulated Crash |
| Safety Goal | SG-03 |
| Governing Hazard | H1-04 |
| ASIL / SIL | ASIL D / SIL 3 |
| Preconditions | POST complete; system in SS-02; baby-seat switch LOW (baby seat detected / D11 = LOW); both channels operational |
| Input Stimuli | Baby-seat switch maintained LOW. POTI-1 advanced to ADC > 1228 (> 30 g). POTI-3 advanced to ADC > 1638 (> 200 kPa). Crash conditions met on both channels. |
| Expected CAN Output | Byte 0: Bit 0 = 1 (driver DEPLOY); Bit 1 = 0 (passenger SUPPRESSED); Bit 2 = 1 (suppression active); Bit 3 = 1 (crash detected). CRC valid. |
| Expected DWI State | OFF. Baby-seat active suppression is normal operation per LCP05 SCD Section 7.4 — it is NOT a fault condition. DWI shall NOT illuminate for active baby-seat suppression. |
| Pass Criterion | Driver airbag deploys (Airbag1 LED ON); passenger airbag suppressed (Airbag2 LED OFF, Byte 0 Bit 1 = 0, Bit 2 = 1); CAN frame received within 40 ms; DWI remains OFF; SG-03 satisfied |
| Notes | CORRECTION from draft V1.0: The draft incorrectly stated DWI = ON for active baby-seat suppression. Active suppression (babySeat=TRUE) is the intended fail-safe default — it is not a fault. DWI activates only for fault conditions (systemFault=TRUE). Reference: SADS Section 4.4.1, Deployment Control logic. |
| Status | NOT TESTED |

### TC-VAL-006 — Use Case B2: Baby Seat Line Open-Circuit — Suppress-on-Fault

Table 22: TC-VAL-006

| Test ID | TC-VAL-006 |
| --- | --- |
| Title | Use Case B2 — Baby Seat Line Open-Circuit: Suppress-on-Fault Strategy |
| Safety Goal | SG-03 |
| Governing Hazard | H1-04 (stuck-HIGH fault) |
| ASIL / SIL | ASIL D / SIL 3 |
| Preconditions | System armed; baby-seat switch initially HIGH (adult mode); CAN bus connected |
| Input Stimuli | Physically disconnect the baby-seat digital input wire (simulates open-circuit / stuck-HIGH fault per GAP-02). Then advance POTI-1 beyond deployment threshold (ADC > 1228). |
| Expected Output | After line disconnect: pull-up holds D11 HIGH → babySeat=FALSE in current firmware (GAP-02: line continuity check not implemented, so stuck-HIGH not actively detected). During crash simulation: passenger airbag bit = 0 (suppressed only if babySeat forced TRUE by continuity check — see GAP-02). DWI: ON within 800 ms if continuity check detects fault (GAP-02 required). |
| Expected DWI State | ON within 800 ms (requires GAP-02 continuity check to be implemented) |
| Pass Criterion | When GAP-02 is resolved: DWI activates within 800 ms of disconnect; DTC logged for baby-seat line fault; passenger airbag suppressed despite input reading HIGH; driver airbag remains armed. Until GAP-02 resolved: this test is BLOCKED for the suppress-on-fault aspect. |
| Notes | *GAP-01: Debounce not implemented — affects TC-VAL-006 signal stability sub-test. *GAP-02: Line continuity monitoring not implemented — primary reason this test is partially BLOCKED. |
| Status | NOT TESTED (BLOCKED pending GAP-01 and GAP-02) |

### TC-VAL-007 — Use Case C(i): Sensor Out-of-Range — DWI and Safe State

Table 23: TC-VAL-007

| Test ID | TC-VAL-007 |
| --- | --- |
| Title | Use Case C(i) — POTI Sensor Out-of-Range: DWI Activation and Safe State |
| Safety Goal | SG-06 |
| Governing Hazard | H4-01 |
| ASIL / SIL | ASIL C / SIL 2 |
| Preconditions | System in normal operation (SS-02 / ST-03); baby-seat switch HIGH |
| Input Stimuli | Force POTI-1 input above 3.3 V (e.g., connect to 5 V supply) or disconnect to create open-circuit (ADC reads 0 or max rail at out-of-range detection boundary). Maintain for > 200 ms. |
| Expected CAN Output | Byte 4 sensor fault bits set for POTI-1; Byte 0 Bit 6 (DWI active) = 1; DTC for sensor out-of-range stored |
| Expected DWI State | ON within 800 ms of fault detection (FSR-19) |
| Pass Criterion | DWI activates within 800 ms; DTC logged; affected sensor channel isolated (SS-04 state); remaining channels continue monitoring; no spurious deployment command issued during fault condition |
| Status | NOT TESTED |

### TC-VAL-008 — Use Case C(ii): CAN Timeout — Limp-Home Mode

Table 24: TC-VAL-008

| Test ID | TC-VAL-008 |
| --- | --- |
| Title | Use Case C(ii) — CAN Bus Communication Timeout: Limp-Home / Fault Mode |
| Safety Goal | SG-06, SG-07 |
| Governing Hazard | H4-01, H4-02 |
| ASIL / SIL | ASIL B / SIL 2 |
| Preconditions | CAN bus active; both ECUs transmitting heartbeat frames at 500 ms interval; OMS PC receiving frames normally |
| Input Stimuli | Physically disconnect the CAN bus between ECU1 and ECU2 (remove twisted pair / SN65HVD230 connection). Maintain disconnection for > 2100 ms. |
| Expected Output | After 2000 ms timeout: faultCOMM=TRUE → systemFault=TRUE → voteDeploy=FALSE → D8/D9 LOW. DWI (D10) HIGH. System enters ST-04 (FAULT_ACTIVE) / SS-01 (deployment inhibit). |
| Expected DWI State | ON within 2000 ms + 1 cycle (approximately 2001 ms from last valid frame) |
| Pass Criterion | faultCOMM declared ≤ 2050 ms after last valid frame; DWI activates; DTC logged; no unintended deployment or inhibition occurs during timeout period; system recovers to ST-03 when CAN reconnected (TC-09 equivalent) |
| Notes | FSR-15 DEVIATION: FSRS requires timeout ≤ 50 ms; current implementation uses 2000 ms. This test validates the implemented 2000 ms behaviour. An NCR is raised against this deviation (see Section 1.2 Unresolved Topics). |
| Status | NOT TESTED (NCR required for FSR-15 deviation) |

### TC-VAL-009 — Single-Channel Fault: Channel A Forced Fault

Table 25: TC-VAL-009

| Test ID | TC-VAL-009 |
| --- | --- |
| Title | Single-Channel Fault Injection — Channel A Forced Fault, Channel B Continues |
| Safety Goal | SG-01 |
| Governing Hazard | H1-01 |
| ASIL / SIL | ASIL D / SIL 3 |
| Preconditions | Both channels operational; system in SS-02; CAN bus connected; OMS PC monitoring |
| Input Stimuli | Inject a software fault flag on Channel A (e.g., temporarily modify cpuTest() to return false — see SADS Section 4.1.1 WB test). Simultaneously advance POTI-1 beyond ADC 1228 (> 30 g) on both ECUs. |
| Expected Output | Channel A: faultCPU=TRUE → systemFault=TRUE → voteDeploy=FALSE → D8=LOW. Channel A CAN frame Byte 0 Bit 4 (Ch-A fault) = 1. 2oo2 AND gate: both D8 pins must be HIGH for deployment — Channel A D8=LOW prevents AND gate from firing. No deployment output. DWI ON. |
| Expected DWI State | ON |
| Pass Criterion | No deployment command issued with single-channel fault active; DWI ON within 800 ms; AND gate remains LOW (Airbag1 LED OFF); Byte 0 Bit 4 = 1 in CAN frame from Channel A; SC-03 (no single-point fault causes deployment) verified; FSR-07 and FSR-08 satisfied |
| Status | NOT TESTED |

### TC-VAL-010 — EMC Disturbance Rejection: Single Sensor Spike

Table 26: TC-VAL-010

| Test ID | TC-VAL-010 |
| --- | --- |
| Title | Single-Sensor Spike Rejection — EMC Disturbance Simulation |
| Safety Goal | SG-05 |
| Governing Hazard | H1-05 |
| ASIL / SIL | ASIL C / SIL 2 |
| Preconditions | System in SS-02; all sensors below threshold; baby-seat switch HIGH; both channels operational |
| Input Stimuli | Rapidly advance POTI-1 alone above the deployment threshold (ADC > 1228) on ECU1 only. POTI-1 on ECU2 remains below threshold. POTI-2 and POTI-3 remain below threshold on both ECUs. Hold for 500 ms. |
| Expected Output | ECU1: localCrash=1 (POTI-1 exceeded threshold on ECU1). ECU2: localCrash=0 (POTI-1 below threshold on ECU2). ECU1 remoteCrash=0 (from ECU2 CAN frame). ECU1 voteDeploy=FALSE (remoteCrash=0 fails the AND-gate). D8=LOW on ECU1. AND gate cannot fire — both D8 outputs NOT simultaneously HIGH. DWI: OFF (no system fault — single-sensor above threshold is not a fault condition). |
| Expected DWI State | OFF. Single-channel crash detection with no peer agreement is not a fault — it is the expected 2oo2 protection mechanism. |
| Pass Criterion | No deployment output (AND gate remains LOW); Serial on ECU1 shows localCrash=1 but voteDeploy=0; Serial on ECU2 shows localCrash=0; DWI remains OFF; SG-05 and FSR-04 verified; 2oo2 EMC protection demonstrated |
| Notes | CORRECTION from draft V1.0: The draft incorrectly implied single-sensor above threshold triggers localCrash=FALSE. Per SADS Section 4.3.2, localCrash = TRUE if ANY single sensor exceeds its threshold. However, the 2oo2 architecture prevents deployment because remoteCrash (from the peer ECU) = FALSE, making voteDeploy = FALSE. The safety is provided by the 2oo2 vote, not by sensor fusion requiring multiple simultaneous thresholds within a single channel. |
| Status | NOT TESTED |

# 6  Timing Requirements Verification Summary

The following timing parameters shall be verified and validated during V&V execution. All measurements shall be performed with calibrated instrumentation (oscilloscope or CAN analyser with timestamp resolution ≤ 0.1 ms).

Table 27: Timing Requirements Verification

| Function | Max Allowed | Verification Method | Test Case(s) |
| --- | --- | --- | --- |
| Crash detection algorithm cycle | ≤ 1 ms | micros() start/end of loop(); 99th percentile over 10,000 iterations | TC-VER-002 (sub-test timing) |
| Crash detection response (threshold to decision) | ≤ 20 ms | POTI threshold crossing vs. localCrash=TRUE in Serial log | TC-VAL-003, TC-VAL-004 |
| Deployment command transmission (threshold to CAN TX) | ≤ 40 ms | CAN analyser timestamp vs POTI crossing | TC-VAL-002, TC-VAL-004 |
| Side-impact deployment logic | < 8 ms | Oscilloscope: POTI-3 step input to D8 HIGH edge | TC-VER-006 |
| Pretensioner activation (if implemented) | < 5 ms | Output GPIO timing measurement | TC-VER-006 |
| CAN timeout detection (FSR-15 specified) | ≤ 50 ms | Disconnect CAN; measure faultCOMM assertion time | TC-VAL-008 *DEVIATION: impl=2000 ms |
| Diagnostic cycle interval | ≤ 100 ms | Background task Serial timestamp logging | TC-VER-002, TC-VAL-007 |
| POST execution (complete before arming) | < 800 ms | Serial timestamp: POST start vs. ST-03 entry | TC-VER-008 |
| DWI activation after fault detection | ≤ 800 ms | DWI GPIO rising edge vs. fault detection timestamp | TC-VAL-006, TC-VAL-007 |
| Baby-seat debounce stability | ≥ 10 ms stable | Signal generator + oscilloscope (when GAP-01 resolved) | TC-VAL-006 *GAP-01 |
| Crash notification CAN frame | ≤ 15 ms from deployment auth | CAN analyser deployment frame timestamp | TC-VER-007 |
| CAN heartbeat interval | 500 ms ± 50 ms | USBCANV2 inter-frame timestamps; 20 frames | TC-VER-007 |

# 7  Test Report Framework

## 7.1  Test Case Record Template

Every executed test case shall produce a completed Test Case Record using the following standardised template. Test records shall be archived as formal safety evidence artefacts.

Table 28: Test Case Record Template

| Test Case ID | TC-VER-xxx / TC-VAL-xxx |
| --- | --- |
| Test Case Title | <Short descriptive title> |
| Date of Execution | YYYY-MM-DD |
| Executed By (Test Engineer) | <Name / Role — must be independent of development> |
| Reviewed By (V&V Lead) | <Name / Role> |
| FSRS Requirement(s) | FSR-xx, FSR-yy |
| Safety Goal(s) | SG-xx |
| Governing Hazard(s) | Hx-xx |
| ASIL / SIL | ASIL x / SIL x |
| Hardware Configuration | Arduino Due Channel A: FW Rev x.x ¦ Channel B: FW Rev x.x ¦ AND gate: DM74LS08N |
| CAN Interface | OMS Lab PC USB-CAN-A interface model: <model> ¦ USBCANV2 at 500 kbit/s |
| POTI-1 Setting (Accelerometer) | <ADC count> → <Equivalent g value> (threshold boundary if applicable) |
| POTI-2 Setting (Gyroscope) | <ADC count> → <Equivalent °/s value> |
| POTI-3 Setting (Pressure Sensor) | <ADC count> → <Equivalent kPa value> |
| Baby-Seat Switch State | HIGH (adult) / LOW (baby seat) / Disconnected (fault) |
| Expected CAN Frame (Byte 0–Byte 7) | <Hex values, e.g., 0x0B 0x01 0xAB 0xCD 0x00 0x00 0xCR 0xCC> |
| Expected DWI State | ON / OFF — <Rationale> |
| Expected Timing | <Max allowed time, e.g., Deployment within 40 ms> |
| Actual CAN Frame Received | <Captured hex values from OMS PC USBCANV2 log> |
| CRC Verification Status | PASS / FAIL — <Expected CRC vs received CRC> (*GAP-03 if not yet implemented) |
| Parallel Readback Result | PASS / FAIL — <Discrepancy count> (*GAP-04 if not yet implemented) |
| Actual DWI State | ON / OFF — <Observed> |
| Actual Timing Measured | <ms measured from threshold to CAN frame or GPIO edge> |
| Serial Monitor Snapshot | <Key variables: localCrash, remoteCrash, systemFault, voteDeploy, babySeat, A0, A1, A2> |
| Deviation / Anomaly Notes | <Any unexpected behaviour, non-conformances> |
| PASS / FAIL Verdict | ✓ PASS ✗ FAIL ⚠ BLOCKED |
| Non-Conformance Reference | NCR-xxx (if FAIL) / N/A |
| Signature (Test Engineer) | __________________________ Date: ____________ |
| Signature (V&V Lead) | __________________________ Date: ____________ |

## 7.2  Serial Monitor Evidence Collection

For each test case, the Serial Monitor output shall be captured and archived. The following variable set shall be recorded as minimum evidence that deployment thresholds were reached or not reached (from SADS Section 8.5):

Table 29: Required Serial Monitor Variables per Test Case

| Variable | Meaning / Evidence Value |
| --- | --- |
| ECU=1 or ECU=2 | Identifies which channel produced this Serial output |
| A0=<value> | Raw ADC count for POTI-1 (accelerometer); must be 0–4095 range |
| A1=<value> | Raw ADC count for POTI-2 (gyroscope) |
| A2=<value> | Raw ADC count for POTI-3 (pressure sensor) |
| Baby=<0/1> | Baby-seat switch state: 1=LOW (baby seat), 0=HIGH (adult) |
| Local Crash=<0/1> | localCrash flag: 1 if any sensor exceeded threshold on this channel |
| Remote Crash=<0/1> | remoteCrash flag: peer ECU crash detection state from CAN |
| Comm Fault=<0/1> | faultCOMM flag: 1 if peer CAN timeout exceeded 2000 ms |
| System Fault=<0/1> | systemFault aggregate flag: OR of faultCPU, faultRAM, faultADC, faultCOMM |
| Vote Deploy=<0/1> | voteDeploy: final deployment authorization output |

## 7.3  Test Summary Report Structure

Upon completion of all test cases, a Test Summary Report shall be produced consolidating all individual test records. The report shall include:

- Document header: document number, revision, date, system, applicable standards, author

- Test scope summary: total number of test cases planned, executed, passed, failed, blocked

- ASIL D / SIL 3 requirements coverage: table confirming 100% FSR coverage with PASS status

- Safety goal validation summary: table confirming each SG validated by at least one PASS test

- Traceability matrix: final completed RTM with all test statuses populated

- Compliance gap closure: confirmation that GAP-01 to GAP-07 are resolved and re-tested

- Non-conformance summary: list of all NCRs raised (including FSR-15 CAN timeout deviation), status, and resolution

- Residual risks: any identified residual risks not fully mitigated by V&V results

- Approval signatures: Functional Safety Manager, V&V Lead, Project Supervisor

## 7.4  Non-Conformance Management

Any test case resulting in FAIL or BLOCKED shall trigger the following process:

- Raise a Non-Conformance Report (NCR) with unique identifier (NCR-xxx)

- Classify NCR severity: Critical (ASIL D / SIL 3 requirement violated), Major (ASIL B-C / SIL 1-2), or Minor

- Assign corrective action owner and target resolution date

- Re-execute the affected test case after corrective action implementation

- V&V Lead reviews and signs off NCR closure; update RTM status from FAIL to PASS

# 8  Requirements Traceability Matrix (RTM)

The following matrix provides full bidirectional traceability from HARA hazards through safety goals, FSRS requirements, to V&V test cases. All statuses are set to NOT TESTED at plan issue. All ASIL D / SIL 3 requirements shall achieve PASS status before system release. Entries marked *GAP-xx are dependent on firmware compliance gap resolution.

Table 30: Requirements Traceability Matrix

| Hazard ID | Safety Goal | ASIL/SIL | FSR ID | FSR Short Title | SC Mechanism | Code Module(s) | Test Case(s) | Coverage Type | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H2-01 | SG-02 | ASIL D / SIL 3 | FSR-01 | Crash Detection Timing | Cyclic executive ≤1 ms | MOD-02, MOD-03 | TC-VER-002, TC-VAL-003 | BB + Timing | NOT TESTED |
| H2-04 | SG-02 | ASIL D / SIL 3 | FSR-02 | Algorithm Cycle Time | ≤1 ms loop discipline | All modules | TC-VER-002 | Timing | NOT TESTED |
| H1-01 | SG-01 | ASIL D / SIL 3 | FSR-03 | Deployment Auth (6-cond) | 2oo2 AND-gate vote logic | MOD-04 | TC-VER-002, TC-VAL-009 | BB + WB MC/DC | NOT TESTED |
| H1-01, H1-02 | SG-01 | ASIL D / SIL 3 | FSR-04 | Prevention Unintended Deploy | 2oo2 + inhibit logic | MOD-04 | TC-VAL-001, TC-VAL-010 | BB | NOT TESTED |
| H2-01 | SG-02 | ASIL D / SIL 3 | FSR-05 | Deployment Command Timing | ≤40 ms from crash detect | MOD-04, MOD-05 | TC-VAL-002, TC-VAL-004 | Timing | NOT TESTED |
| H2-02 | SG-02 | ASIL D / SIL 3 | FSR-06 | Side-Airbag Timing | <8 ms interrupt path | MOD-03, MOD-04 | TC-VER-006 | Timing | NOT TESTED |
| H1-01 | SG-01 | ASIL D / SIL 3 | FSR-07 | Dual-Channel Redundancy | 2oo2 architecture | MOD-02–MOD-05 | TC-VAL-009 | Integration BB | NOT TESTED |
| H1-01 | SG-01 | ASIL D / SIL 3 | FSR-08 | Channel Agreement Verification | Cross-channel CAN vote | MOD-04, MOD-05 | TC-VAL-009 | BB | NOT TESTED |
| H1-04 | SG-03 | ASIL D / SIL 3 | FSR-09 | Passenger Airbag Suppression | Baby-seat suppress logic | MOD-04 | TC-VAL-005 | BB | NOT TESTED |
| H1-04 | SG-03 | ASIL D / SIL 3 | FSR-10 | Suppress-on-Fault Strategy | Default suppress on fault | MOD-04 | TC-VAL-006 | BB Fault Inject | NOT TESTED |
| H1-04 | SG-03 | ASIL D / SIL 3 | FSR-11 | Digital Input Debounce | 10 ms stable filter | MOD-02 | TC-VAL-006 *GAP-01 | BB | NOT TESTED |
| H1-04 | SG-03 | ASIL D / SIL 3 | FSR-12 | Baby Seat Line Monitoring | Continuity check | MOD-06 | TC-VAL-006 *GAP-02 | BB Fault Inject | NOT TESTED |
| H4-02 | SG-07 | ASIL B / SIL 2 | FSR-13 | CRC-Protected CAN | 16-bit CRC over bytes 0–5 | MOD-05 | TC-VER-004, TC-VER-007 *GAP-03 | Unit + Integration | NOT TESTED |
| H4-02 | SG-07 | ASIL B / SIL 2 | FSR-14 | CAN Bitstream Readback | Parallel readback channel | MOD-05 | TC-VER-004, TC-VER-007 *GAP-04 | Integration | NOT TESTED |
| H4-02 | SG-07 | ASIL B / SIL 2 | FSR-15 | CAN Timeout Detection (≤50 ms) | CAN frame timeout | MOD-05, MOD-06 | TC-VAL-008 *NOTE: impl=2000ms | BB | NOT TESTED |
| H4-02 | SG-07 | ASIL B / SIL 2 | FSR-16 | Crash Notification Timing | CAN TX within 15 ms | MOD-05 | TC-VER-007 | Timing | NOT TESTED |
| H1-01, H2-01, H4-02 | SG-01/SG-07 | ASIL D / SIL 3 | FSR-16A | ASIL D CAN End-to-End Integrity | CRC + readback + vote + timeout | MOD-04, MOD-05 | TC-VER-004, TC-VER-007, TC-VAL-009 | Integration | NOT TESTED |
| H4-01 | SG-06 | ASIL C / SIL 2 | FSR-17 | Power-On Self-Test | POST 800 ms sequence | MOD-01 | TC-VER-008 *GAP-05 | Integration | NOT TESTED |
| H4-01 | SG-06 | ASIL C / SIL 2 | FSR-18 | Continuous Diagnostics ≤100 ms | Background diag cycle | MOD-06 | TC-VAL-007, TC-VAL-008 | BB | NOT TESTED |
| H4-01 | SG-06 | ASIL A / SIL 1 | FSR-19 | DWI Activation ≤800 ms | Fault → D10 HIGH ≤800 ms | MOD-04, MOD-06 | TC-VAL-006, TC-VAL-007 | Timing | NOT TESTED |
| H1-01 | SG-01 | ASIL D / SIL 3 | FSR-20 | Safe-State Transition | systemFault → voteDeploy=FALSE | MOD-04, MOD-06 | TC-VAL-007, TC-VAL-009 | BB Fault Inject | NOT TESTED |
| H4-01 | SG-06 | ASIL A / SIL 1 | FSR-21 | Fault Logging (DTCs) | DTC to EEPROM/flash | MOD-06 | TC-VAL-006, TC-VAL-007, TC-VAL-008 *GAP-07 | BB | NOT TESTED |
| H1-01, H2-01, H4-01 | SG-01/SG-06 | ASIL D / SIL 3 | FSR-22 | Memory Integrity Protection | CRC32 RAM + WDT + stack guard | MOD-01, MOD-06 | TC-VER-003 *GAP-06 | Unit WB | NOT TESTED |

*GAP notation: Test cases marked *GAP-xx cannot achieve PASS status until the corresponding firmware compliance gap (listed in Section 2.4 and SADS Rev 2.0 Section 7) is resolved and verified. The FSR-15 deviation (CAN timeout 2000 ms vs. 50 ms specified) requires a formal NCR and acceptance rationale.

# 9  References and Normative Standards

Table 31: References

| Reference | Description |
| --- | --- |
| ISO 26262:2018 Parts 1–12 | Road Vehicles — Functional Safety |
| IEC 61508:2010 Parts 1–7 | Functional Safety of Electrical/Electronic/Programmable Electronic Safety-related Systems |
| LCP01-AIRBAG-ECU-001 v1.10 | Concept Phase Document — item definition, system boundary, team roles |
| LCP03-AIRBAG-ECU-001 v2.00 | Hazard and Risk Analysis (HARA) — hazards H1-01 to H4-02, safety goals SG-01 to SG-07 |
| LCP04-AIRBAG-ECU-001 v1.00 | Functional Safety Requirements Specification (FSRS) — FSR-01 to FSR-22 |
| LCP05-AIRBAG-ECU-001 v1.00 | Safety Concept Document (SCD) — safety mechanisms, safe states SS-01 to SS-05 |
| SADS-AIRBAG-ECU-001 Rev 2.0 | Software Architecture and Design Specification — module decomposition, test cases TC-01 to TC-18, GAP-01 to GAP-07 |
| ISO 21448:2022 | Safety of the Intended Functionality (SOTIF) |
| ISO 14229-1:2020 | Unified Diagnostic Services (UDS) |
| ISO 11898 | Road Vehicles — Controller Area Network (CAN) |
| SAE J211 | Instrumentation for Impact Tests |
| FMVSS No. 208 | Occupant Crash Protection — U.S. Department of Transportation, NHTSA |

# 10  List of Dependent Documents

Table 32 lists all documents referenced by or related to this V&V Plan.

Table 32: Dependent Documents

| No. | Document Name | Version | Date | File Name |
| --- | --- | --- | --- | --- |
| 1 | Life Cycle Phase 1 — Concept Document | 1.10 | 2026-05-03 | LCP01-AIRBAG-ECU-001_Concept_v110.docx |
| 2 | Life Cycle Phase 2 — Task of the Process or Product | 1.10 | 2026-05-03 | LCP02-AIRBAG-ECU-001_Concept_v_110.docx |
| 3 | Life Cycle Phase 3 — Hazard and Risk Analysis (HARA) | 2.00 | 2026-05-05 | LCP03-AIRBAG-ECU-001_HARA_v110.docx |
| 4 | Life Cycle Phase 4 — Functional Safety Requirements Specification | 1.00 | 2026-05-11 | LCP04-AIRBAG-ECU-001_FSRS_v100.docx |
| 5 | Life Cycle Phase 5 — Safety Concept Document | 1.00 | 2026-05-12 | LCP05-AIRBAG-ECU-001_SCD_v100.docx |
| 6 | Software Architecture and Design Specification | Rev 2.0 | 2026-06-12 | SADS-AIRBAG-ECU-001_Rev2_0.docx |
| 7 | ISO 26262:2018 — Road Vehicles — Functional Safety | 2018 | — | External standard |
| 8 | IEC 61508:2010 — Functional Safety of E/E/PE Systems | 2010 | — | External standard |

# 11  Figure and Table Listing

Table 1: Meetings within the Project

Table 2: Unresolved Topics

Table 3: Glossary

Table 4: Organisational Roles and Independence

Table 5: Firmware Compliance Gaps Affecting V&V

Table 6: RTM Column Definitions

Table 7: Code Coverage Targets

Table 8: ADC Threshold Reference (12-bit, 0–4095)

Table 9: TC-VER-001 — ADC Scaling Verification

Table 10: TC-VER-002 — Deployment Logic MC/DC

Table 11: TC-VER-003 — Memory Integrity and WDT

Table 12: TC-VER-004 — CAN Frame and CRC

Table 13: TC-VER-005 — Sensor ADC Integration

Table 14: TC-VER-006 — GPIO Output Timing

Table 15: TC-VER-007 — CAN Transmission Integration

Table 16: TC-VER-008 — Power-On Self-Test

Table 17: TC-VAL-001 — Minimum Boundary No-Deploy

Table 18: TC-VAL-002 — Maximum Boundary Full Deploy

Table 19: TC-VAL-003 — Threshold Boundary Step Test

Table 20: TC-VAL-004 — Use Case A Frontal Crash

Table 21: TC-VAL-005 — Use Case B Baby Seat Suppression

Table 22: TC-VAL-006 — Use Case B2 Suppress-on-Fault

Table 23: TC-VAL-007 — Use Case C(i) Sensor Out-of-Range

Table 24: TC-VAL-008 — Use Case C(ii) CAN Timeout

Table 25: TC-VAL-009 — Single-Channel Fault Injection

Table 26: TC-VAL-010 — EMC Single-Sensor Spike Rejection

Table 27: Timing Requirements Verification

Table 28: Test Case Record Template

Table 29: Required Serial Monitor Variables

Table 30: Requirements Traceability Matrix

Table 31: References

Table 32: Dependent Documents

Table 33: Version History

# 12  Version History

Table 33: Version History

| Document Name / File | Date | Change Log |
| --- | --- | --- |
| VVP-AIRBAG-ECU-001.docx | 2026-05-15 | Version 1.0 — Initial draft release. Covered verification test cases TC-VER-001 to TC-VER-008 and validation cases TC-VAL-001 to TC-VAL-010. RTM for FSR-01 to FSR-22. Test report template included. |
| VVP-AIRBAG-ECU-001_v110.docx | 2026-06-12 | Version 1.10 — Major revision. Corrections: (1) ADC range corrected from 10-bit (0–1023) to 12-bit (0–4095) throughout, matching Arduino Due hardware configuration in SADS Rev 2.0. (2) TC-VAL-005 DWI state corrected: active baby-seat suppression is normal operation, not a fault — DWI shall be OFF. (3) TC-VAL-010 expected output corrected: single-sensor above threshold triggers localCrash=TRUE on that channel; deployment prevented by 2oo2 vote (remoteCrash=FALSE on peer), not by multi-sensor fusion within one channel. (4) All RTM statuses reset to NOT TESTED (draft incorrectly showed PASS for all entries before any testing). (5) Section 2.4 added: seven firmware compliance gaps GAP-01 to GAP-07 from SADS Rev 2.0 documented and cross-referenced throughout. (6) FSR-15 CAN timeout deviation (50 ms spec vs 2000 ms implementation) formally identified and notated in RTM and TC-VAL-008. (7) Document metadata updated to include SADS Rev 2.0 as input reference. (8) Document style aligned with LCP01 project template. |

# 13  Approval and Sign-Off

Table 34: Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Functional Safety Manager | Kruthika S. Gowda | __________________ | ____________ |
| Testing Engineer V&V Lead | Shivbaraje Bhosale | __________________ | ____________ |
| Software Safety Engineer | Mariana Chavez Avila | __________________ | ____________ |
| Hardware Safety Engineer | Atharva Avsare / Om Naik | __________________ | ____________ |
| Quality Manager | Thirthashri G. Ganesh | __________________ | ____________ |
| Project Supervisor |  | __________________ | ____________ |