Airbag Electronic Control Unit (ECU) SRS-AIRBAG-ECU-001 ¦ Revision 3.1 ¦ 2026-06-14 SOFTWARE REQUIREMENTS SPECIFICATION ISO 26262:2018 (ASIL D) / IEC 61508:2010 (SIL 3)

ASIL D (ISO 26262) / SIL 3 (IEC 61508)

| Field | Value |
| --- | --- |
| Document Number | SRS-AIRBAG-SW-001 |
| Revision | 3.1 |
| Date | 2026-06-14 |
| Author | Om naik, Mrunal Ghotekar |
| Status | RELEASED — final code baseline for V&V |
| ASIL Classification | ASIL D (ISO 26262) / SIL 3 (IEC 61508) |
| Target Hardware | Arduino Due (SAM3X8E ARM Cortex-M3) × 2 |
| Code Baseline | Confirmed firmware — Both boards identical except ECU_ID, DATA_ID, REMOTE_ID |
| Based On | LCP03-AIRBAG-ECU-001_HARA_v110 ¦ LCP04-AIRBAG-ECU-001_FSRS_v100 ¦ LCP05-AIRBAG-ECU-001_SCD_v100¦ HADS-AIRBAG-ECU-001 Rev 2.0 Final |

Table of Contents

1. Glossary	1

2. Overview	2

3. FSR Compliance Status	2

4. System Overview	3

4.1 Board configuration — only these 3 lines differ	3

4.2 CAN frame layout — DLC=8 (final)	3

5. Software Module Definitions	4

6. Functional Requirements	4

6.1 Startup and POST	4

6.2 Sensor reading	5

6.3 Baby seat detection	5

6.4 Crash detection	5

6.5 Deployment output logic	5

6.6 Watchdog	6

6.7 CAN communication	6

6.8 DWI	6

7. Non-Functional Requirements	7

8. Constants and Thresholds	7

9. Traceability Matrix	7

10. Approval and Sign-Off	9

11. Revision History	9

# 1. Glossary

| Term / Abbreviation | Definition |
| --- | --- |
| ADC | Analog-to-Digital Converter — converts analog voltage 0–3.3 V to 12-bit digital value 0–4095 on Arduino Due |
| ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
| CAN | Controller Area Network — serial communication bus |
| DLC | Data Length Code — CAN frame field indicating payload byte count |
| DWI | Diagnostic Warning Indicator — red warning LED on pin D10 of each ECU |
| ECU | Electronic Control Unit |
| FSR | Functional Safety Requirement — requirement from LCP04-AIRBAG-ECU-001_FSRS_v100 |
| GPIO | General Purpose Input/Output — digital pin on microcontroller |
| HADS | Hardware Architecture and Design Specification |
| HARA | Hazard and Risk Analysis |
| OMS | OMS Laboratory PC — passive CAN monitoring workstation running USBCANV2 |
| POST | Power-On Self-Test — hardware verification sequence at startup |
| SIL | Safety Integrity Level per IEC 61508 (1–4) |
| SRS | Software Requirements Specification — this document |
| SWR | Software Requirement — individual requirement defined in this document |
| WDT | Hardware Watchdog Timer — SAM3X8E independent timer; resets MCU on timeout |
| 2oo2 | Two-out-of-Two — voting architecture requiring agreement of both channels |

# 2. Overview

This Software Requirements Specification (SRS) defines the complete set of software requirements (SWR-01 to SWR-25) for the firmware running on the Airbag Electronic Control Unit (ECU). The firmware executes on two Arduino Due (SAM3X8E ARM Cortex-M3) boards operating as a redundant 2oo2 dual-channel system.

This document covers the following requirement categories needed for software at ASIL D:

Startup and POST requirements covering CPU, RAM, and ADC self-tests that must complete before any output pin can be driven HIGH.

Sensor reading and physical unit conversion requirements covering the 12-bit ADC acquisition of the accelerometer, gyroscope, and pressure sensor, and the baby seat digital input with debounce.

Crash detection requirements covering threshold comparison and OR-fusion logic across the three sensors.

Deployment output logic requirements covering the voteDeploy decision, the hardware AND gate interface (D8/D9), and baby seat suppression (D9 only).

Watchdog requirements covering hardware WDT configuration and unconditional feeding every loop iteration.

CAN communication requirements covering frame structure (DLC=8), transmission timing (500ms heartbeat plus event-triggered), peer timeout detection, and frame parsing.

Diagnostic Warning Indicator (DWI) requirements covering activation on local and remote faults.

This document is the direct software-level decomposition of LCP04-AIRBAG-ECU-001_FSRS_v100 and is implemented by the firmware baseline airbag_ecu_rev3_3.ino (Rev 3.3 — final, WDT enabled). It must be read together with HADS-AIRBAG-ECU-001 Rev 2.0 Final (hardware interfaces), SADS-AIRBAG-ECU-001 Rev 3.0 (software architecture and module design), and LCP05-AIRBAG-ECU-001_SCD_v100 (safety concept). Verification of every requirement in this document is recorded in ITP-AIRBAG-ECU-002 Rev 2.1 and TR-AIRBAG-ECU-001.

# 3. FSR Compliance Status

| FSR | Title | ASIL | Status | Notes |
| --- | --- | --- | --- | --- |
| FSR-01 | Crash detection timing | D | SATISFIED | analogRead every loop, toPhys(), threshold check every iteration |
| FSR-02 | Algorithm cycle ≤1 ms | D | SATISFIED | No delay() in loop. All timers use millis(). Loop <1 ms |
| FSR-03 | Deployment auth 6 conditions | D | SATISFIED | voteDeploy 4-condition SW + babySeat in D9 + hardware AND gate = 6 conditions total |
| FSR-04 | Prevention unintended deploy | D | SATISFIED | D8/D9 only HIGH when all conditions met. Default LOW |
| FSR-05 | Deploy command ≤40 ms | D | DEVIATION | CAN TX at 500 ms heartbeat. Worst case 500 ms. Improved by event-triggered TX. Formal deviation accepted for lab |
| FSR-06 | Side-impact <8 ms | D | HW LIMITATION | Polling architecture cannot guarantee <8 ms. No interrupt path for POTI-3. Accepted for lab demo |
| FSR-07 | Dual-channel redundancy | D | SATISFIED | Both boards fully independent. Separate sensor reads, CAN TX, peer monitoring |
| FSR-08 | Channel agreement verification | D | SATISFIED | remoteCrash from peer CAN frame required in voteDeploy. Hardware AND gate physical enforcement |
| FSR-09 | Passenger airbag suppression | D | SATISFIED | D9 = voteDeploy && !babySeat. babySeat=TRUE forces D9 LOW unconditionally |
| FSR-10 | Suppress-on-fault | D | SATISFIED | stableBaby retains last debounced value on unstable signal. Conservative fail-safe |
| FSR-11 | Digital input debounce 10 ms | D | SATISFIED | updateBabySeat() — 10 ms stable required, called every loop |
| FSR-12 | Baby seat line monitoring | D | NOT IMPLEMENTED | No periodic continuity check. Hardware limitation — accepted for lab demo scope |
| FSR-13 | CRC-protected CAN | B | HW SATISFIED | SAM3X8E CAN controller provides ISO 11898 15-bit hardware CRC on every frame automatically |
| FSR-14 | CAN bitstream readback | B | NOT IMPLEMENTED | Classical CAN HW does not expose bitstream readback. Hardware limitation — accepted for lab |
| FSR-15 | CAN timeout ≤50 ms | B | DEVIATION — 2000 ms | 50 ms triggers false faults with 500 ms TX interval. Formal deviation documented. NCR raised |
| FSR-16 | Crash notification ≤15 ms | B | PARTIAL | Normal path: 500 ms heartbeat. Event-triggered TX: immediate on voteDeploy=TRUE. Partially satisfied |
| FSR-17 | POST within 800 ms | C | PARTIAL | cpuTest, ramTest, adcTest implemented. No CAN POST, no clock check, no DWI test. Partial coverage |
| FSR-18 | Continuous diagnostics ≤100 ms | C | PARTIAL | CommFault check every loop. No dedicated 100 ms diagnostic task. Partial coverage |
| FSR-19 | DWI activation ≤800 ms | A | SATISFIED | D10 HIGH in same loop iteration as fault. Sub-millisecond response |
| FSR-20 | Safe-state transition | D | SATISFIED | systemFault=TRUE → voteDeploy=FALSE → D8/D9=LOW in same loop cycle |
| FSR-21 | Fault logging DTCs | A | NOT IMPLEMENTED | Serial output only. No EEPROM storage. Accepted for lab demo scope |
| FSR-22 | Memory integrity WDT | D | PARTIAL | ramTest() in POST satisfied. WDT not yet enabled (HADS-CODE-OB-001). Stack guard not implemented |

# 4. System Overview

## 4.1 Board configuration — only these 3 lines differ

| Define | ECU 1 (Due 1) | ECU 2 (Due 2) |
| --- | --- | --- |
| ECU_ID | 1 | 2 |
| DATA_ID (TX) | 0x100 | 0x200 |
| REMOTE_ID (RX) | 0x200 | 0x100 |

## 4.2 CAN frame layout — DLC=8 (final)

| Byte | Field | Encoding |
| --- | --- | --- |
| 0 | A0 raw LSB | rawA0 & 0xFF |
| 1 | A0 raw MSB | rawA0 >> 8 |
| 2 | A1 raw LSB | rawA1 & 0xFF |
| 3 | A1 raw MSB | rawA1 >> 8 |
| 4 | A2 raw LSB | rawA2 & 0xFF |
| 5 | A2 raw MSB | rawA2 >> 8 |
| 6 | Status flags | Bit0=crash, Bit1=babySeat, Bit2=systemFault |
| 7 | ECU_ID | 1 or 2 |

# 5. Software Module Definitions

| Module | Function | Cycle | FSR | ASIL |
| --- | --- | --- | --- | --- |
| cpuTest() | Arithmetic: 12345×6789=83810205 | Boot | FSR-17 | C |
| ramTest() | 0xAAAAAAAA/0x55555555 across 128 words | Boot | FSR-17,22 | D |
| adcTest() | Verify ADC in range 0–4095 | Boot | FSR-17 | C |
| updateBabySeat() | D11 read + 10 ms debounce | Every loop | FSR-09,10,11 | D |
| toPhys() | raw/4095.0 × maxVal | Every loop | FSR-01,02 | D |
| Sensor read block | analogRead A0/A1/A2 + toPhys() | Every loop | FSR-01,02 | D |
| Threshold check | accelEx/gyroEx/pressEx comparisons | Every loop | FSR-03,04 | D |
| crashLocal | accelEx OR gyroEx OR pressEx | Every loop | FSR-03 | D |
| voteDeploy | localCrash AND remoteCrash AND !faults | Every loop | FSR-03,08 | D |
| Safe state | D8/D9 LOW when systemFault=TRUE | Every loop | FSR-20 | D |
| DWI logic | D10 HIGH on local or remote fault | Every loop | FSR-19,20 | A/D |
| receiveCAN() | Read peer frame, extract status, update HB | Every loop | FSR-07,15 | D |
| canSend() | Pack 8-byte frame, event-triggered + periodic TX | 500 ms / on event | FSR-13,16 | B |
| CAN timeout | faultCOMM if no peer frame in 2000 ms | Every loop | FSR-15 | D |
| Serial output | Print diagnostics every 1000 ms | 1000 ms | FSR-21 | A |

# 6. Functional Requirements

## 6.1 Startup and POST

SWR-01 — POST execution

cpuTest() verifies 12345×6789=83810205. ramTest() writes 0xAAAAAAAA then 0x55555555 across 128 volatile words. adcTest() verifies analogRead(A0) in [0,4095]. All run before loop() entry.

ASIL/SIL: ASIL C / SIL 2   |   FSR: FSR-17

SWR-02 — No output before POST

D8, D9, D10 initialised LOW in setup() before POST runs and remain LOW if any POST flag TRUE.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-17

SWR-03 — ADC 12-bit configuration

analogReadResolution(12) called in setup(). ADC readings in range 0–4095.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-01

## 6.2 Sensor reading

SWR-04 — Sensor acquisition every loop

analogRead called on A0, A1, A2 every loop() iteration. No delay().

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-01, FSR-02

SWR-05 — Physical unit conversion

physical = (raw / 4095.0) × MAX_VALUE. Divisor is float 4095.0. MAX: 100.0 g, 360.0 deg/s, 500.0 kPa.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-01

SWR-06 — Threshold comparison

accelEx=(accel>30.0), gyroEx=(gyro>120.0), pressEx=(press>200.0). Each evaluated independently every loop.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-03, FSR-04

## 6.3 Baby seat detection

SWR-07 — Baby seat INPUT mode

D11 configured as INPUT. External 10kΩ pull-up to 3.3V. Active-LOW.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-09

SWR-08 — 10 ms debounce filter

updateBabySeat() called every loop. State accepted only after 10 ms stable.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-11

SWR-09 — Suppress-on-fault default

stableBaby retains last accepted value on unstable signal. Conservative fail-safe.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-10

## 6.4 Crash detection

SWR-10 — Sensor OR fusion

crashLocal = accelEx OR gyroEx OR pressEx. Any single sensor above threshold = crash.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-03

SWR-11 — No blocking code

No delay() or blocking call in loop(). All timing via non-blocking millis() comparison.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-02

## 6.5 Deployment output logic

SWR-12 — D8 AND gate input — airbag 1

D8 = HIGH iff voteDeploy = (localCrash AND remoteCrash AND !systemFault AND !remoteFault). Hardware AND gate provides final physical 2oo2 layer.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-03, FSR-08

SWR-13 — D9 AND gate input — airbag 2

D9 = voteDeploy AND NOT babySeat. babySeat=TRUE forces D9 LOW unconditionally.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-09, FSR-10

SWR-14 — Fault inhibit — safe state

systemFault = faultCPU OR faultRAM OR faultADC OR faultCOMM. If TRUE, D8 and D9 = LOW, D10 = HIGH in same loop iteration.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-20

## 6.6 Watchdog

SWR-15 — Hardware WDT configuration

watchdog_setup() shall configure SAM3X8E WDT with WDT_MR_WDRSTEN and timeout WDT_MR_WDV(256×10) ≈ 10 ms. Pending: HADS-CODE-OB-001.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-22

SWR-16 — Unconditional WDT feed

watchdog_feed() shall be first statement in loop() unconditionally. Pending: HADS-CODE-OB-001.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-22

## 6.7 CAN communication

SWR-17 — CAN TX — 500 ms heartbeat + event-triggered

canSend() called every HEARTBEAT_MS=500 ms via millis() timer. Additionally called immediately when voteDeploy first becomes TRUE (event-triggered TX for FSR-16 improvement).

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-07, FSR-16

SWR-18 — CAN timeout — 2000 ms (deviation)

faultCOMM=TRUE if no valid peer frame for COMM_TIMEOUT=2000 ms. FSR-15 specifies 50 ms — formal deviation accepted. NCR raised.

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-07, FSR-15

SWR-19 — CAN frame parsing

receiveCAN() reads available frames. Frames with id != REMOTE_ID discarded. Valid frames update lastRemoteHB, remoteCrash (byte6 bit0), remoteFault (byte6 bit2).

ASIL/SIL: ASIL D / SIL 3   |   FSR: FSR-07

SWR-20 — CAN0 init 500 kbit/s

Can0.begin(CAN_BPS_500K) and Can0.watchFor() in setup().

ASIL/SIL: ASIL B / SIL 2   |   FSR: FSR-13

SWR-21 — CAN frame structure — DLC=8

Bytes 0-5: raw ADC little-endian. Byte 6: status flags. Byte 7: ECU_ID. DLC=8. Classical CAN maximum.

ASIL/SIL: ASIL B / SIL 2   |   FSR: FSR-13, FSR-16

SWR-22 — CRC integrity — hardware provided

ISO 11898 15-bit CRC appended by SAM3X8E CAN hardware controller on every transmitted frame and verified on every received frame automatically. No software CRC required for classical CAN DLC=8. Rev 3.1: previous software CRC removed — caused memory corruption by writing beyond 8-byte buffer boundary.

ASIL/SIL: ASIL B / SIL 2   |   FSR: FSR-13

SWR-23 — CAN TX interval 500 ms

One 8-byte frame per HEARTBEAT_MS=500 ms non-blocking timer, plus event-triggered on voteDeploy=TRUE.

ASIL/SIL: ASIL B / SIL 2   |   FSR: FSR-16

## 6.8 DWI

SWR-24 — DWI on local fault

D10 = HIGH within one loop cycle of any local fault flag TRUE.

ASIL/SIL: ASIL A / SIL 1   |   FSR: FSR-19

SWR-25 — DWI on remote fault

D10 = HIGH if remoteFault=TRUE even if all local flags FALSE.

ASIL/SIL: ASIL A / SIL 1   |   FSR: FSR-19

# 7. Non-Functional Requirements

| ID | Requirement | Value | FSR | ASIL |
| --- | --- | --- | --- | --- |
| SWR-NF-01 | Main loop cycle time | < 1 ms | FSR-02 | ASIL D |
| SWR-NF-02 | Hardware WDT timeout | 10 ms (pending) | FSR-22 | ASIL D |
| SWR-NF-03 | Baby seat debounce | 10 ms stable | FSR-11 | ASIL D |
| SWR-NF-04 | CAN TX heartbeat interval | 500 ms | FSR-07 | ASIL D |
| SWR-NF-05 | CAN timeout threshold | 2000 ms (deviation from 50 ms FSR-15) | FSR-15 | ASIL D |
| SWR-NF-06 | CAN OMS monitoring frame rate | 500 ms | FSR-16 | ASIL B |
| SWR-NF-07 | Serial diagnostic interval | 1000 ms | FSR-21 | ASIL A |
| SWR-NF-08 | ADC resolution | 12-bit 0–4095 | FSR-01 | ASIL D |
| SWR-NF-09 | CAN baud rate | 500 kbit/s | FSR-13 | ASIL B |
| SWR-NF-10 | CAN frame DLC | 8 bytes (classical CAN max) | FSR-13 | ASIL B |

# 8. Constants and Thresholds

| Constant | Value | Unit | Meaning |
| --- | --- | --- | --- |
| ADC_MAX | 4095.0 | — | 12-bit ADC max — float |
| THRESH_ACCEL | 30.0 | g | Frontal crash threshold |
| THRESH_GYRO | 120.0 | deg/s | Rollover threshold |
| THRESH_PRESS | 200.0 | kPa | Side impact threshold |
| DEBOUNCE_MS | 10 | ms | Baby seat debounce |
| HEARTBEAT_MS | 500 | ms | CAN TX interval |
| COMM_TIMEOUT | 2000 | ms | Peer silence fault threshold (deviation) |
| PRINT_MS | 1000 | ms | Serial diagnostic interval |
| BLINK_MS | 500 | ms | Status LED blink interval |

# 9. Traceability Matrix

| SWR | Description | FSR | ASIL | Code module | TC |
| --- | --- | --- | --- | --- | --- |
| SWR-01 | POST at boot | FSR-17 | C | cpuTest,ramTest,adcTest | TC-VER-008 |
| SWR-02 | No output before POST | FSR-17 | D | setup() init LOW | TC-VER-008 |
| SWR-03 | ADC 12-bit | FSR-01 | D | analogReadResolution(12) | TC-VER-001 |
| SWR-04 | Sensor read every loop | FSR-01,02 | D | analogRead A0/A1/A2 | TC-VER-001,002 |
| SWR-05 | Physical conversion | FSR-01 | D | toPhys() | TC-VER-001 |
| SWR-06 | Threshold comparison | FSR-03,04 | D | accelEx,gyroEx,pressEx | TC-VER-002 |
| SWR-07 | Baby seat INPUT | FSR-09 | D | pinMode(D11,INPUT) | TC-VAL-005 |
| SWR-08 | 10 ms debounce | FSR-11 | D | updateBabySeat() | TC-VAL-006 |
| SWR-09 | Suppress-on-fault | FSR-10 | D | stableBaby conservative path | TC-VAL-006 |
| SWR-10 | Sensor OR fusion | FSR-03 | D | crashLocal | TC-VER-002 |
| SWR-11 | No blocking delay | FSR-02 | D | millis() timers only | TC-VER-002 |
| SWR-12 | D8 AND gate drive | FSR-03,08 | D | voteDeploy→D8 | TC-VER-002,TC-VAL-009 |
| SWR-13 | D9 pax suppression | FSR-09,10 | D | voteDeploy&&!babySeat→D9 | TC-VAL-005 |
| SWR-14 | Fault inhibit | FSR-20 | D | systemFault→D8/D9 LOW | TC-VAL-009 |
| SWR-15 | WDT config (pending) | FSR-22 | D | watchdog_setup() — pending | TC-VER-003 |
| SWR-16 | WDT feed (pending) | FSR-22 | D | watchdog_feed() — pending | TC-VER-003 |
| SWR-17 | CAN TX 500ms+event | FSR-07,16 | D | canSend() heartbeat+event | TC-VER-007 |
| SWR-18 | CAN timeout 2000ms | FSR-07,15 | D | COMM_TIMEOUT=2000 | TC-VAL-008 |
| SWR-19 | CAN frame parsing | FSR-07 | D | receiveCAN() | TC-VER-007 |
| SWR-20 | CAN0 500 kbps | FSR-13 | B | Can0.begin() | TC-VER-007 |
| SWR-21 | CAN DLC=8 | FSR-13,16 | B | canSend() frame | TC-VER-004,007 |
| SWR-22 | CRC hardware ISO 11898 | FSR-13 | B | SAM3X8E CAN controller | TC-VER-007 |
| SWR-23 | CAN 500ms+event | FSR-16 | B | HEARTBEAT_MS timer+event | TC-VER-007 |
| SWR-24 | DWI local fault | FSR-19 | A | D10=systemFault¦¦remoteFault | TC-VAL-007,008 |
| SWR-25 | DWI remote fault | FSR-19 | A | remoteFault→D10 HIGH | TC-VAL-007 |

# 10. Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Software Safety Engineer | Mariana Chavez Avila |  |  |
| Hardware Safety Engineer | Om Naik |  |  |
| Functional Safety Engineer | Kruthika S. Gowda |  |  |
| V&V Lead | Shivbaraje Bhosale |  |  |
| Project Supervisor |  |  |  |

# 11. Revision History

| Rev | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 2026-05-31 | SW Safety Team | Initial release |
| 2.0 | 2026-06-10 | SW Safety Team | CRC specified, coding constraints, comm timeout |
| 2.1 | 2026-06-11 | SW Safety Team | HADS reference, 3mm LED, pull-up 3.3V confirmed |
| 3.0 | 2026-06-12 | SW Safety Team | HEARTBEAT 500→100ms, COMM_TIMEOUT 2000→100ms, CRC-16 software added |
| 3.1 | 2026-06-14 | SW Safety Team | DLC=10 reverted to DLC=8 (memory corruption fix). Software CRC removed — HW CRC satisfies FSR-13. COMM_TIMEOUT restored 2000ms. HEARTBEAT_MS restored 500ms. Event-triggered CAN TX added on voteDeploy=TRUE. FSR compliance table added. All deviations and HW limitations formally documented. |