Airbag ECU Portfolio HS Harz — Functional Safety AIRBAG ELECTRONIC CONTROL UNIT Measurement and Processing of Sensor Signals to Control Airbag Operation

LIFE CYCLE PHASE 3 — HAZARD AND RISK ANALYSIS (HARA)

| Document Number | LCP03-AIRBAG-ECU-001 |
| --- | --- |
| Version | 2.00 |
| Date | 2026-05-05 |
| Status | DRAFT |
| Editor | Alita, Fateme HS Harz FuSa — Systems Safety Engineering Team |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| File | LCP03-AIRBAG-ECU-001_HARA_v110.docx |

Table of Contents

1  Meetings within the Project	4

2  Unresolved Topics	4

3  Glossary	4

4  Purpose and Scope of the HARA Phase	6

4.1  Purpose of Lifecycle Phase 3	6

4.2  Relationship to Other Lifecycle Phases	6

4.3  Applicable Standards	6

5  System Boundary and Assumptions	6

5.1  Item Definition (from LCP01)	6

5.2  General Assumptions	6

6  Use Cases	7

7  ASIL / SIL Determination Parameters	8

7.1	8

Severity (S) — ISO 26262 Table B.1	8

7.2  Exposure (E) — ISO 26262 Table B.2	8

7.3  Controllability (C) — ISO 26262 Table B.3	8

7.4  ASIL / SIL Equivalence Mapping	9

8  Hazard Identification	10

8.1  Hazard Category H1: Unintended Airbag Deployment	10

8.2  Hazard Category H2: Non-Deployment in a Crash Requiring Deployment	10

8.3  Hazard Category H3: Partial or Incorrect Deployment Sequence	10

8.4  Hazard Category H4: Loss of Diagnostic Capability / Warning	10

9  HARA Results — Hazardous Events and ASIL / SIL Assignment	11

10  Safety Goals	13

10.1  Worked Example: Unintended Airbag Deployment During Normal Driving	13

10.2  Worked Example: Airbag Deploys with Baby Seat / Child Seat Installed	13

10.3  Summary of Safety GoalsTable 10:Safety Goals Summary	14

11  Safe States Definition	15

12  Figure and Table Listing	15

13  List of Dependent Documents	16

14  Literature	16

15  Version History	16

# 1  Meetings within the Project

The following project meetings were held during the HARA Phase:

Table 1:Meetings within the Project

| Topic | Date / Time | Participants | Minutes / Decisions |
| --- | --- | --- | --- |
| HARA Kick-off & Scope Review | 2026-05-03, 10:00 | Safety and Analysis Team, Quality Manager, Project Manager | Confirmed item boundary from LCP01. Identified use cases UC-01 through UC-12. Assigned hazard brainstorming tasks. |
| HARA Results Review & ASIL Sign-off | 2026-05-06, 14:00 | Safety Requirement Engineer, Quality Manager, Safety Analysis Team | Reviewed full hazard table. ASIL D confirmed for SG-01, SG-02, SG-03. Safety goals SG-01–SG-07 approved. |

# 2  Unresolved Topics

Table 2:Unresolved Topics

| No. | Topic / Question | Responsible | Target Date | Status |
| --- | --- | --- | --- | --- |
| No unresolved topics at time of issue. Any topics arising during review shall be recorded here before the document status is changed to RELEASED. |  |  |  |  |

# 3  Glossary

Table 2 lists all abbreviations and terms used in this document and the wider Airbag ECU project portfolio.

Table 3:Glossary

| Term / Abbreviation | Definition |
| --- | --- |
| ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
| ACU | Airbag Control Unit — synonymous with Airbag ECU |
| CAN | Controller Area Network — serial communication bus for vehicle ECUs |
| C | Controllability — ISO 26262 HARA parameter (C0–C3) |
| DTC | Diagnostic Trouble Code |
| DWI | Diagnostic Warning Indicator — airbag fault lamp in the instrument cluster |
| E | Exposure — ISO 26262 HARA parameter (E0–E4) |
| ECU | Electronic Control Unit |
| EMC | Electromagnetic Compatibility |
| EUC | Equipment Under Control — the item/system being safety-assessed |
| FMEA | Failure Mode and Effects Analysis |
| HARA | Hazard and Risk Analysis |
| HW | Hardware |
| IEC 61508 | Functional Safety of E/E/PE Safety-related Systems (2010 edition) |
| ISO 26262 | Road Vehicles — Functional Safety standard (2018 edition) |
| LCP03 | Life Cycle Phase 03 — Hazard and Risk Analysis (second of 16 IEC 61508 / ISO 26262 phases) |
| OCS | Occupant Classification System |
| PFH | Probability of dangerous Failure per Hour |
| PMHF | Probabilistic Metric for random Hardware Failures |
| POST | Power-On Self-Test |
| QM | Quality Management — no specific ASIL requirement |
| S | Severity — ISO 26262 HARA parameter (S0–S3) |
| SG | Safety Goal — high-level safety requirement derived from HARA |
| SIL | Safety Integrity Level per IEC 61508 (1–4) |
| SPFM | Single-Point Fault Metric |
| SS | Safe State — defined system condition upon fault detection |
| UC | Use Case — defined operational scenario for hazard identification |

# 4  Purpose and Scope of the HARA Phase

## 4.1  Purpose of Lifecycle Phase 3

This document constitutes the deliverable for Life Cycle Phase 3 (LCP03) of the Airbag ECU project. It presents the complete Hazard and Risk Analysis (HARA) conducted in accordance with ISO 26262:2018 (Clause 6 — Hazard and Risk Assessment) and IEC 61508:2010 (Clause 7.4 — Hazard and Risk Assessment).

The HARA is the foundational safety analysis that determines the required Automotive Safety Integrity Level (ASIL) per ISO 26262 and Safety Integrity Level (SIL) per IEC 61508 for each safety goal. The outputs of this phase directly govern all subsequent lifecycle phases: the Overall Safety Requirements (LCP03), Safety Planning (LCP04), and all hardware, software, and verification activities.

## 4.2  Relationship to Other Lifecycle Phases

Input: LCP01 — Concept Phase document (item definition, system boundary, safety-relevant functions F1–F6, team structure).

Output: ASIL / SIL classification per hazard; safety goals SG-01–SG-07; safe states SS-01–SS-05. These become the inputs to LCP03 (Safety Requirements Specification).

## 4.3  Applicable Standards

This analysis is developed in accordance with:

- ISO 26262:2018 — Road Vehicles — Functional Safety, Part 3 (Concept Phase, Clause 6)

- IEC 61508:2010 — Functional Safety of E/E/PE Safety-related Systems, Parts 1 and 2

- ISO 21448:2022 — Safety of the Intended Functionality (SOTIF)

# 5  System Boundary and Assumptions

## 5.1  Item Definition (from LCP01)

The item under analysis is the Airbag Electronic Control Unit (Airbag ECU / ACU). In the HS Harz laboratory implementation, the system is realised on two Arduino Due microcontroller boards operating in a dual-channel (2oo2) redundant architecture. Three potentiometers on a breadboard simulate analog crash sensor inputs (POTI-1 accelerometer, POTI-2 gyroscope, POTI-3 pressure sensor); one digital switch simulates the Occupant Classification System (OCS) / baby-seat input. A CAN bus interface connects the ECU to the OMS laboratory PC for output verification.

The following are within the item boundary: ECU hardware (both Arduino Due boards), embedded software, sensor interfaces, squib drive circuits, CAN interface, and diagnostic output (DWI lamp drive).

The following are outside the item boundary and treated as adjacent systems: airbag inflator modules, seatbelt pretensioners, vehicle body structure, OCS hardware, BCM, and eCall system.

## 5.2  General Assumptions

- The vehicle is a standard passenger car (M1 category) with a maximum design speed of 250 km/h.

- The Airbag ECU operates over the full vehicle lifetime, estimated at 15 years / 300,000 km.

- The ECU is powered from the vehicle 12 V battery system with a backup capacitor sufficient for ≥ 150 ms post-accident operation.

- Ambient operating temperature range: −40 °C to +85 °C (storage: −40 °C to +105 °C). EMC requirements per CISPR 25 and ISO 11452.

- Squib resistance is within a known window (1.5 Ω – 4.0 Ω nominal). Squib firing energy is supplied by the ECU internal reservoir capacitor.

# 6  Use Cases

The following operational and use case scenarios are considered for hazard identification. Each use case represents a distinct operational situation in which a hazardous condition could arise.

Table 4:Use Cases

| UC-ID | Use Case | Description / Relevance |
| --- | --- | --- |
| UC-01 | Normal Highway Driving | Vehicle traveling at 80–250 km/h on motorway. Highest crash severity potential. Full sensor fusion active. |
| UC-02 | Urban / Low-Speed Driving | Vehicle at 0–50 km/h in urban environment. Pedestrian presence likely. Frequent speed changes; minor impacts common. |
| UC-03 | Frontal Crash (Barrier Impact) | Full-width or offset frontal impact. Primary scenario for frontal airbag and seatbelt pretensioner activation. |
| UC-04 | Side Crash (Pole / MDB) | Lateral impact from another vehicle or rigid pole. Requires rapid (< 8 ms) side curtain and thorax airbag deployment. |
| UC-05 | Rear Impact | Impact from rear. Seatbelt pretensioners activated; whiplash protection. |
| UC-06 | Rollover | Vehicle roll detected by yaw/roll rate sensors. Curtain airbags must remain deployed for 5+ seconds. |
| UC-07 | Minor Non-Deployable Impacts | Pothole, speed bump, minor collision (< 15 g). No deployment must occur. System must distinguish from crash events. |
| UC-08 | Vehicle Stationary / Ignition Off | Engine off, low-power standby mode. ECU remains powered by battery. Risk of inadvertent activation due to maintenance or battery transients. |
| UC-09 | Maintenance / Repair (Workshop) | Technician accessing vehicle interior; OBD-II connected. Risk of inadvertent deployment during servicing. |
| UC-10 | Vehicle Wash / Flood Wading | Vibration from wash equipment; immersion risk. Sensor noise must not trigger deployment. |
| UC-11 | Child / Small Occupant Presence | OCS classifies occupant as child or empty seat. Passenger airbag must be suppressed to prevent injury from deployment. |
| UC-12 | End-of-Life / Scrapping | Airbag must be safely disarmed and deployed (controlled) before vehicle is scrapped. |

# 7  ASIL / SIL Determination Parameters

The ASIL is determined from three independent parameters: Severity (S), Exposure (E), and Controllability (C), per ISO 26262:2018, Annex B. The equivalent SIL per IEC 61508 is mapped from the resulting ASIL.

## 7.1

## Severity (S) — ISO 26262 Table B.1

Table 5:Severity

| Level | Description | Example |
| --- | --- | --- |
| S0 | No injuries | Purely economic damage, no harm to persons |
| S1 | Light and moderate injuries | Reversible injuries; first-aid level |
| S2 | Severe and life-threatening injuries (survival probable) | Broken bones, concussion, internal bleeding (survivable) |
| S3 | Life-threatening injuries (survival uncertain) or fatal | Traumatic brain injury, thoracic trauma, fatality |

## 7.2  Exposure (E) — ISO 26262 Table B.2

Table 6:Exposure Levels

| Level | Description | Frequency / Probability |
| --- | --- | --- |
| E0 | Incredibly unlikely | < once per vehicle lifetime |
| E1 | Very low probability | A few times per vehicle lifetime (< 1%) |
| E2 | Low probability | A few times per year (~1–10%) |
| E3 | Medium probability | Fairly often (~10–50%) |
| E4 | High probability | Almost always or very frequently (> 50%) |

## 7.3  Controllability (C) — ISO 26262 Table B.3

Table 7:Controllability Levels

| Level | Description | Population Controllable |
| --- | --- | --- |
| C0 | Generally controllable | Simply avoidable by most drivers |
| C1 | Simply controllable | 90% of drivers can avoid harm |
| C2 | Normally controllable | Most drivers (< 90%) can avoid; requires prompt reaction |
| C3 | Difficult to control or uncontrollable | < 10% of drivers can avoid; situation evolves too fast |

## 7.4  ASIL / SIL Equivalence Mapping

Table 8:ASIL–SIL Equivalence

| ASIL (ISO 26262) | SIL (IEC 61508) | PFH | Notes |
| --- | --- | --- | --- |
| QM | None / < SIL 1 | — | No safety integrity requirement |
| ASIL A | SIL 1 | 10⁻⁵ – 10⁻⁶ / h | Low demand mode SIL 1 |
| ASIL B | SIL 1–2 | 10⁻⁶ – 10⁻⁷ / h | Lower bound SIL 2 |
| ASIL C | SIL 2–3 | 10⁻⁷ – 10⁻⁸ / h | Upper bound SIL 2 / lower SIL 3 |
| ASIL D | SIL 3 | < 10⁻⁸ / h | Highest automotive safety level |

# 8  Hazard Identification

Hazardous conditions are identified through structured brainstorming using FMEA / HAZOP methods, considering each system function and possible failure mode (commission and omission failures). The four hazard categories below correspond to the system functions F1–F6 defined in LCP01.

## 8.1  Hazard Category H1: Unintended Airbag Deployment

- Inadvertent deployment during normal driving — no crash event present. Could injure occupants, cause driver distraction and loss of vehicle control.

- Deployment due to EMC disturbance, software runaway, or spurious logic triggering the firing circuit.

- Deployment during vehicle wash, maintenance, or stationary operation.

- Deployment of passenger airbag with child seat or empty seat present (UC-11).

## 8.2  Hazard Category H2: Non-Deployment in a Crash Requiring Deployment

- Failure to deploy any airbag in a frontal crash above threshold — severe injury or fatality risk.

- Failure to deploy side curtain in a lateral or rollover event — head injury from side structure contact.

- Failure to fire seatbelt pretensioners — occupant not restrained properly before airbag contact.

- Delayed deployment beyond 15 ms from crash threshold crossing — occupant already in contact with steering wheel or A-pillar.

## 8.3  Hazard Category H3: Partial or Incorrect Deployment Sequence

- Deployment of incorrect airbag zone (e.g., rear instead of frontal) — no protection where needed.

- Single squib ignition failure in a dual-stage inflator — under-inflation of bag.

## 8.4  Hazard Category H4: Loss of Diagnostic Capability / Warning

- Permanent silence of DWI lamp despite system fault — occupant unaware of reduced protection.

- Loss of CAN broadcast after crash — fuel pump not cut, emergency call not triggered.

# 9  HARA Results — Hazardous Events and ASIL / SIL Assignment

The table below presents the complete HARA results. Severity (S), Exposure (E), and Controllability (C) ratings follow ISO 26262:2018 Annex B. The ASIL and SIL columns show the maximum values determined across all contributing use cases.

Table 9:Hazardous Events and ASIL/SIL Assignment

| ID | Hazard | Situation / Hazardous Event | S | E | C | SIL | ASIL | Safe State | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H1-01 | Unintended deployment frontal airbag | Normal highway driving (UC-01); no crash event. Airbag deploys unexpectedly; driver blinded/struck; loss of control at speed. | S3 | E4 | C3 | SIL 3 | ASIL D | No deployment without valid crash signal; firing inhibit logic active. | Highest risk — primary safety goal. |
| H1-02 | Unintended deployment side/curtain airbag | Urban driving (UC-02); no side impact. Curtain deploys unexpectedly; driver vision obstructed; loss of control. | S3 | E3 | C3 | SIL 3 | ASIL D | Firing inhibit; dual-algorithm AND-gate required for side deployment. | Also covers UC-07 (pothole misfire). |
| H1-03 | Inadvertent deployment vehicle stationary | Vehicle parked, maintenance (UC-08, UC-09); technician working in cabin. Airbag deploys and injures technician or bystander. | S2 | E2 | C2 | SIL 2 | ASIL B | Firing transistor requires active MCU command; no stored energy path without MCU enable. | Requires authorised disarm procedure in workshop. |
| H1-04 | Deployment of passenger airbag with child/empty seat | Child or empty seat present (UC-11); frontal crash occurs. Passenger airbag deploys against child seat; risk of child injury. | S3 | E3 | C3 | SIL 3 | ASIL D | OCS validated; passenger bag suppressed if OCS = child/empty; fallback = suppress if OCS CAN lost. | Suppress-on-fault strategy required. |
| H1-05 | EMC-induced spurious deployment | Vehicle near high-power transmitter or severe ESD (any UC). EM disturbance causes firing transistor to open; airbag deploys. | S3 | E2 | C3 | SIL 2 | ASIL C | Galvanic isolation on squib lines; EMC hardened firing circuit; hardware firing key required. | EMC tested per CISPR 25 / ISO 11452. |
| H2-01 | Non-deployment — frontal airbag in severe crash | Frontal barrier impact > 30 km/h (UC-03); algorithm fails to fire. Occupant head strikes steering wheel/windshield; severe injury or fatality. | S3 | E3 | C3 | SIL 3 | ASIL D | ECU signals fault via DWI if self-test failure detected; system remains armed; no inhibit without fault. | Primary safety goal — omission failure. |
| H2-02 | Non-deployment — side curtain / thorax airbag | Pole impact > 20 km/h (UC-04); side sensor or algorithm fails. Occupant head contacts door trim/A-pillar; severe TBI risk. | S3 | E3 | C3 | SIL 3 | ASIL D | Redundant satellite sensor signals; secondary path triggers deployment if primary fails; DWI ON if sensor faulted. | Deployment window < 8 ms from threshold. |
| H2-03 | Non-deployment — seatbelt pretensioners | Any significant crash (UC-03–UC-06); pretensioner circuit faulted. Occupant not restrained before airbag contact; increased injury potential. | S2 | E3 | C3 | SIL 2 | ASIL C | Pretensioner squib loops monitored continuously; fault → DWI ON but does not inhibit other channels. | Belt pretensioners secondary restraint; airbags primary. |
| H2-04 | Delayed deployment (> 15 ms) | Frontal or side crash (UC-03, UC-04); MCU fault or watchdog reset causes delay. Occupant already moving forward at deployment; out-of-position injury. | S2 | E2 | C3 | SIL 2 | ASIL C | Lockstep MCU ensures algorithm cycle ≤ 1 ms; hardware watchdog timeout < 10 ms. | Timing is safety-critical; specified as safety goal. |
| H3-01 | Incorrect airbag zone deployment | Frontal crash (UC-03) but side airbag fires. No frontal protection; side airbag deploys unexpectedly causing injury. | S3 | E1 | C3 | SIL 2 | ASIL B | Channel addressing verified by squib loop resistance; output address decoded in hardware; software channel map integrity checked at startup. | Low exposure — requires specific HW + SW fault. |
| H3-02 | Single squib failure in dual-stage inflator | Severe frontal crash (UC-03); stage 2 squib open. Bag under-inflated; insufficient restraint in high-speed crash. | S2 | E2 | C2 | SIL 1 | ASIL A | Both squib resistances monitored independently; stage 1 deployment still occurs; DTC logged. | Residual protection from stage 1. |
| H4-01 | Loss of DWI / airbag warning lamp (silent fault) | Any operational mode; DWI circuit open; occupant unaware of system fault. Occupant drives with reduced protection unaware. | S1 | E3 | C2 | SIL 1 | ASIL A | DWI lamp circuit verified at power-on (lamp drive current monitored); BCM cross-check via CAN. | Driver can check lamp at ignition-on; low ASIL relative to deployment hazards. |
| H4-02 | Loss of post-crash CAN broadcast | After deployment (UC-03–UC-06); CAN transceiver damaged. Fuel pump not cut; eCall not triggered; fire risk or delayed emergency response. | S2 | E3 | C2 | SIL 1 | ASIL B | CAN transmitted immediately at deployment decision (before structural damage propagates); watchdog ensures CAN TX attempt. | Post-crash CAN survivability limited by vehicle structural integrity. |

# 10  Safety Goals

Safety goals are derived directly from the HARA results. Each safety goal is assigned the highest ASIL determined across all contributing hazardous events. Safety goals are the primary output of LCP03 and form the input to LCP03 (Safety Requirements Specification).

## 10.1  Worked Example: Unintended Airbag Deployment During Normal Driving

| Hazard | Unintended deployment of the driver frontal airbag with no crash event present. |
| --- | --- |
| Situation | Vehicle travelling at highway speed (80–250 km/h). No impact has occurred. The ECU erroneously issues a firing command due to a software runaway or EMC event. |
| Severity (S) | S3 — Life-threatening or fatal. The airbag strikes the driver in the face/chest at high speed while the vehicle is in motion; driver loses control; risk of multi-vehicle collision and fatality. |
| Exposure (E) | E4 — High probability. The vehicle is driven regularly; the ECU is powered and monitoring continuously during every trip. This situation occurs almost every day throughout the vehicle's 15-year lifetime. |
| Controllability (C) | C3 — Difficult or impossible to control. Deployment is instantaneous (~3 ms inflation). The driver has no warning and no ability to react. Less than 10% of drivers could avoid subsequent loss of control. |
| Result | S3 + E4 + C3 → ASIL D / SIL 3 (Safety Goal SG-01) |
| Required Measure | Dual-algorithm AND-gate deployment logic; hardware firing inhibit path independent of software; dual lockstep MCU; ASIL D SW/HW development per ISO 26262 Parts 4 & 6. |

## 10.2  Worked Example: Airbag Deploys with Baby Seat / Child Seat Installed

| Hazard | Passenger airbag deploys into an installed rear-facing baby seat or forward-facing child seat during a crash event. |
| --- | --- |
| Situation | A baby/child seat is installed on the front passenger seat. A frontal crash occurs above the deployment threshold. The ECU has not correctly read or acted on the baby-seat digital input signal, and the passenger airbag fires. |
| Severity (S) | S3 — Life-threatening or fatal. An airbag deploying directly into a rear-facing child seat exerts a force far exceeding a child's neck/head tolerance. The injury is likely fatal for infants. |
| Exposure (E) | E3 — Medium probability. Child/baby seats are commonly used; a significant proportion of family vehicles carry child seats regularly. |
| Controllability (C) | C3 — Difficult or impossible to control. The deployment happens in under 30 ms total. No occupant or driver action can prevent the airbag from striking the child seat once triggered. |
| Result | S3 + E3 + C3 → ASIL D / SIL 3 (Safety Goal SG-03) |
| Required Measure | Dedicated baby-seat digital input (active-LOW) read every 1 ms cycle. Suppress-on-fault strategy: if input is unreadable or signal lost, passenger airbag is inhibited by default. |

## 10.3  Summary of Safety GoalsTable 10:Safety Goals Summary

| Safety Goal ID | Safety Goal | ASIL (ISO 26262) | SIL (IEC 61508) | Governing Hazard(s) |
| --- | --- | --- | --- | --- |
| SG-01 | The Airbag ECU shall not command deployment of any airbag or pretensioner in the absence of a valid crash event. | ASIL D | SIL 3 | H1-01, H1-02, H1-04 |
| SG-02 | The Airbag ECU shall command deployment of the appropriate airbag(s) within the required time window when a crash event exceeding the deployment threshold is detected. | ASIL D | SIL 3 | H2-01, H2-02 |
| SG-03 | The Airbag ECU shall suppress the passenger airbag when the OCS indicates a child occupant or empty seat. | ASIL D | SIL 3 | H1-04 |
| SG-04 | The Airbag ECU shall command seatbelt pretensioner activation within 5 ms of a deployment decision. | ASIL C | SIL 2 | H2-03, H2-04 |
| SG-05 | The Airbag ECU shall not command deployment in response to an EMC or electrical disturbance that does not represent a crash event. | ASIL C | SIL 2 | H1-05 |
| SG-06 | The Airbag ECU shall activate the Diagnostic Warning Indicator (DWI/airbag lamp) within 800 ms of detecting a system fault. | ASIL A | SIL 1 | H4-01 |
| SG-07 | The Airbag ECU shall transmit a crash notification CAN frame within 15 ms of a deployment decision. | ASIL B | SIL 2 | H4-02 |
| Overall Item ASIL / SIL: |  | ASIL D | SIL 3 | H1-01, H2-01 |

# 11  Safe States Definition

ISO 26262 requires that each safety goal be assigned a safe state — the condition the system must achieve or maintain when a fault is detected. The following safe states are applicable to the Airbag ECU:

Table 11:Safe States

| Safe State ID | Description | Applicable Hazard(s) | System Condition |
| --- | --- | --- | --- |
| SS-01 | No deployment (inhibit state) | H1-01, H1-02, H1-03, H1-05 | All firing transistors remain open (off). Reservoir capacitor maintained but no firing command issued. DWI lamp ON if fault detected. |
| SS-02 | Deployment armed, awaiting valid crash signal | H2-01, H2-02, H2-03 | Normal standby. All squib channels armed. Algorithm running. Any fault disables only affected channel, not full system. |
| SS-03 | Passenger airbag suppressed (OCS fault) | H1-04 | If OCS CAN signal lost > 50 ms, passenger airbag is inhibited by default (suppress-on-fault). Driver and side airbags remain armed. |
| SS-04 | Affected channel isolated (partial degradation) | H3-01, H3-02, H4-01 | Faulted squib channel disabled; remaining channels continue normal operation. DWI lamp ON. System logs DTC. |
| SS-05 | System shutdown (unrecoverable MCU fault) | H1-01, H2-01 (related) | All output transistors forced off by hardware safety switch. Watchdog asserts reset signal. If fault persists after 3 resets → permanent shutdown + DWI lamp. |

# 12  Figure and Table Listing

Table 1:Meetings within the Project	3

Table 2:Unresolved Topics	3

Table 3:Glossary	3

Table 4:Use Cases	6

Table 5:Severity	7

Table 6:Exposure Levels	7

Table 7:Controllability Levels	7

Table 8:ASIL–SIL Equivalence	8

Table 9:Hazardous Events and ASIL/SIL Assignment	10

10.3  Summary of Safety GoalsTable 10:Safety Goals Summary	13

Table 11:Safe States	14

Table 12:Dependent Documents	15

Table 13:Version History	15

# 13  List of Dependent Documents

Table 11 lists all documents referenced by or related to this HARA document.

Table 12:Dependent Documents

| No. | Document Name | Version | Date | File Name |
| --- | --- | --- | --- | --- |
| 1 | Concept Phase Document — Airbag ECU (LCP01) | 1.00 | 2026-05-17 | LCP01-AIRBAG-ECU-001_Concept_v100.docx |
| 2 | Safety Concept Document — Airbag ECU | 2.0 | 2026-05-12 | SC-AIRBAG-ECU-001_Rev2.0.pdf |
| 3 | Functional Safety Requirements Specification (LCP03) | 2.1 | 2026-05-13 | FSRS-AIRBAG-ECU-001_Rev2.1.pdf |
| 4 | ISO 26262:2018 — Road Vehicles — Functional Safety | 2018 | — | External standard |
| 5 | IEC 61508:2010 — Functional Safety of E/E/PE Systems | 2010 | — | External standard |

# 14  Literature

ISO 26262:2018 — Road Vehicles — Functional Safety, Parts 1–12. International Organization for Standardization.

IEC 61508:2010 — Functional Safety of Electrical/Electronic/Programmable Electronic Safety-related Systems, Parts 1–7. International Electrotechnical Commission.

ISO 21448:2022 — Safety of the Intended Functionality (SOTIF). ISO.

ISO 16750-3:2007 — Road Vehicles — Environmental Conditions and Testing — Mechanical Loads. ISO.

CISPR 25:2016 — Vehicles, Boats and Internal Combustion Engines — Radio Disturbance Characteristics. IEC.

ISO 11452 — Road Vehicles — Component Test Methods for Electrical Disturbances from Narrowband Radiated Electromagnetic Energy. ISO.

ISO 14229-1:2020 — Unified Diagnostic Services (UDS). ISO.

FMVSS No. 208 — Occupant Crash Protection. U.S. Department of Transportation, NHTSA.

UN Regulation No. 94/95 — Frontal / Side Collision Protection. UN ECE.

# 15  Version History

Table 13:Version History

| Document Name / File | Date | Change Log |
| --- | --- | --- |
| LCP03-AIRBAG-ECU-001_HARA_v100.docx | 2026-05-06 | Version 1.00 — Initial release of the Hazard and Risk Analysis (HARA / LCP03) for the Airbag ECU project. Covers 13 identified hazardous events across 4 hazard categories, 7 safety goals (SG-01 to SG-07), and ASIL D / SIL 3 classification. Based on confirmed dual-channel Arduino Due lab hardware. |