VALIDATION PLANNING DOCUMENT (V&V PLAN)

Document Number: VVP-AIRBAG-ECU-001
Revision: 1.0
Date: 2026-05-19
System: Airbag Electronic Control Unit (ECU)
Applicable Standards: ISO 26262:2018 / IEC 61508:2010
Classification: Functional Safety Validation Planning Document

1. Purpose

This Validation Planning Document defines the strategy, environment, procedures, resources, and acceptance criteria required for Verification and Validation (V&V) activities of the Airbag Electronic Control Unit (ECU).

The purpose of validation is to ensure that:

- Functional safety requirements are correctly implemented

- Safety goals are achieved

- Failure conditions are handled safely

- Timing constraints are satisfied

- CAN communication integrity is maintained

- Redundant architecture behaves correctly

- Hardware and software interactions satisfy safety requirements

The validation activities support lifecycle phases 6–9 according to IEC 61508.

2. Scope

Validation activities include:

Hardware validation:

- Sensor input validation

- Dual Arduino architecture validation

- CAN communication verification

- Digital input validation

- CRC verification

Software validation:

- Crash detection algorithm testing

- Redundancy logic verification

- Fault handling

- Timing verification

- Diagnostic verification

System validation:

- Complete system behavior verification

- End-to-end deployment logic validation

3. Validation Objectives

| Objective ID | Validation Objective |
| --- | --- |
| VO-01 | Verify correct crash detection |
| VO-02 | Verify prevention of unintended deployment |
| VO-03 | Verify passenger airbag suppression |
| VO-04 | Verify dual-channel decision agreement |
| VO-05 | Verify CAN communication integrity |
| VO-06 | Verify diagnostic behavior |
| VO-07 | Verify timing requirements |
| VO-08 | Verify fault handling |

4. Test Environment

4.1 Hardware Setup

| Component | Description |
| --- | --- |
| Arduino Due #1 | Channel A |
| Arduino Due #2 | Channel B |
| POTI-1 | Accelerometer simulation |
| POTI-2 | Gyroscope simulation |
| POTI-3 | Pressure sensor simulation |
| Digital switch | Baby seat detection |
| Breadboard | Sensor connections |
| CAN Interface | Communication monitoring |
| OMS Lab PC | Logging and monitoring |

The laboratory architecture uses redundant 2oo2 processing with two Arduino Due boards and CAN output verification.

5. Test Resources

| Resource | Quantity |
| --- | --- |
| Laboratory PCs | 18 |
| Arduino Due Boards | 2 |
| Breadboards | 1 |
| Potentiometers | 3 |
| CAN Interface Modules | 1 |
| Validation Engineers | Assigned team members |

The laboratory exercise provides multiple computers and Arduino boards for implementation and testing.

6. Validation Strategy

Validation shall be performed in multiple levels:

| Level | Description |
| --- | --- |
| Module Test | Individual software modules |
| Integration Test | Hardware + software integration |
| Architecture Verification | Dual-channel architecture verification |
| System Verification | Full ECU functionality |
| Fault Injection Test | Failure behavior validation |
| Requirement Validation | Requirement traceability |

7. Requirement Traceability

| Requirement | Validation Method |
| --- | --- |
| FSR-01 Crash detection | Sensor threshold testing |
| FSR-03 Deployment logic | Dual-channel comparison |
| FSR-09 Baby-seat suppression | Digital input testing |
| FSR-13 CAN CRC | CAN verification |
| FSR-17 POST | Startup testing |
| FSR-20 Safe-state transition | Fault injection |

Requirements are derived from the FSRS document.

8. Planned Test Scenarios

TC-01 Normal Operation

Purpose: Verify no deployment under normal driving conditions

Input Conditions

| Sensor | Value |
| --- | --- |
| Accelerometer | 10g |
| Gyroscope | 20°/s |
| Pressure | 50kPa |
| Baby Seat | HIGH |

Expected:

- No deployment

- CAN message status = NORMAL

- CRC valid

TC-02 Frontal Crash Detection

Purpose: Verify deployment in crash condition

Input:

| Sensor | Value |
| --- | --- |
| Accelerometer | 85g |
| Gyroscope | 100°/s |
| Pressure | 450kPa |
| Baby Seat | HIGH |

Expected:

- Deployment command generated

- Both channels agree

- CAN deployment frame sent

TC-03 Child Seat Detection

Input:

| Sensor | Value |
| --- | --- |
| Accelerometer | 90g |
| Gyroscope | 110°/s |
| Pressure | 470kPa |
| Baby Seat | LOW |

Expected:

- Passenger airbag suppressed

- Driver airbag remains active

TC-04 CAN Fault Injection

Expected:

- CRC mismatch detected

- Warning generated

- DTC logged

TC-05 Channel Mismatch

Expected:

- Deployment inhibited

- Safe state activated

TC-06 Sensor Failure

Expected:

- Fault detected

- Warning lamp activated

9. Timing Validation

| Function | Requirement | Validation Method |
| --- | --- | --- |
| Crash detection | ≤20ms | Oscilloscope / timestamps |
| Algorithm cycle | ≤1ms | Execution logging |
| Deployment command | ≤40ms | CAN timestamps |
| Side deployment | ≤8ms | Event timing measurement |
| POST | ≤800ms | Startup timing |

Timing requirements come directly from the FSRS.

10. Test Protocol Template

| Test ID | Scene Settings | Expected Output | Actual Output | CAN Result | CRC Status | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- |
| TC-XX | Sensor values | Expected behavior | Measured behavior | Valid/Invalid | Pass/Fail | Pass/Fail |

11. Acceptance Criteria

Validation shall be considered successful if:

- All safety requirements pass verification

- Timing constraints are met

- No unintended deployment occurs

- CAN communication remains valid

- Fault detection behaves correctly

- Dual-channel architecture prevents single-point failures

- Test coverage reaches target level

12. Deliverables

Validation activities shall produce:

- Test specifications

- Test procedures

- Test execution reports

- Requirement traceability matrix

- Fault injection reports

- Coverage report

- Final validation report