Airbag ECU Portfolio

HS Harz — Functional Safety

USER GUIDE

Airbag Electronic Control Unit (ECU) Laboratory Prototype

UG-AIRBAG-ECU-001  |  Version 2.0  |  2026-06-14

ISO 26262:2018 (ASIL D)  /  IEC 61508:2010 (SIL 3)

| Field | Value |
| --- | --- |
| Document Number | UG-AIRBAG-ECU-001 |
| Version | 2.0 |
| Date | 2026-06-14 |
| Status | RELEASED |
| Firmware Baseline | airbag_ecu_rev3_3.ino (Rev 3.3 — FINAL, WDT enabled) |
| ASIL / SIL | ASIL D (ISO 26262:2018) / SIL 3 (IEC 61508:2010) |
| Target Hardware | Arduino Due (SAM3X8E ARM Cortex-M3, 84 MHz) × 2 |
| Based On | HADS-AIRBAG-ECU-001 Rev 2.0 Final ¦ SADS-AIRBAG-ECU-001 Rev 3.0 ¦ SRS-AIRBAG-SW-001 Rev 3.1 ¦ ITP-AIRBAG-ECU-002 Rev 2.1 ¦ VVP-AIRBAG-ECU-001 v1.20 |
| Author | Airbag ECU Safety Engineering Team |
| Reviewer | Functional Safety Manager |

ℹ  This guide describes how to assemble, power up, operate, monitor, and troubleshoot the Airbag ECU laboratory demonstrator. Written for first-time operators — no prior firmware knowledge required.

# 1. Introduction

## 1.1 Purpose of this Guide

This User Guide explains how to set up, power on, operate, and monitor the Airbag ECU laboratory prototype. It covers hardware assembly references, firmware upload, indicator LED and Serial Monitor interpretation, normal operating procedures, the 12-step HW/SW integration bring-up, and a troubleshooting guide for the most common issues encountered during bring-up and test.

This guide does not replace the formal verification documents. For test procedures and acceptance criteria, refer to ITP-AIRBAG-ECU-002 Rev 2.1. For detailed wiring, refer to HADS-AIRBAG-ECU-001 Rev 2.0 Final. For software design and module details, refer to SADS-AIRBAG-ECU-001 Rev 3.0. For requirements, refer to SRS-AIRBAG-SW-001 Rev 3.1.

## 1.2 What this System Does

The system is a laboratory demonstrator of a redundant automotive airbag Electronic Control Unit (ECU), built to ISO 26262 ASIL D / IEC 61508 SIL 3 principles. Two identical Arduino Due boards (ECU 1 and ECU 2) each independently read three potentiometers simulating an accelerometer, a gyroscope, and a pressure sensor, plus a baby seat occupancy switch.

Each ECU independently decides whether a crash condition exists based on its sensor readings. The two ECUs exchange status over a 500 kbit/s CAN bus. An airbag indicator LED fires only if BOTH ECUs independently agree that a crash has occurred (2oo2 voting), AND neither ECU reports a fault, AND the baby seat suppression is not active for that channel. This agreement is enforced twice: once in firmware (voteDeploy logic) and once physically by a DM74LS08N hardware AND gate driving the indicator LEDs. Neither channel is a master — both are fully peer-equal.

## 1.3 System Architecture Overview

| Block | Description |
| --- | --- |
| Sensors (shared) | 3× potentiometers (POTI-1 accel/POTI-2 gyro/POTI-3 pressure) + baby seat switch. Both ECUs read the same physical signals via shared high-impedance ADC inputs. |
| ECU 1 (Arduino Due 1) | Runs identical firmware. ECU_ID=1, transmits CAN ID 0x100, listens for 0x200. Peer-equal — no master role. |
| ECU 2 (Arduino Due 2) | Runs identical firmware. ECU_ID=2, transmits CAN ID 0x200, listens for 0x100. Peer-equal — no slave role. |
| CAN Bus (500 kbit/s) | Connects ECU 1 ↔ ECU 2 via Waveshare SN65HVD230 transceivers, and to OMS PC via USB-CAN-A for passive monitoring. |
| DM74LS08N AND Gate | Physical 2oo2 enforcement layer. Airbag LED can only illuminate if BOTH ECUs simultaneously drive their output pin HIGH. |
| Indicator LEDs (×3) | Airbag 1 (driver), Airbag 2 (passenger), DWI (fault warning) — ALL are RED 3mm LEDs. |
| OMS PC (USBCANV2) | Passive CAN bus monitor at 500 kbit/s. Verifies frame content and timing. Must NOT transmit on the bus. |

## 1.4 Document References

| Document ID | Title | Revision |
| --- | --- | --- |
| HADS-AIRBAG-ECU-001 | Hardware Architecture and Design Specification | Rev 2.0 Final |
| SADS-AIRBAG-ECU-001 | Software Architecture and Design Specification | Rev 3.0 |
| SRS-AIRBAG-SW-001 | Software Requirements Specification | Rev 3.1 |
| FSRS-AIRBAG-ECU-001 | Functional Safety Requirements Specification | Rev 2.1 |
| ITP-AIRBAG-ECU-002 | Integration Test Plan | Rev 2.1 |
| VVP-AIRBAG-ECU-001 | Verification and Validation Plan | v1.20 |

# 2. Safety Information

⛔  THIS IS A LABORATORY DEMONSTRATOR ONLY. It uses indicator LEDs, NOT real pyrotechnic airbag squibs. NEVER connect this circuit to a real airbag initiator, squib charge, or any pyrotechnic device under any circumstances.

## 2.1 Electrical Safety

- Power the Arduino Due boards via USB from a computer or a regulated 5V supply only. Do not exceed 3.3V on any Arduino Due GPIO pin.

- The baby seat switch pull-up resistor MUST be connected to the 3.3V rail, NOT 5V (HADS-CR-001). Connecting to 5V can damage the SAM3X8E GPIO input permanently.

- The DM74LS08N AND gate VCC (pin 14) uses 5V and GND (pin 7) uses common ground. Do not reverse these connections.

- The Waveshare SN65HVD230 CAN transceiver VCC MUST be 3.3V. Do NOT connect it to 5V.

- Disconnect power before changing any wiring on the breadboard.

- All three LEDs (Airbag 1, Airbag 2, DWI) are RED 3mm LEDs. There are no blue, green, or amber LEDs in this implementation.

## 2.2 Firmware Modification Safety

⚠  Any firmware modification used for fault-injection testing (e.g. forcing cpuTest() to return false, removing watchdog_feed(), changing DLC) MUST be done on a SEPARATE copy of the source file. Always keep a clean unmodified copy of airbag_ecu_rev3_3.ino and restore it to both boards after all fault-injection tests are complete.

## 2.3 Known Compliance Gaps (Lab Use Constraints)

The following FSR deviations and open gaps apply to the Rev 3.3 firmware baseline. All are formally accepted for the laboratory demonstration scope:

| Gap / Deviation | FSR | Description | User Impact |
| --- | --- | --- | --- |
| GAP-01 | FSR-11 | CLOSED — 10ms debounce implemented in updateBabySeat() | No impact — resolved. |
| GAP-02 | FSR-12 | Baby seat line continuity not monitored — open circuit returns HIGH (adult assumed) | An open circuit wire on D11 is NOT detected as a fault. Verify wiring physically. |
| GAP-03 | FSR-13 | CLOSED — hardware CRC provided by SAM3X8E CAN controller per ISO 11898 | No impact — resolved. |
| GAP-05 | FSR-17 | CAN0.begin() success not checked in POST | POST does not detect CAN hardware failure at startup. |
| FSR-15 Dev. | FSR-15 | CAN timeout 2000ms vs 50ms FSR requirement — formal deviation | CommFault is raised 2000ms after last valid frame, not 50ms. NCR raised. |
| FSR-05/06 Dev. | FSR-05/06 | Deploy latency >40ms in heartbeat path; side-impact <8ms not guaranteed with polling | Event-triggered TX partially mitigates. Accepted for lab demo. |

# 3. Hardware Setup

ℹ  Full wiring tables, schematics, and component specifications are in HADS-AIRBAG-ECU-001 Rev 2.0 Final. This section provides a quick-reference summary for assembly and verification.

## 3.1 Bill of Materials

| Item | Qty | Specification / Notes |
| --- | --- | --- |
| Arduino Due (SAM3X8E ARM Cortex-M3, 84 MHz) | 2 | ECU 1 and ECU 2 — identical firmware, only 3 #defines differ |
| 10kΩ potentiometer (linear taper) | 3 | POTI-1 (A0, accel), POTI-2 (A1, gyro), POTI-3 (A2, pressure) — shared between both ECUs |
| Tactile push button (SPST, normally open) | 1 | Baby seat simulation on D11 — active-LOW |
| 10kΩ resistor (¼W) | 1 | Baby seat switch pull-up — connect to 3.3V rail ONLY (not 5V) |
| DM74LS08N Quad 2-input AND gate (14-pin DIP) | 1 | Hardware 2oo2 physical layer. VCC pin 14 = 5V, GND pin 7 = common GND |
| 3mm RED LED | 3 | Airbag 1, Airbag 2, DWI — ALL RED. Vf ~2.0V. No other colours used. |
| 220Ω resistor (¼W) | 4 | Current limiter: one per Airbag 1 LED, one per Airbag 2 LED, two for DWI LED (one per ECU D10 output — see Section 3.4) |
| Waveshare SN65HVD230 CAN transceiver | 2 | 3.3V VCC. One per ECU. CANTX/CANRX to Arduino Due CAN pins. |
| USB-CAN-A adapter | 1 | 500 kbit/s, USBCANV2 compatible. Passive monitoring only. |
| 120Ω termination resistor | 2 | One at each physical end of the CAN bus (two total on the bus) |
| Breadboard (full-size 830 tie-point) | 1 | Standard 0.1" pitch |
| Jumper wires, twisted pair (min 1 twist/cm for CANH/CANL) | 1 set | Various lengths for breadboard connections |

## 3.2 Pin Assignment (Both Boards Identical)

ℹ  Both ECU 1 and ECU 2 use exactly the same pin assignments. The table below applies to both Arduino Due boards without distinction.

| Arduino Due Pin | Signal / Connection | Direction | Notes |
| --- | --- | --- | --- |
| A0 | POTI-1 wiper (accelerometer) | Input | 12-bit ADC, 0–4095 = 0–100g, threshold >30g (raw >1228) |
| A1 | POTI-2 wiper (gyroscope) | Input | 12-bit ADC, 0–4095 = 0–360 deg/s, threshold >120 deg/s (raw >1365) |
| A2 | POTI-3 wiper (pressure) | Input | 12-bit ADC, 0–4095 = 0–500 kPa, threshold >200 kPa (raw >1638) |
| D8 | AND gate input — Airbag 1 vote | Output | HIGH = this ECU votes crash confirmed. Feeds AND gate pin 1 (ECU1) or pin 2 (ECU2) |
| D9 | AND gate input — Airbag 2 vote | Output | HIGH = crash AND no baby seat. Feeds AND gate pin 4 (ECU1) or pin 5 (ECU2) |
| D10 | DWI output (shared anode) | Output | HIGH = this ECU has a fault. Both ECUs D10 drive same LED — OR logic |
| D11 | Baby seat switch | Input | Active-LOW. 10kΩ pull-up to 3.3V. LOW=child seat present. HIGH=adult/empty |
| D13 | Status LED (built-in) | Output | Blinks every 500ms when loop() is running normally |
| CANTX / CANRX (labelled) | Waveshare CANTX / CANRX | Bi-dir | Use pins PHYSICALLY labelled CANTX/CANRX — NOT digital pin 1 |
| 3.3V | Waveshare VCC + baby seat pull-up | Power | Common 3.3V rail |
| 5V | AND gate VCC (pin 14) | Power | Common 5V rail |
| GND | Common ground | Power | All grounds tied together — mandatory single reference |

## 3.3 DM74LS08N AND Gate Wiring

| Pin | Name | Connected To | Function |
| --- | --- | --- | --- |
| 1 | 1A | ECU1 (Due 1) D8 | Airbag 1 deploy vote from Channel A |
| 2 | 1B | ECU2 (Due 2) D8 | Airbag 1 deploy vote from Channel B |
| 3 | 1Y | 220Ω → Airbag 1 LED anode → GND | HIGH only when BOTH D8 pins HIGH simultaneously |
| 4 | 2A | ECU1 (Due 1) D9 | Airbag 2 deploy vote from Channel A |
| 5 | 2B | ECU2 (Due 2) D9 | Airbag 2 deploy vote from Channel B |
| 6 | 2Y | 220Ω → Airbag 2 LED anode → GND | HIGH only when BOTH D9 pins HIGH simultaneously |
| 7 | GND | Common GND rail | IC ground reference |
| 14 | VCC | Common 5V rail | IC power — MUST be 5V |
| 8–13 | n/c | Leave floating | Gates 3 and 4 unused |

## 3.4 DWI Lamp Wiring

ECU1 D10 and ECU2 D10 both connect through separate 220Ω resistors to the anode of a single shared RED 3mm LED (OR logic — either ECU can illuminate it). The cathode connects to common GND. This implements an OR-gate in hardware: if either ECU detects a fault, the DWI illuminates.

## 3.5 Potentiometer Wiring

| Terminal | Connected To | Notes |
| --- | --- | --- |
| Leg 1 (end) | 3.3V rail | Top of resistive track. Use 3.3V to constrain wiper to safe ADC range (0–3.3V) |
| Leg 2 (end) | Common GND rail | Bottom of resistive track |
| Leg 3 (wiper) | ADC pin of BOTH Due 1 and Due 2 (shared) | Both ADC inputs are high-impedance — safe to share on the same wire |

## 3.6 CAN Bus Wiring

- Each Waveshare SN65HVD230: VCC → 3.3V, GND → common GND, CANTX → Arduino CANTX pin, CANRX → Arduino CANRX pin.

- CANH of Waveshare 1 ↔ CANH of Waveshare 2 ↔ USB-CAN-A CANH. Do NOT cross CANH/CANL.

- CANL similarly shared across all three nodes.

- 120Ω termination resistor at each physical end of the bus (two total). Waveshare 2 is the middle node — no terminator on it.

- Use twisted pair for CANH/CANL — minimum 1 twist per cm for noise immunity.

# 4. Software Setup

## 4.1 Required Software

| Software | Version | Purpose |
| --- | --- | --- |
| Arduino IDE | 2.x | Compile and upload firmware to both boards |
| due_can library | v1.0 | Install via Arduino IDE Library Manager — search "due_can" |
| USBCANV2 | Latest stable | CAN bus monitoring software on OMS PC — passive mode only |
| Serial Monitor | Bundled in Arduino IDE | 115200 baud diagnostic output from both ECUs |

## 4.2 Firmware Configuration

The final firmware file is airbag_ecu_rev3_3.ino. Both ECU boards run this exact same file. ONLY the following three lines differ between the two boards:

| #define | ECU 1 (Due 1) | ECU 2 (Due 2) |
| --- | --- | --- |
| ECU_ID | 1 | 2 |
| DATA_ID (TX CAN ID) | 0x100 | 0x200 |
| REMOTE_ID (RX CAN ID) | 0x200 | 0x100 |

⚠  Do NOT change any other line between the two boards. All thresholds, timing constants, pin assignments, and logic must remain identical.

## 4.3 Firmware Upload Procedure

- Open airbag_ecu_rev3_3.ino in Arduino IDE.

- Select Tools → Board → Arduino Due (Programming Port).

- Connect ECU 1 (Due 1) via USB. Select the correct COM port under Tools → Port.

- Verify the three #define lines at the top read: ECU_ID=1, DATA_ID=0x100, REMOTE_ID=0x200.

- Click Upload. Wait for "Done uploading" confirmation.

- Open Serial Monitor (Tools → Serial Monitor), set baud rate to 115200.

- Confirm the startup banner appears with all tests PASS and WDT ENABLED (see Section 5.1).

- Disconnect ECU 1.

- Change the three #defines to: ECU_ID=2, DATA_ID=0x200, REMOTE_ID=0x100.

- Connect ECU 2 (Due 2) via USB. Repeat steps 3, 5, 6, 7 for ECU 2.

# 5. Startup Sequence and Indicators

## 5.1 Power-On Self-Test (POST) Banner

When each board powers on, it runs a Power-On Self-Test (POST) in setup() before entering normal operation. The Serial Monitor (115200 baud) shows the following banner:

================================
  ECU 1
  AIRBAG ECU Rev 3.3
================================
  CPU TEST : PASS
  RAM TEST : PASS
  ADC TEST : PASS
  WDT      : ENABLED
================================

| Banner Line | What it Tests | Meaning if PASS / ENABLED | Meaning if FAIL |
| --- | --- | --- | --- |
| CPU TEST | 12345 × 6789 = 83810205 (ALU multiply) | SAM3X8E ALU verified | faultCPU=TRUE — DWI ON permanently, deployment inhibited for session |
| RAM TEST | 128-word checkerboard: 0xAAAAAAAA then 0x55555555 | Memory read-write integrity verified | faultRAM=TRUE — DWI ON permanently, deployment inhibited |
| ADC TEST | analogRead(A0) in range 0–4095 | ADC hardware responding within range | faultADC=TRUE — DWI ON permanently, deployment inhibited |
| WDT : ENABLED | SAM3X8E watchdog timer configured at ~10ms hardware timeout (WDT_MR_WDV(256×10)) | Hardware watchdog active — MCU resets within ~10ms if loop() stalls | If missing from banner: firmware is not Rev 3.3 — re-upload correct file |

## 5.2 Indicator LED Reference

ℹ  All three indicator LEDs (Airbag 1, Airbag 2, DWI) are RED 3mm LEDs. There are no blue, green, or amber indicator LEDs in this implementation.

| LED | Colour | Location | ON Means | OFF Means |
| --- | --- | --- | --- | --- |
| Airbag 1 | RED | AND gate pin 3 output | BOTH ECUs voted crash confirmed, no faults on either — driver airbag deploy condition met | Normal monitoring, only one ECU voted, or a fault is present on either ECU |
| Airbag 2 | RED | AND gate pin 6 output | Same as Airbag 1 AND baby seat switch is OPEN (no child seat) | Airbag 1 condition not met, OR baby seat switch is pressed (suppressed) |
| DWI (fault lamp) | RED | Shared anode on D10 (both ECUs) | At least one ECU has a fault: POST failure, CAN loss (>2000ms), or remote fault reported | Both ECUs healthy, POST passed, CAN communicating normally |
| D13 (status) | Amber/orange (built-in) | On the Arduino Due board itself | Blinks every 500ms — loop() is running normally | Solid OFF or solid ON = firmware crashed or stalled — WDT will reset within ~10ms |

## 5.3 Serial Monitor Field Reference

Every 1000ms, each ECU prints a diagnostic status block over USB-Serial at 115200 baud. Field meanings:

| Field | Meaning |
| --- | --- |
| ECU | 1 or 2 — which board this output is from |
| A0, A1, A2 | Raw 12-bit ADC values (0–4095) from POTI-1, POTI-2, POTI-3 respectively |
| Accel | Scaled accelerometer value in g (0–100g). Threshold for crash: >30.0g |
| Gyro | Scaled gyroscope value in deg/s (0–360 deg/s). Threshold: >120.0 deg/s |
| Press | Scaled pressure value in kPa (0–500 kPa). Threshold: >200.0 kPa |
| BabySeat | 1 = baby seat switch pressed (D11 LOW — child seat present), 0 = open (adult/empty) |
| LocalCrash | 1 = this ECU's own sensors exceed at least one threshold (OR of accel/gyro/press) |
| RemoteCrash | 1 = the OTHER ECU reported LocalCrash=1 in its CAN status frame |
| CommFault | 1 = no valid CAN frame received from the peer ECU within the last 2000ms |
| SystemFault | 1 = faultCPU OR faultRAM OR faultADC OR CommFault is TRUE on THIS ECU |
| VoteDeploy | 1 = this ECU's software vote to deploy is TRUE: drives D8 HIGH (Airbag 1 AND gate input) |

# 6. HW/SW Integration Bring-Up Procedure

ℹ  This 12-step procedure is mandatory before executing any integration test cases (IT-01 to IT-16). All steps must be completed and recorded as PASS before proceeding to Section 7 (Normal Operation). This procedure is specified in ITP-AIRBAG-ECU-002 Rev 2.1 Section 7.

⚠  Do not proceed to integration tests until all 12 steps below record a PASS result.

## Step 1 — Hardware Assembly Verification

Verify that the prototype board layout matches HADS-AIRBAG-ECU-001 Rev 2.0 Final wiring tables exactly. Visually confirm:

- Ground and power planes are isolated from shorts.

- Baby seat pull-up (10kΩ) is connected to 3.3V, NOT 5V.

- All three LEDs are RED 3mm devices.

- CAN transceiver VCC is connected to 3.3V, NOT 5V.

## Step 2 — Power Rail Verification

Power up the bench supply. Verify:

- AND gate VCC (pin 14): 5.0V ±0.25V (measured with multimeter).

- Arduino Due 3.3V rail: 3.3V ±0.15V.

- Waveshare SN65HVD230 VCC: 3.3V ±0.15V.

## Step 3 — ECU 1 Firmware Upload

Connect ECU 1 (Due 1) via USB to the development workstation. Configure firmware with: ECU_ID=1, DATA_ID=0x100, REMOTE_ID=0x200. Compile and upload airbag_ecu_rev3_3.ino.

## Step 4 — ECU 1 POST Verification

Open Serial Monitor at 115200 baud. Confirm startup banner shows: "ECU 1 / AIRBAG ECU Rev 3.3 / CPU TEST : PASS / RAM TEST : PASS / ADC TEST : PASS / WDT : ENABLED". Record PASS.

## Step 5 — ECU 2 Firmware Upload

Disconnect ECU 1. Connect ECU 2 (Due 2). Change firmware to: ECU_ID=2, DATA_ID=0x200, REMOTE_ID=0x100. Compile and upload airbag_ecu_rev3_3.ino.

## Step 6 — ECU 2 POST Verification

Open Serial Monitor at 115200 baud. Confirm banner shows "ECU 2" with all PASS and WDT ENABLED. Record PASS.

## Step 7 — CAN Bus Physical Wiring

Power down the entire system. Wire the CAN bus between boards:

- ECU1 Waveshare CANH ↔ ECU2 Waveshare CANH ↔ USB-CAN-A CANH.

- ECU1 Waveshare CANL ↔ ECU2 Waveshare CANL ↔ USB-CAN-A CANL.

- Install 120Ω termination resistors at each physical bus end (two total on the bus).

- Use twisted pair for CANH/CANL wiring.

## Step 8 — Power-On Self-Test (Both ECUs)

Apply system-wide power. Monitor Serial output on both ECUs simultaneously. Confirm POST flags show zero errors on both boards: CPU PASS, RAM PASS, ADC PASS, WDT ENABLED.

## Step 9 — CAN Bus Monitor Setup

Connect the USB-CAN-A adapter to the OMS PC. Open USBCANV2 and set baud rate to 500 kbit/s. Configure the adapter in passive (listen-only) mode.

## Step 10 — CAN Heartbeat Verification

Confirm that cyclic CAN frames from both ID 0x100 (ECU1) and ID 0x200 (ECU2) appear in USBCANV2 at approximately 500ms intervals. Confirm DLC=8 on all frames. On both ECU Serial Monitors, confirm CommFault=0. Record PASS.

## Step 11 — Quiescent State Verification

With all potentiometers at minimum (fully anticlockwise — 0V input) and baby seat switch open:

- Verify Airbag 1 LED: OFF.

- Verify Airbag 2 LED: OFF.

- Verify DWI LED: OFF.

- Verify D13 status LED: blinking every 500ms.

- Verify Serial: LocalCrash=0, VoteDeploy=0, CommFault=0, SystemFault=0 on both ECUs.

- Confirm D8, D9, D10 output 0V with a voltmeter or oscilloscope.

## Step 12 — Integration Sanity Test

Advance POTI-1 (accelerometer, A0) above the threshold position (raw >1228, approximately 30% of full rotation) on both boards simultaneously. Confirm:

- Airbag 1 RED LED illuminates immediately.

- Serial Monitor shows LocalCrash=1, RemoteCrash=1, VoteDeploy=1 on both ECUs.

Return POTI-1 below threshold. Confirm Airbag 1 LED extinguishes and VoteDeploy returns to 0. Record PASS.

| Step | Result (PASS/FAIL) | Operator | Date | Notes |
| --- | --- | --- | --- | --- |
| Step 1 — HW Assembly | ☐ PASS ☐ FAIL |  |  |  |
| Step 2 — Power Rails | ☐ PASS ☐ FAIL |  |  |  |
| Step 3 — ECU1 Upload | ☐ PASS ☐ FAIL |  |  |  |
| Step 4 — ECU1 POST | ☐ PASS ☐ FAIL |  |  |  |
| Step 5 — ECU2 Upload | ☐ PASS ☐ FAIL |  |  |  |
| Step 6 — ECU2 POST | ☐ PASS ☐ FAIL |  |  |  |
| Step 7 — CAN Wiring | ☐ PASS ☐ FAIL |  |  |  |
| Step 8 — POST Both ECUs | ☐ PASS ☐ FAIL |  |  |  |
| Step 9 — CAN Monitor | ☐ PASS ☐ FAIL |  |  |  |
| Step 10 — CAN Heartbeat | ☐ PASS ☐ FAIL |  |  |  |
| Step 11 — Quiescent State | ☐ PASS ☐ FAIL |  |  |  |
| Step 12 — Sanity Test | ☐ PASS ☐ FAIL |  |  |  |

# 7. Normal Operation

## 7.1 Power-Up Sequence

- Connect both Arduino Due boards to USB power or external regulated supply.

- Both boards run POST automatically. Confirm both Serial Monitors show the Rev 3.3 banner with all PASS and WDT ENABLED (Section 5.1). If any POST line shows FAIL, stop and see Section 10.4.

- Within 3 seconds of power-up, CommFault should read 0 on both ECUs — this confirms the CAN bus is operational and both boards are communicating.

- Set all potentiometers to minimum (fully anticlockwise). Confirm baby seat switch is open. All three LEDs should be OFF.

- System is now in MONITORING state — normal operation.

## 7.2 Operating the Potentiometers (Sensor Simulation)

Each potentiometer simulates a physical crash sensor. Turning clockwise increases the simulated physical value. The deployment thresholds are:

| Potentiometer | Simulates | Physical Range | Crash Threshold | ADC at Threshold | ~Rotation |
| --- | --- | --- | --- | --- | --- |
| POTI-1 (A0) | Accelerometer (frontal impact) | 0–100 g | >30.0 g | raw > 1228 | ~30% |
| POTI-2 (A1) | Gyroscope (rollover rate) | 0–360 deg/s | >120.0 deg/s | raw > 1365 | ~33% |
| POTI-3 (A2) | Pressure sensor (side impact) | 0–500 kPa | >200.0 kPa | raw > 1638 | ~40% |

ℹ  Any ONE of the three sensors exceeding its threshold sets LocalCrash=1 on that ECU (logical OR). All three potentiometers are shared between both ECUs — turning one affects both ECUs simultaneously.

## 7.3 Triggering a Full Crash (Both Airbag LEDs)

- Confirm both boards are in MONITORING state: CommFault=0, SystemFault=0, all LEDs OFF, baby seat switch open.

- Turn POTI-1 (or any single potentiometer) clockwise past its threshold (~30% rotation for POTI-1). Since both ECUs share the same physical potentiometer, both ECUs simultaneously read the same signal.

- Both Serial Monitors show LocalCrash=1, RemoteCrash=1, VoteDeploy=1.

- Airbag 1 RED LED illuminates — both ECUs voted crash, no faults, AND gate output HIGH.

- Since baby seat switch is open, Airbag 2 RED LED also illuminates.

- Return the potentiometer below threshold — both LEDs extinguish, VoteDeploy returns to 0.

## 7.4 Baby Seat Suppression (Passenger Airbag Inhibit)

- During a full crash condition (Section 7.3), press and hold the baby seat push button.

- After 10ms debounce, both Serial Monitors show BabySeat=1.

- Airbag 1 RED LED remains ON (driver airbag — not suppressed by baby seat).

- Airbag 2 RED LED turns OFF — the passenger airbag is suppressed because a child seat is detected on D11.

- DWI LED stays OFF — this is normal active suppression, not a fault condition.

- Release the switch. After 10ms debounce, BabySeat returns to 0 and Airbag 2 re-illuminates if the crash condition is still present.

## 7.5 Deployment Authorization Logic (Reference)

For reference, the complete deployment decision logic confirmed in firmware (SADS-AIRBAG-ECU-001 Rev 3.0):

| localCrash | remoteCrash | systemFault | remoteFault | babySeat | D8 — Airbag 1 | D9 — Airbag 2 |
| --- | --- | --- | --- | --- | --- | --- |
| FALSE | any | any | any | any | LOW | LOW |
| TRUE | FALSE | any | any | any | LOW | LOW |
| TRUE | TRUE | TRUE | any | any | LOW | LOW |
| TRUE | TRUE | FALSE | TRUE | any | LOW | LOW |
| TRUE | TRUE | FALSE | FALSE | TRUE | HIGH | LOW (suppressed) |
| TRUE | TRUE | FALSE | FALSE | FALSE | HIGH | HIGH |

# 8. Fault Conditions and the DWI

The DWI (Diagnostic Warning Indicator, shared RED LED on D10) illuminates whenever EITHER ECU detects a fault. When DWI is ON, no airbag LED can fire regardless of sensor readings — this is the fail-safe inhibit direction.

| Fault Condition | How it Occurs | DWI Behaviour | Deployment Effect | Recovery |
| --- | --- | --- | --- | --- |
| POST failure (CPU/RAM/ADC TEST FAIL) | A self-test fails at power-on — only occurs with intentionally modified firmware during fault-injection testing | DWI ON immediately at startup. Fault is sticky for the session. | D8/D9 remain LOW permanently — no deployment possible | Power-cycle with unmodified airbag_ecu_rev3_3.ino. POST failures cannot be cleared without reset. |
| CAN communication loss (CommFault=1) | CAN bus disconnected, miswired, REMOTE_ID wrong, or one ECU powered off | DWI ON within ~1ms of CommFault becoming TRUE. CommFault itself triggers 2000ms after last valid frame. | voteDeploy forced FALSE on affected ECU — deployment inhibited | Reconnect/restore CAN bus. CommFault clears within one heartbeat (~500ms) once valid frames resume. DWI then turns OFF. |
| Remote fault reported | The other ECU has a fault and broadcasts systemFault=1 in its CAN status byte (byte 6 bit 2) | DWI ON on the healthy ECU as well, via D10 HIGH from remoteFault logic | voteDeploy inhibited on the healthy ECU too | Resolve the fault on the faulted ECU. Clears automatically once remote ECU reports no fault. |
| WDT reset (loop stall) | Firmware loop() exceeds ~10ms watchdog timeout (WDT_MR_WDV(256×10) per SRS SWR-15) — e.g. from intentional IT-16 fault injection or added delay() calls | MCU resets automatically within ~10ms. All outputs go LOW during reset (safe state). DWI may briefly extinguish then re-evaluate after POST. | All outputs LOW during reset — no deployment during stall | Automatic. Board restarts, re-runs POST, resumes normal operation. |

ℹ  FSR-15 deviation: the 2000ms CAN timeout is a formally documented deviation from the FSR-15 requirement of 50ms. A 50ms timeout caused constant false CommFaults given the 500ms CAN heartbeat interval. The safety direction is unaffected — CAN loss always leads to deployment inhibit, never to spurious deployment. NCR raised against FSR-15.

# 9. Monitoring with USBCANV2

## 9.1 Setup

- Connect the USB-CAN-A adapter to the shared CANH/CANL bus and to the OMS PC via USB.

- Open USBCANV2. Set baud rate to 500 kbit/s.

- Configure the adapter in PASSIVE / LISTEN-ONLY mode. It must NOT transmit CAN frames onto the bus.

- Start monitoring. You should see frames with ID 0x100 (from ECU 1) and ID 0x200 (from ECU 2) arriving approximately every 500ms. On the first voteDeploy=TRUE rising edge, an event-triggered frame arrives immediately.

## 9.2 CAN Frame Format Reference (DLC = 8)

| Byte(s) | Field | Encoding | Meaning |
| --- | --- | --- | --- |
| 0–1 | A0 raw (accelerometer) | Little-endian 16-bit: raw = (byte1 << 8) ¦ byte0 | Accelerometer ADC value, 0–4095. Crash threshold: raw > 1228 |
| 2–3 | A1 raw (gyroscope) | Little-endian 16-bit: raw = (byte3 << 8) ¦ byte2 | Gyroscope ADC value, 0–4095. Crash threshold: raw > 1365 |
| 4–5 | A2 raw (pressure) | Little-endian 16-bit: raw = (byte5 << 8) ¦ byte4 | Pressure ADC value, 0–4095. Crash threshold: raw > 1638 |
| 6 | Status flags | Bit 0 = LocalCrash, Bit 1 = BabySeat, Bit 2 = SystemFault, Bits 3-7 = 0 (reserved) | Aggregate status. Byte 6 = 0x00: no crash, no fault. Byte 6 = 0x01: crash detected. |
| 7 | ECU_ID | 0x01 = ECU1, 0x02 = ECU2 | Identifies which board sent this frame |

ℹ  CRC integrity: the SAM3X8E hardware CAN controller automatically appends a 15-bit ISO 11898 CRC to every transmitted frame and verifies it on every received frame. This is not visible in the 8 data bytes shown by USBCANV2 — verification is transparent in hardware.

## 9.3 Example Frame Decode

Frame: ID=0x100, DLC=8, Data = 8B 00 DF 02 5B 04 00 01

| Bytes | Raw Value | Calculation | Result |
| --- | --- | --- | --- |
| 0–1: 8B 00 | 0x008B = 139 | 139 / 4095 × 100 | Accel = 3.4 g (below 30g threshold — no crash) |
| 2–3: DF 02 | 0x02DF = 735 | 735 / 4095 × 360 | Gyro = 64.6 deg/s (below 120 deg/s threshold) |
| 4–5: 5B 04 | 0x045B = 1115 | 1115 / 4095 × 500 | Press = 136.1 kPa (below 200 kPa threshold) |
| 6: 00 | Bit 0=0, Bit 1=0, Bit 2=0 | — | No crash, no baby seat, no fault |
| 7: 01 | 0x01 | — | This frame is from ECU 1 |

# 10. Troubleshooting Guide

## 10.1 CommFault=1 / Random CAN IDs in USBCANV2

Symptom: USBCANV2 shows random changing IDs (e.g. 0x6CA, 0x518) with DLC=8, instead of stable 0x100/0x200. Or one ECU shows CommFault=1.

| Possible Cause | Check / Fix |
| --- | --- |
| Wrong firmware — DLC=10 from an old build | Re-upload airbag_ecu_rev3_3.ino. DLC must be 8. DLC=10 corrupts the frame ID register on Arduino Due (SAM3X8E hardware bug with DLC > 8). |
| CAN TX/RX wires on wrong Arduino Due pins | Confirm Waveshare CANTX/CANRX connect to the Arduino pins physically labelled CANTX/CANRX — NOT digital pin 1 (which is Serial TX on Due). |
| CANH and CANL crossed between boards | Trace: Waveshare 1 CANH must connect to Waveshare 2 CANH (not CANL). Verify polarity at both ends. |
| Wrong termination — missing or too many 120Ω resistors | Exactly 2× 120Ω resistors on the whole bus — one at each physical end. Check if Waveshare boards have onboard termination; remove external resistors if duplicate. |
| REMOTE_ID misconfigured | ECU1 must have REMOTE_ID=0x200, ECU2 must have REMOTE_ID=0x100. Verify the #define values in uploaded firmware. |

## 10.2 CommFault=1 but USBCANV2 Shows Correct Frames

| Possible Cause | Check / Fix |
| --- | --- |
| One board running old firmware rev | Confirm BOTH Serial Monitors show "AIRBAG ECU Rev 3.3". If one shows an earlier revision, re-upload. |
| COMM_TIMEOUT / HEARTBEAT_MS mismatch | Rev 3.3 uses COMM_TIMEOUT=2000ms with HEARTBEAT_MS=500ms. Verify these constants in the source. |
| REMOTE_ID pointing to own DATA_ID | If ECU1 REMOTE_ID = 0x100 (instead of 0x200), it will never receive its own frames as "remote". Cross-check the three #defines carefully. |

## 10.3 No LEDs Illuminate During a Full Crash (IT-02 Fails)

| Possible Cause | Check / Fix |
| --- | --- |
| AND gate wiring incorrect | Verify: ECU1 D8 → AND gate pin 1, ECU2 D8 → pin 2, pin 3 → 220Ω → Airbag1 LED anode → GND. Similarly for D9/Airbag2 circuit on pins 4/5/6. |
| AND gate not powered | Measure 5V at pin 14 and GND at pin 7 with a multimeter. Without power, the AND gate output stays LOW regardless of inputs. |
| One ECU has SystemFault=1 | Check Serial — SystemFault=1 on either ECU means that ECU's D8 stays LOW. Resolve the fault first (see Section 8). |
| CommFault=1 on either ECU | RemoteCrash cannot become 1 if CAN is not working, so VoteDeploy stays 0. Fix CAN communication first (see Section 10.1 / 10.2). |
| Both POTIs need to be above threshold | Confirm both boards see the same potentiometer — they share the wiper. If the wiring is correct, one POTI adjustment affects both ECUs simultaneously. |

## 10.4 Both Airbag LEDs OFF, DWI Always ON

| Possible Cause | Check / Fix |
| --- | --- |
| POST failure on one or both boards | Check Serial startup banner. If CPU/RAM/ADC TEST shows FAIL, that board has a sticky fault. Power-cycle with a clean firmware copy. |
| Fault-injection modification still active in source | Confirm the firmware source does NOT contain "return false" inside cpuTest()/ramTest()/adcTest(), and systemFault is not hardcoded TRUE anywhere in loop(). Restore airbag_ecu_rev3_3.ino. |
| CAN loss causing remote fault on healthy ECU | If DWI is ON because CommFault=1 cascades to systemFault, fix CAN communication per Section 10.1. |

## 10.5 Board Continuously Resetting (Repeating Startup Banners)

| Possible Cause | Check / Fix |
| --- | --- |
| watchdog_feed() missing or not first in loop() | This is intentional behaviour for IT-16 (WDT fault injection). For normal operation, confirm watchdog_feed() is the very FIRST statement in loop() in the source file. |
| loop() taking longer than ~10ms WDT timeout | Check for any added delay() calls, blocking I/O, or infinite inner loops — these trigger the WDT. delay() is forbidden in this firmware per SADS Section 8.4. |

## 10.6 Baby Seat Suppression Not Working (Airbag 2 Still Fires)

| Possible Cause | Check / Fix |
| --- | --- |
| Pull-up resistor connected to 5V instead of 3.3V | D11 pull-up MUST go to the 3.3V rail. A 5V pull-up means D11 never reads LOW (the Due I/O clamps it). Fix per HADS-CR-001. |
| Switch polarity reversed | The switch must pull D11 to GND when pressed. Verify terminal 1 goes to GND and terminal 2 goes to D11 + pull-up. |
| Debounce not yet settled | The 10ms debounce requires D11 to be stable for 10ms before BabySeat updates. Press and hold the switch for at least 20ms before observing the effect. |

# 11. Maintenance and Periodic Checks

| Check | Frequency | Procedure |
| --- | --- | --- |
| Power rail voltages | Before each test session | Measure 5V at AND gate pin 14 and 3.3V at Waveshare VCC and baby seat pull-up with a multimeter. Verify within spec. |
| CommFault status | Before each test session | Confirm CommFault=0 on both Serial Monitors within 3 seconds of power-up. |
| LED quiescent state | Before each test session | All POTIs at minimum, baby seat open — confirm all three LEDs (Airbag 1, Airbag 2, DWI) are OFF. |
| Firmware version | After any re-upload | Confirm Serial banner shows "AIRBAG ECU Rev 3.3" and "WDT : ENABLED" on both boards before testing. |
| Wiring continuity | After any hardware change | Visual inspection + multimeter continuity check against HADS-AIRBAG-ECU-001 Rev 2.0 Final wiring tables. |
| Clean firmware restoration | After every fault-injection test session | Re-upload the unmodified airbag_ecu_rev3_3.ino to ALL affected boards. Confirm normal startup banner (all PASS, WDT ENABLED). |
| CAN bus termination | After any CAN wiring change | Verify exactly two 120Ω termination resistors on the bus — one at each physical end. Measure ~60Ω between CANH and CANL with power off. |

# 12. Glossary

| Term | Definition |
| --- | --- |
| 2oo2 | Two-out-of-Two voting — both ECUs must independently agree (both D8/D9 HIGH) before an airbag LED can illuminate via the AND gate. |
| ASIL | Automotive Safety Integrity Level (A to D, highest) per ISO 26262. This system targets ASIL D. |
| CAN | Controller Area Network — the 500 kbit/s differential serial bus connecting ECU 1, ECU 2, and the OMS PC. |
| CommFault | Communication Fault flag. Set TRUE when no valid CAN frame is received from the peer ECU within 2000ms. Triggers systemFault → deployment inhibit. |
| DWI | Diagnostic Warning Indicator — the shared RED LED (D10 on both ECUs, OR-wired). Illuminates on any fault. |
| ECU | Electronic Control Unit — each Arduino Due (SAM3X8E) board is one ECU. |
| FSR | Functional Safety Requirement from FSRS-AIRBAG-ECU-001. |
| GAP | A documented non-conformance between the firmware and a specific FSR. See SADS Rev 3.0 Section 10. |
| LocalCrash | Boolean flag indicating this ECU's own sensors exceed at least one threshold (OR fusion of accel/gyro/press). |
| OMS PC | Off-board Monitoring System PC — the OMS PC running USBCANV2 for passive CAN monitoring. |
| POST | Power-On Self-Test — CPU, RAM, and ADC checks executed once in setup() before loop() begins. |
| RemoteCrash | Boolean flag received from the peer ECU via CAN (byte 6 bit 0). Indicates the other board has detected a crash. |
| SIL | Safety Integrity Level per IEC 61508 (SIL 1–4). This system targets SIL 3. |
| systemFault | Aggregated fault flag: TRUE if faultCPU OR faultRAM OR faultADC OR CommFault. Inhibits deployment when TRUE. |
| voteDeploy | The per-ECU deployment authorisation boolean: localCrash AND remoteCrash AND NOT systemFault AND NOT remoteFault. Drives D8 HIGH when TRUE. |
| WDT | Watchdog Timer — hardware timer on SAM3X8E that resets the MCU if loop() fails to call watchdog_feed() within ~10ms (WDT_MR_WDV(256×10) per SRS SWR-15). Implemented in Rev 3.3. |

# 13. FSR Compliance Summary

ℹ  This table summarises the compliance status of every Functional Safety Requirement against the airbag_ecu_rev3_3.ino firmware baseline, as documented in SRS-AIRBAG-SW-001 Rev 3.1.

| FSR | Title | ASIL | Status |
| --- | --- | --- | --- |
| FSR-01 | Crash detection timing — analogRead every loop, toPhys(), threshold every iteration | D | SATISFIED |
| FSR-02 | Algorithm cycle ≤1ms — no delay() in loop, all timers via millis() | D | SATISFIED |
| FSR-03 | Deployment authorisation — 6 conditions (4 SW + babySeat + HW AND gate) | D | SATISFIED |
| FSR-04 | Prevention of unintended deployment — D8/D9 only HIGH when all conditions met | D | SATISFIED |
| FSR-05 | Deploy command ≤40ms | D | DEVIATION — event-triggered TX improves; worst-case 500ms. Lab accepted. |
| FSR-06 | Side-impact <8ms | D | HW LIMITATION — polling cannot guarantee <8ms. Accepted for lab. |
| FSR-07 | Dual-channel redundancy — both boards fully independent | D | SATISFIED |
| FSR-08 | Channel agreement — remoteCrash required in voteDeploy + HW AND gate | D | SATISFIED |
| FSR-09 | Passenger airbag suppression — D9 = voteDeploy AND NOT babySeat | D | SATISFIED |
| FSR-10 | Suppress-on-fault — stableBaby retains last value on unstable signal | D | SATISFIED |
| FSR-11 | 10ms digital debounce — updateBabySeat() GAP-01 CLOSED | D | SATISFIED |
| FSR-12 | Baby seat line monitoring (open circuit) | D | NOT IMPLEMENTED — GAP-02 open |
| FSR-13 | CRC-protected CAN — SAM3X8E ISO 11898 hardware CRC (GAP-03 CLOSED) | B | HW SATISFIED |
| FSR-14 | CAN bitstream readback | B | NOT IMPLEMENTED — HW limitation, accepted |
| FSR-15 | CAN timeout ≤50ms | B | DEVIATION — 2000ms implemented. NCR raised. |
| FSR-16 | Crash notification ≤15ms | B | PARTIAL — event-triggered TX on voteDeploy=TRUE; 500ms heartbeat otherwise |
| FSR-17 | POST within 800ms | C | PARTIAL — CPU/RAM/ADC tested; CAN POST and clock check not implemented |
| FSR-18 | Continuous diagnostics ≤100ms | C | PARTIAL — CommFault every loop; no dedicated 100ms diagnostic task |
| FSR-19 | DWI activation ≤800ms | A | SATISFIED — D10 HIGH in same loop cycle (<1ms response) |
| FSR-20 | Safe-state transition on fault | D | SATISFIED — systemFault=TRUE → voteDeploy=FALSE → D8/D9 LOW same loop cycle |
| FSR-21 | Fault logging / DTCs | A | NOT IMPLEMENTED — Serial output only, no EEPROM. Accepted for lab. |
| FSR-22 | WDT and memory integrity | D | PARTIAL — ramTest in POST satisfied; WDT enabled in Rev 3.3; stack guard not implemented |

# 14. Revision History

| Version | Date | Author | Description of Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-06-14 | Airbag ECU Safety Engineering Team | Initial release. Covers hardware setup, firmware upload, startup sequence, indicator reference, normal operation, fault conditions, CAN monitoring, troubleshooting, and maintenance for firmware Rev 3.3. |
| 2.0 | 2026-06-14 | Airbag ECU Safety Engineering Team | Finalized version. Added: 12-step HW/SW integration bring-up procedure (Section 6); bring-up sign-off table; FSR compliance summary table (Section 13); complete system architecture section; HW corrections from HADS Rev 2.0 Final (all LEDs RED, pull-up to 3.3V HADS-CR-001, peer-equal architecture confirmed, no master/slave); SRS Rev 3.1 compliance gaps (GAP-01 CLOSED, GAP-03 CLOSED, WDT enabled Rev 3.3); deployment authorization truth table; event-triggered CAN TX note; expanded troubleshooting (Section 10) with baby seat and WDT sections; document references table; corrected WDT hardware timeout to ~10ms per SRS SWR-15; corrected 220Ω resistor BOM count to 4; corrected troubleshooting cross-references. Status changed from DRAFT to RELEASED. |