QUALITY ASSURANCE REPORT

Airbag Electronic Control Unit (ECU)

Functional Safety Project : ISO 26262:2018 / IEC 61508:2010

| Document Number | QAR-AIRBAG-ECU-001 |
| --- | --- |
| Revision | 1.0 : Initial QA Review |
| Date | 2026-05-23 |
| Prepared By | Quality Assurance Engineer |
| Project | Functional Safety 2026/SS : Airbag ECU Portfolio |
| Documents Reviewed | HARA Rev 2.1 ¦ FSRS Rev 2.1 ¦ Safety Concept Rev 2.0 ¦ System Architecture |
| Overall QA Status | SEE SECTION 3 : SUMMARY DASHBOARD |
| Classification | Confidential : Internal QA Document |

This QA Report evaluates all project documents against ISO 26262:2018 and IEC 61508:2010 requirements. Findings are classified as NON-CONFORMANCE, WARNING, or OBSERVATION. Each finding includes a required action and owner.

# 1.  Document Information

| QA Report Number | QAR-AIRBAG-ECU-001 |
| --- | --- |
| Revision | 1.0 |
| Date | May 2026 |
| QA Engineer | [ QA Department] |
| Project | Airbag ECU : Functional Safety Course 2026/SS |
| Documents Under Review | HARA-AIRBAG-ECU-001 Rev 2.1 ¦ FSRS-AIRBAG-ECU-001 Rev 2.1 ¦ SC-AIRBAG-ECU-001 Rev 2.0 ¦ System Architecture Diagram |
| Review Scope | Document completeness, ISO 26262 compliance, cross-document consistency, traceability, and correctness of ASIL/SIL assignments |
| Applicable Standards | ISO 26262:2018 Parts 1–12 ¦ IEC 61508:2010 Parts 1–7 |
| QA Review Method | Document inspection, checklist-based review, cross-reference analysis |

## 1.1  Reviewed Documents

| Ref | Document Title | Doc Number | Revision | Status |
| --- | --- | --- | --- | --- |
| [D1] | Hazard and Risk Analysis (HARA) | HARA-AIRBAG-ECU-001 | Rev 2.1 | PASS |
| [D2] | Functional Safety Requirements Specification (FSRS) | FSRS-AIRBAG-ECU-001 | Rev 2.1 | PASS |
| [D3] | Safety Concept Document (SCD) | SC-AIRBAG-ECU-001 | Rev 2.0 | PASS |
| [D4] | System Architecture Diagram | N/A | Rev 1.0 | WARNING |

# 2.  Executive Summary

This Quality Assurance Report covers the four core functional safety documents produced for the Airbag ECU project. The review is conducted from the perspective of the Quality Manager / QA Engineer role as defined in the project team structure under ISO 26262:2018 Part 2 (Management of Functional Safety).

The project is progressing well. The three main documents (HARA, FSRS, Safety Concept) demonstrate a strong understanding of functional safety principles and are largely compliant with ISO 26262:2018. The dual-channel 2oo2 architecture decision is well-justified. The laboratory-specific adaptations (Arduino Due, potentiometer sensors, CAN to OMS PC) are appropriately documented.

However, the QA review has identified 4 Non-Conformances, 8 Warnings, and 6 Observations that must be addressed before any of these documents can be formally closed for the course submission. The most critical issues relate to incomplete author/approver sign-offs, missing traceability IDs, unresolved [TBD] entries, and minor cross-document inconsistencies.

| Non-Conformances (NC) | Warnings (W) | Observations (OBS) | Overall Status |
| --- | --- | --- | --- |
| 4 | 8 | 6 | CONDITIONAL PASS Resolution required |

The overall QA status is CONDITIONAL PASS. The project may proceed with implementation but all Non-Conformances must be resolved before final submission.

# 3.  Document Status Dashboard

The following table provides a per-document QA status summary across all review dimensions.

| Document | Completeness | Standard Compliance | Traceability | Consistency | Correctness | Overall |
| --- | --- | --- | --- | --- | --- | --- |
| HARA Rev 2.1 | PASS | PASS | WARNING | PASS | PASS | PASS |
| FSRS Rev 2.1 | WARNING | PASS | WARNING | WARNING | PASS | WARNING |
| Safety Concept Rev 2.0 | PASS | PASS | PASS | WARNING | PASS | PASS |
| System Architecture | FAIL | WARNING | FAIL | WARNING | WARNING | FAIL |

## 3.1  Legend

| Status | Definition |
| --- | --- |
| PASS | Document meets the requirement. No action needed. |
| WARNING | Minor gap or improvement needed. Must be resolved before final submission. Does not block progress. |
| FAIL | Significant gap or missing element. Must be resolved immediately. Blocks sign-off of dependent documents. |

# 4.  Findings Register : Summary Table

All findings are categorised as: NON-CONFORMANCE (NC) : must be resolved before document sign-off; WARNING (W) : must be resolved before final submission; OBSERVATION (OBS) : improvement recommended.

| ID | Type | Document | Title | Priority | Status |
| --- | --- | --- | --- | --- | --- |
| NC-01 | NON-CONFORMANCE | All Documents | Author names and approval signatures missing from all documents | CRITICAL | OPEN |
| NC-02 | NON-CONFORMANCE | FSRS Rev 2.1 | RTM references hazard H2-04 but HARA Rev 2.1 does not use H2-04 as a top-level ID | HIGH | OPEN |
| NC-03 | NON-CONFORMANCE | System Architecture | System Architecture has no document number, revision, author, date, or standards reference | HIGH | OPEN |
| NC-04 | NON-CONFORMANCE | FSRS Rev 2.1 | FSRS introduces SG-04 (pretensioner activation) not present as a primary safety goal in HARA | HIGH | OPEN |
| W-01 | WARNING | HARA Rev 2.1 | [TBD] entries not resolved: bleed resistor discharge time and deployment threshold values | HIGH | OPEN |
| W-02 | WARNING | FSRS Rev 2.1 | Timing discrepancy: HARA says ≤15 ms total deployment decision; FSRS says 20 ms crash detection + 40 ms deployment command | HIGH | OPEN |
| W-03 | WARNING | Safety Concept Rev 2.0 | SG-04 and SG-05 have no safety mechanism rows in the traceability matrix | MEDIUM | OPEN |
| W-04 | WARNING | FSRS Rev 2.1 | Watchdog minimum kick window is still [TBD] : this is a safety-critical value | HIGH | OPEN |
| W-05 | WARNING | System Architecture | Architecture diagram does not show CAN bus connection to OMS LAB correctly : signal directions unclear | MEDIUM | OPEN |
| W-06 | WARNING | FSRS Rev 2.1 | V&V requirements section lacks specific test case IDs and traceability to requirements | MEDIUM | OPEN |
| W-07 | WARNING | HARA Rev 2.1 | H4-02 ASIL assignment inconsistency: HARA says ASIL B but remark says SIL 1 | MEDIUM | OPEN |
| W-08 | WARNING | All Documents | FSRS references 'HARA Rev 2.1' but Safety Concept references 'HARA Rev 2.1' : HARA revision history shows only Rev 1.0 and Rev 2.0 | MEDIUM | OPEN |
| OBS-01 | OBSERVATION | HARA Rev 2.1 | Baby seat failure modes in Section 8 are not formally listed as separate hazard IDs in the main hazard table | LOW | OPEN |
| OBS-02 | OBSERVATION | Safety Concept Rev 2.0 | Hardware-enforced deployment inhibit (SC-09) references an 'e-fuse or dedicated supervisory IC' but no specific component is selected | LOW | OPEN |
| OBS-03 | OBSERVATION | FSRS Rev 2.1 | UC-10 (Vehicle Wash) and UC-12 (End-of-Life) are screened out without reference to the residual capacitor hazard | LOW | OPEN |
| OBS-04 | OBSERVATION | All Documents | No formal Engineering Change Request (ECR) process is referenced despite multiple document revisions | LOW | OPEN |
| OBS-05 | OBSERVATION | System Architecture | OR GATE for the DWI lamp is shown in the architecture but its inputs are not documented anywhere | LOW | OPEN |
| OBS-06 | OBSERVATION | FSRS Rev 2.1 | CAN frame definition is well-structured but CAN message ID and baud rate are not specified | LOW | OPEN |

# 5.  Detailed Findings

## 5.1  Non-Conformances

| NC-01 ¦ NON-CONFORMANCE ¦ All Documents ¦ Document Information ¦ Priority: CRITICAL ¦ Status: OPEN |  |
| --- | --- |
| Title | Author names and approval signatures missing from all documents |
| Finding | All four documents contain placeholder names ('[Name Placeholder]') or blank signature fields. ISO 26262 Part 2 Clause 5.4.5 requires that all safety documents are authored, reviewed, and approved by identified persons with defined roles. This is a mandatory requirement for document validity. |
| Impact | ⚠ Documents cannot be formally baselined or submitted without identified authors and approvers. This is a blocking issue for course submission and for any audit. |
| Required Action | → All team members must populate their names against their roles in all four documents. Sign-off blocks must be signed before any document is finalised. |
| Owner | Project Leader / Safety Manager |

| NC-02 ¦ NON-CONFORMANCE ¦ FSRS Rev 2.1 ¦ RTM : Section 10 ¦ Priority: HIGH ¦ Status: OPEN |  |
| --- | --- |
| Title | RTM references hazard H2-04 but HARA Rev 2.1 does not use H2-04 as a top-level ID |
| Finding | FSR-02 in the FSRS (Section 10, RTM) references governing hazard 'H2-04'. In HARA Rev 2.1, the hazard labelled H2-04 is 'Delayed Deployment (>15 ms)'. However, the FSRS requirement FSR-02 is about 'Crash Algorithm Cycle Time' (1 ms deterministic cycle). This is traceable to H2-04 indirectly but the cross-reference creates an inconsistency that breaks formal traceability. |
| Impact | ⚠ Broken traceability between FSRS and HARA is a direct ISO 26262 Part 8 (ASIL D) violation. Test cases traced to FSR-02 will not link correctly back to the HARA hazard. |
| Required Action | → Correct FSR-02 governing hazard reference to H2-03 and H2-04. Add explicit note that algorithm cycle time is a contributing requirement to both timing hazards. |
| Owner | Safety Engineer / Systems Engineer |

| NC-03 ¦ NON-CONFORMANCE ¦ System Architecture ¦ Full Document ¦ Priority: HIGH ¦ Status: OPEN |  |
| --- | --- |
| Title | System Architecture has no document number, revision, author, date, or standards reference |
| Finding | The System Architecture Diagram submitted is a single-page image with no document control metadata whatsoever. ISO 26262 Part 2 requires all safety-relevant documents to be under configuration management with document number, revision, author, date, and applicable standard references. Without this, the diagram cannot be referenced by HARA, FSRS, or Safety Concept as a controlled document. |
| Impact | ⚠ All three other documents reference the system architecture. If the architecture document is uncontrolled, any change to it is undetected and the downstream documents become invalid. |
| Required Action | → Create a proper Architecture Document wrapping the diagram: add document number (e.g. SA-AIRBAG-ECU-001), revision, date, author, and a brief description section. Place under configuration management in the project Git repository. |
| Owner | System Engineer / Hardware Engineer |

| NC-04 ¦ NON-CONFORMANCE ¦ FSRS Rev 2.1 ¦ Section 4 : Safety Goals ¦ Priority: HIGH ¦ Status: OPEN |  |
| --- | --- |
| Title | FSRS introduces SG-04 (pretensioner activation) not present as a primary safety goal in HARA |
| Finding | The FSRS defines SG-04 as 'The ECU shall activate pretensioners within required timing limits (ASIL C / SIL 2)'. The HARA Rev 2.1 covers pretensioner failure under H2-03 but does not list pretensioner activation as a formal top-level safety goal with the same wording. This creates a forward-reference in the FSRS that does not trace back to a HARA safety goal : a direct traceability violation. |
| Impact | ⚠ ASIL C pretensioner requirements in the FSRS are orphaned from the HARA. Any test cases for pretensioner timing will have broken upward traceability. |
| Required Action | → Either: (a) Add SG-04 as a formal safety goal to HARA Rev 2.1 (requires HARA revision to 2.2), or (b) Remove SG-04 from the FSRS safety goals section and absorb pretensioner requirements under SG-02 with an explanatory note. Option (a) is preferred. |
| Owner | Safety Engineer |

## 5.2  Warnings

| W-01 ¦ WARNING ¦ HARA Rev 2.1 ¦ Section 3 : Assumptions ¦ Priority: HIGH ¦ Status: OPEN |  |
| --- | --- |
| Title | [TBD] entries not resolved: bleed resistor discharge time and deployment threshold values |
| Finding | Section 3 of the HARA still contains '[TBD]' entries for backup capacitor bleed-down discharge time and deployment threshold values. Rev 2.1 is marked as RELEASED. A released document should not contain unresolved [TBD] items in safety-relevant fields. These values are referenced by FSR-03 and FSR-05 in the FSRS. |
| Impact | ⚠ Software and hardware engineers cannot implement or verify these requirements without defined values. |
| Required Action | → Hardware team to define bleed resistor discharge time. Software team to define and document deployment thresholds (g-level, kPa, degrees/s) in SRS. Update HARA to Rev 2.2 once values are confirmed. |
| Owner | Hardware Engineer / Software Engineer |

| W-02 ¦ WARNING ¦ FSRS Rev 2.1 ¦ Section 7 : Timing ¦ Priority: HIGH ¦ Status: OPEN |  |
| --- | --- |
| Title | Timing discrepancy: HARA says ≤15 ms total deployment decision; FSRS says 20 ms crash detection + 40 ms deployment command |
| Finding | HARA Rev 2.1 Section 3.2 states 'deployment decision within 10–15 ms from impact onset'. FSRS Section 7 specifies: crash detection response = 20 ms AND deployment command transmission = 40 ms. These values appear inconsistent. 20 ms detection + 40 ms command = 60 ms total, which significantly exceeds the 15 ms HARA target. Even if the 40 ms is end-to-end from crash onset, it still conflicts with the HARA 15 ms figure. |
| Impact | ⚠ Safety Goal SG-02 is jeopardised by conflicting timing requirements across documents. Test engineers will not know which value to verify against. |
| Required Action | → Clarify and align timing requirements across HARA and FSRS. Define: (a) time from crash onset to threshold crossing (algorithm), (b) time from threshold crossing to deployment command output. Both values must be consistent in HARA and FSRS. Update both documents in the same revision cycle. |
| Owner | Systems Engineer / Safety Engineer |

| W-03 ¦ WARNING ¦ Safety Concept Rev 2.0 ¦ Section 8 : Traceability Matrix ¦ Priority: MEDIUM ¦ Status: OPEN |  |
| --- | --- |
| Title | SG-04 and SG-05 have no safety mechanism rows in the traceability matrix |
| Finding | The Safety Mechanism Traceability Matrix in Section 8 shows columns for SG-01 through SG-07, but SG-04 (pretensioner timing) has no checkmarks in any safety mechanism row, and SG-05 (EMC rejection) only has checkmarks in the EMC hardware measures row. This means SG-04 has no traceable safety mechanism : which is a gap. SG-05 coverage also appears incomplete (should include the AND-gate multi-sensor strategy). |
| Impact | ⚠ SG-04 pretensioner safety goal has no documented safety mechanism to achieve it : this is an ASIL C gap in the safety concept. |
| Required Action | → Add pretensioner-specific safety mechanism row (e.g. squib loop monitoring, timing supervision) with SG-04 checkmark. Add AND-gate multi-sensor plausibility check to SG-05 column. |
| Owner | Systems Safety Engineer |

| W-04 ¦ WARNING ¦ FSRS Rev 2.1 ¦ Section 5.1 / 9.1 ¦ Priority: HIGH ¦ Status: OPEN |  |
| --- | --- |
| Title | Watchdog minimum kick window is still [TBD] : this is a safety-critical value |
| Finding | Both HARA and FSRS specify a windowed watchdog requirement but neither document defines the minimum kick window value. This value is safety-critical: if the minimum window is not defined, a stuck software loop that kicks the watchdog very early (but repeatedly) would not be detected. The maximum window (10 ms) is defined, but the minimum is left as [TBD]. |
| Impact | ⚠ Without the minimum watchdog window, the software team cannot implement a compliant windowed watchdog. The entire SW-hang hazard mitigation (H5-02) is incomplete. |
| Required Action | → Software/Hardware team to define the minimum watchdog kick window based on the worst-case fastest legitimate SW cycle time. Document in FSRS Section 7 and Safety Concept Section 7.1. |
| Owner | Software Engineer / Hardware Engineer |

| W-05 ¦ WARNING ¦ System Architecture ¦ Block Diagram ¦ Priority: MEDIUM ¦ Status: OPEN |  |
| --- | --- |
| Title | Architecture diagram does not show CAN bus connection to OMS LAB correctly : signal directions unclear |
| Finding | The architecture diagram shows CAN BUS as a central block connected by a solid red line to Arduino Due 1 and Arduino Due 2. The OMS LAB connection is shown via a red dashed line from CAN BUS to OMS LAB. However: (1) Signal direction arrows on the OMS LAB connection are unclear (is it bidirectional?), (2) The parallel readback channel described in FSRS FSR-14 is not shown in the diagram at all, (3) The AND GATE blocks are not labelled with their inputs/outputs clearly. |
| Impact | ⚠ Engineers implementing the system may misinterpret the architecture. The parallel readback channel is a mandatory ASIL B requirement (FSR-14) : its absence from the diagram creates a verification gap. |
| Required Action | → Update architecture diagram to: show signal direction arrows clearly, add parallel readback channel, label AND GATE inputs/outputs, add legend for line colours (yellow=sensor, blue=digital, green=diagnostic, red=CAN). |
| Owner | System Engineer |

| W-06 ¦ WARNING ¦ FSRS Rev 2.1 ¦ Section 11 : V&V ¦ Priority: MEDIUM ¦ Status: OPEN |  |
| --- | --- |
| Title | V&V requirements section lacks specific test case IDs and traceability to requirements |
| Finding | Section 11 describes what each test case shall document (sensor settings, expected response, CRC status, pass/fail, etc.) but does not provide any actual test case IDs, test case titles, or a test plan structure. For ASIL D, ISO 26262 Part 6 requires a formal Software Test Specification (STS) with test cases traceable to each functional requirement. |
| Impact | ⚠ Without test case IDs linked to FSR-xx requirements, the V&V activity cannot be formally tracked or reported. The RTM in Section 10 has no test case column. |
| Required Action | → Create a separate Test Plan / Software Test Specification document (or extend Section 11) with at minimum: test case IDs (TC-001 etc.), one test case per FSR, and explicit pass/fail criteria. Add TC-IDs to the RTM. |
| Owner | Test Engineer / V&V Lead |

| W-07 ¦ WARNING ¦ HARA Rev 2.1 ¦ Section 10 : Hazard Table ¦ Priority: MEDIUM ¦ Status: OPEN |  |
| --- | --- |
| Title | H4-02 ASIL assignment inconsistency: HARA says ASIL B but remark says SIL 1 |
| Finding | In HARA Rev 2.1 Section 10, hazard H4-02 (Loss of post-crash CAN broadcast) shows: SIL column = 'SIL 1' but ASIL column = 'ASIL B'. Per the ASIL↔SIL equivalence table in Section 9.5, ASIL B maps to SIL 1–2 (not exclusively SIL 1). This creates ambiguity. The FSRS correctly uses ASIL B / SIL 2 for FSR-13 through FSR-16. The HARA should state SIL 1–2 or SIL 2 consistently. |
| Impact | ⚠ If SIL 1 is used, the CAN requirements in the FSRS (ASIL B / SIL 2) are stricter than the HARA basis : this creates an unexplained upward deviation that needs justification. |
| Required Action | → Correct H4-02 SIL value in HARA from SIL 1 to SIL 2 (or SIL 1–2) to align with FSRS. Document the correction in the revision history. |
| Owner | Safety Engineer |

| W-08 ¦ WARNING ¦ All Documents ¦ Cross-document ¦ Priority: MEDIUM ¦ Status: OPEN |  |
| --- | --- |
| Title | FSRS references 'HARA Rev 2.1' but Safety Concept references 'HARA Rev 2.1' : HARA revision history shows only Rev 1.0 and Rev 2.0 |
| Finding | The HARA document header states Rev 2.1 but the revision history table only has entries for Rev 1.0 and Rev 2.0 : there is no Rev 2.1 entry explaining what changed from 2.0 to 2.1. The FSRS and Safety Concept both cite 'HARA Rev 2.1' as their basis. This creates a configuration management gap. |
| Impact | ⚠ If Rev 2.1 has undocumented changes, the downstream documents (FSRS, SC) may be based on a revision that is not properly controlled. |
| Required Action | → Add Rev 2.1 entry to HARA revision history table documenting what changed from Rev 2.0 to Rev 2.1. Confirm that FSRS and Safety Concept are indeed based on the same Rev 2.1 content. |
| Owner | Quality Manager / Safety Engineer |

## 5.3  Observations

| OBS-01 ¦ OBSERVATION ¦ HARA Rev 2.1 ¦ Section 10 ¦ Priority: LOW ¦ Status: OPEN |  |
| --- | --- |
| Title | Baby seat failure modes in Section 8 are not formally listed as separate hazard IDs in the main hazard table |
| Finding | Section 8 of the HARA provides a detailed analysis of baby seat digital input failure modes (stuck-HIGH ASIL D, stuck-LOW ASIL C, intermittent ASIL D) with individual ASIL assignments. However, these are not listed as separate hazard IDs (e.g. H1-04a, H1-04b, H1-04c) in the main hazard table in Section 10. This makes the traceability from FSRS FSR-09 through FSR-12 harder to follow. |
| Impact | ⚠ Minor traceability gap : not blocking, but should be improved for audit quality. |
| Required Action | → Consider adding H1-04a (stuck-HIGH) and H1-04b (stuck-LOW) as sub-entries in the Section 10 hazard table, or add a cross-reference note in H1-04 pointing to Section 8. |
| Owner | Safety Engineer |

| OBS-02 ¦ OBSERVATION ¦ Safety Concept Rev 2.0 ¦ Section 4.5 ¦ Priority: LOW ¦ Status: OPEN |  |
| --- | --- |
| Title | Hardware-enforced deployment inhibit (SC-09) references an 'e-fuse or dedicated supervisory IC' but no specific component is selected |
| Finding | The Safety Concept correctly identifies the hardware firing inhibit requirement (SC-09) and states 'e-fuse or dedicated supervisory IC'. For the Arduino Due prototype, neither component has been specified in any document. This is acceptable for a laboratory prototype but should be noted as a known gap vs. production architecture. |
| Impact | ⚠ No blocking impact for laboratory implementation. Important for completeness. |
| Required Action | → Add a note in Safety Concept Section 4.5 and Architecture document confirming that for the laboratory prototype the hardware inhibit is implemented via [specify: relay / logic gate / MOSFET circuit], and that an e-fuse or supervisory IC would be required for production. |
| Owner | Hardware Engineer |

| OBS-03 ¦ OBSERVATION ¦ FSRS Rev 2.1 ¦ Section 13 : Use Case Screening ¦ Priority: LOW ¦ Status: OPEN |  |
| --- | --- |
| Title | UC-10 (Vehicle Wash) and UC-12 (End-of-Life) are screened out without reference to the residual capacitor hazard |
| Finding | The FSRS screens out UC-12 (End-of-Life scrapping) with the justification that it is outside the laboratory ECU scope. The HARA identifies H5-04 (residual capacitor charge causing post ignition-off deployment) as an ASIL B hazard explicitly linked to UC-12. Screening UC-12 without referencing H5-04 creates an apparent gap in hazard coverage within the FSRS. |
| Impact | ⚠ The bleed resistor / capacitor discharge requirement is not carried forward into the FSRS. This is a known gap in the laboratory implementation scope. |
| Required Action | → Add a note in the FSRS screening table for UC-12 explicitly acknowledging H5-04 (residual capacitor hazard) as a known gap in laboratory scope, with a statement that this hazard is handled at the hardware design level for production vehicles. |
| Owner | Safety Engineer / Systems Engineer |

| OBS-04 ¦ OBSERVATION ¦ All Documents ¦ Document control ¦ Priority: LOW ¦ Status: OPEN |  |
| --- | --- |
| Title | No formal Engineering Change Request (ECR) process is referenced despite multiple document revisions |
| Finding | The project has already produced multiple document revisions (HARA 1.0 → 2.0 → 2.1, FSRS 1.0 → 2.0 → 2.1, Safety Concept 1.0 → 2.0). For ISO 26262 compliance, changes to baselined safety documents should be controlled through a formal change management process. No ECR form or change control procedure has been produced. |
| Impact | ⚠ Without a change control process, it is not possible to verify that all stakeholders have reviewed and approved changes between revisions. |
| Required Action | → Create a simple ECR template (one-page form: change description, affected documents, safety impact assessment, approver). Apply retrospectively to document the changes already made. |
| Owner | Quality Manager |

| OBS-05 ¦ OBSERVATION ¦ System Architecture ¦ Block Diagram ¦ Priority: LOW ¦ Status: OPEN |  |
| --- | --- |
| Title | OR GATE for the DWI lamp is shown in the architecture but its inputs are not documented anywhere |
| Finding | The architecture diagram shows an OR GATE connected to the DWI Lamp, with inputs from both Arduino Due 1 and Arduino Due 2 (green dashed lines). However, the logic of this OR gate : what signals from each Arduino trigger the lamp : is not documented in the FSRS or Safety Concept. FSR-19 and SG-06 mention the DWI lamp but do not define the OR logic. |
| Impact | ⚠ Test engineers cannot write a test case for the DWI lamp without knowing all the conditions that should activate it. |
| Required Action | → Document the DWI lamp activation logic in Safety Concept Section 4 or FSRS Section 5.5: list all conditions that cause each Arduino to assert the DWI signal, and confirm OR-gate behaviour (either channel fault → lamp ON). |
| Owner | Hardware Engineer / Software Engineer |

| OBS-06 ¦ OBSERVATION ¦ FSRS Rev 2.1 ¦ Section 9.4 : CAN Frame ¦ Priority: LOW ¦ Status: OPEN |  |
| --- | --- |
| Title | CAN frame definition is well-structured but CAN message ID and baud rate are not specified |
| Finding | Section 9.4 defines the CAN deployment frame bit-by-bit (Bits 0–7, Bytes 1–7). This is excellent detail. However, the CAN message ID (11-bit or 29-bit arbitration ID) and the CAN bus baud rate are not defined. For lab implementation, the OMS PC CAN interface needs to be configured to match these parameters. |
| Impact | ⚠ Minor : implementation detail. But without the message ID, both the Arduino CAN library and the OMS PC logging software cannot be configured. |
| Required Action | → Add CAN message ID and baud rate to FSRS Section 9.4 or a new CAN Communication Specification appendix. Confirm these with the OMS lab setup requirements. |
| Owner | Software Engineer |

# 6.  Per-Document Detailed Review

## 6.1  HARA Rev 2.1 : HARA-AIRBAG-ECU-001

| Review Item | Finding | Status |
| --- | --- | --- |
| System description is complete and covers all ECU functions | Covers crash detection, deployment, suppression, diagnostics, CAN. Item boundary clearly defined. | PASS |
| Use cases defined (UC-01 to UC-12) | 12 use cases covering all operational scenarios including workshop, child seat, end-of-life. | PASS |
| ASIL parameters (S/E/C) defined with tables | S0–S3, E0–E4, C0–C3 tables provided with ISO 26262 references. | PASS |
| ASIL lookup matrix provided | ISO 26262 Part 3 Table 4 matrix shown. All ASIL values traceable. | PASS |
| All 13 hazards present with ASIL derivation | H1-01 to H4-02 with S+E+C shown. Baby seat failure modes in Section 8. | PASS |
| Safe states defined (SS-01 to SS-05) | 5 safe states with applicable hazards and system conditions. | PASS |
| Safety goals defined (SG-01 to SG-07) | 7 safety goals with ASIL, SIL, governing hazards. | PASS |
| Worked examples for ASIL derivation | Section 11: two worked examples (unintended deploy + baby seat). Clear step-by-step. | PASS |
| Rev 2.1 change entry missing from revision history | History shows Rev 1.0 and Rev 2.0 only. Rev 2.1 not documented. | FAIL |
| Author names populated | All roles show 'Systems Safety Engineering Team' : individual names missing. | FAIL |
| Bleed resistor discharge time still [TBD] | Safety-critical value not yet defined in a RELEASED document. | WARNING |
| Baby seat failure modes not in main hazard table | Section 8 analysis not cross-referenced as sub-IDs in Section 10 hazard table. | WARNING |
| H4-02 SIL value inconsistency | HARA shows SIL 1; FSRS uses SIL 2 for same hazard. | WARNING |
| ASIL↔SIL equivalence table provided | Section 9.5 with PFH values. Correctly maps ASIL D = SIL 3. | PASS |
| References complete | 10 normative standards listed. | PASS |

## 6.2  FSRS Rev 2.1 : FSRS-AIRBAG-ECU-001

| Review Item | Finding | Status |
| --- | --- | --- |
| Based on HARA and references it correctly | References HARA-AIRBAG-ECU-001 Rev 2.1 throughout. | PASS |
| 22 functional safety requirements (FSR-01 to FSR-22) | Good coverage of crash detection, redundancy, OCS, CAN, diagnostics, memory. | PASS |
| Dual 2oo2 architecture justified | ASIL D SPFM requirement used to justify dual-channel. Well-argued. | PASS |
| 6-condition deployment authorization logic (FSR-03) | Clear AND-gate logic with all 6 conditions listed with rationale. | PASS |
| Suppress-on-fault for baby seat (FSR-10) | Correct implementation of default-safe direction. | PASS |
| CAN frame defined (Section 9.4) | Bit-by-bit frame definition. CRC bytes 6–7 defined. | PASS |
| CAN message ID and baud rate not specified | Engineers cannot configure OMS lab CAN interface without these. | WARNING |
| RTM provided (Section 10) | All FSR-xx mapped to SG, hazard, ASIL. Good coverage. | PASS |
| H2-04 reference inconsistency in RTM | FSR-02 linked to H2-04 but HARA H2-04 is 'Delayed Deployment', not algorithm cycle. | FAIL |
| SG-04 (pretensioner) not in HARA safety goals | Forward reference from FSRS SG-04 not traceable to a HARA safety goal. | FAIL |
| Timing values inconsistent with HARA | HARA: 15 ms total. FSRS: 20 ms detection + 40 ms command. Needs alignment. | WARNING |
| Watchdog minimum kick window [TBD] | ASIL D windowed watchdog cannot be implemented without minimum window value. | WARNING |
| V&V section lacks test case IDs | Section 11 is descriptive but no TC-IDs or test plan structure. | WARNING |
| Author sign-off block blank | Approval table has no names or signatures. | FAIL |
| Safety constraints table complete (SC-01 to SC-09) | 9 constraints well-defined. Hardware inhibit SC-09 important and present. | PASS |
| Use case screening section (Section 13.1) | UC-10 and UC-12 screened out. Residual capacitor hazard not acknowledged. | WARNING |
| Sensor interface values complete (Section 9.1) | Voltage ranges for all 4 inputs defined (0–3.3V). | PASS |

## 6.3  Safety Concept Rev 2.0 : SC-AIRBAG-ECU-001

| Review Item | Finding | Status |
| --- | --- | --- |
| Safety concept answers the 3 fundamental questions | 'How prevent / detect / react' clearly addressed in document introduction. | PASS |
| Dual 2oo2 architecture described in full | Section 4.1 : AND-gate logic, cross-comparison, channel failure handling. | PASS |
| 6-condition deployment authorization table | Section 4.2 : consistent with FSR-03 in FSRS. | PASS |
| Sensor fusion thresholds defined | Section 4.3 : deploy thresholds: >30g accel, >120°/s gyro, >200 kPa pressure. | PASS |
| Baby seat suppress-on-fault with pull-up | Section 4.4 : 10kΩ pull-up, debounce 10ms, periodic continuity check. Complete. | PASS |
| Hardware-enforced firing inhibit (SC-09) | Section 4.5 : hardware path independent of SW. Correct. | PASS |
| POST procedure detailed | Section 5.1 : 9 POST items with action-on-failure. Very complete. | PASS |
| Continuous background diagnostics | Section 5.2 : 7 monitoring items at ≤100ms cycle. Good. | PASS |
| Fault detection summary table | Section 5.3 : 12 failure modes with detection, safe state, ASIL. | PASS |
| Safe states defined consistently with HARA | Section 6 : SS-01 to SS-05 aligned with HARA. State transition principles added. | PASS |
| Watchdog architecture dual-layered | Section 7.1 : HW watchdog + SW task monitor. Good defence-in-depth. | PASS |
| CRC applied at 3 levels (Flash, CAN, RAM) | Section 7.3 : comprehensive CRC strategy. | PASS |
| Safety mechanism traceability matrix | Section 8 : 15 mechanisms × 7 SGs. SG-04 column empty. | WARNING |
| Safety constraints (SC-01 to SC-11) | Section 9 : 11 constraints, all traceable to FSRS or standards. | PASS |
| EMC protection strategy | Section 7.5 : galvanic isolation, shielding, hardware key, multi-sensor plausibility. | PASS |
| Author information | 'Systems Safety Engineering Team' : individual names not populated. | WARNING |
| References to HARA Rev 2.1 | Correct document number cited. Rev 2.1 revision history gap noted. | WARNING |
| OBD workshop reset for SS-05 | Section 6 correctly states SS-05 requires authorised OBD procedure to reset. | PASS |

## 6.4  System Architecture Diagram

| Review Item | Finding | Status |
| --- | --- | --- |
| Dual Arduino Due boards shown | Arduino Due 1 and Arduino Due 2 present with CAN BUS between them. | PASS |
| AND GATE deployment logic shown | Two AND GATE blocks between Arduino boards and Airbag 1/2. | PASS |
| Watchdog shown on both channels | Watch DOG block shown for each Arduino with dashed lines. | PASS |
| All 4 sensors shown | Pressure Sensor, Accelerometer, Gyroscope, Baby Seat Digital Switch : all present. | PASS |
| DWI lamp (OR GATE) shown | Lamp and OR GATE block shown. Architecture concept correct. | PASS |
| OMS LAB connection shown | CAN BUS to OMS LAB with dashed line shown. | PASS |
| Document number, revision, date, author : all missing | No document metadata. Single image with no control information. | FAIL |
| Parallel readback channel not shown | FSR-14 mandatory requirement. Not visible in diagram. | FAIL |
| Signal direction arrows unclear on OMS LAB line | Bidirectional? Unidirectional? Cannot determine from diagram. | WARNING |
| OR GATE inputs not documented | Which fault signals from each Arduino trigger OR GATE → Lamp? | WARNING |
| No legend for line colours | Yellow, blue, green, red dashed lines : no legend explaining meaning. | WARNING |
| AND GATE inputs/outputs not labelled | What enters and exits each AND gate not shown. | WARNING |
| No connection to FSRS interface table | Section 9 of FSRS defines interfaces but architecture is not cross-referenced. | WARNING |

# 7.  Action Plan & Resolution Tracking

The following table consolidates all required actions. The project team must assign owners, agree resolution dates, and update the Status column as actions are completed. All NC items must be closed before final project submission.

| ID | Type | Owner | Required Action | Target Date | Status |
| --- | --- | --- | --- | --- | --- |
| NC-01 | NC | Project Leader | Populate all team member names and obtain signatures in all 4 documents | Before submission | OPEN |
| NC-02 | NC | Safety Engineer | Fix FSR-02 hazard reference from H2-04 → H2-03/H2-04 in FSRS RTM | 1 week | OPEN |
| NC-03 | NC | System Engineer | Create proper Architecture Document with doc number, revision, metadata, and add parallel readback channel | 1 week | OPEN |
| NC-04 | NC | Safety Engineer | Add SG-04 as formal safety goal to HARA Rev 2.2 or absorb into SG-02 | 1 week | OPEN |
| W-01 | WARN | HW + SW Eng | Define bleed resistor discharge time and deployment thresholds; update HARA | 2 weeks | OPEN |
| W-02 | WARN | Systems Eng | Align timing values between HARA (15ms) and FSRS (20ms/40ms); update both | 1 week | OPEN |
| W-03 | WARN | Safety Eng | Add SG-04 and SG-05 coverage to Safety Concept traceability matrix | 1 week | OPEN |
| W-04 | WARN | SW + HW Eng | Define minimum watchdog kick window; document in FSRS and Safety Concept | 2 weeks | OPEN |
| W-05 | WARN | System Eng | Update architecture diagram: arrows, parallel readback, AND gate labels, legend | 1 week | OPEN |
| W-06 | WARN | Test Engineer | Create test case IDs (TC-001+) with pass/fail criteria for each FSR-xx | 2 weeks | OPEN |
| W-07 | WARN | Safety Eng | Correct H4-02 SIL 1 → SIL 2 in HARA; add to next revision | 1 week | OPEN |
| W-08 | WARN | QA Manager | Add Rev 2.1 entry to HARA revision history | Immediate | OPEN |
| OBS-01 | OBS | Safety Eng | Add H1-04a/H1-04b sub-entries or cross-reference in HARA Section 10 | Before final submission | OPEN |
| OBS-02 | OBS | HW Engineer | Document lab prototype hardware inhibit implementation in Safety Concept | Before final submission | OPEN |
| OBS-03 | OBS | Safety Eng | Add H5-04 acknowledgement to FSRS UC-12 screening note | Before final submission | OPEN |
| OBS-04 | OBS | QA Manager | Create simple ECR template and apply retrospectively to existing revisions | Before final submission | OPEN |
| OBS-05 | OBS | HW + SW Eng | Document DWI lamp OR gate activation conditions in Safety Concept | Before final submission | OPEN |
| OBS-06 | OBS | SW Engineer | Add CAN message ID and baud rate to FSRS Section 9.4 | Before final submission | OPEN |

# 8.  Project Strengths : What Is Working Well

The QA review notes the following significant strengths in the project documents. These represent best-practice functional safety work that should be maintained and built upon.

| Ref | Area | Commendation |
| --- | --- | --- |
| S-01 | Architecture : 2oo2 Redundancy | The decision to use two independent Arduino Due boards in a 2oo2 architecture is well-justified with explicit reference to ASIL D SPFM requirements. The 6-condition AND-gate deployment authorization is correctly derived and consistently applied across all three documents. |
| S-02 | HARA : Hazard Depth | The HARA Rev 2.1 contains worked ASIL derivation examples (Section 11) showing step-by-step S+E+C → ASIL calculations for key scenarios. This is excellent and demonstrates real understanding of the ISO 26262 risk assessment methodology. |
| S-03 | Baby Seat Analysis | Section 8 of the HARA provides a dedicated hardware failure mode analysis of the baby seat digital input (stuck-HIGH, stuck-LOW, intermittent) : each with its own ASIL, consequence analysis, and mitigation. This level of detail on a single interface is exemplary. |
| S-04 | Safety Concept : Defence in Depth | The Safety Concept implements layered protection: dual algorithm, AND-gate, hardware firing inhibit, watchdog, CRC at 3 levels, parallel CAN readback, EMC isolation. This is genuine defence-in-depth for an ASIL D system. |
| S-05 | CAN Frame Definition | The FSRS defines the CAN deployment frame at bit level (Bits 0–7, Bytes 1–7 with CRC). This is production-quality detail that will directly enable implementation and testing. |
| S-06 | Suppress-on-Fault Strategy | The suppress-on-fault default for the baby seat (both hardware pull-up and software default) is correctly designed in the safe direction. The periodic line continuity check (drive LOW, verify response) is a sophisticated fault detection measure. |
| S-07 | POST Procedure | The Safety Concept Section 5.1 defines a 9-item POST with specific actions on failure for each item. This directly maps to testable requirements and is well above the minimum expected standard. |
| S-08 | Cross-Document Consistency | Despite the findings noted, the three main documents are broadly consistent with each other. The safety goals appear in all three with the same wording. Safe states are defined in both HARA and Safety Concept consistently. This shows disciplined document management. |
| S-09 | Laboratory Context Documentation | The documents clearly distinguish between the production vehicle context (ISO 26262 full requirements) and the laboratory prototype context (Arduino Due, potentiometers, CAN to PC). This transparency is exactly what a functional safety assessor needs to see. |

# 9.  Recommended Next Steps for the Project

Beyond resolving the findings in Section 7, the following documents and activities are recommended to complete the functional safety portfolio for the course submission:

| Priority | Document / Activity | Description | Owner |
| --- | --- | --- | --- |
| 1 | Resolve all NC items | Close NC-01 through NC-04 before any document is finalised. Especially author names (NC-01) : no document is valid without identified authors. | All team |
| 2 | Software Safety Requirements Spec | Derive SW-level requirements from FSRS FSR-xx. Include: SW architecture, module decomposition, coding standards (MISRA-C), MC/DC coverage target, static analysis tool requirements. | SW Engineer |
| 3 | Hardware Safety Requirements Spec | Derive HW-level requirements from FSRS. Include: sensor interface specs (complete [TBD] values), squib circuit design, firing transistor spec, backup cap spec, bleed resistor. | HW Engineer |
| 4 | Test Plan / Software Test Specification | Create TC-001 through TC-0xx covering all FSR-xx. Define: potentiometer settings for each test, expected CAN output, CRC pass/fail criteria. This enables formal V&V. | Test Engineer |
| 5 | Arduino Due Software Implementation | Implement FSR requirements in code: dual-channel crash algorithm, 6-condition AND-gate, CRC library, CAN TX, watchdog kick, baby seat suppression, POST sequence. | SW Engineer |
| 6 | FMEA (Hardware) | Conduct component-level FMEA for the hardware. Calculate SPFM and PMHF. Verify PMHF < 10 FIT for ASIL D safety goals. Required for ISO 26262 Part 5 compliance. | HW Engineer |
| 7 | Test Report | After running the test plan on the lab hardware, produce a formal test report documenting actual CAN outputs vs. expected, CRC results, pass/fail for each TC. | Test Engineer |
| 8 | ECR Process | Create a simple change request form. Apply to any future revisions of the HARA, FSRS, or Safety Concept. | QA Manager |
| 9 | Final V&V Report | Summarise all V&V activities, test coverage, open findings, and safety goal compliance status for the final submission. | Test Engineer / QA |

# 10.  QA Sign-Off

This Quality Assurance Report has been prepared based on review of the four documents listed in Section 1. It represents the formal QA assessment of the project at this stage.

## 10.1  Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-23 | QA Engineer [Name Placeholder] | Initial QA review of HARA Rev 2.1, FSRS Rev 2.1, Safety Concept Rev 2.0, and System Architecture. 4 NCs, 8 Warnings, 6 Observations identified. |

: END OF QUALITY ASSURANCE REPORT :

QAR-AIRBAG-ECU-001 Rev 1.0  |  Confidential : For Internal Use Only  |  ISO 26262 / IEC 61508