Airbag ECU Portfolio HS Harz — Functional Safety AIRBAG ELECTRONIC CONTROL UNIT Quality Assurance Report — Functional Safety Portfolio

QUALITY ASSURANCE (QA) REPORT QA-AIRBAG-ECU-003 ¦ Revision 3.0 ¦ June 2026

| Document Number | QA-AIRBAG-ECU-003 |
| --- | --- |
| Revision | 3.0 |
| Date | June 2026 |
| Supersedes | QAR-AIRBAG-ECU-002 Rev 2.0 (2026-06-10) |
| Classification | Confidential - Internal QA Document |
| Applicable Standards | ISO 26262:2018 (Parts 1–12), IEC 61508:2010 (Parts 1–7), ISO 21448:2022 (SOTIF) |
| Overall ASIL/SIL | ASIL D (ISO 26262) / SIL 3 (IEC 61508) |
| QA Verdict | APPROVED |

1.  Executive Score Summary

The following table provides a headline assessment result at a glance. Detailed evidence is available in the engineering sections below.

| Indicator | Result | Notes |
| --- | --- | --- |
| Overall QA verdict | Approved with Conditions (Conditional PASS) | Ready for SoSe 2026; unconditional PASS after 5 release conditions closed |
| Composite QA index | 90.8 / 100 | Above audit-ready benchmark of ≥ 85 |
| ASPICE process capability | 11/16 at CL 2; 5/16 at CL 1; 0 at CL 0 | Capability index 84% of target |
| SUP.1 Quality Assurance BPs | 4F + 2L of 6 | 22/24 = 92% — CL 2 Managed achieved |
| Work-product quality | 9F + 4L + 1P of 14 | 36/42 = 86% |
| Integration tests (TR Rev 2.0) | EXECUTED — PASS | 16-step sign-off complete; firmware Rev 3.3 |
| Validation tests (VVP §5) | EXECUTED — PASS | TC-VAL-001…008; TC-VAL-005 PASS with caveat (GAP-02) |
| Structural coverage (stmt/branch/MC-DC) | 100% / 100% / 100% | Achieved on 6 SW modules MOD-01..06 |
| Fault detection rate | 6/6 injected faults detected (100%) | ≥ 90% target met |
| FSR coverage in TR §4.1 | 20/22 = 90.9% | FSR-05 and FSR-06 missing — F-2026-02 |
| Findings closed since QA Rev 2.0 | 7 (4 CRITICAL + 3 WARNINGS) | Incl. SRS↔SADS algorithm conflict and COMM_TIMEOUT inconsistency |
| Open Non-Conformities | 3 (NCR-001, F-2026-02, F-2026-04) | 1 Major safety NCR + 1 Major doc-gap + 1 Minor repo-merge gap |
| Open Improvements / Editorial | 4 (F-2026-01, 03, 05, 06, 07) | All scheduled by 2026-06-22 / 2026-06-24 |

Score after closure of the 5 release conditions: composite index ≈ 96 / 100, all 16 processes at CL 2, verdict moves to unconditional PASS and repository can be tagged v1.0.0.

2.  Document Control

2.1  Document History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-25 | QA Lead | Initial QA report — concept-phase documents only (LCP01–LCP05) |
| 2.0 | 2026-06-10 | Thirthashri Gowli Ganesh | Extended review across 9 documents. CRITICAL findings on SRS↔SADS algorithm and COMM_TIMEOUT inconsistency. |
| 3.0 | 2026-06-16 | Thirthashri Gowli Ganesh | Final portfolio review across 13 documents incl. firmware Rev 3.3, TR Rev 2.0 evidence. |

2.2  Documents Reviewed in Rev 3.0

| Ref | Document | ID | Rev | Date | Status |
| --- | --- | --- | --- | --- | --- |
| D-01 | Concept (LCP 1) | LCP01-AIRBAG-ECU-001 | 1.10 | 2026-05-01 | RELEASED |
| D-02 | Task of the Process (LCP 2) | LCP02-AIRBAG-ECU-001 | 1.10 | 2026-05-03 | RELEASED |
| D-03 | Hazard and Risk Analysis (LCP 3) | LCP03-AIRBAG-ECU-001 | 2.00 | 2026-05-05 | RELEASED |
| D-04 | Functional Safety Requirements Specification (LCP 4) | LCP04-AIRBAG-ECU-001 | 1.00 | 2026-05-11 | RELEASED |
| D-05 | Safety Concept Document (LCP 5) | LCP05-AIRBAG-ECU-001 | 1.00 | 2026-05-12 | RELEASED |
| D-06 | Software Requirements Specification | SRS-AIRBAG-SW-001 | 3.1 | 2026-06-14 | RELEASED |
| D-07 | Hardware Architecture & Design Specification | HADS-AIRBAG-ECU-001 | 2.0 Final | 2026-06-14 | RELEASED |
| D-08 | Software Architecture & Design Specification | SADS-AIRBAG-ECU-001 | 3.0 | 2026-06-14 | RELEASED |
| D-09 | Integration Test Plan | ITP-AIRBAG-ECU-001 | 3.0 | 2026-06-13 | RELEASED |
| D-10 | V&V Plan | VVP-AIRBAG-ECU-001 | 1.3 | 2026-06-14 | RELEASED |
| D-11 | Test Report | TR-AIRBAG-ECU-001 | 2.0 | 2026-06-15 | RELEASED |
| D-12 | User Guide / Manual | UG-AIRBAG-ECU-001 | 2 | 2026-06-12 | RELEASED |
| D-13 | Firmware Baseline (source) | FINAL-CODE-AIRBAG-ECU-001_2.ino | 3.3 | 2026-06-15 | RELEASED |

3.  QA Strategy and Methodology

3.1  QA Objectives

This QA Report provides independent quality evidence that the Airbag ECU project portfolio satisfies the planning, processes, traceability, and verification & validation expected by IEC 61508 (Lifecycle phases 1–16) and the ISO 26262 ASIL D classification confirmed in LCP03. The six key objectives are:

1.  Confirm that all 16 IEC 61508 lifecycle phases have associated work products in the project portfolio.

2.  Verify bi-directional traceability: HARA → FSRS → SCD → SRS → SADS → Code → ITP → TR.

3.  Audit document quality (history, glossary, sign-off, dependencies, version control).

4.  Audit software quality, coding constraints, and structural coverage on firmware Rev 3.3.

5.  Audit the executed test campaign (Test Report Rev 2.0) and open non-conformances.

6.  Issue release recommendation for the SoSe 2026 presentation with explicit open items and risks.

3.2  Methods Applied

Document reviews of all 13 deliverables (checklist-based, cross-referenced); static cross-document consistency checks on author/department, version numbers, dates, FSR↔SWR↔TC IDs; audit of executed integration & validation tests on the assembled hardware (TR Rev 2.0 evidence boxes); firmware Rev 3.3 audit against SRS Rev 3.1 and SADS Rev 3.0; audit of compliance gaps GAP-01…GAP-08 and NCR-001 status; and carry-over check of the 4 CRITICAL and 7 WARNING findings from QA Rev 2.0.

4.  Process Compliance: IEC 61508 Lifecycle Phases 1–16

Summary: 14 lifecycle phases are rated OK and 2 are Partial. Zero phases are Non-Compliant.

| LCP | Phase | Work Product | Status | Finding |
| --- | --- | --- | --- | --- |
| 1 | Concept | D-01 LCP01 Concept | OK | - |
| 2 | Overall scope definition | D-02 LCP02 Task of the Process | OK | - |
| 3 | Hazard & Risk Analysis | D-03 LCP03 HARA | OK | - |
| 4 | Overall safety requirements | D-04 LCP04 FSRS | OK | F-2026-01 |
| 5 | Safety requirements allocation | D-05 LCP05 SCD | OK | - |
| 6 | Operation & maintenance planning | D-12 UG v2 | OK | - |
| 7 | Validation planning | D-10 VVP Rev 1.3 | OK | - |
| 8 | Installation & commissioning planning | ITP §8 / TR §1 12-step bring-up | OK | - |
| 9 | Realisation — E/E/PE system | SRS Rev 3.1, HADS Rev 2.0, SADS Rev 3.0, FW Rev 3.3 | OK | F-2026-02 |
| 10 | Other risk reduction measures | LCP05 §10.5 EMC + hardware firing key | Partial | F-2026-03 |
| 11 | Overall installation & commissioning | TR Rev 2.0 §1 — 12 bring-up steps PASS | OK | - |
| 12 | Overall safety validation | VVP §5 TC-VAL-001…008 + TR Rev 2.0 IT-01…IT-16 | OK | - |
| 13 | Overall operation, maintenance & repair | D-12 UG v2 §Operation/Maintenance | OK | - |
| 14 | Overall modification & retrofit | GitHub repo; document revision histories | OK | F-2026-04 |
| 15 | Decommissioning / disposal | Out of lab scope — referenced in LCP01 §5.1 | Partial | F-2026-05 |
| 16 | Verification (transversal) | ITP Rev 3.0, VVP §4, TR Rev 2.0 (16 ITs) | OK | - |

5.  Document Reviews

5.1  Per-Document Quality Scoring

| # | Document | Completeness | Compliance | Traceability | Consistency | Correctness | Overall |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LCP01 Concept | PASS | PASS | PASS | PASS | PASS | PASS |
| 2 | LCP02 Task | PASS | PASS | PASS | PASS | PASS | PASS |
| 3 | LCP03 HARA v2.0 | PASS | PASS | PASS | PASS | PASS | PASS |
| 4 | LCP04 FSRS v1.00 | PASS | PASS | PASS | PASS | PASS | PASS |
| 5 | LCP05 SCD v1.00 | PASS | PASS | PASS | PASS | PASS | PASS |
| 6 | SRS Rev 3.1 | PASS | PASS | PASS | PASS | PASS | PASS |
| 7 | HADS Rev 2.0 Final | PASS | PASS | PASS | PASS | PASS | PASS |
| 8 | SADS Rev 3.0 | PASS | PASS | PASS | PASS | PASS | PASS |
| 9 | ITP Rev 3.0 | PASS | PASS | PASS | PASS | PASS | PASS |
| 10 | VVP Rev 1.3 | PASS | PASS | PASS | PASS | PASS | PASS |
| 11 | TR Rev 2.0 | PASS | PASS | PASS | WARNING | PASS | CONDITIONAL PASS |
| 12 | User Guide v2 | PASS | PASS | PASS | PASS | PASS | PASS |
| 13 | Firmware Rev 3.3 | PASS | PASS | PASS | PASS | PASS | PASS |

5.2  Why TR Rev 2.0 is rated CONDITIONAL PASS

The Test Report is technically sound — all 16 integration tests (IT-01…IT-16) executed and PASS, structural coverage is 100% statement / 100% branch / 100% MC/DC, and every executed result is supported by archived evidence. The Consistency dimension is downgraded to WARNING and the Overall to CONDITIONAL PASS for a single, well-defined reason captured as F-2026-02 (Major):

| Issue | Detail |
| --- | --- |
| Where | TR Rev 2.0 §4.1 FSR Traceability Final Status table |
| What is wrong | FSR-05 (Deploy command ≤40ms, ASIL D) and FSR-06 (Side-impact deployment <8ms, ASIL D) are absent from the TR coverage table. |
| Why it matters | Both are ASIL D requirements with deviation/limitation status in SRS Rev 3.1, so they must appear in the TR coverage table. |
| Safety impact | None. The deviations are already documented in SRS Rev 3.1 and are in the fail-safe direction. This is a documentation / traceability gap only. |

5.3  Open Findings

| ID | Severity | Document / Topic | Description | Owner | Due | Status |
| --- | --- | --- | --- | --- | --- | --- |
| NCR-001 | Major | SWE.3/SWE.6 | FSR-15 deviation (50ms vs 2000ms) — accepted, fail-safe direction | SW Safety Eng. | Open | Open |
| F-2026-01 | Minor | FSRS LCP04 | SC-10/SC-11 inconsistency between FSRS §8 and SCD §12 | Mobina Ketabi | 2026-06-22 | Open |
| F-2026-02 | Major | Test Report §4.1 | FSR-05 (≤40ms) and FSR-06 (<8ms) missing from TR Rev 2.0 §4.1 coverage table | M. Chavez Avila | 2026-06-22 | Open |
| F-2026-03 | Minor | LCP05 §10.5 EMC | EMC measures defined but untested — declare lab-scope deviation | FSM | 2026-06-22 | Open |
| F-2026-04 | Minor | Repo | Repo holds placeholders only; merge full doc set before v1.0.0 tag | Paresh Patil | 2026-06-22 | Open |
| F-2026-05 | Minor | User Guide | User Guide v2 missing LCP-15 decommissioning section | Doc Lead | 2026-06-24 | Open |
| F-2026-06 | Editorial | LCP01 §4.3 | Role labels inconsistent with README (FSM and FS Assessor) | Alita Shrestha | 2026-06-22 | Open |
| F-2026-07 | Editorial | LCP02 §9 | Dependent-doc table cites LCP04 instead of LCP05 for SCD | Alita Shrestha | 2026-06-22 | Open |

6.  Requirements Traceability Audit

6.1  Forward Traceability Chain

HARA → FSRS → SCD → SRS → SADS → Firmware Rev 3.3 → ITP/VVP → TR Rev 2.0

6.2  FSR Coverage in TR Rev 2.0 §4.1

| FSR | Title | ASIL | TR Status | Notes |
| --- | --- | --- | --- | --- |
| FSR-01 | Crash detection timing | D | PASS (IT-02, 06, 12) | OK |
| FSR-02 | Algorithm cycle ≤1ms | D | PASS (IT-11) | OK |
| FSR-03 | Deployment authorisation | D | PASS (IT-02, 06, 09) | OK |
| FSR-04 | Prevention of unintended deploy | D | PASS (IT-01, 03, 06) | OK |
| FSR-05 | Deploy command ≤40ms | D | MISSING from TR §4.1 | SRS Rev 3.1 §3: DEVIATION — F-2026-02 |
| FSR-06 | Side-impact deployment <8ms | D | MISSING from TR §4.1 | SRS Rev 3.1 §3: HW LIMITATION — F-2026-02 |
| FSR-07 | Dual-channel redundancy | D | PASS (IT-06) | OK |
| FSR-08 | Channel agreement verification | D | PASS (IT-06) | OK |
| FSR-09 | Passenger airbag suppression | D | PASS (IT-04) | OK |
| FSR-10 | Suppress-on-fault | D | PASS (IT-04) | GAP-02 still open |
| FSR-11 | Digital input debounce 10ms | D | PASS (IT-04) | GAP-01 CLOSED Rev 3.3 |
| FSR-12 | Baby seat line monitoring | D | BLOCKED | GAP-02 OPEN — accepted lab scope |
| FSR-13 | CRC-protected CAN | B | PASS (IT-05, 13) | GAP-03 CLOSED SAM3X8E HW CRC |
| FSR-14 | CAN bitstream readback | B | BLOCKED | GAP-04 OPEN — HW limitation |
| FSR-15 | CAN timeout ≤50ms | B | PASS @ 2000ms | NCR-001 OPEN — accepted deviation |
| FSR-16 | Crash notification ≤15ms | B | PASS (IT-05, 13) | Event-triggered TX added Rev 3.0 |
| FSR-17 | POST | C | PASS (IT-10) partial | GAP-05 OPEN Can0.begin() not checked |
| FSR-18 | Continuous diagnostics ≤100ms | C | PASS (IT-07) | OK |
| FSR-19 | DWI ≤800ms | A | PASS (IT-15) | OK |
| FSR-20 | Safe-state transition | D | PASS (IT-07, 10) | OK |
| FSR-21 | DTC logging | A | BLOCKED | GAP-07 OPEN — Serial only, lab scope |
| FSR-22 | Memory integrity / WDT | D | PASS (IT-16) | GAP-06 CLOSED WDT enabled Rev 3.3 |

6.3  Coverage Statistics

| Metric | Value |
| --- | --- |
| Total FSRs | 22 |
| FSRs verified PASS | 16 (FSR-01..04, 07..11, 13, 15, 16, 17, 18, 19, 20, 22) |
| FSRs BLOCKED (open GAPs accepted) | 3 (FSR-12, FSR-14, FSR-21) |
| FSRs missing from TR coverage table | 2 (FSR-05, FSR-06) — F-2026-02 |
| FSR coverage in TR (PASS + BLOCKED tracked) | 20/22 = 90.9% |
| FSR coverage including F-2026-02 documented | 100% once F-2026-02 is closed |

7.  Software Quality Audit

7.1  Module Status (SADS Rev 3.0 §4.1)

| Module ID | Module Name | Primary FSRs | ASIL | Status Rev 3.3 |
| --- | --- | --- | --- | --- |
| MOD-01 | Power-On Self-Test | FSR-17 | C | Implemented (CPU/RAM/ADC) — GAP-05 open |
| MOD-02 | Sensor Acquisition | FSR-01, FSR-02, FSR-11 | D | Implemented — debounce added (GAP-01 CLOSED) |
| MOD-03 | Crash Detection | FSR-01..04 | D | Implemented — OR-fusion of 3 sensors |
| MOD-04 | Deployment Control | FSR-03, 08, 09, 10, 20 | D | Implemented — voteDeploy + event-trig TX |
| MOD-05 | CAN Communication | FSR-13..16 | B | Implemented — HW CRC + 500ms HB + event TX |
| MOD-06 | Diagnostic Monitor | FSR-17, 18, 19, 20, 22 | C/D | Implemented — systemFault aggregation |

7.2  Structural Coverage (TR Rev 2.0 §2.1, §4.2)

| Metric | ASIL D Target | Achieved | Method / Evidence |
| --- | --- | --- | --- |
| Statement coverage | 100% | 100% | All MOD-01…06 exercised across IT-01…IT-16 |
| Branch coverage | 100% | 100% | TR §4.2 — both TRUE/FALSE outcomes of all conditionals covered |
| MC/DC coverage | 100% | 100% | voteDeploy 4-condition AND, baby-seat gate, OR-fusion — TC-VER-002 |
| Fault detection rate | ≥ 90% | 100% (6/6) | IT-07, IT-09, IT-10, IT-14, IT-15, IT-16 — all faults detected |

7.3  Coding Constraints Compliance

Audit of FINAL-CODE-AIRBAG-ECU-001_2.ino Rev 3.3 confirms compliance with all SADS §8.4 coding constraints — 0 violations detected.

Permitted: analogRead, digitalRead/Write, Can0.sendFrame/read, millis(), Serial.print.

Forbidden (confirmed absent): malloc, new, delete, recursion, unbounded loops, delay() inside loop(), clearing sticky faultCPU/RAM/ADC after set, runtime modification of ECU_ID/DATA_ID/REMOTE_ID.

7.4  Compliance Gap Status (SADS Rev 3.0 §10)

| GAP ID | FSR | Description | Status Rev 3.3 | QA Disposition |
| --- | --- | --- | --- | --- |
| GAP-01 | FSR-11 | Baby-seat debounce | CLOSED | Accepted |
| GAP-02 | FSR-12 | Baby-seat line continuity monitoring | OPEN | Accepted for laboratory scope; flagged in UG |
| GAP-03 | FSR-13 | Software CRC removed (HW CRC used) | CLOSED | Accepted — ISO 11898 HW CRC |
| GAP-04 | FSR-14 | Parallel CAN bitstream readback | OPEN — HW limit | Accepted deviation |
| GAP-05 | FSR-17 | Can0.begin() return not checked | OPEN | Recommend fix before V&V completion |
| GAP-06 | FSR-22 | Hardware WDT enable | CLOSED Rev 3.3 | Accepted |
| GAP-07 | FSR-21 | DTC logging to EEPROM | OPEN — lab scope | Accepted deviation |
| GAP-08 | FSR-15 | CAN timeout 2000ms vs FSRS 50ms | OPEN — NCR-001 | Conditional accept; safe-direction confirmed |

8.  Hardware / Integration Test Audit (TR Rev 2.0)

8.1  Bring-Up Sign-Off (TR §1)

All 12 bring-up steps PASS. Date: 2026-06-15. Test location: HS Harz Laboratory, ambient 16°C, supply 3.3V measured. Executed by Om Naik, reviewed by Mariana Chavez Avila on firmware Rev 3.3 (both ECU 1 and ECU 2).

8.2  Integration Test Execution (TR §2.1)

Aggregate result: 16 integration tests (IT-01…IT-16) executed against firmware Rev 3.3 — 16 PASS, 0 FAIL, 0 BLOCKED.

Coverage areas: no-crash baseline, full + single-channel crash (2oo2 AND gate), baby-seat suppression, CAN heartbeat/disconnect/reconnect, peer fault bit, POST fault, ≤1ms loop, ≤20ms crash response, DWI ≤800ms, WDT reset (GAP-06 closed). Single accepted deviation NCR-001 (FSR-15 timeout 2000ms) materialises in IT-07 and IT-14.

8.3  Validation Test Execution (VVP Rev 1.3 §5)

8 validation cases TC-VAL-001…TC-VAL-008 covering SG-01, SG-02, SG-03, SG-05, SG-06 and SG-07 — all PASS. TC-VAL-005 carries the GAP-02 caveat (baby-seat line continuity, accepted lab scope); TC-VAL-006 references NCR-001.

8.4  Evidence Archive (TR §6)

All evidence (per IT case: ECU 1 Serial log, ECU 2 Serial log, USBCANV2 CSV, LED photo, timing log) is recorded as ARCHIVED under TEST_SESSION_2026-06-15/. Evidence record is complete; LED photos and CAN logs referenced in each IT-XX record are present in the test session folder.

9.  Non-Conformance Register

| NCR ID | Source IT | Severity | Description | Root Cause | Corrective Action | Status |
| --- | --- | --- | --- | --- | --- | --- |
| NCR-001 | IT-14 | Major | FSR-15 specifies ≤50ms CAN timeout; implementation uses 2000ms. | A 50ms timeout would produce continuous false faultCOMM because the heartbeat is 500ms. | Formal deviation rationale: timeout is in the fail-safe direction. Long-term: shorten heartbeat below 50ms or move to event-driven inter-ECU sync. | OPEN — accepted |

10.  Risks and Mitigations

| ID | Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- | --- |
| R-01 | F-2026-02 — FSR-05/FSR-06 deviations not visible in TR §4.1 | Medium | High | Add both FSRs to TR §4.1 with status DEVIATION; reference SRS Rev 3.1 |
| R-02 | NCR-001 timeout 40× larger than FSR — visibility to assessor | High | Medium | Already accepted; ensure formal NCR is signed off by FSM and QM |
| R-03 | Arduino Due is not automotive-qualified — exercise-only platform | High | Low | Documented as laboratory constraint in LCP01 §5 and User Guide |
| R-04 | GAP-02 — baby-seat open-circuit not detected (FSR-12 BLOCKED) | Medium | Medium | Pull-up holds line HIGH on open circuit (fail-safe). Document residual risk. |
| R-05 | Repo snapshot has only role placeholders (F-2026-04) | High | Medium | Merge all 13 documents and firmware Rev 3.3 into repo before v1.0.0 tag |
| R-06 | EMC measures (LCP05 §10.5) untested | Medium | Low | Declared out-of-scope for laboratory exercise |

11.  Metrics Summary

| Metric | Value |
| --- | --- |
| Lifecycle phases planned (IEC 61508) | 16 |
| Lifecycle phases with released work products | 14 OK + 2 Partial |
| Hazards identified (HARA) | 12 (H1-01 … H4-02) |
| Use cases (HARA) | 12 (UC-01 … UC-12) |
| Safety Goals | 7 (3×ASIL D, 2×ASIL C, 1×ASIL B, 1×ASIL A) |
| Functional Safety Requirements | 22 (FSR-01 … FSR-22) |
| Safety Constraints | 11 (SC-01 … SC-11) — note: FSRS lists SC-01..SC-09 (F-2026-01) |
| Software Requirements (SRS Rev 3.1) | 25 (SWR-01 … SWR-25) |
| Software modules | 6 (MOD-01 … MOD-06) |
| Integration test cases executed (TR Rev 2.0) | 16 (IT-01 … IT-16) |
| Integration test pass rate | 16/16 = 100% |
| Validation test cases | 8 (TC-VAL-001 … 008) — all PASS |
| Verification test cases | 10 (TC-VER-001 … 010) — all PASS |
| Statement / Branch / MC-DC coverage | 100% / 100% / 100% |
| Open GAPs | 5 (GAP-02, 04, 05, 07, 08) |
| Closed GAPs | 3 (GAP-01, 03, 06) |
| Open NCRs | 1 (NCR-001) |
| QA findings open (this report) | 7 (1 Major, 4 Minor, 2 Editorial) |
| QA findings closed since Rev 2.0 | 7 (4 NC + 3 W from Rev 2.0) |

12.  ASPICE Process Capability Level Results

Target: CL 2 (Managed) for SYS, SWE, MAN.3, SUP.1, SUP.8, SUP.10; CL 1 (Performed) for SUP.9 per VDA scope.

| Process | Target CL | Achieved CL | Conditions / Gap |
| --- | --- | --- | --- |
| MAN.3 | 2 | CL 2 | — |
| SYS.1 | 2 | CL 2 | — |
| SYS.2 | 2 | CL 2 | Close F-2026-01 to reach steady CL 2 |
| SYS.3 | 2 | CL 2 | — |
| SYS.4 | 2 | CL 1 Largely | F-2026-02 must be closed for CL 2 |
| SYS.5 | 2 | CL 1 Largely | F-2026-02 must be closed for CL 2 |
| SWE.1 | 2 | CL 2 | — |
| SWE.2 | 2 | CL 2 | — |
| SWE.3 | 2 | CL 2 | — |
| SWE.4 | 2 | CL 2 | — |
| SWE.5 | 2 | CL 2 | — |
| SWE.6 | 2 | CL 1 Largely | F-2026-02 close + NCR-002/003 raise = CL 2 |
| SUP.1 | 2 | CL 2 | — |
| SUP.8 | 2 | CL 1 Largely | F-2026-04 repo merge required for CL 2 |
| SUP.9 | 1 | CL 1 | NCR-001 handled per process |
| SUP.10 | 2 | CL 1 Largely | F-2026-02 close required for CL 2 |

Overall assessment outcome: Target CL 2 reached for 11/16 processes. 5 processes are CL 1 Largely achieved with a clear path to CL 2 via the F-2026-02, F-2026-04, F-2026-01 closure plan.

13.  Conclusion and Release Recommendation

The Airbag ECU portfolio (LCP01 → TR Rev 2.0 + firmware Rev 3.3) demonstrates a mature, well-traced ASIL D / SIL 3 development for the SoSe 2026 functional-safety exercise:

■  All 16 lifecycle phases are represented in the released document set; 14 OK, 2 Partial.

■  HARA, FSRS and SCD correctly classify the item as ASIL D / SIL 3 with 7 safety goals derived from 12 hazards across 12 use cases.

■  The peer-equal 2oo2 architecture is consistently specified across SRS Rev 3.1, HADS Rev 2.0 Final and SADS Rev 3.0 — the algorithm conflict raised in QA Rev 2.0 is fully resolved.

■  Firmware Rev 3.3 is fully traceable to 25 SWRs and 22 FSRs; coding constraints comply with SADS §8.4.

■  All 16 integration tests + 8 validation tests PASS, with structural coverage at 100% statement / 100% branch / 100% MC/DC for safety-critical conditionals.

■  7 of the previous QA Rev 2.0 findings (4 CRITICAL + 3 WARNINGS) have been closed in this revision.

Release Recommendation: CONDITIONAL PASS — APPROVED for the SoSe 2026 presentation and Functional Safety Assessment.

Conditions to be closed before the final repository tag v1.0.0:

| # | Finding | Action Required | Owner | Due |
| --- | --- | --- | --- | --- |
| 1 | F-2026-02 (Major) | Add FSR-05 and FSR-06 to TR §4.1 with DEVIATION status; raise NCR-002/NCR-003; re-release as TR Rev 2.1 | M. Chavez Avila | 2026-06-22 |
| 2 | F-2026-04 (Minor) | Commit all 13 work products + firmware Rev 3.3 to GitHub repo; replace placeholder READMEs; create baseline tags | Paresh Patil | 2026-06-22 |
| 3 | F-2026-01 (Minor) | Synchronise SC-10 and SC-11 between FSRS §8 and SCD §12 | Mobina Ketabi | 2026-06-22 |
| 4 | F-2026-06 / F-2026-07 (Editorial) | Reconcile role labels in LCP01 §4.3; fix dependent-document table in LCP02 §9 | Alita Shrestha | 2026-06-22 |
| 5 | NCR-001 | Obtain FSM sign-off on NCR-001 rationale; embed in safety case | FSM / QM | 2026-06-22 |

14.  Approvals

| Role | Name |
| --- | --- |
| Quality Manager (lead, SUP.1) | Thirthashri Gowli Ganesh |
| Functional Safety Manager | Fateme Hosseinzadeh |
| Functional Safety Assessor (TÜV) | Mariana Chavez Avila |
| Project Manager | Alita Shrestha |
| Configuration Manager (SUP.8) | Paresh Sanjiv Patil |

Appendix : Glossary

| Term | Meaning |
| --- | --- |
| ASIL | Automotive Safety Integrity Level (ISO 26262: QM, A, B, C, D) |
| SIL | Safety Integrity Level (IEC 61508: 1–4) |
| ACU | Airbag Control Unit — synonymous with Airbag ECU |
| HARA | Hazard and Risk Analysis |
| FSRS | Functional Safety Requirements Specification |
| SCD | Safety Concept Document |
| SRS | Software Requirements Specification |
| SADS | Software Architecture and Design Specification |
| HADS | Hardware Architecture and Design Specification |
| ITP | Integration Test Plan |
| VVP | Verification & Validation Plan |
| TR | Test Report |
| OCS | Occupant Classification System (baby-seat) |
| DWI | Diagnostic Warning Indicator (D10) |
| POST | Power-On Self-Test |
| FSM | Functional Safety Manager |
| 2oo2 | Two-out-of-Two voting (both ECUs must agree before deployment) |
| MC/DC | Modified Condition / Decision Coverage |
| WDT | Hardware Watchdog Timer |
| NCR | Non-Conformance Report |
| DTC | Diagnostic Trouble Code |
| SPFM | Single-Point Fault Metric |
| PMHF | Probabilistic Metric for random Hardware Failures |
| ASPICE | Automotive SPICE — Process Assessment / Reference Model (v3.1 / v4.0) |
| BP | Base Practice (one numbered activity of an ASPICE process) |
| CL 0…5 | Capability Level (0 Incomplete → 5 Innovating) |