Airbag ECU Portfolio HS Harz — Functional Safety AIRBAG ELECTRONIC CONTROL UNIT Measurement and Processing of Sensor Signals to Control Airbag Operation

LIFE CYCLE PHASE 4 FUNCTIONAL SAFETY REQUIREMENTS SPECIFICATION (FSRS)

| Document Number | LCP04-AIRBAG-ECU-001 |
| --- | --- |
| Version | 1.00 |
| Date | 2026-05-11 |
| Editor | Fateme, Mobina, Alita |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| File | LCP04-AIRBAG-ECU-001_FSRS_v100.docx |

Table of Contents

1  Meetings within the Project	3

2  Unresolved Topics	3

3  Glossary	3

4  Purpose and Scope of the FSRS Phase	5

4.1  Purpose of Lifecycle Phase 4	5

4.2  Relationship to Other Lifecycle Phases	5

4.3  Applicable Standards	5

5  System Overview	5

5.1  System Description	5

5.2  System Architecture	6

5.3  Sensor Inputs	6

5.4  Output Interfaces	7

6  Safety Goals	7

7  Functional Safety Requirements	8

7.1  Crash Detection and Deployment Requirements	8

7.2  Redundancy and Decision Integrity Requirements	9

7.3  Occupant Classification Requirements	9

7.4  CAN Communication Safety Requirements	9

7.5  Diagnostic and Fault Handling Requirements	10

8  Safety Constraints	12

9  Timing Requirements	12

10  Safe States Definition	12

11  Interface Requirements	13

11.1  Sensor Interfaces	13

11.2  Processing Interfaces	13

11.3  Communication Interfaces	13

11.4  CAN Deployment Frame Definition	14

12  Requirement Traceability Matrix	15

13  Verification and Validation Requirements	17

14  Figure and Table Listing	17

15  List of Dependent Documents	18

16  Literature	18

17  Version History	18

# 1  Meetings within the Project

The following project meetings were held during the FSRS Phase:

Table 1: Meetings within the Project

| Topic | Date / Time | Participants | Minutes / Decisions |
| --- | --- | --- | --- |
| FSRS Phase Kick-off & Scope Review | 2026-05-11, 10:00 | Safety Requirement Engineers, Functional Safety Manager, Software Architect, Quality Manager | Confirmed safety goals SG-01–SG-07 from HARA. Reviewed system architecture. Assigned requirements derivation tasks. |
| FSRS Results Review & Sign-off | 2026-05-11, 15:00 | Functional Safety Engineering Team, Quality Manager, Project Manager | Reviewed full requirements set FSR-01–FSR-22. Traceability matrix verified. Document approved for DRAFT release. |

# 2  Unresolved Topics

Table 2: Unresolved Topics

| No. | Topic / Question | Responsible | Target Date | Status |
| --- | --- | --- | --- | --- |
| No unresolved topics at time of issue. Any topics arising during review shall be recorded here before the document status is changed to RELEASED. |  |  |  |  |

# 3  Glossary

Table 3 lists all abbreviations and terms used in this document and the wider Airbag ECU project portfolio.

Table 3: Glossary

| Term / Abbreviation | Definition |
| --- | --- |
| ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
| ACU | Airbag Control Unit — synonymous with Airbag ECU |
| CAN | Controller Area Network — serial communication bus for vehicle ECUs |
| CRC | Cyclic Redundancy Check — error-detection mechanism for data integrity verification |
| DTC | Diagnostic Trouble Code |
| DWI | Diagnostic Warning Indicator — airbag fault lamp in the instrument cluster |
| ECU | Electronic Control Unit |
| EMC | Electromagnetic Compatibility |
| FSRS | Functional Safety Requirements Specification |
| HARA | Hazard and Risk Analysis |
| HW | Hardware |
| IEC 61508 | Functional Safety of E/E/PE Safety-related Systems (2010 edition) |
| ISO 26262 | Road Vehicles — Functional Safety standard (2018 edition) |
| LCP04 | Life Cycle Phase 04 — Functional Safety Requirements Specification |
| OCS | Occupant Classification System |
| PFH | Probability of dangerous Failure per Hour |
| PMHF | Probabilistic Metric for random Hardware Failures |
| POST | Power-On Self-Test |
| QM | Quality Management — no specific ASIL requirement |
| S | Severity — ISO 26262 HARA parameter (S0–S3) |
| SG | Safety Goal — high-level safety requirement derived from HARA |
| SIL | Safety Integrity Level per IEC 61508 (1–4) |
| SPFM | Single-Point Fault Metric |
| SS | Safe State — defined system condition upon fault detection |
| SW | Software |

# 4  Purpose and Scope of the FSRS Phase

## 4.1  Purpose of Lifecycle Phase 4

This document constitutes the deliverable for Life Cycle Phase 4 (LCP04) of the Airbag ECU project. It presents the complete Functional Safety Requirements Specification (FSRS) derived from the Hazard Analysis and Risk Assessment (HARA) conducted in LCP03, in accordance with ISO 26262:2018 and IEC 61508:2010.

The purpose of this document is to translate identified hazards, safety goals, ASIL classifications, and SIL targets into concrete, verifiable, and testable safety requirements applicable to the ECU hardware and embedded software implementation.

## 4.2  Relationship to Other Lifecycle Phases

Input: LCP03 — Hazard and Risk Analysis (HARA) document, providing hazardous events H1-01 through H4-02, safety goals SG-01–SG-07, safe states SS-01–SS-05, and ASIL/SIL classifications.

Output: Functional safety requirements FSR-01–FSR-22, safety constraints SC-01–SC-09, timing requirements, interface definitions, and requirement traceability matrix. These become the inputs to LCP05 (Hardware Architecture & Design Documentation) and LCP06 (Software Architecture & Source Code).

## 4.3  Applicable Standards

This specification is developed in accordance with:

• ISO 26262:2018 — Road Vehicles — Functional Safety, Part 3 (Concept Phase) and Part 4 (Product Development at the System Level)

• IEC 61508:2010 — Functional Safety of E/E/PE Safety-related Systems, Parts 1 and 2

• ISO 21448:2022 — Safety of the Intended Functionality (SOTIF)

# 5  System Overview

## 5.1  System Description

The Airbag ECU continuously monitors crash-related sensor information and determines whether airbag deployment is required based on sensor fusion algorithms and occupant classification logic.

The system is safety-critical because both unintended deployment and non-deployment during a crash can directly result in severe injury or fatality.

The Airbag ECU is responsible for:

• Monitoring crash sensor signals continuously

• Detecting frontal, lateral, rollover, and impact events

• Making deterministic deployment decisions

• Preventing unintended airbag deployment

• Suppressing passenger airbag deployment during child-seat occupancy

• Performing continuous diagnostics and fault monitoring

• Broadcasting deployment and diagnostic information via CAN communication

This specification applies to the confirmed laboratory architecture consisting of:

• Two Arduino Due boards operating in a redundant 2oo2 safety architecture

• Three potentiometer-based analog crash sensor simulations

• One digital baby-seat switch input

• CAN bus communication to the OMS laboratory PC

• CRC-protected CAN communication

• Parallel CAN bitstream readback verification

The physical pyrotechnic inflator, airbag cushion, and vehicle body structure are treated as adjacent systems outside the laboratory implementation boundary.

## 5.2  System Architecture

The laboratory implementation uses a dual-channel redundant architecture (2oo2 — two-out-of-two voting logic). Two Arduino Due boards independently:

• Read all sensor inputs

• Execute crash detection algorithms

• Calculate deployment decisions

• Generate CRC-protected CAN frames

• Verify communication integrity

A deployment command is issued only if both channels independently reach identical deployment decisions. This redundancy architecture is required to satisfy ISO 26262 ASIL D and IEC 61508 SIL 3.

A single Arduino Due processing channel does not satisfy the ASIL D single-point fault metric (SPFM) targets required by ISO 26262. Therefore, two independent processing channels operating in a 2oo2 redundant architecture are required to achieve the necessary fault tolerance, decision integrity, and prevention of unintended deployment caused by single-point failures.

## 5.3  Sensor Inputs

Table 4: Sensor Inputs (Breadboard)

| # | Real Sensor | Laboratory Substitute | Signal Type | Represents |
| --- | --- | --- | --- | --- |
| 1 | Accelerometer | POTI-1 | Analog (ADC) 0–3.3 V | 0–100 g crash pulse |
| 2 | Gyroscope / Angular Rate | POTI-2 | Analog (ADC) 0–3.3 V | 0–360°/s rollover / yaw |
| 3 | Pressure Sensor | POTI-3 | Analog (ADC) 0–3.3 V | 0–500 kPa impact pressure |
| 4 | Baby Seat / OCS | Digital Switch | Digital GPIO LOW/HIGH | Passenger airbag suppression. LOW = baby seat detected; HIGH = adult occupant / empty seat. |

## 5.4  Output Interfaces

Table 5: Output Interfaces

| Interface | Description |
| --- | --- |
| CAN Bus | Deployment and diagnostic communication |
| CRC Field | CAN communication integrity verification |
| Parallel Readback Channel | Bitstream verification |
| OMS Laboratory PC | Monitoring and logging interface |

# 6  Safety Goals

The following safety goals are derived from LCP03 (HARA). Each safety goal drives the functional safety requirements defined in Section 7.

Table 6: Safety Goals

| SG-ID | Safety Goal | ASIL | SIL | Governing Hazard(s) |
| --- | --- | --- | --- | --- |
| SG-01 | The ECU shall not deploy airbags without a valid crash event. | ASIL D | SIL 3 | H1-01, H1-02, H1-04 |
| SG-02 | The ECU shall deploy airbags within the required timing window during valid crash events. | ASIL D | SIL 3 | H2-01, H2-02 |
| SG-03 | The ECU shall suppress passenger airbag deployment when a child seat is detected. | ASIL D | SIL 3 | H1-04 |
| SG-04 | The ECU shall activate pretensioners within required timing limits. | ASIL C | SIL 2 | H2-03, H2-04 |
| SG-05 | The ECU shall reject EMC disturbances and invalid electrical events. | ASIL C | SIL 2 | H1-05 |
| SG-06 | The ECU shall detect and report safety-critical faults. | ASIL A | SIL 1 | H4-01 |
| SG-07 | The ECU shall transmit crash notification CAN frames reliably. | ASIL B | SIL 2 | H4-02 |

# 7  Functional Safety Requirements

Functional safety requirements are derived directly from the safety goals and HARA results of LCP03. Each requirement is assigned an ASIL/SIL classification, a source hazard reference, and a safety rationale.

## 7.1  Crash Detection and Deployment Requirements

FSR-01 — Crash Detection Timing

Requirement: The Airbag ECU shall detect crash conditions within a maximum of 20 ms after crash sensor thresholds are exceeded.

Safety Rationale: Delayed crash detection reduces occupant protection effectiveness and may result in occupant contact with vehicle structures before airbag inflation.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H2-01, H2-02

FSR-02 — Crash Algorithm Cycle Time

Requirement: The crash detection algorithm shall execute with a deterministic cycle time not exceeding 1 ms.

Safety Rationale: Deterministic execution is required to ensure predictable deployment timing behavior.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H2-04

FSR-03 — Deployment Authorization Logic

Requirement: The ECU shall authorize airbag deployment only when all of the following conditions are simultaneously satisfied:

(1) Crash thresholds are exceeded;

(2) Channel A independently detects a valid crash;

(3) Channel B independently detects a valid crash;

(4) Both channels agree on the deployment decision;

(5) No critical fault condition exists;

(6) Passenger suppression conditions are not active.

Safety Rationale: All six conditions are required to prevent unintended deployment caused by sensor faults, software faults, communication corruption, or single-channel processing failures.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H1-01, H2-01

FSR-04 — Prevention of Unintended Deployment

Requirement: The ECU shall inhibit all deployment outputs whenever crash thresholds are not exceeded.

Safety Rationale: Unintended deployment during normal driving represents the highest-severity hazard identified in the HARA.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H1-01, H1-02

FSR-05 — Deployment Command Timing

Requirement: The ECU shall transmit the deployment command within 40 ms after crash detection.

Safety Rationale: The 40 ms deployment command transmission limit is derived from occupant protection timing requirements and representative frontal crash pulse characteristics. Delayed deployment beyond this interval may significantly reduce airbag effectiveness and increase occupant injury risk.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H2-01

FSR-06 — Side-Airbag Timing Constraint

Requirement: Side-impact deployment logic shall complete within 8 ms after side-impact threshold detection.

Safety Rationale: Side-impact crash events develop significantly faster than frontal impacts due to reduced crush space between the occupant and intrusion zone. The 8 ms requirement ensures sufficient occupant protection before lateral intrusion reaches the occupant compartment.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H2-02

## 7.2  Redundancy and Decision Integrity Requirements

FSR-07 — Dual-Channel Redundancy

Requirement: Both Arduino Due channels shall independently acquire sensor data, execute crash algorithms, calculate deployment decisions, generate CAN communication frames, and calculate CRC values.

Safety Rationale: Dual-channel redundancy is required to satisfy ASIL D fault tolerance objectives and reduce the probability of unsafe deployment decisions caused by hardware faults, software faults, timing failures, or corrupted sensor processing within a single processing channel.

ASIL / SIL: ASIL D / SIL 3

FSR-08 — Channel Agreement Verification

Requirement: The ECU shall issue deployment outputs only if both redundant channels independently generate identical deployment decisions.

Safety Rationale: This requirement prevents single-point failures from causing unintended deployment.

ASIL / SIL: ASIL D / SIL 3

## 7.3  Occupant Classification Requirements

FSR-09 — Passenger Airbag Suppression

Requirement: The ECU shall suppress passenger airbag deployment whenever the baby-seat signal is LOW.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H1-04

FSR-10 — Suppress-on-Fault Strategy

Requirement: If the baby-seat signal becomes invalid, intermittent, unstable, or unavailable, the ECU shall default to passenger airbag suppression.

Safety Rationale: Suppressing deployment during uncertain occupancy conditions minimizes risk to child occupants.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H1-04

FSR-11 — Digital Input Debounce

Requirement: The baby-seat digital input shall remain stable for at least 10 ms before a state transition is accepted as valid.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: Intermittent/noisy signal condition

FSR-12 — Baby Seat Input Line Monitoring

Requirement: The ECU shall periodically verify continuity of the baby-seat digital input line.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: Open-circuit and stuck-HIGH faults

## 7.4  CAN Communication Safety Requirements

FSR-13 — CRC-Protected CAN Communication

Requirement: All CAN frames shall include CRC protection calculated using the Arduino CRC library.

ASIL / SIL: ASIL B / SIL 2

FSR-14 — CAN Bitstream Readback Verification

Requirement: The ECU shall implement a secondary parallel readback channel to verify transmitted CAN bitstreams.

Safety Rationale: Parallel verification detects communication corruption and CAN transmission faults.

ASIL / SIL: ASIL B / SIL 2

FSR-15 — CAN Communication Timeout Detection

Requirement: The ECU shall detect CAN communication timeout conditions exceeding 50 ms.

ASIL / SIL: ASIL B / SIL 2

FSR-16 — Crash Notification Transmission

Requirement: The ECU shall transmit a crash notification CAN frame within 15 ms after deployment authorization.

ASIL / SIL: ASIL B / SIL 2 — Source Hazard: H4-02

FSR-16A — Safety-Critical Deployment Communication Integrity

Requirement: Although the CAN communication channel is classified ASIL B / SIL 2 for general diagnostic communication, deployment-related CAN frames shall achieve end-to-end ASIL D integrity through redundant channel agreement verification, CRC protection, parallel bitstream readback validation, timeout supervision, and deployment authorization cross-checking between both processing channels.

Safety Rationale: In the laboratory implementation, deployment authorization information is transmitted over CAN communication. Additional end-to-end integrity mechanisms are therefore required to ensure that ASIL D deployment decisions are not degraded by the lower inherent integrity classification of the CAN transport layer.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H1-01, H2-01, H4-02

## 7.5  Diagnostic and Fault Handling Requirements

FSR-17 — Power-On Self-Test (POST)

Requirement: The ECU shall execute a complete power-on self-test within 800 ms after startup. The POST shall verify: ADC functionality, sensor plausibility, CAN communication, CRC integrity, memory integrity, clock integrity, and digital input continuity.

ASIL / SIL: ASIL C / SIL 2

FSR-18 — Continuous Diagnostic Monitoring

Requirement: The ECU shall execute continuous background diagnostics with intervals not exceeding 100 ms.

ASIL / SIL: ASIL C / SIL 2

FSR-19 — Diagnostic Warning Indicator

Requirement: The ECU shall activate the Diagnostic Warning Indicator (DWI) within 800 ms after detection of a safety-critical fault.

ASIL / SIL: ASIL A / SIL 1

FSR-20 — Safe-State Transition

Requirement: Upon detection of an unrecoverable safety-critical fault, the ECU shall transition into a safe inhibit state in which all deployment outputs are disabled.

ASIL / SIL: ASIL D / SIL 3

FSR-21 — Fault Logging

Requirement: Detected faults shall be stored as Diagnostic Trouble Codes (DTCs).

ASIL / SIL: ASIL A / SIL 1

FSR-22 — Memory Integrity Protection

Requirement: The ECU software shall implement memory integrity protection mechanisms including: RAM integrity verification using CRC or equivalent techniques, stack boundary monitoring, detection of invalid memory access conditions, watchdog supervision for runaway software execution, and safe-state transition upon detection of unrecoverable memory faults.

Safety Rationale: Memory corruption, stack overflow, or uncontrolled software execution may lead to loss of deployment control, incorrect deployment authorization, or failure to enter a safe inhibit state. Memory integrity supervision is therefore required to maintain deterministic and safe ECU operation.

ASIL / SIL: ASIL D / SIL 3 — Source Hazard: H1-01, H2-01, H4-01

# 8  Safety Constraints

Table 7: Safety Constraints

| Constraint ID | Safety Constraint |
| --- | --- |
| SC-01 | Overall system classification shall remain ASIL D / SIL 3. |
| SC-02 | PMHF shall remain below 10⁻⁸ failures/hour. |
| SC-03 | No single-point fault shall cause unintended deployment. |
| SC-04 | Deployment requires agreement of both redundant channels. |
| SC-05 | CAN communication shall include CRC verification. |
| SC-06 | Critical faults shall force transition to a safe inhibit state. |
| SC-07 | Safety-related software shall execute deterministically. |
| SC-08 | Diagnostic monitoring shall execute continuously during operation. |
| SC-09 | Deployment inhibit functionality shall remain enforceable by hardware independent of software execution. |

# 9  Timing Requirements

Table 8: Timing Requirements

| Function | Maximum Allowed Time |
| --- | --- |
| Crash detection cycle | 1 ms |
| Deployment decision | 10–15 ms |
| Crash detection response | 20 ms |
| Deployment command transmission | 40 ms |
| Side-impact deployment | < 8 ms |
| Pretensioner activation | < 5 ms |
| CAN timeout detection | 50 ms |
| Diagnostic cycle interval | 100 ms |
| POST execution | 800 ms |
| Baby-seat debounce | 10 ms |

# 10  Safe States Definition

ISO 26262 requires that each safety goal be assigned a safe state — the condition the system must achieve or maintain when a fault is detected. The following safe states are applicable to the Airbag ECU:

Table 9: Safe States Definition

| Safe State ID | Description | System Condition |
| --- | --- | --- |
| SS-01 | Deployment Inhibit State | All firing outputs disabled. Reservoir capacitor maintained but no firing command issued. DWI lamp ON if fault detected. |
| SS-02 | Deployment Armed State | Normal standby. Crash monitoring active. All squib channels armed. Algorithm running. |
| SS-03 | Passenger Airbag Suppressed | Passenger airbag disabled. If OCS signal lost > 50 ms, passenger airbag inhibited by default (suppress-on-fault). Driver and side airbags remain armed. |
| SS-04 | Partial Degradation State | Faulted channels isolated. Faulted squib channel disabled; remaining channels continue normal operation. DWI lamp ON. System logs DTC. |
| SS-05 | System Shutdown State | Hardware safety inhibit active and deployment outputs forced LOW independent of software state. Watchdog asserts reset signal. If fault persists after 3 resets → permanent shutdown + DWI lamp. |

# 11  Interface Requirements

## 11.1  Sensor Interfaces

Table 10: Sensor Interfaces

| Interface | Type / Voltage Range | Description |
| --- | --- | --- |
| POTI-1 | Analog ADC 0–3.3 V | Accelerometer simulation |
| POTI-2 | Analog ADC 0–3.3 V | Gyroscope simulation |
| POTI-3 | Analog ADC 0–3.3 V | Pressure sensor simulation |
| Baby Seat Switch | Digital GPIO LOW/HIGH | OCS / passenger airbag suppression input |

## 11.2  Processing Interfaces

Table 11: Processing Interfaces

| Interface | Description |
| --- | --- |
| Channel A | Arduino Due #1 |
| Channel B | Arduino Due #2 |
| Cross-Comparison Interface | Redundant decision synchronization between Channel A and Channel B |

## 11.3  Communication Interfaces

Table 12: Communication Interfaces

| Interface | Description |
| --- | --- |
| CAN Bus | Deployment and diagnostic communication |
| CRC Verification | Frame integrity verification |
| Parallel Readback | Bitstream validation |
| OMS Laboratory PC | Logging and monitoring interface |

CAN Communication Integrity Note: Deployment-related communication transmitted over the CAN interface shall use CRC protection, timeout monitoring, redundant channel agreement verification, and parallel readback validation to maintain end-to-end ASIL D deployment integrity.

## 11.4  CAN Deployment Frame Definition

Table 13: CAN Deployment Frame Definition

| Field / Bit | Description |
| --- | --- |
| Bit 0 | Driver airbag deployment command |
| Bit 1 | Passenger airbag deployment command |
| Bit 2 | Passenger suppression active |
| Bit 3 | Crash event detected |
| Bit 4 | Channel A fault detected |
| Bit 5 | Channel B fault detected |
| Bit 6 | Diagnostic warning indicator active |
| Bit 7 | Safe-state inhibit active |
| Byte 1 | Deployment sequence counter |
| Bytes 2–3 | Sensor status information |
| Bytes 4–5 | Diagnostic status information |
| Bytes 6–7 | CRC field (16-bit) |

CRC Definition: CRC protection shall use a 16-bit CRC algorithm implemented consistently on both processing channels and verified during CAN frame reception and parallel readback validation.

# 12  Requirement Traceability Matrix

Table 14: Requirement Traceability Matrix

| Req. ID | Safety Goal | Governing Hazard | ASIL | SIL | Summary |
| --- | --- | --- | --- | --- | --- |
| FSR-01 | SG-02 | H2-01 | ASIL D | SIL 3 | Crash detection timing |
| FSR-02 | SG-02 | H2-04 | ASIL D | SIL 3 | Algorithm cycle time |
| FSR-03 | SG-01 | H1-01 | ASIL D | SIL 3 | Deployment authorization logic |
| FSR-04 | SG-01 | H1-01, H1-02 | ASIL D | SIL 3 | Prevention of unintended deployment |
| FSR-05 | SG-02 | H2-01 | ASIL D | SIL 3 | Deployment command timing |
| FSR-06 | SG-02 | H2-02 | ASIL D | SIL 3 | Side-airbag timing constraint |
| FSR-07 | SG-01 | H1-01 | ASIL D | SIL 3 | Dual-channel redundancy |
| FSR-08 | SG-01 | H1-01 | ASIL D | SIL 3 | Channel agreement verification |
| FSR-09 | SG-03 | H1-04 | ASIL D | SIL 3 | Passenger airbag suppression |
| FSR-10 | SG-03 | H1-04 | ASIL D | SIL 3 | Suppress-on-fault strategy |
| FSR-11 | SG-03 | H1-04 | ASIL D | SIL 3 | Digital input debounce |
| FSR-12 | SG-03 | H1-04 | ASIL D | SIL 3 | Baby seat input line monitoring |
| FSR-13 | SG-07 | H4-02 | ASIL B | SIL 2 | CRC-protected CAN communication |
| FSR-14 | SG-07 | H4-02 | ASIL B | SIL 2 | CAN bitstream readback verification |
| FSR-15 | SG-07 | H4-02 | ASIL B | SIL 2 | CAN communication timeout detection |
| FSR-16 | SG-07 | H4-02 | ASIL B | SIL 2 | Crash notification transmission |
| FSR-16A | SG-01 / SG-07 | H1-01, H2-01, H4-02 | ASIL D | SIL 3 | Deployment communication integrity |
| FSR-17 | SG-06 | H4-01 | ASIL C | SIL 2 | Power-on self-test (POST) |
| FSR-18 | SG-06 | H4-01 | ASIL C | SIL 2 | Continuous diagnostic monitoring |
| FSR-19 | SG-06 | H4-01 | ASIL A | SIL 1 | Diagnostic warning indicator |
| FSR-20 | SG-01 | H1-01 | ASIL D | SIL 3 | Safe-state transition |
| FSR-21 | SG-06 | H4-01 | ASIL A | SIL 1 | Fault logging |
| FSR-22 | SG-01 / SG-06 | H1-01, H2-01, H4-01 | ASIL D | SIL 3 | Memory integrity protection |

# 13  Verification and Validation Requirements

The laboratory implementation shall include a formal Verification and Validation (V&V) protocol. Each test case shall document:

• Test identifier

• Sensor potentiometer settings

• Expected ECU response

• Actual CAN output

• CRC verification status

• Parallel readback verification result

• Pass/fail status

• Fault injection scenario

• Timestamp and operator information

The verification process shall validate:

• Crash detection functionality

• Non-deployment conditions

• Redundant channel agreement

• Communication integrity

• Diagnostic fault handling

• Safe-state transitions

# 14  Figure and Table Listing

Table 1: Meetings within the Project

Table 2: Unresolved Topics

Table 3: Glossary

Table 4: Sensor Inputs (Breadboard)

Table 5: Output Interfaces

Table 6: Safety Goals

Table 7: Safety Constraints

Table 8: Timing Requirements

Table 9: Safe States Definition

Table 10: Sensor Interfaces

Table 11: Processing Interfaces

Table 12: Communication Interfaces

Table 13: CAN Deployment Frame Definition

Table 14: Requirement Traceability Matrix

Table 15: Dependent Documents

Table 16: Version History

# 15  List of Dependent Documents

Table 15: Dependent Documents

| No. | Document Name | Version | Date | File Name |
| --- | --- | --- | --- | --- |
| 1 | Life Cycle Phase 1 — Concept Document | 1.00 | 2026-05-01 | LCP01-AIRBAG-ECU-001_Concept_v110.docx |
| 2 | Life Cycle Phase 2 — Task of the Process or Product | 1.10 | 2026-05-03 | LCP02-AIRBAG-ECU-001_Concept_v_110.docx |
| 3 | Life Cycle Phase 3 — Hazard and Risk Analysis (HARA) | 2.00 | 2026-05-05 | LCP03-AIRBAG-ECU-001_HARA_v110.docx |
| 4 | ISO 26262:2018 — Road Vehicles — Functional Safety | 2018 | — | External standard |
| 5 | IEC 61508:2010 — Functional Safety of E/E/PE Systems | 2010 | — | External standard |

# 16  Literature

[1] ISO 26262:2018 — Road Vehicles — Functional Safety, Parts 1–12. International Organization for Standardization.

[2] IEC 61508:2010 — Functional Safety of Electrical/Electronic/Programmable Electronic Safety-related Systems, Parts 1–7. International Electrotechnical Commission.

[3] ISO 21448:2022 — Safety of the Intended Functionality (SOTIF). ISO.

[4] ISO 16750-3:2007 — Road Vehicles — Environmental Conditions and Testing — Mechanical Loads. ISO.

[5] CISPR 25:2016 — Vehicles, Boats and Internal Combustion Engines — Radio Disturbance Characteristics. IEC.

[6] ISO 11452 — Road Vehicles — Component Test Methods for Electrical Disturbances from Narrowband Radiated Electromagnetic Energy. ISO.

[7] ISO 14229-1:2020 — Unified Diagnostic Services (UDS). ISO.

[8] ISO 11898 — Road Vehicles — Controller Area Network (CAN). ISO.

[9] FMVSS No. 208 — Occupant Crash Protection. U.S. Department of Transportation, NHTSA.

# 17  Version History

Table 16: Version History

| Document Name / File | Date | Change Log |
| --- | --- | --- |
| LCP04-AIRBAG-ECU-001_FSRS_v100.docx | 2026-05-11 | Version 1.00 — Initial release of the Functional Safety Requirements Specification (FSRS / LCP04) for the Airbag ECU project. Derived from HARA (LCP03). Covers requirements FSR-01 through FSR-22, safety constraints SC-01–SC-09, timing requirements, safe states SS-01–SS-05, interface definitions, and requirement traceability matrix. Based on confirmed dual-channel Arduino Due laboratory implementation. Classified ASIL D / SIL 3. |

LCP04-AIRBAG-ECU-001 — Functional Safety Requirements Specification  v1.00  |  2026-05-11