SADS-AIRBAG-ECU-001 | Rev 2.1 | 2026-06-12

Software Architecture & Design Specification | ASIL D / SIL 3

Airbag ECU Portfolio

HS Harz — Functional Safety Engineering

SOFTWARE ARCHITECTURE AND

DESIGN SPECIFICATION (SADS)

Measurement and Processing of Sensor Signals to Control Airbag Operation

| Document Number | SADS-AIRBAG-ECU-001 |
| --- | --- |
| Version | 2.1 |
| Date | 2026-06-12 |
| Status | RELEASED |
| ASIL Classification | ASIL D (ISO 26262) / SIL 3 (IEC 61508) |
| Target Hardware | Arduino Due (SAM3X8E ARM Cortex-M3, 84 MHz) ×2 |
| Applicable Standards | ISO 26262:2018 / IEC 61508:2010 |
| Author | Software Safety Engineering Team |
| Based On | HADS-AIRBAG-ECU-001 Rev 2.1 ¦ FSRS-AIRBAG-ECU-001 Rev 2.1 |

# Table of Contents

Table of Contents	1

1  Purpose and Scope	1

1.1  Software Safety Goals Addressed	1

2  Selection of Tools and Programming Languages	1

2.1  Programming Language	1

2.2  Development Tools	1

2.3  Tool Classification (ISO 26262-8 Clause 11)	1

3  Software Architecture Overview	1

3.1  Module Decomposition	1

3.2  Data Flow Between Modules	1

3.3  Execution Control Flow — Cyclic Executive	1

4  System State Machine	1

4.1  State Definitions	1

4.2  State Transition Diagram	1

4.3  Safe State Guarantees	1

5  Detailed Design of Software Modules	1

5.1  MOD-01 — Power-On Self-Test (POST)	1

5.1.1  CPU Test	1

5.1.2  RAM Test	1

5.1.3  ADC Test	1

5.1.4  POST Completion Criteria	1

5.2  MOD-02 — Sensor Acquisition	1

5.3  MOD-03 — Crash Detection	1

5.3.1  Scaling Formulas	1

5.3.2  Threshold Comparison	1

5.3.3  Threshold Rationale	1

5.4  MOD-04 — Deployment Control	1

5.4.1  Deployment Vote Logic (Full)	1

5.4.2  Deployment Authorization Truth Table	1

5.5  MOD-05 — CAN Communication	1

5.5.1  Transmit Frame (sendData)	1

5.5.2  Receive and Timeout	1

5.6  MOD-06 — Diagnostic Monitor	1

6  Occupant Classification and Baby Seat Logic	1

6.1  Signal Interpretation	1

6.2  FSR-11 Debounce Requirement (Required Implementation)	1

7  Safety Mechanisms in Software	1

7.1  Hardware Watchdog Timer (WDT)	1

7.2  2oo2 Redundant Decision Architecture	1

7.3  Fail-Safe Direction Analysis	1

7.4  Permitted and Forbidden Software Operations	1

Permitted:	1

Forbidden:	1

8  Unit Testing of Software Modules	1

8.1  Unit Test Environment	1

8.2  MOD-01 Unit Tests — POST	1

8.3  MOD-02 Unit Tests — Sensor Acquisition	1

8.4  MOD-03 Unit Tests — Crash Detection	1

8.5  MOD-04 Unit Tests — Deployment Control	1

9  Integration Testing of Software Modules	1

9.1  Integration Test Environment	1

9.2  Integration Test Cases — Normal Operation	1

9.3  Integration Test Cases — Fault Injection	1

9.4  Integration Test Cases — Timing Verification	1

10  Integration of Programmable Electronics (HW and SW)	1

10.1  HW/SW Integration Procedure	1

10.2  Firmware Version Identification	1

11  Software Verification	1

11.1  Verification Methods	1

11.2  Code Review Checklist	1

11.3  Verification Coverage Target	1

12  Validation of the Software	1

12.1  Validation Scope and Approach	1

12.2  Validation Test Cases	1

12.3  Validation Evidence Requirements	1

13  Assessment of Functional Safety	1

13.1  ASIL D Software Requirements Conformance	1

13.2  Single-Point Fault Metric (SPFM) Analysis	1

13.3  Residual Risk Assessment	1

14  Compliance Gaps and Required Improvements	1

15  Modification and Change Management Procedure	1

15.1  Change Categories	1

15.2  Change Control Process	1

15.3  Version Naming Convention	1

15.4  Configuration Items Under Change Control	1

16  Requirement Traceability Matrix	1

17  References	1

17.1  Project Documents	1

17.2  Normative Standards	1

18  Approval and Sign-Off	1

19  Revision History	1

# 1  Purpose and Scope

This Software Architecture and Design Specification (SADS) defines the internal software structure, module organisation, data flow, control flow, safety mechanisms, state machines, and implementation logic for the Airbag Electronic Control Unit (ECU) firmware running on two Arduino Due (SAM3X8E) microcontrollers in a redundant 2oo2 safety architecture.

The document covers all phases required by ISO 26262 Part 6 for software at ASIL D, including:

- Software architecture design and tool/language selection

- Detailed module design and data structures

- Unit and integration test specifications

- HW/SW integration procedures

- Software verification methods and evidence

- Software validation plan

- Functional safety assessment

- Modification and change management procedure

This specification is the primary reference for software verification, V&V testing, functional safety assessments, and code reviews. It must be read in conjunction with FSRS-AIRBAG-ECU-001 Rev 2.1 and HADS-AIRBAG-ECU-001 Rev 2.1.

## 1.1  Software Safety Goals Addressed

| Safety Goal | ASIL/SIL | Software Responsibility |
| --- | --- | --- |
| SG-01 | ASIL D / SIL 3 | 2oo2 vote logic, fault inhibit, threshold gating |
| SG-02 | ASIL D / SIL 3 | ≤1 ms cycle time, ≤20 ms detection, ≤40 ms command |
| SG-03 | ASIL D / SIL 3 | Baby seat logic with debounce and fail-safe suppression |
| SG-06 | ASIL A / SIL 1 | POST, continuous diagnostics, DWI activation ≤800 ms |
| SG-07 | ASIL B / SIL 2 | 500 ms heartbeat, 2 s timeout detection, CRC framing |

# 2  Selection of Tools and Programming Languages

Per ISO 26262-6:2018 Clause 5 and IEC 61508-3:2010 Clause 7.4, all tools and languages used in the development of safety-related software shall be qualified or otherwise justified. This section documents the selection rationale, qualification status, and constraints for each tool and language used.

## 2.1  Programming Language

| Attribute | Detail | Safety Justification |
| --- | --- | --- |
| Language | C/C++ (Arduino framework, Arduino IDE 2.x) | C/C++ is supported by ISO 26262-6 Annex C as a high-coverage language for embedded safety systems. The Arduino framework wraps Atmel SAM3X8E hardware abstraction. |
| Language standard | C++11 (GCC 4.8.3 ARM compiler bundled with Arduino Due board package) | C++11 constrained to avoid dynamic memory (heap) and recursive functions per permitted/forbidden operations list (Section 6.4). |
| Subset restrictions | No dynamic allocation (new/delete/malloc/free), no recursion, no unbounded loops, no exceptions | Restrictions eliminate non-deterministic timing and stack overflow risks — ASIL D software coding guidelines. |
| Compiler flags | Default Arduino Due build flags: -O2 optimisation, -DARDUINO=10819, -mthumb -mcpu=cortex-m3 | Compiler version and flags pinned in board package. No custom flag changes allowed without re-qualification. |

## 2.2  Development Tools

| Tool | Version | Role | Qualification / Justification |
| --- | --- | --- | --- |
| Arduino IDE | 2.x | Code editor, compiler, upload tool | Tool output (compiled binary) verified by functional testing. No claim placed on tool correctness beyond standard compiler qualification. |
| GCC ARM (avr-g++) | 4.8.3 (bundled) | Cross-compiler for SAM3X8E (ARM Cortex-M3) | Widely used in safety projects. Code output validated via module testing (Section 8) and integration tests (Section 10). |
| due_can library | v1.0 (open source) | Arduino Due CAN0 hardware abstraction | Library limited to CAN frame send/receive. Qualified by code review + communication tests TC-08, TC-16, TC-17. |
| USBCANV2 (OMS PC) | Latest stable | Passive CAN monitoring on OMS Lab PC | Used for test evidence collection only. Not part of deployed firmware. No safety claim placed on monitor tool. |
| Serial Monitor (Arduino IDE) | Bundled | Debug/diagnostic output at 115200 baud | Used for test observation only. No safety claim. Serial.print() calls are non-critical diagnostics. |
| Git (version control) | Any | Source code configuration management | All firmware source under version control. Tagged releases correspond to documented revisions. See Section 13. |

## 2.3  Tool Classification (ISO 26262-8 Clause 11)

Under ISO 26262-8:2018 Clause 11, each tool is assigned a Tool Confidence Level (TCL) based on Tool Impact (TI) — whether tool output could directly cause a safety violation — and Tool Error Detection (TD) — whether erroneous tool output would be detected.

| Tool | TI | TD | TCL | Qualification Method |
| --- | --- | --- | --- | --- |
| GCC ARM Compiler | TI2 | TD2 | TCL2 | Functional testing of compiled output against specification (test cases TC-01 to TC-18) provides sufficient confidence. |
| Arduino IDE Upload | TI1 | TD1 | TCL1 | Flash read-back verification: confirm checksum post-upload via serial bootloader acknowledgement. |
| due_can library | TI2 | TD2 | TCL2 | Code review + CAN communication test cases (TC-08, TC-16, TC-17, TC-09). |
| USBCANV2 Monitor | TI1 | TD3 | TCL1 | Test/monitoring tool only; not in safety path. No qualification required. |

# 3  Software Architecture Overview

The firmware follows a single-task cyclic executive model executing on the bare-metal Arduino Due environment (no RTOS). One main loop iteration constitutes one execution cycle, targeted at ≤1 ms per FSR-02. The architecture is divided into six functional modules that map directly onto FSR requirements.

## 3.1  Module Decomposition

| Module ID | Module Name | Primary FSR(s) | ASIL |
| --- | --- | --- | --- |
| MOD-01 | Power-On Self-Test (POST) | FSR-17 | ASIL C |
| MOD-02 | Sensor Acquisition | FSR-01, FSR-02 | ASIL D |
| MOD-03 | Crash Detection | FSR-01, FSR-02, FSR-03, FSR-04 | ASIL D |
| MOD-04 | Deployment Control | FSR-03, FSR-08, FSR-09, FSR-10 | ASIL D |
| MOD-05 | CAN Communication | FSR-13, FSR-14, FSR-15, FSR-16 | ASIL B |
| MOD-06 | Diagnostic Monitor | FSR-17, FSR-18, FSR-19, FSR-20, FSR-22 | ASIL C/D |

## 3.2  Data Flow Between Modules

- MOD-02 (Sensor Acquisition) reads raw 12-bit ADC values (0–4095) from A0, A1, A2 and digital pin D11, storing them in global variables rawA0, rawA1, rawA2, and babySeatRaw.

- MOD-03 (Crash Detection) reads rawA0–rawA2, scales to engineering units, applies threshold comparisons, and writes localCrash (boolean) and scaled float values accel, gyro, press.

- MOD-05 (CAN Communication) transmits the local ECU status frame and receives the peer ECU frame, writing remoteCrash, remoteFault, and updating lastRemoteHB timestamp.

- MOD-06 (Diagnostic Monitor) evaluates fault flags, computing the aggregated systemFault boolean written to MOD-04.

- MOD-04 (Deployment Control) reads localCrash, remoteCrash, systemFault, remoteFault, and babySeat to compute voteDeploy, then drives D8, D9, and D10 output pins.

## 3.3  Execution Control Flow — Cyclic Executive

| Step | Action | Period / Trigger | FSR |
| --- | --- | --- | --- |
| 1 | receiveCAN() — drain RX buffer, update remoteCrash, remoteFault, lastRemoteHB | Every cycle (non-blocking) | FSR-15 |
| 2 | Heartbeat timeout check — if (millis() - lastRemoteHB > 2000 ms) → faultCOMM = true | Every cycle | FSR-15 |
| 3 | Read sensors — analogRead A0, A1, A2 + digitalRead D11 | Every cycle | FSR-01, FSR-02 |
| 4 | Scale sensors — convert ADC to g, deg/s, kPa | Every cycle | FSR-01 |
| 5 | Crash detection — threshold comparisons → localCrash | Every cycle | FSR-01, FSR-04 |
| 6 | systemFault aggregation — OR of faultCPU, faultRAM, faultADC, faultCOMM | Every cycle | FSR-20 |
| 7 | Deployment vote — voteDeploy logic → drive D8, D9, D10 | Every cycle | FSR-03, FSR-08 |
| 8 | CAN transmit heartbeat — sendData() if 500 ms elapsed | Every 500 ms | FSR-16 |
| 9 | Status LED blink — toggle D13 if 500 ms elapsed | Every 500 ms | — |
| 10 | Serial diagnostic print — if 1000 ms elapsed | Every 1000 ms | FSR-21 |

# 4  System State Machine

## 4.1  State Definitions

| State ID | State Name | Conditions to Enter | Outputs | FSRS Ref |
| --- | --- | --- | --- | --- |
| ST-01 | POWER_ON | Reset / power-up event | All outputs LOW, POST executing | SS-01 |
| ST-02 | POST_FAILED | Any POST check returns FALSE (CPU, RAM, ADC) | D10 (DWI) HIGH, D8/D9 LOW — permanent safe inhibit | SS-05 |
| ST-03 | MONITORING | POST passed, no crash, no fault, peer CAN active | D10 LOW, D8/D9 LOW — watching sensors | SS-02 |
| ST-04 | FAULT_ACTIVE | systemFault=TRUE (any fault flag set) | D10 HIGH (DWI), D8/D9 LOW — inhibit deployment | SS-01, SS-04 |
| ST-05 | CRASH_DETECTED | localCrash=TRUE AND remoteCrash=TRUE AND !systemFault AND !remoteFault | D8 HIGH (Airbag1 vote), D9 HIGH if !babySeat (Airbag2 vote) | SS-02 |

## 4.2  State Transition Diagram

Transitions described textually (graphical diagram produced in HADS Appendix):

ST-01 POWER_ON

|-- POST executes (CPU test, RAM test, ADC test)

|-- POST FAIL ────────────────────────> ST-02 POST_FAILED (permanent — no exit)

|-- POST PASS

v

ST-03 MONITORING

|-- systemFault TRUE ────────────────> ST-04 FAULT_ACTIVE

|       faultCOMM (peer timeout >2000 ms)     |-- fault clears --> ST-03

|-- localCrash=TRUE AND remoteCrash=TRUE

|   AND !systemFault AND !remoteFault ───────> ST-05 CRASH_DETECTED

|                                              |-- sensors below threshold -> ST-03

|                                              |-- fault occurs -----------> ST-04

## 4.3  Safe State Guarantees

- ST-02 (POST_FAILED): Software sets D8/D9 LOW in setup() and never reaches loop(). Hardware AND gate outputs LOW permanently — ultimate safe inhibit (SS-05).

- ST-04 (FAULT_ACTIVE): systemFault=TRUE forces voteDeploy=FALSE each cycle, pulling D8/D9 LOW. DWI lamp (D10) activates within one cycle (~1 ms), satisfying FSR-19 (≤800 ms) by a large margin.

- Loss of CAN peer (faultCOMM=TRUE): After 2000 ms without a valid peer frame, faultCOMM=TRUE propagates through systemFault into voteDeploy, inhibiting deployment. Fail-safe direction: inhibit rather than spurious deploy.

# 5  Detailed Design of Software Modules

Each module is described with its interface, internal logic, data structures, and pseudo-code. This section satisfies the ISO 26262-6 Clause 8 requirement for detailed software unit design.

## 5.1  MOD-01 — Power-On Self-Test (POST)

Executed once in setup() before loop() begins. Results stored in persistent boolean fault flags. Any failed test sets the corresponding flag.

### 5.1.1  CPU Test

Tests ALU correctness by performing a deterministic integer addition and verifying the result.

bool cpuTest() {

volatile uint32_t a = 100;

volatile uint32_t b = 200;

return ((a + b) == 300);  // volatile prevents compiler optimisation

}

### 5.1.2  RAM Test

Writes a known bit pattern (0xAAAAAAAA — alternating 1/0 bits stressing adjacent bit coupling) to a stack variable and reads it back.

bool ramTest() {

uint32_t test = 0xAAAAAAAA;

return (test == 0xAAAAAAAA);

}

NOTE: This is a lightweight single-word test appropriate for ASIL C within an ASIL D system backed by dual-channel redundancy. Full memory sweep testing would be required for standalone ASIL D certification.

### 5.1.3  ADC Test

Confirms ADC hardware is operational and returning values within the valid 12-bit range (0–4095).

bool adcTest() {

int v = analogRead(A0);

return (v >= 0 && v <= 4095);

}

### 5.1.4  POST Completion Criteria

| Test | Pass Condition | Fail Action | Flag |
| --- | --- | --- | --- |
| CPU arithmetic | a + b == 300 | faultCPU = true → systemFault = true → DWI ON, deploy inhibited | faultCPU |
| RAM read-back | 0xAAAAAAAA read back correctly | faultRAM = true | faultRAM |
| ADC range | 0 ≤ v ≤ 4095 | faultADC = true | faultADC |
| CAN init | Can0.begin() success | Not currently checked — GAP-05 (see Section 12) | — |

## 5.2  MOD-02 — Sensor Acquisition

Reads all sensor inputs at the start of each loop iteration using the Arduino Due 12-bit ADC (analogReadResolution(12) set in setup()).

uint16_t rawA0 = analogRead(A0);             // Accelerometer POTI-1: 0-4095

uint16_t rawA1 = analogRead(A1);             // Gyroscope    POTI-2: 0-4095

uint16_t rawA2 = analogRead(A2);             // Pressure     POTI-3: 0-4095

bool babySeat  = (digitalRead(PIN_BABY) == LOW); // Active-LOW switch

| Sensor | Pin | ADC Range | Scaled Range | Deploy Threshold | ADC Threshold |
| --- | --- | --- | --- | --- | --- |
| Accelerometer (POTI-1) | A0 | 0–4095 | 0–100 g | > 30.0 g | raw > 1228 |
| Gyroscope (POTI-2) | A1 | 0–4095 | 0–360 °/s | > 120.0 °/s | raw > 1365 |
| Pressure (POTI-3) | A2 | 0–4095 | 0–500 kPa | > 200.0 kPa | raw > 1638 |
| Baby Seat Switch | D11 | HIGH / LOW | TRUE / FALSE | LOW = baby seat | — |

## 5.3  MOD-03 — Crash Detection

Converts raw ADC values to engineering units and applies threshold comparisons. An OR relationship between three sensors is used: any single sensor exceeding its threshold asserts localCrash = TRUE.

### 5.3.1  Scaling Formulas

float accel = (rawA0 / 4095.0) * 100.0;   // 0-100 g

float gyro  = (rawA1 / 4095.0) * 360.0;   // 0-360 deg/s

float press = (rawA2 / 4095.0) * 500.0;   // 0-500 kPa

### 5.3.2  Threshold Comparison

bool localCrash = (accel > THRESH_ACCEL)   // > 30.0 g

|| (gyro  > THRESH_GYRO)    // > 120.0 deg/s

|| (press > THRESH_PRESS);  // > 200.0 kPa

### 5.3.3  Threshold Rationale

| Sensor | Threshold | Physical Meaning | Rationale |
| --- | --- | --- | --- |
| Accelerometer | 30.0 g | 30× gravitational acceleration — severe frontal impact pulse characteristic | Below hard-braking (~1 g) and road-bump range (<5 g); above representative airbag-required crash pulse |
| Gyroscope | 120.0 °/s | One-third of full rotation per second — rollover onset angular rate | Well above aggressive cornering (<30 °/s); below confirmed rollover rates (>200 °/s) for laboratory sensitivity |
| Pressure | 200.0 kPa | ~2 bar gauge — door panel structural deformation pressure | Representative of side-impact crush pressure exceeding normal ambient variation |

## 5.4  MOD-04 — Deployment Control

Computes the deployment vote and drives all output pins. This is the safety-critical decision point combining all module inputs.

### 5.4.1  Deployment Vote Logic (Full)

// Step 1: Aggregate system fault

systemFault = faultCPU || faultRAM || faultADC || faultCOMM;

// Step 2: Compute deployment vote

bool voteDeploy = localCrash    // This ECU detects crash

&& remoteCrash   // Peer ECU independently detects crash

&& !systemFault  // No fault on this ECU

&& !remoteFault; // No fault on peer ECU

// Step 3: Drive output pins

digitalWrite(PIN_AIRBAG1, voteDeploy);              // Driver airbag vote

digitalWrite(PIN_AIRBAG2, voteDeploy && !babySeat); // Passenger suppressed if babySeat

digitalWrite(PIN_WARNING, systemFault);             // DWI lamp

### 5.4.2  Deployment Authorization Truth Table

| localCrash | remoteCrash | systemFault | remoteFault | babySeat | Airbag1 (D8) | Airbag2 (D9) |
| --- | --- | --- | --- | --- | --- | --- |
| FALSE | any | any | any | any | LOW | LOW |
| TRUE | FALSE | any | any | any | LOW | LOW |
| TRUE | TRUE | TRUE | any | any | LOW | LOW |
| TRUE | TRUE | FALSE | TRUE | any | LOW | LOW |
| TRUE | TRUE | FALSE | FALSE | FALSE | HIGH | HIGH |
| TRUE | TRUE | FALSE | FALSE | TRUE | HIGH | LOW (suppressed) |

## 5.5  MOD-05 — CAN Communication

### 5.5.1  Transmit Frame (sendData)

Transmits an 8-byte CAN frame every 500 ms. The 500 ms heartbeat interval ensures the peer ECU can detect communication loss within the 2000 ms faultCOMM timeout window.

| Byte | Field | Encoding | Notes |
| --- | --- | --- | --- |
| 0 | A0 low byte | rawA0 & 0xFF | Accelerometer ADC low byte |
| 1 | A0 high byte | rawA0 >> 8 | Accelerometer ADC high byte |
| 2 | A1 low byte | rawA1 & 0xFF | Gyroscope ADC low byte |
| 3 | A1 high byte | rawA1 >> 8 | Gyroscope ADC high byte |
| 4 | A2 low byte | rawA2 & 0xFF | Pressure ADC low byte |
| 5 | A2 high byte | rawA2 >> 8 | Pressure ADC high byte |
| 6 | Status byte | Bit0=crash, Bit1=baby, Bit2=fault | Peer reads bits to extract remoteCrash and remoteFault |
| 7 | ECU ID | 1 or 2 | Identifies transmitting ECU |

### 5.5.2  Receive and Timeout

void receiveCAN() {

while (Can0.rx_avail()) {

Can0.read(incoming);

if (incoming.id == REMOTE_ID) {       // Filter for peer ECU only

lastRemoteHB = millis();           // Reset timeout watchdog

remoteCrash = (status & 0x01);    // Extract crash bit

remoteFault = (status & 0x04);    // Extract fault bit

}

}

}

// Timeout check in main loop:

if (millis() - lastRemoteHB > 2000) faultCOMM = true;

else                                faultCOMM = false;

## 5.6  MOD-06 — Diagnostic Monitor

Runs every cycle. Aggregates hardware fault flags into systemFault. POST fault flags (faultCPU, faultRAM, faultADC) are sticky for the session; faultCOMM is evaluated dynamically.

| Flag | Set When | Cleared When | Effect on System |
| --- | --- | --- | --- |
| faultCPU | cpuTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultRAM | ramTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultADC | adcTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultCOMM | millis() - lastRemoteHB > 2000 ms | Peer CAN frame received within 2000 ms | Dynamic: ST-04 while active, recovers to ST-03 when peer resumes |

# 6  Occupant Classification and Baby Seat Logic

The baby seat detection implements a fail-safe suppress-by-default strategy per FSR-09 and FSR-10. The switch is wired active-LOW with a 10 kΩ pull-up resistor to the Arduino Due 3.3 V rail.

## 6.1  Signal Interpretation

| D11 State | babySeat Value | Airbag 2 (D9) | Meaning |
| --- | --- | --- | --- |
| HIGH (~3.3 V via pull-up) | FALSE | Follows voteDeploy | No child seat — adult or empty — passenger airbag armed |
| LOW (switch pressed) | TRUE | Forced LOW | Child seat present — passenger airbag suppressed |

## 6.2  FSR-11 Debounce Requirement (Required Implementation)

FSR-11 requires the baby seat digital input to remain stable for at least 10 ms before a state transition is accepted as valid. The current firmware reads the pin directly without a debounce timer (GAP-01). Required implementation:

static bool lastBabyRaw = HIGH;

static unsigned long babyStableTime = 0;

bool currentRaw = digitalRead(PIN_BABY);

if (currentRaw != lastBabyRaw) {

babyStableTime = millis();

lastBabyRaw = currentRaw;

}

if ((millis() - babyStableTime) >= 10) {

babySeat = (lastBabyRaw == LOW);

}

# 7  Safety Mechanisms in Software

## 7.1  Hardware Watchdog Timer (WDT)

The SAM3X8E microcontroller includes a dedicated Hardware WDT independent of the main CPU. When enabled, it resets the MCU if the main loop fails to execute within the configured timeout window.

Current status: The WDT is NOT explicitly enabled in the current firmware (GAP-06). Required implementation for FSR-22 compliance:

// In setup():

WDT_Enable(WDT, WDT_MR_WDD(0xFFF) | WDT_MR_WDRSTEN | WDT_MR_WDV(500));

// At end of each loop() iteration:

WDT_Restart(WDT);  // Feed the watchdog

## 7.2  2oo2 Redundant Decision Architecture

The primary safety mechanism is the dual-channel 2oo2 architecture. Both ECUs independently read identical sensor inputs, execute identical crash detection algorithms, compute independent deployment votes, and assert D8/D9 only when their local vote is TRUE. The DM74LS08N AND gate forms the hardware enforcement layer.

This architecture prevents: single-point hardware failures, single-channel software errors, single-sensor noise events, and communication corruption.

## 7.3  Fail-Safe Direction Analysis

| Failure Mode | Software Response | Hardware Response | Safe? |
| --- | --- | --- | --- |
| Peer CAN timeout (>2000 ms) | faultCOMM=TRUE → systemFault=TRUE → voteDeploy=FALSE → D8/D9 LOW | AND gate output LOW | Yes — inhibit on comm loss |
| POST failure (CPU/RAM/ADC) | faultXXX=TRUE, loop() inhibits via systemFault | D8/D9 never driven HIGH in fault state | Yes — inhibit on startup fault |
| Single sensor above threshold | localCrash=TRUE on this ECU only | AND gate requires BOTH D8 HIGH — peer must agree | Yes — no single-channel deploy |
| Single ECU software crash | Loop stops, D8/D9 go LOW (default) | AND gate output LOW | Yes — fail LOW |
| babySeat line open circuit | Pull-up holds D11 HIGH → babySeat=FALSE → airbag NOT suppressed | D9 follows voteDeploy | GAP-02 — see Section 12 |

## 7.4  Permitted and Forbidden Software Operations

Per ASIL D software development guidelines (ISO 26262-6 Annex A):

### Permitted:

- Reading ADC pins A0, A1, A2 via analogRead()

- Reading digital pin D11 via digitalRead()

- Writing D8, D9 (deployment votes) and D10 (DWI) via digitalWrite()

- Transmitting CAN frames via Can0.sendFrame()

- Reading received CAN frames via Can0.read()

- Using millis() for timing comparisons

- Printing to Serial for diagnostics

### Forbidden:

- Dynamic memory allocation (malloc, new) — non-deterministic timing prohibited at ASIL D

- Recursive functions — stack depth unpredictable

- Unbounded loops (while(true) without exit) — threatens deterministic cycle time

- Direct register access bypassing safety flags

- Clearing faultCPU, faultRAM, faultADC once set — these are sticky faults

- Modifying ECU_ID, DATA_ID, REMOTE_ID at runtime — configuration constants must be compile-time only

# 8  Unit Testing of Software Modules

This section defines unit-level tests for each software module (MOD-01 through MOD-06). Unit tests verify each module in isolation before integration. All tests are executed on the target hardware (Arduino Due) to avoid simulator discrepancies.

## 8.1  Unit Test Environment

- Each module tested by toggling inputs and observing outputs via Serial Monitor (115200 baud) and/or oscilloscope on output pins.

- Potentiometers POTI-1/2/3 set to specific ADC counts as specified per test case.

- Baby seat switch wired to D11 with 10 kΩ pull-up as per HADS.

- Isolated firmware builds used where needed to test modules independently.

## 8.2  MOD-01 Unit Tests — POST

| UT-ID | Description | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- |
| UT-01-A | CPU test passes — nominal ALU | Serial: faultCPU=0; DWI OFF | faultCPU = false at startup | FSR-17 |
| UT-01-B | CPU test fail simulation (hardcode return false) | Serial: faultCPU=1; DWI ON; deploy inhibited throughout session | System enters ST-04 permanently | FSR-17, FSR-20 |
| UT-01-C | RAM test passes — nominal SRAM | Serial: faultRAM=0 | faultRAM = false at startup | FSR-17 |
| UT-01-D | RAM test fail simulation (hardcode return false) | Serial: faultRAM=1; systemFault=1; DWI ON | Deploy inhibited permanently | FSR-22 |
| UT-01-E | ADC test passes — nominal ADC | Serial: faultADC=0 | faultADC = false at startup | FSR-17 |

## 8.3  MOD-02 Unit Tests — Sensor Acquisition

| UT-ID | Description | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- |
| UT-02-A | ADC read minimum — POTI-1/2/3 at GND end (ADC ~0) | Serial: A0=0, A1=0, A2=0 (±5 counts) | All ADC reads ≤ 5 counts | FSR-01 |
| UT-02-B | ADC read maximum — POTI-1/2/3 at 3.3 V end (ADC ~4095) | Serial: A0≈4095, A1≈4095, A2≈4095 | All ADC reads ≥ 4090 counts | FSR-01 |
| UT-02-C | Baby seat switch open (HIGH) — no baby seat | Serial: Baby=0 | babySeat = false | FSR-09 |
| UT-02-D | Baby seat switch pressed (LOW) — baby seat present | Serial: Baby=1 | babySeat = true | FSR-09 |
| UT-02-E | ADC threshold crossing — POTI-1 set to ADC 1228 (30g boundary) | Serial: accel ≈ 30.0 g; localCrash=0 (not exceeded) | localCrash = false at ADC 1228 | FSR-01, FSR-04 |

## 8.4  MOD-03 Unit Tests — Crash Detection

| UT-ID | Description | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- |
| UT-03-A | Accel threshold — POTI-1 just above 1228 (ADC 1229) | localCrash=1 from accelerometer | localCrash = true within one cycle | FSR-01 |
| UT-03-B | Gyro threshold — POTI-2 just above 1365 (ADC 1366) | localCrash=1 from gyroscope | localCrash = true within one cycle | FSR-01 |
| UT-03-C | Pressure threshold — POTI-3 just above 1638 (ADC 1639) | localCrash=1 from pressure | localCrash = true within one cycle | FSR-01 |
| UT-03-D | All sensors below threshold | localCrash=0 | localCrash = false | FSR-04 |
| UT-03-E | OR logic — two sensors below threshold, one above | localCrash=1 (OR satisfied) | localCrash = true | FSR-01 |

## 8.5  MOD-04 Unit Tests — Deployment Control

| UT-ID | Description | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- |
| UT-04-A | voteDeploy TRUE — both crash, no fault, no baby seat | D8=HIGH, D9=HIGH, D10=LOW | Both output pins HIGH | FSR-03 |
| UT-04-B | systemFault=TRUE inhibits deployment | voteDeploy=0; D8=LOW, D9=LOW, D10=HIGH | D8/D9 LOW when fault active | FSR-20 |
| UT-04-C | remoteFault=TRUE inhibits deployment | voteDeploy=0; D8=LOW, D9=LOW | D8/D9 LOW when peer faulted | FSR-03 |
| UT-04-D | babySeat=TRUE suppresses Airbag2 only | D8=HIGH, D9=LOW | D9 LOW when baby seat active | FSR-09 |
| UT-04-E | DWI activates within one cycle of fault onset | D10=HIGH within ~1 ms | D10 HIGH within 1 cycle | FSR-19 |

# 9  Integration Testing of Software Modules

Integration tests verify the correct interaction between modules and between software and hardware. Tests are executed on the full assembled system with both ECUs, CAN bus, AND gate, and all output LEDs connected per HADS-AIRBAG-ECU-001 Rev 2.1.

## 9.1  Integration Test Environment

- Both Arduino Due ECUs assembled on breadboard per HADS wiring tables.

- OMS PC connected via USB-CAN-A, USBCANV2 running at 500 kbit/s, monitoring IDs 0x100 and 0x200.

- Serial Monitor (115200 baud) open on both ECUs simultaneously.

- Potentiometers at mid-position (ADC ~2048, all above threshold) as baseline unless specified.

- Baby seat switch open (HIGH) as baseline unless specified.

## 9.2  Integration Test Cases — Normal Operation

| IT-ID | Type | Description | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- | --- |
| IT-01 | BB | No-crash baseline — all sensors below threshold on both ECUs | Both LEDs OFF; Serial: localCrash=0, voteDeploy=0 | LEDs off ≥ 5 s | FSR-04 |
| IT-02 | BB | Full dual-channel crash — both ECUs above threshold simultaneously | Both LEDs ON; Serial: voteDeploy=1 on both ECUs; USBCANV2 shows crash bit in 0x100 and 0x200 | Airbag1 LED ON within observed cycle | FSR-01, FSR-03 |
| IT-03 | BB | Crash ceases — sensors returned below threshold | LEDs extinguish within one cycle | LEDs OFF on sensor drop | FSR-04 |
| IT-04 | BB | Baby seat suppression during crash | Airbag1 LED ON, Airbag2 LED OFF when baby seat pressed during crash | Airbag2 stays OFF | FSR-09 |
| IT-05 | BB | CAN heartbeat visible on USBCANV2 monitor | Frames at ~500 ms intervals on both 0x100 and 0x200 | All intervals 450–550 ms | FSR-16 |

## 9.3  Integration Test Cases — Fault Injection

| IT-ID | Type | Description | Expected Output | Pass Criterion | FSR |
| --- | --- | --- | --- | --- | --- |
| IT-06 | BB | Single-channel crash — only ECU1 above threshold; ECU2 below | Both LEDs OFF; ECU1 remoteCrash=0 (from ECU2 CAN); voteDeploy=FALSE | AND gate cannot fire | FSR-03, FSR-08 |
| IT-07 | BB | CAN bus disconnected — peer timeout after 2000 ms | DWI LED ON within 2000 ms; both airbag LEDs OFF | DWI ON within 2 s | FSR-15, FSR-20 |
| IT-08 | BB+WB | CAN reconnected after timeout — system recovers | faultCOMM clears; DWI OFF; system returns to MONITORING | Recovery within one CAN frame period | FSR-15 |
| IT-09 | WB | Remote fault bit set — peer ECU broadcasts fault in CAN byte 6 | ECU1 reads remoteFault=TRUE; voteDeploy=FALSE even with both sensors above threshold | Airbag LEDs remain OFF | FSR-03 |
| IT-10 | BB | POST failure simulation — faultCPU forced TRUE | DWI ON; voteDeploy=FALSE for entire session regardless of sensor state | No LED fires even with full crash | FSR-17, FSR-20 |

## 9.4  Integration Test Cases — Timing Verification

| IT-ID | Description | Method | Acceptance Criterion | FSR |
| --- | --- | --- | --- | --- |
| IT-11 | Loop cycle time ≤ 1 ms | micros() timestamps logged via Serial over 10,000 iterations | 99th percentile cycle ≤ 1000 µs | FSR-02 |
| IT-12 | Crash detection response ≤ 20 ms | Timestamp pot threshold crossing vs localCrash=TRUE in Serial log | Delta ≤ 20 ms over 10 trials | FSR-01 |
| IT-13 | CAN heartbeat interval 500 ms ± 50 ms | USBCANV2 frame timestamp log over 20 frames | All intervals 450–550 ms | FSR-16 |
| IT-14 | faultCOMM declared ≤ 2000 ms after CAN loss | Disconnect CAN, log millis() at last frame and at faultCOMM=TRUE | faultCOMM declared ≤ 2050 ms | FSR-15 |
| IT-15 | DWI activates within 800 ms of fault | Inject fault at known millis(); observe D10 HIGH transition | D10 HIGH within 800 ms | FSR-19 |

# 10  Integration of Programmable Electronics (HW and SW)

This section describes the procedure for integrating the firmware with the hardware assembly. It covers flash programming, pin verification, CAN bus bring-up, and the acceptance criteria that must be met before the integrated system is released to functional testing.

## 10.1  HW/SW Integration Procedure

| Step | Action | Acceptance Criterion | Owner |
| --- | --- | --- | --- |
| 1 | Assemble hardware per HADS-AIRBAG-ECU-001 Rev 2.1 — breadboard wiring, potentiometers, baby seat switch, CAN transceivers, AND gate, LEDs | All wiring verified against HADS Tables 5–14. No short circuits. Continuity check on all signal lines. | HW Engineer |
| 2 | Verify power rails: 5 V rail to AND gate VCC, 3.3 V rail to CAN transceivers, GND common to all | Measure 5 V ± 0.25 V at AND gate pin 14. Measure 3.3 V ± 0.15 V at Waveshare VCC. GND continuity confirmed. | HW Engineer |
| 3 | Flash ECU 1 firmware (ECU_ID=1, DATA_ID=0x100, REMOTE_ID=0x200) via Arduino IDE USB upload | Upload completes with 'Done uploading'. Serial Monitor shows ECU 1 STARTED message at 115200 baud. | SW Engineer |
| 4 | Flash ECU 2 firmware (ECU_ID=2, DATA_ID=0x200, REMOTE_ID=0x100) via Arduino IDE USB upload | Upload completes with 'Done uploading'. Serial Monitor shows ECU 2 STARTED message. | SW Engineer |
| 5 | Verify POST results on both ECUs via Serial Monitor | Serial output shows: faultCPU=0, faultRAM=0, faultADC=0. Status LED (D13) blinking at ~500 ms on both ECUs. | SW Engineer |
| 6 | Verify CAN bus bring-up — connect OMS PC via USB-CAN-A, open USBCANV2 at 500 kbit/s | CAN frames with ID 0x100 and 0x200 visible at ~500 ms intervals in USBCANV2 log. | HW/SW Engineer |
| 7 | Verify CAN communication between ECUs — check faultCOMM=0 on both ECUs | Serial output: faultCOMM=0 on both ECUs. Remote crash and fault bits correctly parsed. | SW Engineer |
| 8 | Verify pin output levels at rest — all sensors below threshold | D8=LOW, D9=LOW, D10=LOW, D13 blinking. Airbag LEDs OFF. DWI OFF. | HW Engineer |
| 9 | Verify AND gate logic — set POTI-1 on ECU1 only above threshold | Airbag1 LED stays OFF (only one ECU asserting D8). AND gate pin 3 LOW. | HW Engineer |
| 10 | Verify full crash scenario — set both ECU POTI-1 above threshold simultaneously | Airbag1 LED fires (AND gate pin 3 HIGH). Serial: voteDeploy=1 on both ECUs. | HW/SW Engineer |
| 11 | Verify baby seat suppression — press baby seat switch during step 10 crash | Airbag1 LED remains ON; Airbag2 LED OFF. D9 LOW on both ECUs. | HW/SW Engineer |
| 12 | Sign off HW/SW integration — record results and release to functional testing | All steps passed. Integration sign-off form completed. | Project Lead |

## 10.2  Firmware Version Identification

Each firmware binary shall be uniquely identified. The following version information shall be printed to Serial at startup and included in the integration sign-off record:

// Printed in setup():

Serial.println("SADS-AIRBAG-ECU-001 Rev 2.1");

Serial.print("ECU ID: "); Serial.println(ECU_ID);

Serial.print("Build date: "); Serial.println(__DATE__);

Serial.print("Build time: "); Serial.println(__TIME__);

# 11  Software Verification

Software verification confirms that the software implementation correctly and completely satisfies the requirements in FSRS-AIRBAG-ECU-001 Rev 2.1. Per ISO 26262-6:2018 Clause 9, verification activities at ASIL D include code review, static analysis, and test-based verification.

## 11.1  Verification Methods

| Method | Description | ISO 26262-6 Reference | Applicability |
| --- | --- | --- | --- |
| Code Review (Walkthrough) | Manual review of source code against SADS module specifications. Two-person review (author + independent reviewer). Review checklist covers: forbidden constructs, fault flag handling, threshold values, output pin logic, CAN frame encoding. | Clause 9.4.2 (code review) | All modules — mandatory ASIL D |
| Static Analysis | Review of code structure for: unreachable code, unused variables, potential integer overflow in ADC scaling, uninitialized variable access, and adherence to forbidden operations list. | Clause 9.4.2 (static analysis) | MOD-02, MOD-03, MOD-04 |
| Unit Testing | Black-box and white-box tests per Section 8. Tests executed on target hardware. Results logged as test evidence. | Clause 9.4.3 (dynamic analysis) | MOD-01 through MOD-06 |
| Integration Testing | Tests per Section 9 verifying inter-module and HW/SW integration. | Clause 9.4.3 | Full system |
| Timing Analysis | Cycle time measurement via micros() log. Worst-case cycle time must be ≤ 1000 µs (FSR-02). CAN heartbeat timing verified via USBCANV2 log. | Clause 9.4.4 (timing analysis) | Main loop, CAN TX |
| Requirement Traceability Review | Each FSR-XX requirement traced to implementing module(s) and at least one passing test case. Untraced requirements documented as gaps. | Clause 9.4.1 (traceability) | All FSR requirements |

## 11.2  Code Review Checklist

Each code review shall verify all items in the following checklist. All items must be PASS before the firmware version is accepted for functional testing.

| # | Checklist Item | Result | Reviewer |
| --- | --- | --- | --- |
| CR-01 | ECU_ID, DATA_ID, REMOTE_ID are correct compile-time constants for the target board (ECU 1 vs ECU 2). No runtime modification. | PASS / FAIL |  |
| CR-02 | analogReadResolution(12) called in setup(). ADC max divisor is 4095.0 (float), not 4096. | PASS / FAIL |  |
| CR-03 | localCrash uses OR of all three sensor thresholds: accel > 30.0 ¦¦ gyro > 120.0 ¦¦ press > 200.0. | PASS / FAIL |  |
| CR-04 | voteDeploy = localCrash && remoteCrash && !systemFault && !remoteFault — all four conditions present. | PASS / FAIL |  |
| CR-05 | D9 driven as voteDeploy && !babySeat — baby seat suppression applied to passenger airbag only. | PASS / FAIL |  |
| CR-06 | D10 driven as systemFault — DWI activated on any fault condition. | PASS / FAIL |  |
| CR-07 | faultCOMM evaluated dynamically each cycle from millis() - lastRemoteHB > 2000. | PASS / FAIL |  |
| CR-08 | receiveCAN() filters on REMOTE_ID before accepting peer data. Frames with wrong IDs ignored. | PASS / FAIL |  |
| CR-09 | CAN frame byte 6 bit 0 = crashLocal, bit 2 = systemFault. Status byte encoding matches HADS Table 18. | PASS / FAIL |  |
| CR-10 | No dynamic memory allocation (no new, malloc, delete, free) in any module. | PASS / FAIL |  |
| CR-11 | No recursive function calls in any module. | PASS / FAIL |  |
| CR-12 | No unbounded loops (while(1) without break or timeout check). | PASS / FAIL |  |
| CR-13 | POST fault flags (faultCPU, faultRAM, faultADC) are never cleared after being set. | PASS / FAIL |  |
| CR-14 | Status LED blink (D13) is non-critical and does not interfere with timing of safety-critical outputs. | PASS / FAIL |  |
| CR-15 | Serial.print() calls are inside 1000 ms elapsed check — do not execute every cycle. | PASS / FAIL |  |

## 11.3  Verification Coverage Target

ISO 26262-6 ASIL D requires MC/DC (Modified Condition/Decision Coverage) or equivalent structural coverage for software at the highest integrity level. For this laboratory implementation, the agreed coverage target is:

- Statement coverage: 100% of executable statements exercised by test cases TC-01 through TC-18 / IT-01 through IT-15.

- Branch coverage: All TRUE and FALSE outcomes of every conditional (if/else, || and && operators in voteDeploy and localCrash) exercised.

- Fault detection coverage: ≥ 90% of identifiable single-point software faults detectable through defined test cases.

# 12  Validation of the Software

Software validation confirms that the complete integrated software fulfils the intended safety purpose in the final hardware environment. Per ISO 26262-6:2018 Clause 10, validation is performed at system level — not module level — and must demonstrate that all safety goals (SG-01 through SG-07) are achievable through the software.

## 12.1  Validation Scope and Approach

Validation is performed on the fully assembled laboratory system (both ECUs, CAN bus, AND gate, all LEDs) using the scenarios derived from the FSRS safety goals. Validation differs from verification in that it asks "does the system do the right thing?" rather than "does it match the specification?"

## 12.2  Validation Test Cases

| VT-ID | Safety Goal | Scenario | Expected System Behaviour | Pass Criterion |
| --- | --- | --- | --- | --- |
| VT-01 | SG-01 | No-crash condition — all potentiometers below deployment threshold for 10 minutes of continuous operation | No airbag LED fires at any point. DWI remains OFF. System stays in ST-03 (MONITORING). | Zero false deployments in 10-minute run |
| VT-02 | SG-02 | Simulated crash event — both ECUs POTI-1 simultaneously moved above threshold | Airbag1 LED fires within observed cycle. Deployment vote visible in Serial log and USBCANV2 within 1 loop cycle after threshold crossing. | LED fires; Serial confirms voteDeploy=1 |
| VT-03 | SG-03 | Baby seat present during crash — switch pressed before and during crash event | Airbag1 LED fires (driver protected). Airbag2 LED does NOT fire (passenger suppressed). babySeat=1 in Serial log. | Airbag2 LED remains OFF throughout |
| VT-04 | SG-01 | Single ECU fault scenario — faultCPU forced TRUE on ECU1; both sensors above threshold | No airbag LED fires. DWI ON. System stays safe even with full crash signal. | Zero deployment when faulted ECU |
| VT-05 | SG-01 | CAN loss scenario — CAN bus disconnected during nominal monitoring | After 2000 ms: DWI ON, both airbag LEDs OFF. System inhibits deployment on comm loss. | DWI ON within 2 s; no spurious deploy |
| VT-06 | SG-06 | Fault detection and DWI — POST failure injected | DWI activates within 800 ms of fault. All deployment outputs inhibited. | DWI ON ≤ 800 ms after fault |
| VT-07 | SG-07 | CAN reliability — OMS PC logs crash notification frames during deployment | USBCANV2 shows crash bit (byte 6 bit 0 = 1) in 0x100 and 0x200 frames within one heartbeat period after crash onset. | Crash notification visible in OMS log |

## 12.3  Validation Evidence Requirements

The following evidence shall be collected and archived for each validation test case:

- Serial Monitor log file (timestamped) showing all relevant variables during the test.

- USBCANV2 CAN frame log file confirming CAN frame content and timing.

- Photograph or video of LED states (airbag1, airbag2, DWI) at key moments.

- Test operator name, date, firmware version (from startup Serial print), and hardware revision.

- Pass/fail determination against acceptance criterion.

# 13  Assessment of Functional Safety

This section provides a functional safety assessment of the software implementation against the requirements and constraints defined in FSRS-AIRBAG-ECU-001 Rev 2.1. The assessment evaluates whether the software architecture and implementation are consistent with ASIL D / SIL 3 objectives.

## 13.1  ASIL D Software Requirements Conformance

| ISO 26262-6 Requirement | Implementation Status | Evidence | Status |
| --- | --- | --- | --- |
| Hierarchical software structure with defined modules | Six modules defined (MOD-01 through MOD-06) with documented interfaces, responsibilities, and ASIL assignments. | Section 3.1 | COMPLIANT |
| Defined data flow between modules | Data flow documented in Section 3.2. All global variables identified (rawA0, rawA1, rawA2, localCrash, remoteCrash, systemFault, etc.). | Section 3.2 | COMPLIANT |
| Deterministic execution (no RTOS, bounded loops) | Single-task cyclic executive with no dynamic allocation, no recursion, bounded timing per Section 3.3. | Section 3.3, 7.4 | COMPLIANT |
| Defensive programming (fault detection checks) | POST (MOD-01), continuous fault monitoring (MOD-06), fail-safe direction analysis documented. | Section 5.1, 7.2, 7.3 | COMPLIANT |
| Safety-related software shall fail safely | All fault paths lead to voteDeploy=FALSE and D8/D9=LOW. Fail-safe direction is inhibit. Hardware AND gate enforces. | Section 7.3 | COMPLIANT |
| Redundant independent processing channels | Two independent Arduino Due boards with identical algorithms operating in 2oo2 architecture. | Section 7.2, HADS 2.2 | COMPLIANT |
| Code review required for ASIL D | Code review checklist defined in Section 11.2. Two-person review required. | Section 11.2 | REQUIRED — not yet completed |
| WDT supervision for ASIL D | WDT required by FSR-22. Implementation not yet in firmware. | Section 7.1 (GAP-06) | OPEN GAP |
| CRC-protected communication | CRC not yet implemented in CAN frame. | Section 12 (GAP-03) | OPEN GAP |
| Baby seat input debounce | 10 ms debounce not yet implemented. | Section 6.2 (GAP-01) | OPEN GAP |

## 13.2  Single-Point Fault Metric (SPFM) Analysis

ISO 26262 ASIL D requires SPFM ≥ 99%. The 2oo2 dual-channel architecture addresses the SPFM target as follows:

- A single hardware fault in ECU1 (CPU, RAM, ADC, or software error) will at most cause ECU1 to assert D8/D9 HIGH spuriously. However, the AND gate requires ECU2 to independently assert its D8/D9 simultaneously. ECU2 will not agree unless its own sensor data also indicates a crash. A single-point hardware fault therefore cannot cause unintended airbag deployment.

- A single communication fault (corrupted CAN frame) sets remoteCrash to an unknown value. The worst case is remoteCrash=FALSE, which inhibits deployment (safe direction). In the case where a corrupted frame spuriously sets remoteCrash=TRUE, the hardware AND gate still requires both physical D8 pins HIGH simultaneously.

- The CRC gap (GAP-03) means corrupted frames are not detected at the CAN layer. However, the hardware AND gate provides a physical second check. SPFM for deployment-induced faults remains high due to this hardware layer.

SPFM assessment: ASIL D SPFM target achievable through 2oo2 architecture and hardware AND gate enforcement, conditional on closure of open gaps (GAP-01, GAP-03, GAP-06) before production deployment.

## 13.3  Residual Risk Assessment

| Risk ID | Scenario | Likelihood | Severity | Mitigation |
| --- | --- | --- | --- | --- |
| RR-01 | GAP-03: Corrupted CAN frame sets remoteCrash=TRUE spuriously + ECU local crash also TRUE → spurious deployment vote | Low (CAN CRC hardware layer provides partial protection) | High (LED fires — in real system: airbag deploys) | Hardware AND gate mitigates; CRC implementation (GAP-03) required for final closure |
| RR-02 | GAP-01: Baby seat switch signal bounce causes babySeat=FALSE during baby seat presence → passenger airbag fires | Medium (switch noise probable during crash vibration) | High | Implement 10 ms debounce per Section 6.2 |
| RR-03 | GAP-06: WDT not enabled; software lockup possible in extreme fault scenarios | Low (no evidence of lockup in testing) | High (system state unknown on lockup) | Enable WDT per Section 7.1 |
| RR-04 | Open circuit on D11 pull-up: pull-up holds D11 HIGH → babySeat=FALSE → passenger airbag not suppressed when baby seat present | Low (hardware dependent on pull-up continuity) | High | GAP-02: implement D11 line monitoring |

# 14  Compliance Gaps and Required Improvements

The following gaps were identified between the current firmware implementation and the requirements in FSRS-AIRBAG-ECU-001 Rev 2.1. Each gap carries an assigned priority and required action.

| Gap ID | FSR | Description | Risk | Required Action |
| --- | --- | --- | --- | --- |
| GAP-01 | FSR-11 | Baby seat debounce not implemented — D11 read directly without 10 ms stability filter | Medium | Implement debounce timer as shown in Section 6.2 |
| GAP-02 | FSR-12 | Baby seat line continuity monitoring not implemented — open-circuit stuck-HIGH not detected | High | Add periodic D11 continuity test or input monitoring |
| GAP-03 | FSR-13 | CRC not included in CAN frame — bytes 6–7 carry status and ECU_ID, not a 16-bit CRC | Medium | Compute 16-bit CRC over bytes 0–5, store in bytes 6–7, verify on receive |
| GAP-04 | FSR-14 | Parallel CAN bitstream readback not implemented | Low | Implement readback channel or loopback verification |
| GAP-05 | FSR-17 | CAN communication not verified during POST — Can0.begin() success not checked | Medium | Check Can0.begin() return value; set faultCOMM on failure |
| GAP-06 | FSR-22 | Hardware WDT not enabled in firmware | High | Enable WDT in setup() and feed at end of every loop() iteration (Section 7.1) |
| GAP-07 | FSR-21 | Fault logging to DTC/non-volatile memory not implemented | Low | Implement DTC logging to EEPROM or flash |
| GAP-08 | FSR-15 | CAN timeout threshold is 2000 ms vs FSR-15 requirement of 50 ms | Medium | Reduce COMM_TIMEOUT to 50 ms (or justify the 2000 ms as a design choice with safety rationale) |

# 15  Modification and Change Management Procedure

Per ISO 26262-8:2018 Clause 8 (Configuration Management) and Clause 9 (Change Management), all modifications to safety-related software must follow a controlled change process. This section defines the procedure for requesting, evaluating, implementing, and releasing firmware changes.

## 15.1  Change Categories

| Category | Definition | Required Activities |
| --- | --- | --- |
| CAT-1 — Safety-Critical Change | Any change to: deployment logic (voteDeploy), threshold values, fault flag handling, CAN encoding, output pin assignments, or safety mechanism code (POST, WDT) | Full impact analysis → SADS update → code review (2-person) → full unit test re-run → integration re-run → functional safety re-assessment → new document version release |
| CAT-2 — Non-Safety-Critical Change | Changes to: Serial diagnostic prints, status LED blink timing, comments, variable naming (no logic change) | Limited impact analysis → SADS note update → author self-review → targeted unit test → minor revision |
| CAT-3 — Gap Closure Change | Changes that close GAP-01 through GAP-08 listed in Section 14 | Gap-specific analysis → SADS gap status update → relevant unit tests → integration re-run → safety assessment update |

## 15.2  Change Control Process

- Change Request (CR) raised — describe the change, category (CAT-1/2/3), reason, and affected modules/FSRs.

- Impact Analysis — identify all modules affected, FSR requirements impacted, and test cases requiring update or re-run.

- SADS Update — update relevant sections of this document. Increment version number per revision history.

- Implementation — implement the change in firmware. Commit to version control with CR reference.

- Review — code review per Section 11.2 checklist (full for CAT-1; targeted for CAT-2/3).

- Test — execute all affected test cases. For CAT-1, re-run complete test suite (Sections 8–9).

- Release — tag firmware version in version control. Update revision history. Obtain approval signatures.

## 15.3  Version Naming Convention

Firmware and document versions shall follow the naming convention:

- Major version (X.0): Architecture change, new module, change to safety-critical logic.

- Minor version (X.Y): Gap closure, additional test cases, document additions, non-safety-critical changes.

- Hotfix (X.Y.Z): Typographic correction, comment update — no code logic change.

## 15.4  Configuration Items Under Change Control

| Configuration Item | Current Version | Repository Location |
| --- | --- | --- |
| Firmware source code (ECU 1) | Rev 2.1 | Git repository — tagged SADS-2.1-ECU1 |
| Firmware source code (ECU 2) | Rev 2.1 | Git repository — tagged SADS-2.1-ECU2 |
| SADS (this document) | Rev 2.1 | Project document repository |
| FSRS-AIRBAG-ECU-001 | Rev 2.1 | Project document repository |
| HADS-AIRBAG-ECU-001 | Rev 2.1 | Project document repository |
| Test evidence logs (Serial + USBCANV2) | Per test run | Test archive folder — date-stamped |

# 16  Requirement Traceability Matrix

| FSR ID | FSR Title | Implementing Module(s) | Test Case(s) | Gap |
| --- | --- | --- | --- | --- |
| FSR-01 | Crash Detection Timing (≤20 ms) | MOD-02, MOD-03 | IT-02, IT-12 | — |
| FSR-02 | Algorithm Cycle Time (≤1 ms) | All modules (loop discipline) | IT-11 | — |
| FSR-03 | Deployment Authorization Logic (6 conditions) | MOD-04 | IT-02, IT-04, IT-06, IT-09 | — |
| FSR-04 | Prevention of Unintended Deployment | MOD-04 | IT-01, IT-03, IT-06 | — |
| FSR-07 | Dual-Channel Redundancy | MOD-02 to MOD-05 | IT-06 | — |
| FSR-08 | Channel Agreement Verification | MOD-04, MOD-05 | IT-06 | — |
| FSR-09 | Passenger Airbag Suppression | MOD-04 | IT-04, VT-03 | — |
| FSR-10 | Suppress-on-Fault (baby seat) | MOD-04 | IT-04 | GAP-02 |
| FSR-11 | Digital Input Debounce (10 ms) | MOD-02 (not yet) | — | GAP-01 |
| FSR-12 | Baby Seat Line Monitoring | MOD-06 (not yet) | — | GAP-02 |
| FSR-13 | CRC-Protected CAN | MOD-05 (not yet) | — | GAP-03 |
| FSR-14 | CAN Bitstream Readback | MOD-05 (not yet) | — | GAP-04 |
| FSR-15 | CAN Timeout Detection (≤50 ms) | MOD-05, MOD-06 | IT-07, IT-14 | GAP-08 (timeout = 2000 ms) |
| FSR-16 | Crash Notification Transmission | MOD-05 | IT-13 | — |
| FSR-17 | Power-On Self-Test | MOD-01 | UT-01-A to UT-01-E, IT-10 | GAP-05 |
| FSR-18 | Continuous Diagnostic Monitoring | MOD-06 | IT-07 | — |
| FSR-19 | DWI Activation (≤800 ms) | MOD-06, MOD-04 | IT-15, VT-06 | — |
| FSR-20 | Safe-State Transition | MOD-04, MOD-06 | IT-07, IT-10 | — |
| FSR-22 | Memory Integrity Protection | MOD-01, MOD-06 (partial) | UT-01-D | GAP-06 |

# 17  References

## 17.1  Project Documents

| Document ID | Title | Revision | Relationship to this Document |
| --- | --- | --- | --- |
| HARA-AIRBAG-ECU-001 | Hazard Analysis and Risk Assessment | Rev 2.1 | Source of hazard IDs and ASIL derivation |
| FSRS-AIRBAG-ECU-001 | Functional Safety Requirements Specification | Rev 2.1 | Primary source for all FSR-XX requirements |
| SC-AIRBAG-ECU-001 | Safety Concept Document | Rev 2.0 | Defines safety constraints SC-01 through SC-09 |
| HADS-AIRBAG-ECU-001 | Hardware Architecture and Design Specification | Rev 2.1 | HW wiring, BOM, CAN IDs, pin assignments |

## 17.2  Normative Standards

- ISO 26262:2018 — Road Vehicles — Functional Safety (Part 6: Product development at the software level)

- IEC 61508:2010 — Functional Safety of E/E/PE Safety-Related Systems (Part 3: Software requirements)

- ISO 26262-8:2018 — Supporting processes (tool qualification, configuration management, change management)

- ISO 11898 — Road Vehicles — Controller Area Network (CAN)

# 18  Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Software Safety Engineer |  |  |  |
| Hardware Safety Engineer |  |  |  |
| Functional Safety Engineer |  |  |  |
| V&V Lead |  |  |  |
| Project Supervisor |  |  |  |

# 19  Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-28 | Software Safety Engineering Team | Initial release (UART architecture). Full SADS covering architecture, module decomposition, state machine, safety mechanisms, and SWR traceability. |
| 2.0 | 2026-06-12 | Software Safety Engineering Team | Major revision — CAN architecture migration. UART inter-ECU link replaced by CAN bus (500 kbit/s). Master/Slave removed; peer-equal architecture. CAN frame byte layout documented. Compliance gap analysis (GAP-01–GAP-07). Test specification (TC-01–TC-18). FSR traceability matrix. |