Airbag Electronic Control Unit (ECU) HADS-AIRBAG-ECU-001 ¦ Revision 2.0 Final ¦ 2026-06-14 HARDWARE ARCHITECTURE AND DESIGN SPECIFICATION ISO 26262:2018 (ASIL D) / IEC 61508:2010 (SIL 3)

ASIL D (ISO 26262) / SIL 3 (IEC 61508)

| Field | Value |
| --- | --- |
| Document Number | HADS-AIRBAG-ECU-001 |
| Revision | 2.0 Final |
| Date | 2026-06-14 |
| Author | Om naik, Mrunal Ghotekar |
| Status | RELEASED |
| ASIL Classification | ASIL D (ISO 26262) / SIL 3 (IEC 61508) |
| Architecture | Peer-equal dual-channel |
| Target Hardware | Arduino Due (SAM3X8E) x2, DM74LS08N, Waveshare SN65HVD230 x2, USB-CAN-A |
| Diagnostic Tool | USBCANV2 software |
| Code Baseline | Confirmed firmware — ECU1 and ECU2 identical except first 3 #define lines |
| Based On | HARA-AIRBAG-ECU-001 Rev 2.1 ¦ FSRS-AIRBAG-ECU-001 Rev 2.1 ¦ SC-AIRBAG-ECU-001 Rev 2.0 |

Table of Contents

1. Glossary	1

2 Purpose and Scope	2

3 System Architecture Overview	2

3.1 Peer-equal design principle	2

3.2 Deployment decision architecture	2

3.3 Node inventory	2

4 Bill of Materials	3

5 Pin Assignments and Electrical Connections	4

5.1 Arduino Due — pin assignment (identical for both ECU 1 and ECU 2)	4

5.2 DM74LS08N AND gate — pin assignment	4

5.3 Waveshare SN65HVD230 CAN transceiver (applies to both boards)	5

5.4 USB-CAN-A adapter	5

6 Sensor Input Connections	5

6.1 Potentiometer wiring — confirmed from code	5

6.2 Baby seat switch — confirmed from code	6

7 Output Connections	6

7.1 Airbag 1 LED — driver airbag indicator (RED)	6

7.2 Airbag 2 LED — passenger airbag indicator (RED)	7

7.3 Warning lamp — DWI (RED LED, shared between both ECUs)	7

8 CAN Bus Architecture	7

8.1 Physical bus configuration	7

8.2 CAN ID assignment — confirmed from firmware	8

8.3 CAN frame byte layout — confirmed from sendData() function	8

8.4 Status byte bit fields (byte 6)	8

9 Deployment Logic — Confirmed from Firmware	8

9.1 Software vote — identical on both ECUs	8

9.2 Communication timeout fault	9

9.3 Hardware AND gate — final physical layer	9

10 Power Architecture	9

10.1 Power rails	9

10.2 Voltage domain boundaries	9

11 Functional Safety Engineer Review	10

12 Hardware Items to Replace or Modify	11

13 Approval and Sign-Off	11

14 Revision History	12

# 1. Glossary

| Term | Definition |
| --- | --- |
| ADC | Analog-to-Digital Converter — converts 0–3.3 V analog input to 12-bit digital value 0–4095 on Arduino Due |
| ASIL | Automotive Safety Integrity Level — risk classification per ISO 26262 (QM, A, B, C, D) |
| CAN | Controller Area Network — serial communication bus |
| DWI | Diagnostic Warning Indicator — shared red warning LED driven by both ECUs on pin D10 |
| ECU | Electronic Control Unit |
| FSR | Functional Safety Requirement — from FSRS-AIRBAG-ECU-001 |
| GPIO | General Purpose Input/Output — digital pin on microcontroller |
| HADS | Hardware Architecture and Design Specification — this document |
| HARA | Hazard and Risk Analysis |
| OMS | OMS Laboratory PC — passive CAN monitoring workstation running USBCANV2 |
| SIL | Safety Integrity Level per IEC 61508 (1–4) |
| VIH | Voltage Input High — minimum voltage logic gate recognises as logic HIGH |
| VOH | Voltage Output High — output voltage when gate output is logic HIGH |
| WDT | Hardware Watchdog Timer — SAM3X8E independent timer; resets MCU on timeout |
| 2oo2 | Two-out-of-Two — voting architecture requiring simultaneous agreement of both channels |

# 2 Purpose and Scope

This Hardware Architecture and Design Specification (HADS) defines the complete hardware design of the Airbag ECU laboratory prototype. It covers system architecture, bill of materials, wiring connections, pin assignments, electrical connections, power architecture, and functional descriptions of all hardware components.

The document reflects the confirmed firmware architecture where both ECUs operate as peers — there is no master and no slave. Each ECU independently reads sensors, evaluates crash conditions, transmits its own status on its own CAN ID, receives the remote ECU status, and makes its own deployment vote. The hardware AND gate provides a final physical 2oo2 layer on top of the software vote.

Scope: sensor input circuits, processing boards, AND gate deployment logic, output indicator LEDs (all red), CAN bus communication hardware, power distribution, and grounding.

Out of scope: physical airbag inflator, airbag cushion (removed — replaced by USBCANV2).

# 3 System Architecture Overview

## 3.1 Peer-equal design principle

Both ECUs are identical in hardware and nearly identical in firmware. Neither ECU is a master or slave. The only differences between ECU 1 and ECU 2 are the three configuration defines at the top of the firmware:

| Define | ECU 1 (Due 1) | ECU 2 (Due 2) |
| --- | --- | --- |
| ECU_ID | 1 | 2 |
| DATA_ID | 0x100 | 0x200 |
| REMOTE_ID | 0x200 | 0x100 |

All hardware wiring, all sensor connections, all output connections, and all firmware logic are identical between the two boards.

## 3.2 Deployment decision architecture

The system implements two independent AND conditions that must both be satisfied before an airbag LED fires:

- Software AND (in firmware, identical on both ECUs): voteDeploy = localCrash AND remoteCrash AND NOT systemFault AND NOT remoteFault. Each ECU independently confirms a crash with its own sensors before asserting its output pins HIGH.

- Hardware AND (DM74LS08N): the physical AND gate ensures the airbag LED physically cannot illuminate unless BOTH ECUs have simultaneously asserted their output pin HIGH. This operates independently of all software.

This two-layer approach means a single software fault or single hardware fault in either channel cannot cause unintended deployment. This satisfies FSR-03 and FSR-08.

## 3.3 Node inventory

| Node | Component | CAN TX ID | CAN RX ID | Role |
| --- | --- | --- | --- | --- |
| ECU 1 | Arduino Due 1 | 0x100 | 0x200 | Peer — reads sensors, votes, transmits status |
| ECU 2 | Arduino Due 2 | 0x200 | 0x100 | Peer — reads sensors, votes, transmits status |
| OMS lab PC | USBCANV2 software | — | 0x100 and 0x200 | Passive monitor — logs both channels |
| CAN transceiver 1 | Waveshare SN65HVD230 | — | — | ECU 1 CAN0 to CANH/CANL |
| CAN transceiver 2 | Waveshare SN65HVD230 | — | — | ECU 2 CAN0 to CANH/CANL |
| CAN adapter | USB-CAN-A | — | — | CANH/CANL to USB for OMS PC |

# 4 Bill of Materials

| Item | Description | Qty | Specification | Safety role | FSR |
| --- | --- | --- | --- | --- | --- |
| Arduino Due | SAM3X8E ARM Cortex-M3 84 MHz microcontroller | 2 | 3.3 V logic, 12-bit ADC, CAN0 hardware | Processing channel — ASIL D | FSR-07 |
| Potentiometer 10 kΩ | Rotary potentiometer — sensor simulation | 3 | Linear taper, 10 kΩ, 3-terminal | Accel / gyro / pressure simulation | FSR-01 |
| Tactile push button | Momentary push button — baby seat switch | 1 | Active-LOW, SPST, normally open | Baby seat occupant detection — ASIL D | FSR-09 |
| DM74LS08N | Quad 2-input AND gate IC 14-pin DIP | 1 | 74LS TTL, VCC 5 V, VIH min 2.0 V | Hardware 2oo2 physical layer — ASIL D | FSR-03 |
| Waveshare SN65HVD230 | CAN transceiver board | 2 | 3.3 V supply, 500 kbit/s, CANH/CANL output | CAN bus physical layer — ASIL B | FSR-13 |
| USB-CAN-A | USB to CAN bus adapter | 1 | 500 kbit/s, USBCANV2 compatible | OMS PC CAN interface | FSR-16 |
| Resistor 10 kΩ | Pull-up for baby seat switch on D11 | 1 | 1/4 W — connect to 3.3 V (NOT 5 V) | Safe default HIGH state for D11 | FSR-10 |
| Resistor 220 Ω — airbag 1 | LED current limiter — airbag 1 | 1 | 1/4 W | Current limit for airbag 1 LED | — |
| Resistor 220 Ω — airbag 2 | LED current limiter — airbag 2 | 1 | 1/4 W | Current limit for airbag 2 LED | — |
| Resistor 220 Ω — warning | LED current limiter — warning lamp | 2 | 1/4 W (one per ECU D10 output) | Current limit for DWI LED | FSR-19 |
| LED red — airbag 1 | Airbag 1 deployment indicator | 1 | 3 mm RED LED, Vf ~2.0 V | Simulates driver airbag deployment | — |
| LED red — airbag 2 | Airbag 2 deployment indicator | 1 | 3 mm RED LED, Vf ~2.0 V | Simulates passenger airbag deployment | — |
| LED red — warning lamp | Diagnostic Warning Indicator (DWI) | 1 | 3 mm RED LED, Vf ~2.0 V — shared anode | Warning lamp from both ECU D10 pins | FSR-19 |
| Breadboard | Full-size 830 tie-point solderless breadboard | 1 | Standard 0.1 inch pitch | All passive and sensor wiring | — |
| Jumper wires M-M | Dupont jumper wire set | 1 set | Various lengths | All breadboard connections | — |
| Twisted pair wire | CANH/CANL bus wiring | 30 cm | Min 1 twist per cm | CAN bus noise immunity | FSR-13 |

# 5 Pin Assignments and Electrical Connections

Both ECUs use identical pin assignments. The table below applies to Arduino Due 1 (ECU 1) and Arduino Due 2 (ECU 2) without distinction.

## 5.1 Arduino Due — pin assignment (identical for both ECU 1 and ECU 2)

| Pin | Signal | Connected to | Dir | Notes |
| --- | --- | --- | --- | --- |
| A0 | Accelerometer POTI-1 | Potentiometer 1 wiper (leg 3) | In | 0–3.3 V analog, 12-bit ADC, shared with other ECU — high impedance safe |
| A1 | Gyroscope POTI-2 | Potentiometer 2 wiper (leg 3) | In | 0–3.3 V analog, 12-bit ADC, shared |
| A2 | Pressure POTI-3 | Potentiometer 3 wiper (leg 3) | In | 0–3.3 V analog, 12-bit ADC, shared |
| D8 | Airbag 1 AND gate input | DM74LS08N pin 1 (ECU1) or pin 2 (ECU2) | Out | HIGH = vote to deploy AB1; LOW = inhibit |
| D9 | Airbag 2 AND gate input | DM74LS08N pin 4 (ECU1) or pin 5 (ECU2) | Out | HIGH = vote to deploy AB2; LOW = inhibit |
| D10 | Warning lamp | 220 Ω → DWI LED positive (shared anode) | Out | HIGH = any fault. Both ECUs drive same LED — OR logic |
| D11 | Baby seat switch | Push button + 10 kΩ pull-up to 3.3 V | In | Active-LOW. LOW=baby seat. HIGH=adult/empty. Pull-up connected to 3.3 V — correct. |
| D13 | Status LED | Built-in LED on Arduino Due board | Out | Blinks 500 ms — indicates loop running |
| CAN TX | CAN transmit | Waveshare CANTX pin | Out | CAN0 controller — ECU1 ID 0x100, ECU2 ID 0x200 |
| CAN RX | CAN receive | Waveshare CANRX pin | In | ECU1 listens for 0x200, ECU2 listens for 0x100 |
| 3.3 V | 3.3 V supply | Waveshare VCC pin | Pwr | Powers SN65HVD230 — must be 3.3 V not 5 V |
| 5 V | 5 V supply | Common breadboard 5 V rail | Pwr | Feeds DM74LS08N VCC pin 14 |
| GND | Ground | Common GND rail, Waveshare GND | Pwr | Single common reference — mandatory |

## 5.2 DM74LS08N AND gate — pin assignment

| Pin | Name | Connected to | Function |
| --- | --- | --- | --- |
| 1 | 1A | ECU 1 (Due 1) D8 | Airbag 1 deploy vote from Channel A |
| 2 | 1B | ECU 2 (Due 2) D8 | Airbag 1 deploy vote from Channel B |
| 3 | 1Y | 220 Ω resistor → Airbag 1 LED positive (anode) | HIGH only when both D8 pins HIGH |
| 4 | 2A | ECU 1 (Due 1) D9 | Airbag 2 deploy vote from Channel A |
| 5 | 2B | ECU 2 (Due 2) D9 | Airbag 2 deploy vote from Channel B |
| 6 | 2Y | 220 Ω resistor → Airbag 2 LED positive (anode) | HIGH only when both D9 pins HIGH |
| 7 | GND | Common breadboard GND rail | IC ground reference |
| 8–13 | n/c | Unconnected | Gates 3 and 4 unused — leave floating |
| 14 | VCC | Common breadboard 5 V rail | IC power — 5 V |

## 5.3 Waveshare SN65HVD230 CAN transceiver (applies to both boards)

| Pin | Connected to | Notes |
| --- | --- | --- |
| CANTX | Arduino Due CAN TX pin | Logic-level TX input from CAN0 controller |
| CANRX | Arduino Due CAN RX pin | Logic-level RX output to CAN0 controller |
| 3.3 V | Arduino Due 3.3 V pin | VCC — MUST be 3.3 V. Do NOT connect to 5 V |
| GND | Common GND rail | Signal reference — common with Arduino Due GND |
| CANH | Common CANH bus rail | Differential CAN high — shared by both transceivers and USB-CAN-A |
| CANL | Common CANL bus rail | Differential CAN low — shared by both transceivers and USB-CAN-A |

## 5.4 USB-CAN-A adapter

| Pin | Connected to | Notes |
| --- | --- | --- |
| CANH | Common CANH bus rail | Connects to both Waveshare CANH pins |
| CANL | Common CANL bus rail | Connects to both Waveshare CANL pins |
| USB | OMS lab PC USB port | USBCANV2 software — set to 500 kbit/s |

# 6 Sensor Input Connections

## 6.1 Potentiometer wiring — confirmed from code

The code uses analogReadResolution(12) — all ADC readings are 12-bit (0 to 4095). Both ECUs read identical shared potentiometer signals. The conversion formulas confirmed in firmware are:

accel = (rawA0 / 4095.0) * 100.0   // 0–100 g,  threshold 30.0 g

gyro  = (rawA1 / 4095.0) * 360.0   // 0–360 deg/s, threshold 120.0 deg/s

press = (rawA2 / 4095.0) * 500.0   // 0–500 kPa, threshold 200.0 kPa

| Terminal | Connected to | Notes |
| --- | --- | --- |
| Leg 1 (end) | Common 3.3 V rail (RECOMMENDED — see HADS-OB-002) | Top of resistive track. Use 3.3 V to constrain wiper to safe ADC range |
| Leg 2 (end) | Common GND rail | Bottom of resistive track |
| Leg 3 (wiper) | ADC pin of BOTH Due 1 and Due 2 (shared wire) | Output voltage 0–3.3 V. Both ADC inputs are high-impedance — safe to share |

| Potentiometer | Due 1 pin | Due 2 pin | Physical range | Deploy threshold | ADC threshold (approx) |
| --- | --- | --- | --- | --- | --- |
| POTI-1 Accelerometer | A0 | A0 | 0–100 g | 30.0 g | raw > 1228 |
| POTI-2 Gyroscope | A1 | A1 | 0–360 deg/s | 120.0 deg/s | raw > 1365 |
| POTI-3 Pressure | A2 | A2 | 0–500 kPa | 200.0 kPa | raw > 1638 |

## 6.2 Baby seat switch — confirmed from code

The code reads: babySeat = (digitalRead(PIN_BABY) == LOW). Pin D11, configured as INPUT (external pull-up).

| Connection | Detail |
| --- | --- |
| Switch terminal 1 | Common GND rail — pressing button pulls D11 to 0 V |
| Switch terminal 2 | D11 of Due 1 AND D11 of Due 2 (shared) AND 10 kΩ pull-up |
| Pull-up resistor | 10 kΩ — one end to D11 signal line, other end to Arduino Due 3.3 V pin |
| Logic HIGH (open) | ~3.3 V via pull-up — no baby seat / adult occupant |
| Logic LOW (pressed) | ~0 V — baby seat present — passenger airbag suppressed |

# 7 Output Connections

All LEDs in this implementation are red (3 mm, Vf approximately 2.0 V). No blue or amber LEDs are used.

## 7.1 Airbag 1 LED — driver airbag indicator (RED)

The AND gate output (pin 3) drives the LED directly through a 220 Ω resistor.

| Connection | Detail |
| --- | --- |
| AND gate pin 3 (1Y) | → 220 Ω resistor → Airbag 1 LED positive (anode) |
| LED positive (anode) | → 220 Ω resistor → Airbag 1 LED positive (anode) |
| LED cathode | → Common GND rail |
| LED cathode | → Common GND rail |
| LED colour | RED — Vf ~2.0 V |
| LED current | 3 mm red LED, Vf ~2.0 V, direct drive from AND gate output |
| Active condition | AND gate pin 3 HIGH — requires ECU1 D8 HIGH AND ECU2 D8 HIGH AND voteDeploy TRUE on both |

## 7.2 Airbag 2 LED — passenger airbag indicator (RED)

Same circuit as Airbag 1. AND gate pin 6 drives the LED directly through a 220 Ω resistor.

| Connection | Detail |
| --- | --- |
| AND gate pin 6 (2Y) | → 220 Ω resistor → Airbag 2 LED positive (anode) |
| LED positive (anode) | → 220 Ω resistor → Airbag 2 LED positive (anode) |
| LED cathode | → Common GND rail |
| LED colour | RED — 3 mm red LED, same type as Airbag 1 |
| Active condition | AND gate pin 6 HIGH — requires ECU1 D9 HIGH AND ECU2 D9 HIGH. D9 is only HIGH when voteDeploy=TRUE AND babySeat=FALSE |

## 7.3 Warning lamp — DWI (RED LED, shared between both ECUs)

Both ECU D10 pins connect to the same LED through separate 220 Ω resistors. Either pin driving HIGH illuminates the lamp — OR gate behaviour implemented in hardware.

| Connection | Detail |
| --- | --- |
| ECU 1 D10 | → 220 Ω resistor → DWI LED positive (anode) |
| ECU 2 D10 | → 220 Ω resistor → same DWI LED positive (anode) |
| LED cathode | → Common GND rail |
| LED colour | RED — 3 mm red LED, same type as airbag LEDs |
| Active condition | Either ECU detects systemFault = TRUE (CPU / RAM / ADC / COMM fault) |
| FSR | FSR-19 — DWI must activate within 800 ms of fault detection |

# 8 CAN Bus Architecture

## 8.1 Physical bus configuration

| Parameter | Value |
| --- | --- |
| Bus topology | Single segment — three nodes on one twisted pair |
| Baud rate | 500 kbit/s — confirmed in firmware: Can0.begin(CAN_BPS_500K) |
| Termination | 120 Ω at each physical end of the bus — two resistors total |
| Cable type | Twisted pair — minimum 1 twist per cm |
| Bus nodes | Waveshare 1 (ECU 1), Waveshare 2 (ECU 2), USB-CAN-A (OMS PC) |
| Terminator position A | At the ECU 1 / Waveshare 1 end of the bus |
| Terminator position B | At the USB-CAN-A / OMS PC end of the bus |
| Waveshare 2 position | Middle node — no terminator on Waveshare 2 |

## 8.2 CAN ID assignment — confirmed from firmware

| Board | TX ID | RX filter | Frame rate | Receive filter code |
| --- | --- | --- | --- | --- |
| Arduino Due 1 (ECU 1) | 0x100 | Accept all (watchFor with no filter) | ~500 ms (heartbeat loop) | Can0.watchFor() — accepts all IDs. ECU filters by checking incoming.id == REMOTE_ID (0x200) in software |
| Arduino Due 2 (ECU 2) | 0x200 | Accept all (watchFor with no filter) | ~500 ms | Same logic — filters for incoming.id == 0x100 in receiveCAN() |
| OMS PC USBCANV2 | — | 0x100 and 0x200 | Passive | Configure USBCANV2 to display both IDs |

## 8.3 CAN frame byte layout — confirmed from sendData() function

| Byte | Field | Encoding | Code reference |
| --- | --- | --- | --- |
| 0 | A0 raw low byte | rawA0 & 0xFF | frame.data.bytes[0] = a0 & 0xFF |
| 1 | A0 raw high byte | rawA0 >> 8 | frame.data.bytes[1] = a0 >> 8 |
| 2 | A1 raw low byte | rawA1 & 0xFF | frame.data.bytes[2] = a1 & 0xFF |
| 3 | A1 raw high byte | rawA1 >> 8 | frame.data.bytes[3] = a1 >> 8 |
| 4 | A2 raw low byte | rawA2 & 0xFF | frame.data.bytes[4] = a2 & 0xFF |
| 5 | A2 raw high byte | rawA2 >> 8 | frame.data.bytes[5] = a2 >> 8 |
| 6 | Status byte | Bit0=crash, Bit1=babySeat, Bit2=fault | frame.data.bytes[6] = status (bit flags) |
| 7 | ECU ID | 1 or 2 | frame.data.bytes[7] = ECU_ID |

Note: This frame format transmits raw ADC values (12-bit, split across two bytes). The OMS PC (USBCANV2) must reconstruct the 16-bit value as: raw = (byte1 << 8) | byte0, then convert to physical units using the same formula as the firmware.

## 8.4 Status byte bit fields (byte 6)

| Bit | Field | Value | Meaning |
| --- | --- | --- | --- |
| Bit 0 | localCrash | 1 = crash detected | At least one sensor exceeded threshold on this ECU |
| Bit 1 | babySeat | 1 = baby seat present | D11 LOW — passenger airbag suppressed on this ECU |
| Bit 2 | systemFault | 1 = fault active | CPU, RAM, ADC, or COMM fault on this ECU |
| Bit 3–7 | Reserved | 0 | Unused in current firmware |

# 9 Deployment Logic — Confirmed from Firmware

## 9.1 Software vote — identical on both ECUs

The vote decision confirmed from firmware (same logic on both ECUs):

localCrash  = (accel > 30.0) || (gyro > 120.0) || (press > 200.0)

systemFault = faultCPU || faultRAM || faultADC || faultCOMM

voteDeploy  = localCrash && remoteCrash && !systemFault && !remoteFault

D8 (Airbag 1 AND gate input) = voteDeploy

D9 (Airbag 2 AND gate input) = voteDeploy && !babySeat

D10 (Warning lamp)           = systemFault

remoteCrash and remoteFault are extracted from the received CAN frame byte 6 of the other ECU:

remoteCrash = (status & 0x01)   // bit 0 of received byte 6

remoteFault  = (status & 0x04)  // bit 2 of received byte 6

## 9.2 Communication timeout fault

The firmware monitors the remote CAN frame reception timestamp:

if (millis() - lastRemoteHB > 2000) faultCOMM = true

If no frame is received from the remote ECU for 2000 ms, faultCOMM becomes TRUE. systemFault becomes TRUE. voteDeploy becomes FALSE. D8 and D9 are driven LOW. The AND gate output drops LOW. No airbag LED fires. This is the safe state on communication loss.

## 9.3 Hardware AND gate — final physical layer

On top of the software vote, the DM74LS08N AND gate provides a physical 2oo2 layer:

| ECU1 D8 | ECU2 D8 | AND gate pin 3 output | Airbag 1 LED |
| --- | --- | --- | --- |
| LOW | LOW | LOW | OFF — no crash on either |
| HIGH | LOW | LOW | OFF — ECU2 not confirming |
| LOW | HIGH | LOW | OFF — ECU1 not confirming |
| HIGH | HIGH | HIGH | FIRE — both ECUs confirm crash |

# 10 Power Architecture

## 10.1 Power rails

| Rail | Source | Voltage | Consumers | Notes |
| --- | --- | --- | --- | --- |
| 5 V | Arduino Due 5 V pin (USB powered) | 5 V | DM74LS08N VCC (pin 14) only | Shared rail on breadboard. Baby seat pull-up uses 3.3 V pin, not this rail. |
| 3.3 V — ECU 1 | Arduino Due 1 3.3 V pin | 3.3 V | Waveshare 1 VCC, 10 kΩ baby seat pull-up | Powers CAN transceiver 1 and baby seat 10 kΩ pull-up resistor. |
| 3.3 V — ECU 2 | Arduino Due 2 3.3 V pin | 3.3 V | Waveshare 2 VCC | Powers CAN transceiver 2 |
| GND | Common breadboard GND rail | 0 V | All components | Single ground reference — mandatory for all signal integrity |

## 10.2 Voltage domain boundaries

| Interface | Concern | Status | Action required |
| --- | --- | --- | --- |
| Potentiometer leg 1 to ADC | If leg 1 = 5 V, wiper can reach 5 V — over-voltages 3.3 V ADC | OPEN — HADS-OB-002 | Move leg 1 from 5 V to 3.3 V rail |
| Baby seat pull-up to D11 | Pull-up connected to 3.3 V — D11 reads 3.3 V when open. No over-voltage. | RESOLVED — HADS-CR-001 closed | Hardware updated — pull-up now on 3.3 V. No further action required. |
| Due D8/D9 (3.3 V) to DM74LS08N input | 74LS VIH min = 2.0 V — 3.3 V is sufficient | OK — no action | Verified safe |
| DM74LS08N output to LED (direct drive) | 74LS VOH ~2.7 V — 3 mm red LED with Vf ~2.0 V, direct drive confirmed working | OK — confirmed | Direct drive — 3 mm LED confirmed in use |
| Waveshare VCC | SN65HVD230 requires 3.3 V — 5 V will destroy IC | Must verify wiring | Confirm 3.3 V connected, not 5 V |

# 11 Functional Safety Engineer Review

HADS-CR-001 — CLOSED: Pull-up moved to 3.3 V — resolved

Previous issue: 10 kΩ baby seat pull-up was connected to 5 V rail. D11 was reading 5 V when button open.

Risk was: Arduino Due GPIO maximum input is 3.3 V. A 5 V signal on D11 exceeded the absolute maximum rating of the SAM3X8E.

Resolution applied: The free end of the 10 kΩ pull-up resistor has been moved from the 5 V breadboard rail to the Arduino Due 3.3 V pin. D11 now reads 3.3 V when button open (valid) and 0 V when pressed (valid). No firmware change required.

FSR: FSR-09, FSR-10, FSR-11 — ASIL D | Status: CLOSED — pull-up moved to 3.3 V

HADS-OK-002 — Confirmed correct: direct LED drive working (3 mm red LEDs)

Implementation: DM74LS08N output pins 3 and 6 connected directly to 3 mm red airbag LEDs through 220 Ω resistors.

Status: Direct drive confirmed working in the laboratory. 3 mm red LEDs operate correctly with this circuit. No issues observed.

Implementation: AND gate output → 220 Ω → 3 mm red LED anode → LED cathode → GND. Confirmed working in the laboratory.

FSR: FSR-03, FSR-08, FSR-20 | Status: OPEN

HADS-OB-001 — WARNING: Waveshare board onboard termination — verify

Some Waveshare SN65HVD230 boards have an onboard 120 Ω termination resistor. If enabled on both boards, the CAN bus will have too many terminators causing communication errors.

Action: Inspect each Waveshare board for a termination jumper or onboard resistor near CANH/CANL. The bus needs exactly two 120 Ω terminators — one at each physical end. Disable onboard termination if present.  | Status: OPEN — verify before CAN commissioning

HADS-OB-002 — ADVISORY: Potentiometer leg 1 connected to 5 V

Potentiometer leg 1 connected to 5 V rail. At full clockwise rotation wiper approaches 5 V, over-voltaging the 3.3 V ADC input.

Recommended fix: move potentiometer leg 1 from 5 V to 3.3 V rail. Wiper then ranges 0–3.3 V — exactly the ADC input range. Full 0–4095 range preserved.  | Status: ADVISORY — recommend fixing

Code observation HADS-CODE-OB-001 — NOTE: watchdog not implemented in current firmware

The current firmware does not include the hardware watchdog (WDT) implementation from the earlier safety concept. FSR-22 requires hardware watchdog supervision. The sam.h watchdog_setup() and watchdog_feed() functions should be added to the firmware.

Status: ADVISORY — add before final V&V

Confirmed correct — Peer-equal architecture

Both ECUs are independent peers. No master/slave dependency. Each ECU independently reads sensors, votes, and transmits. If one ECU fails, its output goes LOW, the AND gate output goes LOW, no deployment occurs. Communication timeout correctly handled by faultCOMM. Architecture is correct for ASIL D.

Confirmed correct — CAN frame reception

receiveCAN() is called at the top of every loop iteration. Can0.watchFor() accepts all IDs. Software filter (incoming.id == REMOTE_ID) correctly isolates peer frames. lastRemoteHB is updated on every valid peer frame. faultCOMM activates after 2000 ms silence. Correct.

Confirmed correct — All LEDs red

All three LEDs (airbag 1, airbag 2, warning lamp) are 3 mm red LEDs with Vf approximately 2.0 V. No blue or amber LEDs.

# 12 Hardware Items to Replace or Modify

| Item | Current | Change to | Priority | Reason |
| --- | --- | --- | --- | --- |
| Baby seat pull-up — supply connection | 5 V rail | 3.3 V (Arduino Due 3.3 V pin) | CONFIRMED — no change | HADS-CR-001 closed — MCU input now within 3.3 V safe range |
| Potentiometer leg 1 — supply connection | 5 V rail | 3.3 V rail | RECOMMENDED | Prevents ADC over-voltage at full rotation (HADS-OB-002) |
| Airbag LED 1 circuit | AND gate pin 3 → 220 Ω → 3 mm red LED | No change — confirmed correct | CONFIRMED — no change | Confirmed working — 3 mm red LEDs operate correctly with direct drive |
| Airbag LED 2 circuit | AND gate pin 6 → 220 Ω → 3 mm red LED | No change — confirmed correct | CONFIRMED — no change | Same as above |
| Airbag LEDs colour | Previously blue/red/amber mixed | All 3 mm red LEDs | DONE — confirmed | All three LEDs are red |
| Waveshare onboard termination | Unknown state | Verify and disable if present | IMPORTANT | Excess termination causes CAN errors (HADS-OB-001) |
| Hardware watchdog in firmware | Not implemented | Add sam.h WDT setup and feed calls | RECOMMENDED | Required by FSR-22 for ASIL D compliance (HADS-CODE-OB-001) |

# 13 Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Software Safety Engineer | Mariana Chavez Avila |  |  |
| Hardware Safety Engineer | Om Naik |  |  |
| Functional Safety Engineer | Kruthika S. Gowda |  |  |
| V&V Lead | Shivbaraje Bhosale |  |  |
| Project Supervisor |  |  |  |

# 14 Revision History

| Rev | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 2026-06-11 | Hardware Safety Engineering Team | Initial release. Full HADS covering confirmed breadboard wiring, BOM, pin assignments, power architecture, CAN bus, safety review with two critical issues. |
| 2.0 | 2026-06-14 | Hardware Safety Engineering Team | Updated to peer-equal architecture (no master/slave). Confirmed firmware reviewed — voteDeploy logic, CAN frame format, byte layout, and timeout handling documented. All LEDs confirmed as 3 mm red. Direct LED drive from AND gate outputs confirmed correct. Watchdog observation added. HADS-CR-001 closed — baby seat pull-up moved from 5 V to 3.3 V. No critical issues remain open. |