Airbag ECU Portfolio HS Harz — Functional Safety Engineering AIRBAG ELECTRONIC CONTROL UNIT Measurement and Processing of Sensor Signals to Control Airbag Operation

ASIL D / SIL 3

| Field | Value |
| --- | --- |
| Document Number | SADS-AIRBAG-ECU-001 |
| Revision | 3.0 |
| Date | 2026-06-14 |
| Status | RELEASED |
| ASIL Classification | ASIL D (ISO 26262) / SIL 3 (IEC 61508) |
| Target Hardware | Arduino Due (SAM3X8E ARM Cortex-M3, 84 MHz) ×2 |
| Author | Mariana Chavez Avila |
| Based On | HADS-AIRBAG-ECU-001 Rev 2.0 ¦ LCP04-AIRBAG-ECU-001_FSRS_v100 ¦ SRS-AIRBAG-SW-001 Rev 3.1 ¦ LCP05-AIRBAG-ECU-001_SCD_v100 ¦ LCP03-AIRBAG-ECU-001_HARA_v110 ¦ VVP-AIRBAG-ECU-001_Rev1.2 |
| Firmware references | FINAL-CODE-AIRBAG-ECU-001 |

Table of Contents

1	Glossary	3

2	Overview	3

3	Tools and Programming Languages	4

3.1	Programming Language	4

3.2	Development Tools	4

4	Software Architecture Overview	5

4.1	Module Decomposition	5

4.2	Data Flow Between Modules	5

4.3	Execution Control Flow	5

5	System State Machine	6

5.1	State Definitions	6

5.2	Safe State Guarantees	6

6	Detailed Design of Software Modules	7

6.1	MOD-01 — Power-On Self-Test (POST)	7

6.1.1	CPU Test	7

6.1.2	RAM Test	7

6.1.3	ADC Test	7

6.1.4	POST Completion Criteria	7

6.2	MOD-02 — Sensor Acquisition	7

6.3	MOD-03 — Crash Detection	8

6.3.1	Scaling Formulas	8

6.3.2	Threshold Comparison	8

6.3.3	Threshold Rationale	8

6.4	MOD-04 — Deployment Control	8

6.4.1	Deployment Vote Logic	9

6.4.2	Event-triggered CAN TX (Rev 3.0 addition)	9

6.4.3	Deployment Authorization Truth Table	9

6.5	MOD-05 — CAN Communication	9

6.5.1	CAN Frame Layout (DLC=8 - Classical CAN Maximum)	9

6.5.2	Receive and Timeout	10

6.6	MOD-06 — Diagnostic Monitor	10

7	Occupant Classification and Baby Seat Logic	10

7.1	Signal Interpretation	10

7.2	Debounce Implementation (GAP-01 CLOSED Rev 3.0)	11

7.3	Pull-up Configuration (HADS-CR-001 RESOLVED)	11

8	Safety Mechanisms in Software	11

8.1	Hardware Watchdog Timer (WDT) — Pending	11

8.2	2oo2 Redundant Decision Architecture	11

8.3	Fail-Safe Direction Analysis	11

8.4	Permitted and Forbidden Software Operations	12

9	HW/SW Integration Procedure	12

9.1	Integration Steps	12

10	Compliance Gaps and Required Improvements	13

11	Verification Methods Overview	14

11.1	Applicable Verification Methods	14

11.2	Structural Coverage Target	15

12	Requirement Traceability Matrix	15

13	Approval and Sign-Off	16

14	Revision History	16

# Glossary

| Abbreviation | Definition |
| --- | --- |
| 2oo2 | Two-out-of-Two voting. Both ECUs must agree to trigger airbag deployment. |
| ADC | Analog-to-Digital Converter. Converts sensor voltages to 12-bit values (0–4095). |
| ASIL | Automotive Safety Integrity Level. Risk classification per ISO 26262 (QM, A, B, C, D). |
| ASIL D | Highest safety integrity level. Applied to the airbag deployment logic. |
| CAN | Controller Area Network. Communication bus between the two ECUs at 500 kbit/s. |
| CRC | Cyclic Redundancy Check. Error-detection code for CAN frames. Hardware CRC automatically generated and verified by SAM3X8E CAN controller. |
| DWI | Dash Warning Indicator. Fault warning lamp on pin D10. Must activate within ≤800 ms. |
| ECU | Electronic Control Unit. Each Arduino Due board acting as a redundant controller. |
| FSRS | Functional Safety Requirement. Traceable requirement from LCP04-AIRBAG-ECU-001_FSRS_v100. |
| GAP | Identified non-conformance between current firmware and an FSR requirement. |
| HADS | Hardware Architecture and Design Specification. Defines wiring, BOM, and pin assignments. |
| HARA | Hazard Analysis and Risk Assessment. Identifies hazards and derives ASIL classifications. |
| ITP | Implementation Test Plan. Defines unit tests, integration tests, and the code review checklist. |
| OCS | Occupant Classification System. Detects child seat presence via the baby seat switch (D11). |
| POST | Power-On Self-Test. Startup routine testing CPU, RAM, and ADC before the main loop. |
| SADS | Software Architecture and Design Specification. This document portfolio’s SW design reference. |
| SIL | Safety Integrity Level. Risk classification per IEC 61508 (SIL 1–4). ECU is SIL 3. |
| SPFM | Single-Point Fault Metric. ASIL D requires ≥99% of single-point faults to be covered. |
| TR | Test Report. Records test execution results, coverage evidence, and sign-off. |
| V&V | Verification and Validation. Confirms software is built correctly and meets safety goals. |
| WDT | Watchdog Timer. Resets the MCU if the main loop stalls. Required by FSR-22. |
| HW CRC | Hardware CRC by SAM3X8E CAN controller per ISO 11898 – 15-bit automatic CRC |
| voteDeploy | Deployment authorization computed in MOD-04 |

# Overview

This Software Architecture and Design Specification (SADS) defines the internal software structure, module organisation, data flow, control flow, safety mechanisms, state machines, and implementation logic for the Airbag Electronic Control Unit (ECU) firmware running on two Arduino Due (SAM3X8E) microcontrollers in a redundant 2oo2 safety architecture.

The document covers the following architecture and design activities required by ISO 26262 for software at ASIL D:

- Software architecture design and tool/language selection

- Detailed module design and data structures

- HW/SW integration procedure

- Verification methods overview and requirement traceability

- Modification and change management procedure

Unit testing, integration testing, validation test cases, and test evidence are documented separately in the Implementation Test Plan (ITP-AIRBAG-ECU-001) and Test Report (TR-AIRBAG-ECU-001). The safety assessment is documented in the Functional Safety Assessment (FSA-AIRBAG-ECU-001).

This specification must be read in conjunction with LCP04-AIRBAG-ECU-001_FSRS_v100, SRS-AIRBAG-SW-001 Rev 3.1 and HADS-AIRBAG-ECU-001 Rev 2.0.

# Tools and Programming Languages

Per ISO 26262-6:2018 Clause 5 and IEC 61508-3:2010 Clause 7.4, all tools and languages used in the development of safety-related software shall be qualified or otherwise justified. This section documents the selection rationale, qualification status, and constraints for each tool and language used.

## Programming Language

| Attribute | Detail | Safety Justification |
| --- | --- | --- |
| Language | C/C++ (Arduino framework, Arduino IDE 2.3.6) | ISO 26262-6 Annex C supported language for embedded safety systems |
| Standard | C++11 (GCC 4.8.3 ARM bundled with Arduino Due board package) | Dynamic memory and recursion forbidden per ASIL D constraints |
| Forbidden constructs | malloc, new, delete, recursion, unbounded loops, delay() in loop() | Non-deterministic timing prohibited at ASIL D |

## Development Tools

| Tool | Version | Role | Qualification / Justification |
| --- | --- | --- | --- |
| Arduino IDE | 2.3.6 | Code editor, compiler, upload tool | Tool output (compiled binary) verified by functional testing. No claim placed on tool correctness beyond standard compiler qualification. |
| GCC ARM (avr-g++) | 4.8.3 (bundled) | Cross-compiler for SAM3X8E (ARM CortexM3) | Widely used in safety projects. Code output validated via module testing and integration tests documented in ITPAIRBAG-ECU-001. |
| due_can library | v1.0 (open source) | Arduino Due CAN0 hardware abstraction | Limited to CAN frame send/receive. Qualified by code review + communication tests documented in ITPAIRBAG-ECU-001. |
| USBCANV2 (OMS PC) | Latest stable | Passive CAN monitoring on OMS Lab PC | Used for test evidence collection only. Not part of deployed firmware. No safety claim placed on monitor tool. |
| Serial Monitor (Arduino IDE) | Bundled | Debug/diagnostic output at 115200 baud | Used for test observation only. No safety claim. Serial.print() calls are noncritical diagnostics. |

# Software Architecture Overview

The architecture is divided into six functional modules that map directly onto FSR requirements.

## Module Decomposition

| Module ID | Module Name | Primary FSR(s) | ASIL | Status Rev 3.0 |
| --- | --- | --- | --- | --- |
| MOD-01 | Power-On Self-Test (POST) | FSR-17 | ASIL C | Implemented — CPU/RAM/ADC tests |
| MOD-02 | Sensor Acquisition | FSR-01, FSR-02 | ASIL D | Implemented — 12-bit ADC, debounce added (GAP-01 CLOSED) |
| MOD-03 | Crash Detection | FSR-01, FSR-02, FSR-03, FSR-04 | ASIL D | Implemented — OR fusion of three sensors |
| MOD-04 | Deployment Control | FSR-03, FSR-08, FSR-09, FSR-10 | ASIL D | Implemented — voteDeploy + eventtriggered TX |
| MOD-05 | CAN Communication | FSR-13, FSR-14, FSR-15, FSR-16 | ASIL B | Implemented — HW CRC (GAP-03 CLOSED), event TX added |
| MOD-06 | Diagnostic Monitor | FSR-17, FSR-18, FSR-19, FSR-20, FSR-22 | ASIL C/D | Implemented — CommFault, DWI, systemFault aggregation |

## Data Flow Between Modules

- MOD-02 reads rawA0, rawA1, rawA2 from 12-bit ADC and digital pin D11. Stores in global variables. Applies 10ms debounce to D11 (updateBabySeat()).

- MOD-03 reads rawA0–rawA2, scales to engineering units via toPhys(), applies threshold comparisons, writes localCrash boolean and float values accel, gyro, press.

- MOD-05 transmits local ECU status frame every 500ms and immediately on voteDeploy rising edge. Receives peer frame, writes remoteCrash, remoteFault, updates lastRemoteHB.

- MOD-06 evaluates fault flags (faultCPU, faultRAM, faultADC, faultCOMM), computes aggregated systemFault boolean.

- MOD-04 reads localCrash, remoteCrash, systemFault, remoteFault, babySeat. Computes voteDeploy. Drives D8, D9, D10.0

## Execution Control Flow

| Step | Action | Period / Trigger | FSR |
| --- | --- | --- | --- |
| 1 | receiveCAN() — drain RX buffer, verify peer frame ID, update remoteCrash/remoteFault/lastRemoteHB | Every loop (nonblocking) | FSR-15 |
| 2 | CAN timeout check — if millis()lastRemoteHB > 2000ms → faultCOMM=TRUE | Every loop | FSR-15 |
| 3 | updateBabySeat() — read D11, apply 10ms debounce filter | Every loop | FSR-11 |
| 4 | analogRead A0, A1, A2 — raw 12-bit ADC values | Every loop | FSR-01, FSR-02 |
| 5 | toPhys() — scale rawA0/A1/A2 to g, deg/s, kPa | Every loop | FSR-01 |
| 6 | Threshold comparison — accelEx, gyroEx, pressEx → localCrash | Every loop | FSR-01, FSR-04 |
| 7 | systemFault aggregation — OR of faultCPU, faultRAM, faultADC, faultCOMM | Every loop | FSR-20 |
| 8 | voteDeploy logic — drive D8, D9, D10 | Every loop | FSR-03, FSR-08 |
| 9 | Event-triggered CAN TX — if voteDeploy rising edge → canSend() immediately | On rising edge | FSR-16 |
| 10 | Periodic CAN TX — canSend() if 500ms elapsed | Every 500ms | FSR-16 |
| 11 | Status LED blink — toggle D13 if 500ms elapsed | Every 500ms | — |
| 12 | Serial diagnostic print — if 1000ms elapsed | Every 1000ms | FSR-21 |

# System State Machine

## State Definitions

| State ID | State Name | Conditions to Enter | Outputs | FSRS Ref |
| --- | --- | --- | --- | --- |
| ST-01 | POWER_ON | Reset/power-up | All outputs LOW, POST executing | SS-01 |
| ST-02 | POST_FAILED | Any POST returns FALSE | D10 HIGH, D8/D9 LOW — permanent safe inhibit | SS-05 |
| ST-03 | MONITORING | POST passed, no crash, no fault, peer CAN active | D10 LOW, D8/D9 LOW | SS-02 |
| ST-04 | FAULT_ACTIVE | systemFault=TRUE | D10 HIGH (DWI), D8/D9 LOW — deployment inhibit | SS-01, SS-04 |
| ST-05 | CRASH_DETECTED | localCrash AND remoteCrash AND !systemFault AND !remoteFault | D8 HIGH, D9 HIGH if !babySeat | SS-02 |

State transitions described in the graphical diagram produced in SADS Appendix (FLOW-CHART-AIRBAG-ECU-001).

## Safe State Guarantees

- ST-02 (POST_FAILED): setup() never enters loop(). D8/D9 stay LOW. AND gate output permanently LOW. Safe inhibit SS-05.

- ST-04 (FAULT_ACTIVE): systemFault=TRUE forces voteDeploy=FALSE every cycle. D8/D9 LOW. D10 HIGH within one loop cycle (~1ms). FSR-19 (800ms) satisfied by large margin.

- CAN peer loss (faultCOMM=TRUE): After 2000ms without valid peer frame, faultCOMM propagates to systemFault, inhibiting deployment. Fail-safe direction: inhibit, not spurious deploy.

- Baby seat (babySeat=TRUE): D9 forced LOW unconditionally, even during valid crash. Passenger airbag suppressed. DWI stays OFF — active suppression is normal operation, not a fault.

# Detailed Design of Software Modules

Each module is described with its interface, internal logic, data structures, and pseudo-code. Executed once in setup() before loop(). Results stored in sticky boolean fault flags. Any failed test transitions system to ST-02.

## MOD-01 — Power-On Self-Test (POST)

Executed once in setup() before loop(). Results stored in sticky boolean fault flags. Any failed test transitions system to ST-02.

### CPU Test

bool cpuTest() { volatile uint32_t a = 12345; volatile uint32_t b = 6789; volatile uint32_t c = a * b; return (c == 83810205UL); // volatile prevents compiler optimisation }

Note: Rev 3.0 strengthened from addition test (a+b==300) to multiplication (12345*6789=83810205). More thorough ALU verification.

### RAM Test

bool ramTest() {

static volatile uint32_t mem[128];     for (i=0;i<128;i++) mem[i]=0xAAAAAAAA;     for (i=0;i<128;i++) { if(mem[i]!=0xAAAAAAAA) return false; }     for (i=0;i<128;i++) mem[i]=0x55555555;     for (i=0;i<128;i++) { if(mem[i]!=0x55555555) return false; }     return true;

}

Rev 3.0: expanded from single-word to 128-word checkerboard sweep. Write 0xAAAAAAAA (alternating bits), verify, then write 0x55555555 (complement), verify. Satisfies FSR-22 RAM integrity at ASIL D.

### ADC Test

bool adcTest() {     int v = analogRead(A0);     return (v >= 0 && v <= 4095);

}

### POST Completion Criteria

| Test | Pass Condition | Fail Action | Flag |
| --- | --- | --- | --- |
| CPU arithmetic | 12345 * 6789 == 83810205 | faultCPU=TRUE → systemFault → DWI ON, deploy inhibited | faultCPU |
| RAM checkerboard | 128-word 0xAAAAAAAA / 0x55555555 verified | faultRAM=TRUE | faultRAM |
| ADC range | 0 <= v <= 4095 | faultADC=TRUE | faultADC |
| CAN init | Can0.begin() success | Not currently checked — GAP-05 open | — |

## MOD-02 — Sensor Acquisition

Reads all sensor inputs every loop iteration. Arduino Due analogReadResolution(12) set in setup().

uint16_t rawA0 = analogRead(A0); // Accelerometer POTI-1: 0-4095 uint16_t rawA1 = analogRead(A1); // Gyroscope    POTI-2: 0-4095 uint16_t rawA2 = analogRead(A2); // Pressure     POTI-3: 0-4095 updateBabySeat();               // D11 with 10ms debounce (GAP-01 CLOSED)

| Sensor | Pin | ADC Range | Scaled Range | Deploy Threshold | ADC Threshold |
| --- | --- | --- | --- | --- | --- |
| Accelerometer POTI-1 | A0 | 0–4095 | 0–100 g | >30.0 g | raw > 1228 |
| Gyroscope POTI2 | A1 | 0–4095 | 0–360 deg/s | >120.0 deg/s | raw > 1365 |
| Pressure POTI-3 | A2 | 0–4095 | 0–500 kPa | >200.0 kPa | raw > 1638 |
| Baby Seat Switch | D11 | HIGH/LOW | TRUE/FALSE | LOW = baby seat | — |

## MOD-03 — Crash Detection

Converts raw ADC values to engineering units and applies threshold comparisons. An OR relationship between three sensors is used: any single sensor exceeding its threshold asserts localCrash = TRUE.

### Scaling Formulas

float accel = (rawA0 / 4095.0) * 100.0;  // 0-100 g

float gyro  = (rawA1 / 4095.0) * 360.0;  // 0-360 deg/s float press = (rawA2 / 4095.0) * 500.0;  // 0-500 kPa

### Threshold Comparison

bool localCrash = (accel > 30.0)   // > 30.0 g                || (gyro  > 120.0)  // > 120.0 deg/s

|| (press > 200.0); // > 200.0 kPa

### Threshold Rationale

| Sensor | Threshold | Physical Meaning | Rationale |
| --- | --- | --- | --- |
| Accelerometer | 30.0 g | Severe frontal impact pulse | Below hard braking (~1g), road bump (<5g); above required airbag crash pulse |
| Gyroscope | 120.0 deg/s | Rollover onset angular rate | Above cornering (<30 deg/s); below confirmed rollover (>200 deg/s) — lab sensitivity |
| Pressure | 200.0 kPa | ~2 bar door panel deformation | Representative side-impact crush pressure exceeding ambient variation |

## MOD-04 — Deployment Control

Computes the deployment vote and drives all output pins. This is the safety-critical decision point combining all module inputs.

### Deployment Vote Logic

// Step 1: aggregate system fault systemFault = faultCPU ¦¦ faultRAM ¦¦ faultADC ¦¦ faultCOMM; // Step 2: compute deployment vote bool voteDeploy = localCrash // this ECU detects crash && remoteCrash // peer ECU independently detects crash && !systemFault // no fault on this ECU && !remoteFault; // no fault on peer ECU // Step 3: drive output pins digitalWrite(PIN_AIRBAG1, voteDeploy); // airbag 1 AND gate input digitalWrite(PIN_AIRBAG2, voteDeploy && !babySeat); // airbag 2 — suppressed if baby seat digitalWrite(PIN_WARNING, systemFault ¦¦ remoteFault); // DWI on any fault

### Event-triggered CAN TX (Rev 3.0 addition)

bool deployRisingEdge = (voteDeploy && !lastVoteDeploy); lastVoteDeploy = voteDeploy;

if (deployRisingEdge || (millis() - lastTX > HEARTBEAT_MS)) {     canSend(rawA0, rawA1, rawA2, localCrash, babySeat, systemFault);     lastTX = millis();

}

Purpose: on first voteDeploy=TRUE, transmit CAN frame immediately rather than waiting up to 500ms for next heartbeat. Improves FSR-16 worst-case notification latency from 500ms to approximately 1 loop cycle (~1ms).

### Deployment Authorization Truth Table

| localCrash | remoteCrash | systemFault | remoteFault | babySeat | Airbag1 (D8) | Airbag2 (D9) |
| --- | --- | --- | --- | --- | --- | --- |
| FALSE | any | any | any | any | LOW | LOW |
| TRUE | FALSE | any | any | any | LOW | LOW |
| TRUE | TRUE | TRUE | any | any | LOW | LOW |
| TRUE | TRUE | FALSE | TRUE | any | LOW | LOW |
| TRUE | TRUE | FALSE | FALSE | FALSE | HIGH | HIGH |
| TRUE | TRUE | FALSE | FALSE | TRUE | HIGH | LOW (suppressed) |

## MOD-05 — CAN Communication

### CAN Frame Layout (DLC=8 - Classical CAN Maximum)

| Byte | Field | Encoding | Notes |
| --- | --- | --- | --- |
| 0 | A0 low byte | rawA0 & 0xFF | Accelerometer ADC low byte |
| 1 | A0 high byte | rawA0 >> 8 | Accelerometer ADC high byte |
| 2 | A1 low byte | rawA1 & 0xFF | Gyroscope ADC low byte |
| 3 | A1 high byte | rawA1 >> 8 | Gyroscope ADC high byte |
| 4 | A2 low byte | rawA2 & 0xFF | Pressure ADC low byte |
| 5 | A2 high byte | rawA2 >> 8 | Pressure ADC high byte |
| 6 | Status byte | Bit0=crash, Bit1=baby, Bit2=fault | Peer reads bits to extract remoteCrash and remoteFault |
| 7 | ECU ID | 1 or 2 | Identifies transmitting ECU |

CRC integrity: The SAM3X8E CAN hardware controller automatically appends a 15-bit CRC to every transmitted frame and verifies it on every received frame per ISO 11898. No software CRC is required or implemented. GAP-03 CLOSED.

### Receive and Timeout

void receiveCAN() { while (Can0.rx_avail()) { Can0.read(incoming); if (incoming.id != REMOTE_ID) continue; // filter peer ECU only lastRemoteHB = millis(); remoteCrash = (incoming.data.bytes[6] & 0x01); remoteFault = (incoming.data.bytes[6] & 0x04); } } // Timeout: COMM_TIMEOUT = 2000ms (FSR-15 deviation — see GAP-08) if (millis() - lastRemoteHB > COMM_TIMEOUT) faultCOMM = true; else faultCOMM = false;

## MOD-06 — Diagnostic Monitor

Runs every cycle. Aggregates hardware fault flags into systemFault. POST fault flags (faultCPU, faultRAM, faultADC) are sticky for the session; faultCOMM is evaluated dynamically.

| Flag | Set When | Cleared When | Effect on System |
| --- | --- | --- | --- |
| faultCPU | cpuTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultRAM | ramTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultADC | adcTest() returns FALSE at POST | Never — sticky | systemFault=TRUE, DWI ON, deployment inhibited |
| faultCOMM | millis() − lastRemoteHB > 2000 ms | Peer CAN frame received within 2000 ms | Dynamic: ST-04 while active, recovers to ST-03 when peer resumes |

# Occupant Classification and Baby Seat Logic

## Signal Interpretation

| D11 State | babySeat Value | Airbag 2 (D9) | Meaning |
| --- | --- | --- | --- |
| HIGH (~3.3 V via pull-up) | FALSE | Follows voteDeploy | No child seat — adult or empty — passenger airbag armed |
| LOW (switch pressed) | TRUE | Forced LOW | Child seat present — passenger airbag suppressed |

Important: Active baby seat suppression (babySeat=TRUE) is normal fail-safe operation, not a fault condition.

D10 (DWI) does NOT illuminate when baby seat suppression is active. DWI activates only on systemFault=TRUE.

## Debounce Implementation (GAP-01 CLOSED Rev 3.0)

FSR-11 requires 10ms stable before state transition accepted. Fully implemented in Rev 3.2 firmware:

void updateBabySeat() { int current = digitalRead(PIN_BABY); if (current != lastRawBaby) { lastDebounce = millis(); lastRawBaby = current; } if ((millis() - lastDebounce) >= DEBOUNCE_MS) stableBaby = current; // only accepted after 10ms stable }

## Pull-up Configuration (HADS-CR-001 RESOLVED)

Baby seat switch pull-up resistor (10kΩ) connected to Arduino Due 3.3V pin (not 5V). Previous connection to 5V rail would have over-voltaged the 3.3V SAM3X8E GPIO input. This was identified as HADS-CR-001 and resolved in hardware. D11 now reads 3.3V when button open (valid) and 0V when pressed (valid).

# Safety Mechanisms in Software

## Hardware Watchdog Timer (WDT) — Pending

FSR-22 requires hardware WDT. The SAM3X8E has an independent hardware WDT. Status: NOT YET ENABLED (HADS-CODE-OB-001). Required implementation before final V&V:

// In setup():

WDT->WDT_MR = WDT_MR_WDRSTEN | WDT_MR_WDV(256 * 10); // ~10ms timeout // First line of loop():

WDT->WDT_CR = WDT_CR_KEY(0xA5) | WDT_CR_WDRSTT;      // feed watchdog

## 2oo2 Redundant Decision Architecture

Both ECUs independently read identical sensor inputs, execute identical algorithms, compute independent deployment votes, and assert D8/D9 only when their local vote is TRUE. The DM74LS08N AND gate forms the hardware physical enforcement layer — LED fires only when BOTH boards simultaneously drive their respective input HIGH.

This architecture prevents: single-point hardware failures, single-channel software errors, single-sensor noise events, EMC spikes on one channel, and communication corruption.

## Fail-Safe Direction Analysis

| Failure Mode | Software Response | Hardware Response | Safe? |
| --- | --- | --- | --- |
| Peer CAN timeout (>2000ms) | faultCOMM=TRUE → systemFault → voteDeploy=FALSE → D8/D9 LOW | AND gate output LOW | Yes — inhibit on comm loss |
| POST failure (CPU/RAM/ADC) | faultXXX=TRUE → systemFault → deployment inhibited | D8/D9 never HIGH | Yes — inhibit on startup fault |
| Single sensor above threshold | localCrash=TRUE on this ECU only | AND gate requires BOTH D8 HIGH — peer must agree | Yes — no single-channel deploy |
| Single ECU software crash | Loop stops, D8/D9 go LOW (output default) | AND gate output LOW | Yes — fail LOW |
| Baby seat line open circuit | Pull-up holds D11 HIGH → babySeat=FALSE | D9 follows voteDeploy — not suppressed (GAP-02) | PARTIAL — GAP-02 open |
| voteDeploy=TRUE | Event-triggered CAN TX fires immediately | CAN frame reaches OMS PC within ~1ms | Improved FSR-16 coverage |

## Permitted and Forbidden Software Operations

Per ASIL D software development guidelines (ISO 26262-6 Annex A):

Permitted:

- Reading ADC pins A0, A1, A2 via analogRead()

- Reading digital pin D11 via digitalRead()

- Writing D8, D9 (deployment votes) and D10 (DWI) via digitalWrite()

- Transmitting CAN frames via Can0.sendFrame()

- Reading received CAN frames via Can0.read()

- Using millis() for timing comparisons

- Printing to Serial for diagnostics

Forbidden:

- Dynamic memory allocation (malloc, new) — non-deterministic timing prohibited at ASIL D

- Recursive functions — stack depth unpredictable

- Unbounded loops (while(true) without exit) — threatens deterministic cycle time

- Direct register access bypassing safety flags

- Clearing faultCPU, faultRAM, faultADC once set — these are sticky faults

- Modifying ECU_ID, DATA_ID, REMOTE_ID at runtime — compile-time constants only

# HW/SW Integration Procedure

This section describes the procedure for integrating the firmware with the hardware assembly. It covers flash programming, pin verification, CAN bus bring-up, and the acceptance criteria that must be met before the integrated system is released to functional testing. Execution results and sign-off evidence are recorded in TR-AIRBAG-ECU-001.

## Integration Steps

| Step | Action | Acceptance Criterion | Owner |
| --- | --- | --- | --- |
| 1 | Assemble hardware per HADS-AIRBAG-ECU-001 Rev 2.0 — wiring, POTIs, baby seat switch (pull-up to 3.3V), CAN transceivers, AND gate, LEDs | All wiring verified against HADS Tables. No shorts. All signal lines continuous. | HW Engineer |
| 2 | Verify power rails: 5V to AND gate VCC (pin14), 3.3V to Waveshare VCC, 3.3V to baby seat pull-up, GND common | 5V ±0.25V at AND gate. 3.3V ±0.15V at Waveshare. GND continuity. | HW Engineer |
| 3 | Flash ECU 1: ECU_ID=1, DATA_ID=0x100, REMOTE_ID=0x200 | Upload complete. Serial: ECU 1 AIRBAG ECU Rev 3.2 STARTED. CPU/RAM/ADC: PASS. | SW Engineer |
| 4 | Flash ECU 2: ECU_ID=2, DATA_ID=0x200, REMOTE_ID=0x100 | Upload complete. Serial: ECU 2 AIRBAG ECU Rev 3.2 STARTED. CPU/RAM/ADC: PASS. | SW Engineer |
| 5 | Verify CAN bus — connect OMS PC via USB-CAN-A, open USBCANV2 at 500 kbit/s | Frames ID 0x100 and 0x200 visible at ~500ms intervals. DLC=8. Byte 7 = 0x01 and 0x02. | HW/SW Engineer |
| 6 | Verify CommFault=0 on both ECUs via Serial Monitor | Serial: CommFault=0 on both. SystemFault=0. VoteDeploy=0. | SW Engineer |
| 7 | Verify rest state — all sensors below threshold | D8=LOW, D9=LOW, D10=LOW, D13 blinking. All LEDs OFF. | HW Engineer |
| 8 | Verify 2oo2 hardware AND gate — raise POTI-1 on ECU1 only above threshold | Airbag1 LED stays OFF. AND gate pin 3 LOW. ECU1: localCrash=1, voteDeploy=0. | HW Engineer |
| 9 | Verify full crash — raise POTI-1 on BOTH ECUs above threshold simultaneously | Airbag1 LED fires. AND gate pin 3 HIGH. Both ECUs: voteDeploy=1. | HW/SW Engineer |
| 10 | Verify baby seat suppression — press switch during step 9 crash | Airbag1 LED ON, Airbag2 LED OFF. D9 LOW on both ECUs. | HW/SW Engineer |
| 11 | Verify DWI — disconnect CAN between ECUs, wait >2100ms | D10 HIGH (DWI ON) on both ECUs. CommFault=1. VoteDeploy=0. | HW/SW Engineer |
| 12 | Sign off — all steps passed, record in TR-AIRBAG-ECU001 | Integration complete. Release to functional testing. | Project Lead |

# Compliance Gaps and Required Improvements

Status updated to reflect Rev 3.2 firmware baseline.

| Gap ID | FSR | Description | Risk |  | Required Action |
| --- | --- | --- | --- | --- | --- |
| GAP-01 | FSR-11 | Baby seat debounce | Medium | CLOSED Rev 3.2 | updateBabySeat() 10ms debounce implemented and tested |
| GAP-02 | FSR-12 | Baby seat line continuity monitoring — open-circuit stuck-HIGH not detected | High | OPEN | Add periodic D11 continuity test. Hardware limitation for lab scope. |
| GAP-03 | FSR-13 | Software CRC not in CAN frame | Medium | CLOSED — HW CRC | SAM3X8E hardware CAN controller provides ISO 11898 15-bit CRC automatically. No software CRC needed. |
| GAP-04 | FSR-14 | Parallel CAN bitstream readback not implemented | Low | OPEN — HW limit | Hardware limitation of Arduino Due classical CAN. Accepted for lab demo. |
| GAP-05 | FSR-17 | Can0.begin() success not checked in POST | Medium | OPEN | Check return value; set faultCOMM on failure. |
| GAP-06 | FSR-22 | Hardware WDT not enabled | High | CLOSED – SRS Rev 3.1 | Enable WDT in setup(). Feed in loop(). |
| GAP-07 | FSR-21 | Fault logging to EEPROM not implemented | Low | OPEN — lab scope | Serial output only. Accepted for lab demo scope. |
| GAP-08 | FSR-15 | CAN timeout 2000ms vs FSR15 requirement 50ms | Medium | OPEN — NCR required | 50ms causes false faults with 500ms TX interval. NCR raised. 2000ms accepted with rationale. |

# Verification Methods Overview

This section defines the software verification approach required by ISO 26262-6:2018 at ASIL D. Detailed test cases, test execution records, pass/fail results, and coverage evidence are documented in ITP-AIRBAG-ECU001 (test plan) and TR-AIRBAG-ECU-001 (test report).

## Applicable Verification Methods

| Method | Description | ISO 26262-6 Reference | Applicability |
| --- | --- | --- | --- |
| Code Review | Manual two-person review against SADS specs. Forbidden constructs, fault flags, threshold values, output logic, CAN encoding. | Clause 9.4.2 | All modules — mandatory ASIL D |
| Static Analysis | Unreachable code, unused variables, ADC integer overflow, uninitialized variables, forbidden operations. | Clause 9.4.2 | MOD-02, 03, 04 |
| Unit Testing | Black-box and white-box per VVP-AIRBAG-ECU-001_Rev1.2. | Clause 9.4.3 | MOD-01 through MOD-06 |
| Integration Testing | HW/SW integration per Section 9 above and VVP-AIRBAG-ECU-001_Rev1.2 | Clause 9.4.3 | Full system |
| Timing Analysis | Cycle time via micros(). CAN heartbeat timing via USBCANV2. Worst-case loop ≤1ms. | Clause 9.4.4 | Main loop, CAN TX |
| Requirements Traceability | Each FSR traced to module and test case. RTM in VVP-AIRBAG-ECU-001_Rev1.2 | Clause 9.4.1 | All FSR requirements |

## Structural Coverage Target

| Coverage Metric | ASIL D Target | Applicable Modules |
| --- | --- | --- |
| Statement Coverage | 100% | All safety-relevant modules MOD-01 to MOD-06 |
| Branch Coverage | 100% | Crash threshold, baby seat suppression, safe-state transitions, POST pass/fail |
| MC/DC Coverage | 100% | voteDeploy 4-condition AND, babySeat gate, crash OR fusion |
| Path Coverage | Best effort | Complex conditional paths in 2oo2 cross-comparison |

# Requirement Traceability Matrix

Forward traceability from every software requirement in SRS-AIRBAG-SW-001 Rev 3.1 to the software design element defined in this document:

| SWR | Description | FSR | ASIL | Software Design Element |
| --- | --- | --- | --- | --- |
| SWR- 01 | POST at boot | FSR- 17 | C | MOD-01 — cpuTest(), ramTest(), adcTest() |
| SWR- 02 | No output before POST | FSR- 17 | D | setup() — all outputs LOW before POST |
| SWR- 03 | ADC 12-bit | FSR- 01 | D | setup() — analogReadResolution(12) |
| SWR- 04 | Sensor read every loop | FSR- 01,02 | D | MOD-02 — analogRead A0/A1/A2 |
| SWR- 05 | Physical conversion | FSR- 01 | D | MOD-03 — toPhys() |
| SWR- 06 | Threshold comparison | FSR- 03,04 | D | MOD-03 — accelEx/gyroEx/pressEx |
| SWR- 07 | Baby seat INPUT | FSR- 09 | D | setup() — pinMode(D11, INPUT) + 3.3V pull-up |
| SWR- 08 | 10ms debounce | FSR- 11 | D | MOD-02 — updateBabySeat() (GAP-01 CLOSED) |
| SWR- 09 | Suppress-on-fault | FSR- 10 | D | MOD-02 — stableBaby retains last safe value |
| SWR- 10 | Sensor OR fusion | FSR- 03 | D | MOD-03 — localCrash = accelEx¦¦gyroEx¦¦pressEx |
| SWR- 11 | No blocking delay | FSR- 02 | D | Architecture — millis() timers throughout |
| SWR- 12 | D8 AND gate drive | FSR- 03,08 | D | MOD-04 — voteDeploy → D8 |
| SWR- 13 | D9 pax suppression | FSR- 09,10 | D | MOD-04 — voteDeploy && !babySeat → D9 |
| SWR- 14 | Fault inhibit | FSR- 20 | D | MOD-04/06 — systemFault → D8/D9 LOW |
| SWR- 15 | WDT config (pending) | FSR- 22 | D | MOD-01 — watchdog_setup() PENDING GAP-06 |
| SWR- 16 | WDT feed (pending) | FSR- 22 | D | loop() first statement PENDING GAP-06 |
| SWR- 17 | CAN TX 500ms + event | FSR- 07,16 | D | MOD-05 — canSend() heartbeat + rising edge |
| SWR- 18 | CAN timeout 2000ms | FSR- 07,15 | D | MOD-05/06 — COMM_TIMEOUT=2000ms (deviation) |
| SWR- 19 | CAN frame parsing | FSR- 07 | D | MOD-05 — receiveCAN() byte 6 extraction |
| SWR- 20 | CAN0 500kbps | FSR- 13 | B | MOD-05 — Can0.begin(CAN_BPS_500K) |
| SWR- 21 | CAN DLC=8 | FSR- 13,16 | B | MOD-05 — canSend() frame layout |
| SWR- 22 | HW CRC ISO 11898 | FSR- 13 | B | SAM3X8E CAN controller — automatic (GAP-03 CLOSED) |
| SWR- 23 | CAN 500ms + event | FSR- 16 | B | MOD-05 — HEARTBEAT_MS + deployRisingEdge |
| SWR- 25 | DWI remote fault | FSR- 19 | A | MOD-04 — remoteFault → D10 HIGH |

# Approval and Sign-Off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Software Safety Engineer | Mariana Chavez Avila |  |  |
| Hardware Safety Engineer | Om Naik |  |  |
| Functional Safety Engineer | Kruthika S. Gowda |  |  |
| V&V Lead | Shivbaraje Bhosale |  |  |
| Project Supervisor |  |  |  |

# Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-05-28 | Mariana Chavez Avila | Initial release (UART architecture). Full SADS covering architecture, module decomposition, state machine, safety mechanisms, and SWR traceability. |
| 2.0 | 2026-06-12 | Mariana Chavez Avila | Major revision – added sections, CAN architecture migration. UART inter-ECU link replaced by CAN bus (500 kbit/s). Master/Slave removed; peer-equal architecture. CAN frame byte layout documented. Compliance gap analysis (GAP-01–GAP07). SRS traceability matrix. |
| 3.0 | 2026-06-13 | Mariana Chavez Avila | Firmware baseline updated to Rev 3.2. GAP-01 CLOSED (debounce implemented). GAP-03 CLOSED (hardware CRC confirmed). Event-triggered CAN TX added in MOD-04/05. canSend() replaces sendData(). DLC=8 confirmed. HADS-CR001 resolved (pull-up 3.3V). All dependent document references updated. Execution flow updated with event TX step. Compliance gap table updated. |